PROFESSIONAL LED EFFECT FIXTURE

AL0580T

5 x 80 W LED TORNADO

USER MANUAL

English Edition  |  Professionally edited from the supplied manual

5 independent RGBW heads  |  5 RGB aura rings  |  Motorized focus

| Main LEDs | Input | Control | DMX Modes |
| --- | --- | --- | --- |
| 5 x 80 W RGBW | AC 100-240 V | DMX / Auto / Sound | 28 / 32 / 85 / 88 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Retain it for future reference.

Contents

1  Product Information & Safety

2  Installation & Data Connections

3  Display Menu & Service Functions

4  DMX512 Control & Channel Charts

5  Troubleshooting, Inspection & Maintenance

EDITORIAL NOTE  The model and 5 x 80 W designation were supplied by the user. Terminology, grammar, menu labels, and DMX descriptions have been standardized for professional lighting use; unsupported legacy lamp content has been removed.

1  Product Information & Safety

1.1 Intended Use

The AL0580T is an indoor professional effect fixture with five independently moving RGBW heads, five RGB aura rings, motorized focus, and continuous Pan/Tilt rotation effects. It supports DMX512, two automatic programs, sound-active operation, and manual control from the display.

Inspect the fixture and accessories immediately after unpacking. Do not operate a product that appears damaged in transit.

1.2 Package Contents

| Item | Quantity |
| --- | --- |
| AL0580T fixture | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |
| User manual | 1 |

1.3 Safety Instructions

This fixture is intended for professional indoor use and must be installed and serviced by qualified personnel.

Connect only to a properly grounded Protection Class I circuit within the rated AC input range shown on the fixture label.

Disconnect mains power before cleaning, servicing, connecting data cables, or when the fixture is not in use. Pull the plug, never the cable.

Keep the fixture dry. Do not expose it to rain, water, condensation, or high humidity.

After a severe temperature change, allow the fixture to reach room temperature and let condensation evaporate before applying power.

Do not operate the fixture if its power cable, housing, lens, mounting hardware, or safety attachment is damaged.

Keep ventilation openings unobstructed and maintain the clearances shown on the product label or required by local regulations.

Do not look directly into the light source. Strobe lighting may trigger photosensitive seizures; follow venue and local warning requirements.

Stop operation immediately if the fixture overheats, emits abnormal noise, reports repeated sensor errors, or behaves unexpectedly.

WARNING  Never stand directly below the fixture during installation, removal, or service. Always use a correctly rated independent secondary safety attachment.

1.4 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V; frequency not stated in the supplied source |
| Main light sources | 5 x 80 W RGBW LEDs (red, green, blue, white) |
| Aura light sources | 5 x RGB LED aura rings |
| Optical control | Independent motorized Focus channels for heads 1-5 |
| Pan / Tilt | 0-540 degrees plus continuous rotation; 16-bit fine adjustment |
| Dimming | 0-100% electronic dimming |
| Color functions | RGBW mixing, cool-to-warm color-temperature control, color macro |
| Effect functions | Main LED macros, aura macros, synchronized and random strobe |
| Control modes | DMX512, Auto 1, Auto 2, Sound-active, manual control |
| DMX personalities | 28 CH / 32 CH / 85 CH / 88 CH |
| Operating environment | Indoor, dry locations only; formal IP rating not stated |

RATING-LABEL CHECK  The supplied source does not state total fixture power, mains frequency, IP rating, beam angle, dimensions, or weight. Verify these items on the fixture rating label or current manufacturer data before installation.

2  Installation & Data Connections

2.1 Rigging

The fixture may be installed horizontally, at an angle, or inverted. Before mounting, verify that the supporting structure and all hardware can support at least ten times the fixture weight, or the higher load required by local regulations. Complete all rigging before connecting mains power.

Use correctly rated clamps and M10 mounting hardware where specified by the fixture mounting points.

Attach a correctly rated safety cable to a designated safety attachment point. Do not use a carrying handle as the safety anchor.

Keep the area below the fixture clear while it is being installed, removed, or serviced.

Inspect clamps, fasteners, mounting points, safety cables, and the supporting structure before every use.

2.2 DMX512 Data Connection

Use shielded twisted-pair DMX cable with a nominal 120 ohm characteristic impedance, low capacitance, and 22-24 AWG conductors. Connect the controller output to the first fixture input, then connect each fixture output to the next fixture input.

| XLR Contact | Signal |
| --- | --- |
| Pin 1 | Ground / shield |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  For long cable runs or electrically noisy environments, connect a 120 ohm, 1/4 W resistor between pins 2 and 3 in a male XLR plug and install it at the DMX output of the final fixture.

2.3 RDM Connection Notes

The supplied source includes RDM operating guidance. Confirm that RDM is supported by the installed fixture firmware before relying on remote discovery or configuration.

Use an RDM-compatible controller and bidirectional RDM splitters. A conventional one-way DMX splitter blocks return data.

Set all fixtures on the data line to DMX mode and allow only one controller to act as the bus master.

Terminate the final fixture with 120 ohms between Data - and Data +.

If DMX control works but RDM discovery fails, check the splitter first, then inspect pins 2 and 3 throughout the data line.

3  Display Menu & Service Functions

3.1 Panel Controls & Indicators

| Control / Indicator | Function |
| --- | --- |
| MENU / Back | Open the main menu or return to the previous menu. |
| UP | Move upward or increase the selected value. |
| DOWN | Move downward or decrease the selected value. |
| ENTER | Open the selected item, confirm a value, or save a setting. |
| DMX indicator (blue) | Shows DMX data reception. |
| Error indicator (red) | Shows a detected fixture fault. Open Information for details. |

DISPLAY CARE  Operate the panel only with a fingertip. Do not press the display with sharp or hard objects.

The temperature field normally shows a numeric value. A display such as --- degrees C may indicate an open sensor circuit; *** degrees C or an implausible value may indicate a short circuit. Stop operation and have the fixture inspected by qualified service personnel.

3.2 Menu Map

| Menu | Option | Setting / Function |
| --- | --- | --- |
| Address | DMX Address | 001-512 |
| Settings | Run Mode | DMX / Auto 1 / Auto 2 / Sound |
| Settings | Channel Mode | 28 CH / 32 CH / 85 CH / 88 CH |
| Settings | Pan Reverse | Off / On |
| Settings | Tilt Reverse | Off / On |
| Settings | Hall Sensor Correction | Off / On |
| Settings | Optical Sensor Correction | Off / On; Pan/Tilt position correction |
| Settings | DMX Signal Hold | Off: return to post-reset state / On: hold last DMX frame |
| Settings | Screen Saver | Off / On |
| Settings | Display Invert | Off / On / Auto orientation |
| Settings | Synchronized Update | Off / On; transfer settings through the DMX line |
| Settings | Language | Chinese / English |
| Settings | MCU ID | Off / X1-X5 / Y1-Y5 / Z1-Z5; service use |
| Settings | Factory Reset | Cancel / Confirm |
| Manual | Channel Values | Current personality channels; 000-255 |
| Calibration | Service Calibration | Authorized technicians only |
| Reset | Effect Motor Reset | Run reset |
| Reset | Pan/Tilt Reset | Run reset |
| Reset | Full Fixture Reset | Run reset |
| Information | Error / Reset Information | Communication and position-sensor errors |
| Information | DMX Data Monitor | View live channel data |
| Information | Sensor Information | View sensor status |
| Information | Hardware / Software Version | Read-only version information |

FIRMWARE NOTE  A shortened source menu lists only 28 CH and 85 CH, while the detailed settings text and channel charts also define 32 CH and 88 CH. Confirm the available personalities on the installed firmware.

3.3 Service Calibration

SERVICE ONLY  Calibration changes can alter movement limits, LED balance, and maximum output. This menu is intended only for authorized technicians.

| Calibration Group | Range / Function |
| --- | --- |
| Home Position | Pan, Tilt, and Focus: 000-255; 127 = neutral compensation |
| Travel / Output | Pan, Tilt, Focus, and RGBW LED 1-5 calibration: 000-255; 127 = neutral |
| Maximum Output | 000-255; 255 = maximum configured output |
| Password | Service password management |

3.4 Error Information

The Information menu can report X1-X5 and Y1-Y5 motion-board communication errors, Z1-Z5 LED-driver communication errors, and optical or Hall sensor errors for the corresponding head. Record the exact error, power the fixture down safely, and arrange qualified service if the error returns after reset.

4  DMX512 Control

Set the fixture DMX start address and select the required personality. Channel values are shown as decimal DMX values from 000 to 255. Function availability can vary with firmware revision.

OUTPUT NOTE  To produce light output, set the relevant dimmer above zero and place the applicable strobe channel in an Open range.

4.1  28 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine, 16-bit adjustment |
| 3 | 000-255 | Pan Speed, fast to slow |
| 4 | 000-255 | Pan Macro; mapping not specified in source |
| 5 | 000-255 | Tilt, 0-540 degrees |
| 6 | 000-255 | Tilt Fine, 16-bit adjustment |
| 7 | 000-255 | Tilt Speed, fast to slow |
| 8 | 000-255 | Tilt Macro; mapping not specified in source |
| 9 | 000-255 | Pan Continuous Rotation; mapping not specified |
| 10 | 000-255 | Tilt Continuous Rotation; mapping not specified |
| 11 | 000-255 | Focus, far to near |
| 12 | 000-255 | Master Dimmer, 0-100% |
| 13 | 000-255 | Main LED Strobe; see 4.5 |
| 14 | 000-255 | Aura Ring Strobe; see 4.5 |
| 15 | 000-255 | Main LED Red, 0-100% |
| 16 | 000-255 | Main LED Green, 0-100% |
| 17 | 000-255 | Main LED Blue, 0-100% |
| 18 | 000-255 | Main LED White, 0-100% |
| 19 | 000-255 | Color Temperature, cool to warm |
| 20 | 000-255 | Color Macro; mapping not specified in source |
| 21 | 000-255 | Aura Ring Red, 0-100% |
| 22 | 000-255 | Aura Ring Green, 0-100% |
| 23 | 000-255 | Aura Ring Blue, 0-100% |
| 24 | 000-255 | Main LED Macro; see 4.6 |
| 25 | 000-255 | Main Macro Speed / Fade; see 4.6 |
| 26 | 000-255 | Aura Ring Macro; see 4.7 |
| 27 | 000-255 | Aura Macro Speed / Direction; see 4.7 |
| 28 | 000-255 | Reset; see 4.5 |

4.2  32 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine, 16-bit adjustment |
| 3 | 000-255 | Pan Speed, fast to slow |
| 4 | 000-255 | Pan Macro; mapping not specified in source |
| 5 | 000-255 | Tilt, 0-540 degrees |
| 6 | 000-255 | Tilt Fine, 16-bit adjustment |
| 7 | 000-255 | Tilt Speed, fast to slow |
| 8 | 000-255 | Tilt Macro; mapping not specified in source |
| 9 | 000-255 | Pan Continuous Rotation; mapping not specified |
| 10 | 000-255 | Tilt Continuous Rotation; mapping not specified |
| 11 | 000-255 | Head 1 Focus, far to near |
| 12 | 000-255 | Head 2 Focus, far to near |
| 13 | 000-255 | Head 3 Focus, far to near |
| 14 | 000-255 | Head 4 Focus, far to near |
| 15 | 000-255 | Head 5 Focus, far to near |
| 16 | 000-255 | Master Dimmer, 0-100% |
| 17 | 000-255 | Main LED Strobe; see 4.5 |
| 18 | 000-255 | Aura Ring Strobe; see 4.5 |
| 19 | 000-255 | Main LED Red, 0-100% |
| 20 | 000-255 | Main LED Green, 0-100% |
| 21 | 000-255 | Main LED Blue, 0-100% |
| 22 | 000-255 | Main LED White, 0-100% |
| 23 | 000-255 | Color Temperature, cool to warm |
| 24 | 000-255 | Color Macro; mapping not specified in source |
| 25 | 000-255 | Aura Ring Red, 0-100% |
| 26 | 000-255 | Aura Ring Green, 0-100% |
| 27 | 000-255 | Aura Ring Blue, 0-100% |
| 28 | 000-255 | Main LED Macro; see 4.6 |
| 29 | 000-255 | Main Macro Speed / Fade; see 4.6 |
| 30 | 000-255 | Aura Ring Macro; see 4.7 |
| 31 | 000-255 | Aura Macro Speed / Direction; see 4.7 |
| 32 | 000-255 | Reset; see 4.5 |

4.3  85 CH Mode

Channels 4-83 contain five identical 16-channel head blocks. The table below provides a deterministic mapping for every channel.

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Master Dimmer, 0-100% |
| 2 | 000-255 | Main LED Strobe; see 4.5 |
| 3 | 000-255 | Aura Ring Strobe; see 4.5 |
| 84 | 000-255 | Pan/Tilt Speed, fast to slow |
| 85 | 000-255 | Reset; see 4.5 |

| Head | Channel Range | Start Channel |
| --- | --- | --- |
| Head 1 | 4-19 | 4 |
| Head 2 | 20-35 | 20 |
| Head 3 | 36-51 | 36 |
| Head 4 | 52-67 | 52 |
| Head 5 | 68-83 | 68 |

| Relative Position | Function |
| --- | --- |
| 1 | Pan, 0-540 degrees |
| 2 | Pan Fine |
| 3 | Pan Continuous Rotation |
| 4 | Tilt, 0-540 degrees |
| 5 | Tilt Fine |
| 6 | Tilt Continuous Rotation |
| 7 | Focus, far to near |
| 8 | Main LED Red |
| 9 | Main LED Green |
| 10 | Main LED Blue |
| 11 | Main LED White |
| 12 | Aura Ring Red |
| 13 | Aura Ring Green |
| 14 | Aura Ring Blue |
| 15 | Aura Ring Macro; see 4.7 |
| 16 | Aura Macro Speed / Direction; see 4.7 |

MAPPING RULE  Add the relative position minus 1 to the head start channel. Unless a detailed value table is referenced, the parameter uses 000-255.

4.4  88 CH Mode

Channels 4-88 contain five identical 17-channel head blocks. Unlike 85 CH mode, 88 CH mode adds an individual dimmer to every head and places Reset at channel 3.

| Head | Channel Range | Start Channel |
| --- | --- | --- |
| Head 1 | 4-20 | 4 |
| Head 2 | 21-37 | 21 |
| Head 3 | 38-54 | 38 |
| Head 4 | 55-71 | 55 |
| Head 5 | 72-88 | 72 |

| Relative Position | Function |
| --- | --- |
| 1 | Pan, 0-540 degrees |
| 2 | Pan Fine |
| 3 | Tilt, 0-540 degrees |
| 4 | Tilt Fine |
| 5 | Pan Continuous Rotation |
| 6 | Tilt Continuous Rotation |
| 7 | Focus, far to near |
| 8 | Head Dimmer, 0-100% |
| 9 | Main LED Red |
| 10 | Main LED Green |
| 11 | Main LED Blue |
| 12 | Main LED White |
| 13 | Aura Ring Red |
| 14 | Aura Ring Green |
| 15 | Aura Ring Blue |
| 16 | Aura Ring Macro; see 4.7 |
| 17 | Aura Macro Speed / Direction; see 4.7 |

MAPPING RULE  Add the relative position minus 1 to the head start channel. Unless a detailed value table is referenced, the parameter uses 000-255.

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Master Dimmer, 0-100% |
| 2 | 000-255 | Main LED Strobe; see 4.5 |
| 3 | 000-255 | Reset; see 4.5 |

4.5  Shared Strobe & Reset Tables

Main LED Strobe

| DMX Value | Function |
| --- | --- |
| 000-007 | Blackout |
| 008-015 | Open |
| 016-131 | Synchronous strobe |
| 132-139 | Open |
| 140-181 | Pulse strobe: slow open / fast close, slow to fast |
| 182-189 | Open |
| 190-231 | Pulse strobe: fast open / slow close, slow to fast |
| 232-239 | Open |
| 240-247 | Random strobe |
| 248-255 | Open |

Aura Ring Strobe

| DMX Value | Function |
| --- | --- |
| 000-004 | Open |
| 005-250 | Synchronous strobe |
| 251-255 | Open |

Reset

| DMX Value | Function |
| --- | --- |
| 000-214 | No function / not defined in the supplied source |
| 215-225 | Head / effect motor reset |
| 226-229 | No function / not defined in the supplied source |
| 230-240 | Pan/Tilt motor reset |
| 241-244 | No function / not defined in the supplied source |
| 245-255 | Full fixture reset |

RESET CAUTION  Reset commands interrupt movement and may temporarily interrupt output. Use only when the fixture can move safely.

4.6  Main LED Macro

| DMX Value | Main LED Macro |
| --- | --- |
| 000-007 | No function |
| 008-015 | Macro 1 |
| 016-023 | Macro 2 |
| 024-031 | Macro 3 |
| 032-039 | Macro 4 |
| 040-047 | Macro 5 |
| 048-055 | Macro 6 |
| 056-063 | Macro 7 |
| 064-071 | Macro 8 |
| 072-079 | Macro 9 |
| 080-087 | Macro 10 |
| 088-095 | Macro 11 |
| 096-103 | Macro 12 |
| 104-111 | Macro 13 |
| 112-119 | Macro 14 |
| 120-127 | Macro 15 |
| 128-135 | Macro 16 |
| 136-143 | Macro 17 |
| 144-151 | Macro 18 |
| 152-159 | Macro 19 |
| 160-167 | Macro 20 |
| 168-175 | Macro 21 |
| 176-183 | Macro 22 |
| 184-191 | Macro 23 |
| 192-199 | Macro 24 |
| 200-207 | Macro 25 |
| 208-215 | Macro 26 |
| 216-223 | Macro 27 |
| 224-231 | Macro 28 |
| 232-239 | Macro 29 |
| 240-255 | No macro / main LEDs off (verify with installed firmware) |

| DMX Value | Main Macro Speed / Fade |
| --- | --- |
| 000-127 | Macro speed, slow to fast |
| 128-255 | Macro fade, slow to fast |

4.7  Aura Ring Macro

| DMX Value | Aura Ring Macro |
| --- | --- |
| 000-015 | No function |
| 016-031 | Macro 1 |
| 032-047 | Macro 2 |
| 048-063 | Macro 3 |
| 064-079 | Macro 4 |
| 080-095 | Macro 5 |
| 096-111 | Macro 6 |
| 112-127 | Macro 7 |
| 128-143 | Macro 8 |
| 144-159 | Macro 9 |
| 160-175 | Macro 10 |
| 176-191 | Macro 11 |
| 192-207 | Macro 12 |
| 208-223 | Macro 13 |
| 224-239 | Macro 14 |
| 240-255 | Macro 15 |

| DMX Value | Aura Macro Speed / Direction |
| --- | --- |
| 000-127 | Forward, slow to fast |
| 128-255 | Reverse, slow to fast |

4.8  Personality Summary

| Mode | Control Resolution |
| --- | --- |
| 28 CH | Shared Pan/Tilt movement and focus; shared RGBW, color, aura, strobe, macro, and dimmer control. |
| 32 CH | As 28 CH, with independent Focus control for heads 1-5. |
| 85 CH | Independent Pan/Tilt, continuous rotation, Focus, RGBW, and aura control for every head; global dimmer and strobe. |
| 88 CH | Independent Pan/Tilt, continuous rotation, Focus, dimmer, RGBW, and aura control for every head. |

5  Troubleshooting, Inspection & Maintenance

5.1 Troubleshooting

| Symptom | Checks / Corrective Action |
| --- | --- |
| Fixture does not power on | Verify the AC source, plug, cable, protective device, and all accessible connectors. Disconnect power before inspection. |
| Fixture resets but does not respond to DMX | Confirm the start address and personality; check DMX polarity and cable continuity; add a 120 ohm terminator at the final fixture. |
| Output is dim or uneven | Check dimmer, strobe, color, and macro channels; clean the lenses and ventilation system; review reported temperature or driver errors. |
| Intermittent operation or overheating | Stop operation. Check fans, ventilation openings, ambient conditions, and accumulated dust. Arrange qualified service if the problem returns. |
| Movement or Focus error | Remove obstructions, run the appropriate reset, and review sensor/error information. Do not force the mechanism by hand. |
| DMX works but RDM discovery fails | Use an RDM-compatible controller and bidirectional splitter; inspect pins 2 and 3 and verify one bus master. |

5.2 Inspection

Confirm that all installation screws, clamps, and fasteners are tight and free from corrosion.

Check the housing, lenses, mounting points, brackets, and secondary safety attachment for deformation, cracks, or other damage.

Check moving parts for excessive wear, imbalance, abnormal noise, or restricted travel.

Inspect power and data cables for cuts, crushing, heat damage, material fatigue, or contamination.

Correct every safety issue before returning the fixture to service.

5.3 Cleaning

Disconnect mains power and allow the fixture to cool before cleaning.

Clean lens surfaces regularly with a soft, lint-free cloth to prevent output loss.

Remove dust from fans and ventilation openings regularly. Cleaning frequency depends on the operating environment.

Use a cloth lightly moistened with water. Do not use alcohol, solvents, abrasives, or pressurized liquid.

There are no user-serviceable parts inside. Internal service must be performed by qualified personnel.

END OF MANUAL  Retain this manual with the fixture. Before operation, verify the product label, local electrical code, rigging requirements, and venue safety procedures.