AL0660WV

6 x 60 W RGBW Zoom
Moving Head Bar

INDEPENDENT TILT + ZOOM - CONTINUOUS PAN - USER MANUAL

Professional beam, wash and synchronized multi-head effect fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Function-Channel Reference |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition uses model AL0660WV following the product owner's confirmed model correction. Confirm the model, voltage rating and firmware shown on the actual fixture before installation or programming.

SOURCE CORRECTION  The overview lists 28CH, while the menu and complete channel table identify 38CH. This edition uses 38CH consistently.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool before opening, cleaning, inspection, fuse replacement or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Use only AC 100-240 V, 50/60 Hz as specified by the source and product label. Connect to a protected, properly grounded Class I supply.

If an external flexible cable is damaged, have it replaced by the manufacturer, its service agent or similarly qualified personnel.

Do not operate the fixture with a damaged housing, lens, connector, mounting point or cable.

After a large temperature change, leave the fixture disconnected until it reaches ambient temperature and all condensation has evaporated.

Keep flammable materials at least 0.2 m away and maintain clear ventilation around the fixture.

Do not look directly into the LEDs at close range. Flashing light may trigger seizures in susceptible people; follow local restrictions.

For overhead installation, use rated clamps and an independently rated safety cable attached to the designated point.

Carry the fixture only by its handles or mounting structure. Do not lift it by the moving head bar or individual heads.

INDOOR USE  The source specifies IP20. Protect the fixture from rain, splashing water, condensation and excessive dust. Do not use it outdoors without a correctly rated enclosure.

2  Product Overview

The AL0660WV combines six 60 W RGBW four-in-one LEDs with independent head tilt and motorized zoom on a moving base. Continuous pan rotation, color macros, patterns and synchronized effects support beam, wash and multi-head fan looks for stages, clubs, studios and live events.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Rated power | 360 W (source specification) |
| Light source | 6 x 60 W RGBW 4-in-1 LEDs |
| Control | DMX512 / Auto / Master-Slave / RDM |
| DMX personalities | 23CH / 38CH / 62CH / 92CH / 94CH |
| Dimming | 32-bit, 0-100% linear dimming |
| Movement / optics | Base pan with continuous rotation; six independent tilt and motorized zoom heads |
| Primary effects | Beam, wash, patterns, color macros and synchronized multi-head movement |
| Operating temperature | -30 °C to 50 °C (source specification) |
| Strobe | 1-30 Hz |
| Connections | DMX512 input/output; power input/output |
| Ingress protection | IP20 |
| Construction / finish | Metal, black |
| Dimensions | 544.42 x 225.7 x 350.85 mm |
| Net weight | 15.5 kg |

2.2 Main Functions

Six independently controllable tilt and zoom heads with RGBW color mixing.

Base pan, pan fine and continuous pan rotation.

Built-in tilt macros, color macros, patterns, LED effects and background colors.

Linear master dimming, multiple dimmer curves and detailed strobe modes.

Five controller personalities from compact control to independent head color and movement control.

RDM support for compatible controllers and firmware.

3  Setup and Installation

3.1 AC Power

Before connecting power, verify that the product-label voltage and frequency match the local supply. Complete all mounting and data connections before applying power. Disconnect by holding the plug or connector, not by pulling the cable.

3.2 Rigging

STRUCTURAL RATING  The support structure and every attachment must be rated for the fixture and applicable safety factors. Keep people clear of the area below during installation and service.

Inspect the fixture, clamps, fasteners, mounting points, safety cable and support structure.

Attach correctly rated clamps to the designated mounting points and tighten all fasteners.

Attach an independently rated safety cable to the designated safety point; never attach it to a moving head.

Confirm clearance for full pan, tilt and zoom travel, and maintain at least 0.2 m from combustible materials.

Recheck every attachment before applying power.

3.3 DMX512 / RDM Connection

Use shielded, twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. RDM functions require a compatible controller, correct cabling and supported firmware.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the last fixture in a long or electrically noisy DMX chain, install a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use MENU to enter or return, UP/DOWN to select an item or change a value, and ENTER to open an item or save the edited value. Menu labels may vary slightly with firmware revision.

PROTECTED SETTINGS  Factory home-position and color calibration are service-only. Record every original value before editing; incorrect calibration can damage movement, optics or output.

| Category | Item | Option / Range | Operation |
| --- | --- | --- | --- |
| DMX Features | DMX address | 1-493 | Set the start address (source range 1-493; verify against the selected personality and firmware). |
| DMX Features | Channel | 23CH | Select the 23CH personality. |
| DMX Features | Channel | 38CH | Select the 38CH personality. |
| DMX Features | Channel | 62CH | Select the 62CH personality. |
| DMX Features | Channel | 92CH | Select the 92CH personality. |
| DMX Features | Channel | 94CH | Select the 94CH personality. |
| DMX Features | DMX status | Standby | Select the response after loss of DMX signal. |
| DMX Features | DMX status | Manual | Select the response after loss of DMX signal. |
| DMX Features | DMX status | Hold | Select the response after loss of DMX signal. |
| DMX Features | Observe channel values | Channel1 | Monitor received controller data. |
| DMX Features | Observe channel values | Channel2 | Monitor received controller data. |
| DMX Features | Observe channel values | ... | Monitor received controller data. |
| DMX Features | Observe channel values | ... | Monitor received controller data. |
| Control Settings | Tilt Reverse | No | Reverse or restore the tilt direction. |
| Control Settings | Tilt Reverse | Yes | Reverse or restore the tilt direction. |
| Control Settings | Tilt Range | 200 | Set the available tilt travel. |
| Control Settings | Tilt Range | 90 | Set the available tilt travel. |
| Control Settings | Tilt Range | 150 | Set the available tilt travel. |
| Control Settings | Tilt Self-Test | On | When enabled, tilt resets automatically if the home Hall sensor is not detected. |
| Control Settings | Tilt Self-Test | Off | When enabled, tilt resets automatically if the home Hall sensor is not detected. |
| Control Settings | Color mode | RGB | Red green Blue mode |
| Control Settings | Color mode | CMY | CMY mode |
| Control Settings | Dimming curve | Square | Square |
| Control Settings | Dimming curve | Linear | Linear |
| Control Settings | Dimming curve | S Curve | Sine |
| Control Settings | Dimming curve | Lnv SQ | Inverse square |
| Control Settings | Dimming speed | Smooth | Smooth |
| Control Settings | Dimming speed | Fast | Fast |
| Control Settings | LED frequency | 16KHz |  |
| Control Settings | Pixel Order Reverse | Yes | Reverse or restore the tilt-head and LED sequence. |
| Control Settings | Pixel Order Reverse | No | Reverse or restore the tilt-head and LED sequence. |
| Control Settings | Reset | No Function | Run the selected reset function. |
| Control Settings | Reset | All Motors | Run the selected reset function. |
| Control Settings | Reset | Zoom Motor | Run the selected reset function. |
| Control Settings | Reset | Tilt Motor | Run the selected reset function. |
| Control Settings | White Balance | Red | Service white-balance calibration for the selected color. |
| Control Settings | White Balance | Green | Service white-balance calibration for the selected color. |
| Control Settings | White Balance | Blue | Service white-balance calibration for the selected color. |
| Control Settings | White Balance | White | Service white-balance calibration for the selected color. |
| Control Settings | Restore Factory Settings | factory data reset | Restore factory data. |
| Display Settings | Display reverse | Yes/No |  |
| Display Settings | Backlight brightness | 1-10 | Adjust display brightness from 1 (dark) to 10 (bright). |
| Display Settings | Temperature unit | °C | Degrees Celsius |
| Display Settings | Temperature unit | °F | Degrees Fahrenheit |
| Display Settings | Language | Chinese/English |  |
| Equipment Testing | Automated testing | Off | Enable or disable the automatic fixture test. |
| Equipment Testing | Automated testing | On | Enable or disable the automatic fixture test. |
| Equipment Testing | Manual testing | Tilt | Select and test an individual function. |
| Equipment Testing | Manual testing | Tilt Speed | Select and test an individual function. |
| Equipment Testing | Manual testing | ... | Select and test an individual function. |
| Equipment Testing | Manual testing | ... | Select and test an individual function. |
| System information | Software version | Display the software version number | Display the software version number |
| System information | Temperature | LED1 |  |
| System information | Temperature | ... |  |
| System information | Hall condition | tilt axis Hall | 0 when magnetic is detected, 1 otherwise |
| System information | Hall condition | Zoom Hall | 0 when magnetic is detected, 1 otherwise |
| System information | Hall condition | ... | 0 when magnetic is detected, 1 otherwise |
| System information | Error recording |  | View error log |
| System information | Equipment time |  | Equipment usage time |
| System information | LED time |  | LED usage time |
| System information | Permission duration |  | 9999 unlimited; Other values have time limits |
| Factory Settings | Reset calibration | Head 1 Tilt | Service home-position offset, -128 to +127; 0 = no adjustment. Record the original value before editing. |
| Factory Settings | Reset calibration | Head 1 Zoom | Service home-position offset, -128 to +127; 0 = no adjustment. Record the original value before editing. |
| Factory Settings | Reset calibration | Head 2 Tilt | Service home-position offset, -128 to +127; 0 = no adjustment. Record the original value before editing. |
| Factory Settings | Reset calibration | ... | Service home-position offset, -128 to +127; 0 = no adjustment. Record the original value before editing. |
| Factory Settings | Color Calibration | Head 1 Red | Service-only per-head color calibration. Record the original value before editing. |
| Factory Settings | Color Calibration | Head 2 Red | Service-only per-head color calibration. Record the original value before editing. |
| Factory Settings | Color Calibration | .... | Service-only per-head color calibration. Record the original value before editing. |

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. The source menu lists start addresses 1-493; the usable highest address also depends on the selected personality and controller patch, so verify the installed firmware before programming.

SOURCE NORMALIZATION  The overview typo 28CH is corrected to 38CH. Pattern value 001-002 is corrected to 001-255 where the source description explicitly states Pattern 1-255.

| Personality | Coverage |
| --- | --- |
| 23CH | Compact shared movement, color, effects, dimming, strobe and zoom control |
| 38CH | Independent tilt/fine for six heads plus shared color, effects and zoom |
| 62CH | 38CH-style movement with independent RGBW color for each head |
| 92CH | Expanded independent movement, zoom, color and color-temperature control |
| 94CH | Control channel plus maximum independent movement, zoom and color control |

5.1 23-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-100% |
| 2 | Pan Fine | 000-255 | 0-100% |
| 3 | Continuous Pan Rotation | 001-126 / 127-129 / 130-255 | Forward rotation, fast to slow; Stop; Reverse rotation, slow to fast |
| 4 | Heads 1-6 Tilt | 000-255 | 0-100% |
| 5 | Heads 1-6 Tilt Fine | 000-255 | 0-100% |
| 6 | Tilt Speed | 000-255 | 0-100% |
| 7 | Tilt Macro | 000-004 / 005-255 | No function; See Tilt Macro reference |
| 8 | CTC | 000 / 001-255 | No function; From 19,000 K to 2,700 K |
| 9 | Color Macro | 000 / 001-255 | No function; See Color Macro reference |
| 10 | Pattern | 000 / 001-255 | No function; Pattern 1-255 |
| 11 | LED Effect | 000-015 / 016-255 | No function; See LED Effect reference |
| 12 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow; Stop; Slow to fast |
| 13 | LED Effect Delay | 000-255 | Fast to slow |
| 14 | Background Color | 000 / 001-255 | No function; See Background Color reference |
| 15 | Background color Dimmer | 000-255 | 0-100% |
| 16 | Master Dimmer | 000-255 | 0-100% |
| 17 | Strobe | 000-019 / 020-024 | Blackout; See Strobe reference |
| 18 | Heads 1-6 Zoom | 000-255 | 0-100% |
| 19 | Control | 000-009 / 010-255 | No function; See Control reference |
| 20 | Red / Cyan | 000-255 | 0-100% |
| 21 | Green / Magenta | 000-255 | 0-100% |
| 22 | Blue / Yellow | 000-255 | 0-100% |
| 23 | White | 000-255 | 0-100% |

5.2 38-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-100% |
| 2 | Pan Fine | 000-255 | 0-100% |
| 3 | Continuous Pan Rotation | 001-126 / 127-129 / 130-255 | Forward rotation, fast to slow; Stop; Reverse rotation, slow to fast |
| 4 | Head 1 Tilt | 000-255 | 0-100% |
| 5 | Head 1 Tilt Fine | 000-255 | 0-100% |
| 6 | Head 2 Tilt | 000-255 | 0-100% |
| 7 | Head 2 Tilt Fine | 000-255 | 0-100% |
| 8 | Head 3 Tilt | 000-255 | 0-100% |
| 9 | Head 3 Tilt Fine | 000-255 | 0-100% |
| 10 | Head 4 Tilt | 000-255 | 0-100% |
| 11 | Head 4 Tilt Fine | 000-255 | 0-100% |
| 12 | Head 5 Tilt | 000-255 | 0-100% |
| 13 | Head 5 Tilt Fine | 000-255 | 0-100% |
| 14 | Head 6 Tilt | 000-255 | 0-100% |
| 15 | Head 6 Tilt Fine | 000-255 | 0-100% |
| 16 | Tilt Speed | 000-255 | Fast to slow |
| 17 | Tilt Macro | 000-004 / 005-255 | No function; See Tilt Macro reference |
| 18 | CTC | 000 / 001-255 | No function; From 19,000 K to 2,700 K |
| 19 | Color Macro | 000 / 001-255 | No function; See Color Macro reference |
| 20 | Pattern | 000 / 001-255 | No function; Pattern 1-255 |
| 21 | LED Effect | 000-015 / 016-255 | No function; See LED Effect reference |
| 22 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow; Stop; Slow to fast |
| 23 | LED Effect Delay | 000-255 | Fast to slow |
| 24 | Background Color | 000 / 001-255 | No function; See Background Color reference |
| 25 | Background color Dimmer | 000-255 | 0-100% |
| 26 | Master Dimmer | 000-255 | 0-100% |
| 27 | Strobe | 000-019 / 020-024 | Blackout; See Strobe reference |
| 28 | Head 1 Zoom | 000-255 | 0-100% |
| 29 | Head 2 Zoom | 000-255 | 0-100% |
| 30 | Head 3 Zoom | 000-255 | 0-100% |
| 31 | Head 4 Zoom | 000-255 | 0-100% |
| 32 | Head 5 Zoom | 000-255 | 0-100% |
| 33 | Head 6 Zoom | 000-255 | 0-100% |
| 34 | Control | 000-009 / 010-255 | No function; See Control reference |
| 35 | Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 36 | Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 37 | Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 38 | White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |

5.3 62-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-100% |
| 2 | Pan Fine | 000-255 | 0-100% |
| 3 | Continuous Pan Rotation | 001-126 / 127-129 / 130-255 | Forward rotation, fast to slow; Stop; Reverse rotation, slow to fast |
| 4 | Head 1 Tilt | 000-255 | 0-100% |
| 5 | Head 1 Tilt Fine | 000-255 | 0-100% |
| 6 | Head 2 Tilt | 000-255 | 0-100% |
| 7 | Head 2 Tilt Fine | 000-255 | 0-100% |
| 8 | Head 3 Tilt | 000-255 | 0-100% |
| 9 | Head 3 Tilt Fine | 000-255 | 0-100% |
| 10 | Head 4 Tilt | 000-255 | 0-100% |
| 11 | Head 4 Tilt Fine | 000-255 | 0-100% |
| 12 | Head 5 Tilt | 000-255 | 0-100% |
| 13 | Head 5 Tilt Fine | 000-255 | 0-100% |
| 14 | Head 6 Tilt | 000-255 | 0-100% |
| 15 | Head 6 Tilt Fine | 000-255 | 0-100% |
| 16 | Tilt Speed | 000-255 | Fast to slow |
| 17 | Tilt Macro | 000-004 / 005-255 | No function; See Tilt Macro reference |
| 18 | CTC | 000 / 001-255 | No function; From 19,000 K to 2,700 K |
| 19 | Color Macro | 000 / 001-255 | No function; See Color Macro reference |
| 20 | Pattern | 000 / 001-255 | No function; Pattern 1-255 |
| 21 | LED Effect | 000-015 / 016-255 | No function; See LED Effect reference |
| 22 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow; stop; Slow to fast |
| 23 | LED Effect Delay | 000-255 | Fast to slow |
| 24 | Background Color | 000 / 001-255 | No function; See Background Color reference |
| 25 | Background color Dimmer | 000-255 | 0-100% |
| 26 | Master Dimmer | 000-255 | 0-100% |
| 27 | Strobe | 000-019 / 020-024 | Blackout; See Strobe reference |
| 28 | Head 1 Zoom | 000-255 | 0-100% |
| 29 | Head 2 Zoom | 000-255 | 0-100% |
| 30 | Head 3 Zoom | 000-255 | 0-100% |
| 31 | Head 4 Zoom | 000-255 | 0-100% |
| 32 | Head 5 Zoom | 000-255 | 0-100% |
| 33 | Head 6 Zoom | 000-255 | 0-100% |
| 34 | Control | 000-009 / 010-255 | No function; See Control reference |
| 35 | Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 36 | Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 37 | Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 38 | White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 39 | Head 1 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 40 | Head 1 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 41 | Head 1 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 42 | Head 1 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 43 | Head 2 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 44 | Head 2 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 45 | Head 2 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 46 | Head 2 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 47 | Head 3 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 48 | Head 3 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 49 | Head 3 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 50 | Head 3 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 51 | Head 4 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 52 | Head 4 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 53 | Head 4 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 54 | Head 4 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 55 | Head 5 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 56 | Head 5 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 57 | Head 5 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 58 | Head 5 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 59 | Head 6 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 60 | Head 6 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 61 | Head 6 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 62 | Head 6 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |

5.4 92-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-100% |
| 2 | Pan Fine | 000-255 | 0-100% |
| 3 | Continuous Pan Rotation | 001-126 / 127-129 / 130-255 | Forward rotation, fast to slow; Stop; Reverse rotation, slow to fast |
| 4 | Head 1 Tilt | 000-255 | 0-100% |
| 5 | Head 1 Tilt Fine | 000-255 | 0-100% |
| 6 | Head 2 Tilt | 000-255 | 0-100% |
| 7 | Head 2 Tilt Fine | 000-255 | 0-100% |
| 8 | Head 3 Tilt | 000-255 | 0-100% |
| 9 | Head 3 Tilt Fine | 000-255 | 0-100% |
| 10 | Head 4 Tilt | 000-255 | 0-100% |
| 11 | Head 4 Tilt Fine | 000-255 | 0-100% |
| 12 | Head 5 Tilt | 000-255 | 0-100% |
| 13 | Head 5 Tilt Fine | 000-255 | 0-100% |
| 14 | Head 6 Tilt | 000-255 | 0-100% |
| 15 | Head 6 Tilt Fine | 000-255 | 0-100% |
| 16 | Tilt Speed | 000-255 | Fast to slow |
| 17 | Tilt Macro | 000-004 / 005-255 | No function; See Tilt Macro reference |
| 18 | CTC | 000 / 001-255 | No function; From 19,000 K to 2,700 K |
| 19 | Color Macro | 000 / 001-255 | No function; See Color Macro reference |
| 20 | Pattern | 000 / 001-255 | No function; Pattern 1-255 |
| 21 | LED Effect | 000-015 / 016-255 | No function; See LED Effect reference |
| 22 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow; Stop; Slow to fast |
| 23 | LED Effect Delay | 000-255 | Fast to slow |
| 24 | Background Color | 000 / 001-255 | No function; See Background Color reference |
| 25 | Background color Dimmer | 000-255 | 0-100% |
| 26 | Background color Dimmer Fine | 000-255 | 0-100% |
| 27 | Dimming | 000-255 | 0-100% |
| 28 | Dimming Fine | 000-255 | 0-100% |
| 29 | Shutter | 000-019 / 020-024 | Blackout; See Strobe reference |
| 30 | Head 1 Zoom | 000-255 | 0-100% |
| 31 | Head 2 Zoom | 000-255 | 0-100% |
| 32 | Head 3 Zoom | 000-255 | 0-100% |
| 33 | Head 4 Zoom | 000-255 | 0-100% |
| 34 | Head 5 Zoom | 000-255 | 0-100% |
| 35 | Head 6 Zoom | 000-255 | 0-100% |
| 36 | Control | 000-009 / 010-255 | No function; See Control reference |
| 37 | Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 38 | Red Fine Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 39 | Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 40 | Green Fine Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 41 | Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 42 | Blue Fine Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 43 | White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 44 | White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 45 | Head 1 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 46 | Head 1 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 47 | Head 1 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 48 | Head 1 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 49 | Head 1 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 50 | Head 1 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 51 | Head 1 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 52 | Head 1 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 53 | Head 2 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 54 | Head 2 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 55 | Head 2 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 56 | Head 2 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 57 | Head 2 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 58 | Head 2 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 59 | Head 2 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 60 | Head 2 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 61 | Head 3 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 62 | Head 3 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 63 | Head 3 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 64 | Head 3 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 65 | Head 3 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 66 | Head 3 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 67 | Head 3 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 68 | Head 3 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 69 | Head 4 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 70 | Head 4 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 71 | Head 4 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 72 | Head 4 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 73 | Head 4 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 74 | Head 4 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 75 | Head 4 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 76 | Head 4 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 77 | Head 5 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 78 | Head 5 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 79 | Head 5 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 80 | Head 5 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 81 | Head 5 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 82 | Head 5 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 83 | Head 5 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 84 | Head 5 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 85 | Head 6 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 86 | Head 6 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 87 | Head 6 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 88 | Head 6 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 89 | Head 6 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 90 | Head 6 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 91 | Head 6 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 92 | Head 6 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |

5.5 94-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-100% |
| 2 | Pan Fine | 000-255 | 0-100% |
| 3 | Continuous Pan Rotation | 001-126 / 127-129 / 130-255 | Forward rotation, fast to slow; Stop; Reverse rotation, slow to fast |
| 4 | Control | 000-009 / 010-255 | No function; See Control reference |
| 5 | Tilt Speed | 000-255 | Fast to slow |
| 6 | Tilt Macro | 000-004 / 005-255 | No function; See Tilt Macro reference |
| 7 | CTC | 000 / 001-255 | No function; From 19,000 K to 2,700 K |
| 8 | Color Macro | 000 / 001-255 | No function; See Color Macro reference |
| 9 | Pattern | 000 / 001-255 | No function; Pattern 1-255 |
| 10 | LED Effect | 000-015 / 016-255 | No function; See LED Effect reference |
| 11 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow; Stop; Slow to fast |
| 12 | LED Effect Delay | 000-255 | Fast to slow |
| 13 | Background Color | 000 / 001-255 | No function; See Background Color reference |
| 14 | Background color Dimmer | 000-255 | 0-100% |
| 15 | Background color Dimmer fine | 000-255 | 0-100% |
| 16 | Shutter | 000-019 / 020-024 | Blackout; See Strobe reference |
| 17 | Head 1 Tilt | 000-255 | 0-100% |
| 18 | Head 1 Tilt Fine | 000-255 | 0-100% |
| 19 | Head 1 Zoom | 000-255 | 0-100% |
| 20 | Head 1 Dimmer | 000-255 | 0-100% |
| 21 | Head 1 Dimmer Fine | 000-255 | 0-100% |
| 22 | Head 1 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 23 | Head 1 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 24 | Head 1 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 25 | Head 1 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 26 | Head 1 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 27 | Head 1 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 28 | Head 1 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 29 | Head 1 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 30 | Head 2 Tilt | 000-255 | 0-100% |
| 31 | Head 2 Tilt Fine | 000-255 | 0-100% |
| 32 | Head 2 Zoom | 000-255 | 0-100% |
| 33 | Dimmer2 | 000-255 | 0-100% |
| 34 | Head 2 Dimmer Fine | 000-255 | 0-100% |
| 35 | Head 2 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 36 | Head 2 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 37 | Head 2 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 38 | Head 2 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 39 | Head 2 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 40 | Head 2 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 41 | Head 2 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 42 | Head 2 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 43 | Head 3 Tilt | 000-255 | 0-100% |
| 44 | Head 3 Tilt Fine | 000-255 | 0-100% |
| 45 | Head 3 Zoom | 000-255 | 0-100% |
| 46 | Head 3 Dimmer | 000-255 | 0-100% |
| 47 | Head 3 Dimmer Fine | 000-255 | 0-100% |
| 48 | Head 3 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 49 | Head 3 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 50 | Head 3 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 51 | Head 3 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 52 | Head 3 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 53 | Head 3 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 54 | Head 3 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 55 | Head 3 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 56 | Head 4 Tilt | 000-255 | 0-100% |
| 57 | Head 4 Tilt Fine | 000-255 | 0-100% |
| 58 | Head 4 Zoom | 000-255 | 0-100% |
| 59 | Head 4 Dimmer | 000-255 | 0-100% |
| 60 | Head 4 Dimmer Fine | 000-255 | 0-100% |
| 61 | Head 4 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 62 | Head 4 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 63 | Head 4 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 64 | Head 4 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 65 | Head 4 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 66 | Head 4 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 67 | Head 4 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 68 | Head 4 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 69 | Head 5 Tilt | 000-255 | 0-100% |
| 70 | Head 5 Tilt Fine | 000-255 | 0-100% |
| 71 | Head 5 Zoom | 000-255 | 0-100% |
| 72 | Head 5 Dimmer | 000-255 | 0-100% |
| 73 | Head 5 Dimmer Fine | 000-255 | 0-100% |
| 74 | Head 5 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 75 | Head 5 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 76 | Head 5 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 77 | Head 5 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 78 | Head 5 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 79 | Head 5 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 80 | Head 5 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 81 | Head 5 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 82 | Head 6 Tilt | 000-255 | 0-100% |
| 83 | Head 6 Tilt Fine | 000-255 | 0-100% |
| 84 | Head 6 Zoom | 000-255 | 0-100% |
| 85 | Dimmer6 | 000-255 | 0-100% |
| 86 | Head 6 Dimmer Fine | 000-255 | 0-100% |
| 87 | Head 6 Red / Cyan | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 88 | Head 6 Red Fine / Cyan Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 89 | Head 6 Green / Magenta | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 90 | Head 6 Green Fine / Magenta Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 91 | Head 6 Blue / Yellow | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 92 | Head 6 Blue Fine / Yellow Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 93 | Head 6 White | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |
| 94 | Head 6 White Fine | 000-255 | RGBW mode: 0-100% / CMY mode: 100-0% |

6  Function-Channel Reference

Use these tables for channels that point to a shared macro or control reference. Keep all DMX ranges matched to the installed firmware.

LED EFFECT CORRECTION  The source range 163-179 overlaps the next range 171-178 and repeats Effect 31. This edition normalizes the continuous sequence to 163-170 and labels the second repeated entry Effect 32.

6.1 Strobe Ranges

| DMX Value | Function |
| --- | --- |
| 000-019 | Blackout |
| 020-024 | Open |
| 025-064 | Strobe from fast to slow |
| 065-069 | Open |
| 070-084 | 100-0% strobe, from fast to slow |
| 085-089 | Open |
| 090-104 | 0-100% strobe, from fast to slow |
| 105-109 | Open |
| 110-124 | Random strobe, from fast to slow |
| 125-129 | Open |
| 130-144 | Random 100-0% strobe, from fast to slow |
| 145-149 | Open |
| 150-164 | Random 0-100% strobe from fast to slow |
| 165-169 | Open |
| 170-184 | Pulse Strobe 1 fast to slow |
| 185-189 | Open |
| 190-204 | Random pulse strobe, fast to slow |
| 205-209 | Open |
| 210-224 | 100-0-100% strobe, from fast to slow |
| 225-229 | Open |
| 230-244 | Pulse Strobe 2 fast to slow |
| 245-255 | Open |

6.2 Tilt Macro Ranges

| DMX Value | Function |
| --- | --- |
| 000-005 | No function |
| 006-011 | Tilt Macro 1 |
| 012-017 | Tilt Macro 2 |
| 018-023 | Tilt Macro 3 |
| 024-029 | Tilt Macro 4 |
| 030-035 | Tilt Macro 5 |
| 036-041 | Tilt Macro 6 |
| 042-047 | Tilt Macro 7 |
| 048-053 | Tilt Macro 8 |
| 054-059 | Tilt Macro 9 |
| 060-065 | Tilt Macro 10 |
| 066-071 | Tilt Macro 11 |
| 072-077 | Tilt Macro 12 |
| 078-083 | Tilt Macro 13 |
| 084-089 | Tilt Macro 14 |
| 090-095 | Tilt Macro 15 |
| 096-101 | Tilt Macro 16 |
| 102-107 | Tilt Macro 17 |
| 108-113 | Tilt Macro 18 |
| 114-119 | Tilt Macro 19 |
| 120-125 | Tilt Macro 20 |
| 126-131 | Tilt Macro 21 |
| 132-137 | Tilt Macro 22 |
| 138-143 | Tilt Macro 23 |
| 144-149 | Tilt Macro 24 |
| 150-155 | Tilt Macro 25 |
| 156-161 | Tilt Macro 26 |
| 162-167 | Tilt Macro 27 |
| 168-173 | Tilt Macro 28 |
| 174-179 | Tilt Macro 29 |
| 180-185 | Tilt Macro 30 |
| 186-191 | Tilt Macro 31 |
| 192-197 | Tilt Macro 32 |
| 198-203 | Tilt Macro 33 |
| 204-209 | Tilt Macro 34 |
| 210-215 | Tilt Macro 35 |
| 216-221 | Tilt Macro 36 |
| 222-227 | Tilt Macro 37 |
| 228-233 | Tilt Macro 38 |
| 234-239 | Tilt Macro 39 |
| 240-245 | Tilt Macro 40 |
| 246-251 | Tilt Macro 41 |
| 252-255 | Tilt Macro 42 |

6.3 Color Macro Ranges

| DMX Value | Function |
| --- | --- |
| 000 | No function |
| 001-002 | 2700 K |
| 003-004 | 3200 K |
| 005-006 | 4200 K |
| 007-008 | 5600 K |
| 009-010 | 8000 K |
| 011 | Blue |
| 012-048 | Blue to cyan |
| 049 | Cyan |
| 050-086 | Cyan to green |
| 087 | Green |
| 088-124 | Green to yellow |
| 125 | Yellow |
| 126-162 | Yellow to red |
| 163 | Red |
| 164-200 | Red to magenta |
| 201 | Magenta |
| 202-238 | Magenta to blue |
| 239 | Blue |
| 240-247 | Color off, from fast to slow |
| 248-255 | Color on, fast to slow |

6.4 LED Effect Ranges

| DMX Value | Function |
| --- | --- |
| 000-000 | No effect |
| 001-006 | Effect 1 |
| 007-015 | Effect 2 |
| 016-023 | Effect 3 |
| 024-031 | Effect 4 |
| 032-038 | Effect 5 |
| 039-046 | Effect 6 |
| 047-054 | Effect 7 |
| 055-062 | Effect 8 |
| 063-069 | Effect 9 |
| 070-077 | Effect 10 |
| 078-085 | Effect 11 |
| 086-093 | Effect 12 |
| 094-100 | Effect 13 |
| 101-108 | Effect 14 |
| 109-116 | Effect 15 |
| 117-124 | Effect 16 |
| 125-131 | Effect 17 |
| 132-139 | Effect 18 |
| 140-147 | Effect 19 |
| 148-155 | Effect 20 |
| 156-162 | Effect 21 |
| 163-170 | Effect 22 |
| 171-178 | Effect 23 |
| 179-186 | Effect 24 |
| 187-193 | Effect 25 |
| 194-201 | Effect 26 |
| 202-209 | Effect 27 |
| 210-217 | Effect 28 |
| 218-224 | Effect 29 |
| 225-232 | Effect 30 |
| 233-240 | Effect 31 |
| 241-248 | Effect 32 |
| 249-255 | Effect 33 |

6.5 Background Color Ranges

| DMX Value | Function |
| --- | --- |
| 000 | No function |
| 001-002 | 2700 K |
| 003-004 | 3200 K |
| 005-006 | 4200 K |
| 007-008 | 5600 K |
| 009-010 | 8000 K |
| 011 | Blue |
| 012-048 | Blue to cyan |
| 049 | Cyan |
| 050-086 | Cyan to green |
| 087 | Green |
| 088-124 | Green to yellow |
| 125 | Yellow |
| 126-162 | Yellow to red |
| 163 | Red |
| 164-200 | Red to magenta |
| 201 | Magenta |
| 202-238 | Magenta to blue |
| 239 | Blue |
| 240-247 | Color gradient, from fast to slow |
| 248-255 | Color jumps, from fast to slow |

6.6 Control Ranges

| DMX Value | Function |
| --- | --- |
| 000-009 | No function |
| 010-019 | Reserved |
| 020-024 | RGBW (additive) color-mixing mode |
| 025-029 | CMY (subtractive) color-mixing mode |
| 030-054 | Reserved |
| 055-059 | Tilt Reset |
| 060-064 | Zoom reset |
| 065-069 | Reserved |
| 070-074 | All Motor Reset |
| 075-079 | Sequence 1-12 |
| 080-084 | Sequence 12-1 |
| 085-089 | Reserved |
| 090-094 | Tilt Reverse On |
| 095-099 | Reserved |
| 100-104 | Tilt Reverse Off |
| 105-134 | Reserved |
| 135-139 | Fast Dimmer Response |
| 140-144 | Smooth Dimmer Response |
| 145-149 | Linear Dimmer Curve |
| 150-154 | Square Dimmer Curve |
| 155-159 | Inverse Square Dimmer Curve |
| 160-164 | S-Curve Dimmer Curve |
| 165-255 | Reserved |

7  Routine Maintenance

ATTENTION  Disconnect AC power and allow the fixture to cool completely before inspection or cleaning.

Inspect the housing, lenses, mounting points, clamps, safety cable, power cable and data connectors before every use.

Replace damaged lenses, cables and structural parts before returning the fixture to service.

Clean optical surfaces with a soft, lint-free cloth and a suitable glass-cleaning fluid; dry every part carefully.

Keep fans and ventilation openings free of dust, fog-fluid residue and obstructions.

Inspect pan, tilt and zoom travel for unusual noise, resistance, imbalance or positioning errors.

Switch the fixture off completely at the circuit breaker or plug when it is not in use to reduce wear and extend service life.

8  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| No light output | Verify supply voltage, power connection, circuit protection, available supply capacity and controller output. |
| No DMX response after reset | Confirm DMX mode, start address, selected 23/38/62/92/94CH personality, controller patch, cable polarity and connectors. |
| Intermittent or unstable DMX | Shorten the data run, separate signal and high-voltage cables, use shielded twisted-pair cable, inspect amplifiers and terminate the final fixture with 120 ohms. |
| Fixture does not start | Check the supply rating and inspect for transport damage, moisture, loose internal connectors or damaged electronics. Refer internal inspection to qualified service. |
| Abnormal pan, tilt or zoom | Remove obstructions, run the appropriate reset, inspect menu limits/reversals, and refer belts, Hall sensors and internal mechanisms to qualified service. |
| Reduced or unstable output | Check temperature, ventilation, fan operation, optical cleanliness, supply quality and error records. |

SERVICE  There are no user-serviceable high-voltage parts inside. Refer internal repair, light-source replacement, motor/belt service and calibration to the manufacturer, its service agent or similarly qualified personnel.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Include the model, serial number, firmware version, active personality and a clear fault description when requesting support.

End of manual