AL0760WP

7 x 60 W LED Bee Eye
Moving Head IP65

RGBW BEE-EYE - RGB AURA FILAMENT - 7° TO 45° ZOOM

Outdoor beam, wash and pixel-effect moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | Photometric Data |
| 6 | DMX Channel Reference |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition is organized for model AL0760WP, 7 x 60 W LED Bee Eye Moving Head IP65. Confirm the model, voltage rating and firmware shown on the actual fixture before installation or programming.

This edition uses the user-supplied AL0760WP model designation consistently throughout the manual.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool completely before opening, cleaning, inspection or service.

Use the fixture only with the voltage and frequency marked on its product label. Connect it to a properly grounded Class I supply.

If the fixture has experienced a large temperature change, leave it disconnected until it reaches room temperature and condensation has evaporated.

Protect power and data cables from crushing, sharp edges, heat and other damage. Disconnect by holding the plug, not by pulling the cable.

Keep at least 0.5 m clearance from combustible materials and maintain clear ventilation around the fixture.

Do not look directly into the output at close range and do not direct strong light into the fixture lens.

Carry the fixture by its base, never by the moving head. Protect it from severe shock and vibration.

Installation, operation and repair must be performed by qualified personnel. Do not modify the fixture.

For suspended installation, use rated clamps, an omega bracket and an independently rated safety cable attached only to the designated point.

IP65 USE  IP65 protection depends on intact seals, closed caps and correctly mated waterproof connectors. Do not immerse the fixture, open it while wet, or allow water to enter an uncapped connector interface.

LENS CONDITION  Do not operate the fixture with a damaged, severely scratched or loose lens or cover. Replace damaged optical or structural parts before use.

2  Product Overview

The AL0760WP combines seven 60 W RGBW bee-eye emitters with twelve RGB filament/Aura sources in an IP65 moving-head platform. Motorized zoom, lens rotation, color macros and independent pixel control provide beam, wash, kaleidoscope and animated Aura effects.

2.1 Package Contents

| Item | Quantity |
| --- | --- |
| AL0760WP fixture | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |
| Safety cable | 1 |
| User manual | 1 |

2.2 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 90-240 V, 50/60 Hz |
| Rated power | 550 W |
| Main light source | 7 x 60 W RGBW 4-in-1 LEDs |
| Aura / filament source | 12 x 3 W RGB 3-in-1 LED filaments |
| Zoom | 7°-45°, motorized |
| Strobe | 1-25 Hz with multiple effects |
| Dimming | 0-100%; Linear / Square / Inverse Square / S-curve |
| DMX personalities | 21CH / 30CH / 36CH / 57CH / 78CH / 93CH |
| Control | DMX512 / Auto / RDM; Art-Net with optional additional interface |
| Pan / tilt | 540° / 270°, both with 16-bit fine control |
| Display | 1.77-inch OLED; Chinese/English menu; waterproof touch keys |
| Screen activation password | 1111 (source specification) |
| Ingress protection | IP65 |
| Cooling | Intelligent fan control and overheat protection |
| Construction | Die-cast aluminum and flame-retardant, UV-resistant engineering plastic |
| Fixture dimensions | 386 x 291 x 533 mm |
| Carton size | 630 x 470 x 360 mm |
| Net / gross weight | 19.5 kg / 22.5 kg |
| Flight case (2 fixtures) | 960 x 450 x 780 mm; case N.W. 31 kg; loaded G.W. 72 kg |

2.3 Main Functions

RGBW mixing with saturated colors, pastel tones, white output, tungsten emulation and virtual color wheel.

High-precision PMMA optics and quartz light pipe for smooth wash coverage.

Foreground/background macros with speed and direction control.

Three-dimensional pixel mapping of the seven main beam pixels and twelve Aura filaments.

High-speed pan, tilt, motorized zoom and rotating-lens effects.

Automatic fan-speed adjustment and overheating protection.

3  Setup and Installation

3.1 Rigging

STRUCTURAL RATING  The support structure and attachment point must support at least 10 times the fixture weight. Keep people clear of the area below during installation and service.

Inspect the fixture, omega bracket, clamp, fasteners, safety cable and support structure.

Attach a correctly rated clamp to the omega bracket using an M10 fastener through the center hole.

Lock the omega bracket to the fixture base and tighten all fasteners.

Attach an independent rated safety cable to the designated safety point; never attach it to a carrying handle.

Confirm at least 0.5 m clearance from combustible materials and adequate clearance for the full pan, tilt and zoom range.

3.2 DMX512 Connection

Use shielded, twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Keep data wiring away from mains cabling where practical.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the last fixture in a long or electrically noisy DMX chain, install a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use MENU to select a function, ENTER to open or save it, and UP/DOWN to change the selected option or value. Menu labels may vary slightly with firmware revision.

| Menu | Options | Operation |
| --- | --- | --- |
| Address | 001-255 | Set the DMX start address (source menu range; verify firmware). |
| Running Mode | DMX / Auto Test / Manual Test / Auto / Channel Test | Select console, automatic or local test operation. |
| DMX Personality | 21 / 30 / 36 / 57 / 78 / 93CH | Select the matching controller profile. |
| Pan / Tilt Reverse | Yes / No | Reverse movement direction. |
| Screen Reverse / Lock | Yes / No | Invert or lock the display. |
| No-Signal Hold | Yes / No | Retain or release the last state after signal loss. |
| Pan Angle | 540 / 360 / 180 | Limit pan travel. |
| Tilt Angle | 270 / 180 / 90 | Limit tilt travel. |
| Arefaction Time | 085 (source label) | Source-listed timing parameter; verify firmware meaning. |
| Language | EN / CH | Select the display language. |
| Dimmer Speed | Fast / Smooth | Select dimmer response. |
| Dimmer Curve | Linear / Square / Inverse Square / S-curve | Select the dimmer curve. |
| Reset / Factory Reset | Confirm | Reset the fixture or restore factory settings. |
| System Information | Version / Mode / Address / Temperature / Hours | View operating and service information. |

TEMPERATURE MESSAGES  Thermistor Open indicates an open sensor circuit; Thermistor Short indicates a short circuit; Thermistor Hot indicates overheating. Stop operation and correct the fault before reuse.

5  Photometric Data

The following 7° beam-angle illuminance values are reproduced from the source manual. Measurements are nominal and can vary with LED binning, optical condition, voltage and measurement method.

| Color | 3 m | 5 m | 7 m | 10 m |
| --- | --- | --- | --- | --- |
| Red | 12,400 lx | 4,300 lx | 2,200 lx | 1,100 lx |
| Green | 32,500 lx | 12,000 lx | 6,500 lx | 3,200 lx |
| Blue | 55,500 lx | 21,800 lx | 12,000 lx | 6,000 lx |
| White | 55,800 lx | 22,000 lx | 11,400 lx | 5,800 lx |
| RGBW | 10,800 lx* | 54,000 lx | 29,700 lx | 14,600 lx |

SOURCE INCONSISTENCY  The source lists RGBW as 10,800 lx at 3 m but 54,000 lx at 5 m. The 3 m value is reproduced without alteration and must be verified before publication or photometric planning.

6  DMX Channel Reference

Select the matching personality on the fixture and controller. The 57CH and 93CH modes extend the 36CH core; the 78CH mode extends the 21CH core. Pixel-extension tables below complete those personalities without repeating the shared core channels.

| Mode | Channel map |
| --- | --- |
| 21CH | Complete compact RGBW, movement, zoom and lens-rotation control |
| 30CH | Movement, effects, backlight color and resets |
| 36CH | Fine RGB, effects and Aura/backlight control |
| 57CH | 36CH core + seven RGB main pixels (37-57) |
| 78CH | 21CH core + seven RGB main pixels (22-42) + twelve RGB Aura filaments (43-78) |
| 93CH | 36CH core + seven RGB main pixels (37-57) + twelve RGB Aura filaments (58-93) |

SOURCE NORMALIZATION  The source labels Channel 7 as 'g' in 36CH-derived tables; this edition restores Channel 7 Green Fine. 'Focus' is normalized to Zoom where it controls the stated 7°-45° zoom system. Reserved channels remain undocumented.

6.1 21-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Coarse red dimmer |
| 2 | Red Fine | 0-255 | Fine red adjustment |
| 3 | Green | 0-255 | Coarse green dimmer |
| 4 | Green Fine | 0-255 | Fine green adjustment |
| 5 | Blue | 0-255 | Coarse blue dimmer |
| 6 | Blue Fine | 0-255 | Fine blue adjustment |
| 7 | White | 0-255 | Coarse white dimmer |
| 8 | White Fine | 0-255 | Fine white adjustment |
| 9 | CTO | 0-255 | Color temperature, high to low |
| 10 | Color Macro | 0-255 | Built-in color macro |
| 11 | Strobe | 0-255 | 0-3 blackout; 4-103 strobe slow to fast; 104-107 open; 108-207 pulse fast to slow; 208-212 open; 213-251 random; 252-255 open |
| 12 | Master Dimmer | 0-255 | Linear dimming, dark to full |
| 13 | Dimmer Fine | 0-255 | Fine master-dimmer adjustment |
| 14 | Pan | 0-255 | Pan movement |
| 15 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 16 | Tilt | 0-255 | Tilt movement |
| 17 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 18 | Reserved Macro | 0-255 | Reserved macro function |
| 19 | Reset / Fan | 0-255 | 0-25 none; 26-76 effect reset; 77-127 pan/tilt reset; 128-129 none; 130-134 fan mute; 135-139 fan standard; 140-144 fan full; 145-191 none; 192-255 full reset |
| 20 | Zoom | 0-255 | Narrow to wide |
| 21 | Lens Rotation | 0-255 | 0-127 position 0°-60°; 128-190 CCW fast to slow; 191-192 stop; 193-255 CW slow to fast |

6.2 30-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan movement |
| 2 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 3 | Tilt | 0-255 | Tilt movement |
| 4 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 5 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 6 | Zoom | 0-255 | Narrow to wide |
| 7 | Lens Rotation | 0-255 | 0-127 position 0°-60°; 128-190 CCW fast to slow; 191-192 stop; 193-255 CW slow to fast |
| 8 | Master Dimmer | 0-255 | Linear dimming, dark to full |
| 9 | Strobe | 0-255 | 0-3 open; 4-200 strobe slow to fast; 201-255 random strobe slow to fast |
| 10 | Red | 0-255 | Red dimmer |
| 11 | Green | 0-255 | Green dimmer |
| 12 | Blue | 0-255 | Blue dimmer |
| 13 | White | 0-255 | White dimmer |
| 14 | CTO | 0-255 | 0-9 none; 10-255 color temperature high to low |
| 15 | Color Macro | 0-255 | Built-in color macro |
| 16 | Static Effect | 0-255 | 0-9 none; 10-255 one effect per 5 values |
| 17 | Dynamic Effect | 0-255 | 0-4 none; 5-255 one effect per 5 values |
| 18 | Dynamic Effect Speed | 0-255 | 0-127 CCW fast to slow; 128-255 CW slow to fast |
| 19 | Effect Background Red | 0-255 | Dynamic-effect background red |
| 20 | Effect Background Green | 0-255 | Dynamic-effect background green |
| 21 | Effect Background Blue | 0-255 | Dynamic-effect background blue |
| 22 | Effect Background White | 0-255 | Dynamic-effect background white |
| 23 | Backlight Master Dimmer | 0-255 | Backlight master dimmer |
| 24 | Backlight Strobe | 0-255 | 0-3 open; 4-250 strobe slow to fast; 251-255 open |
| 25 | Backlight Red | 0-255 | Backlight red dimmer |
| 26 | Backlight Green | 0-255 | Backlight green dimmer |
| 27 | Backlight Blue | 0-255 | Backlight blue dimmer |
| 28 | Backlight Macro | 0-255 | 0-1 none; 2-51 static colors, two values each; 52-255 dynamic effects, three values each |
| 29 | Backlight Macro Speed | 0-255 | Slow to fast |
| 30 | Reset / Fan | 0-255 | 0-25 none; 26-76 effect reset; 77-127 pan/tilt reset; 128-129 none; 130-134 fan mute; 135-139 fan standard; 140-144 fan full; 145-191 none; 192-255 full reset |

6.3 36-Channel Core Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Strobe | 0-255 | 0-19 blackout; 20-49 open; 50-200 strobe slow to fast; 201-210 open; 211-255 random |
| 2 | Master Dimmer | 0-255 | Linear dimming |
| 3 | Dimmer Fine | 0-255 | Fine master dimmer |
| 4 | Red | 0-255 | Coarse red |
| 5 | Red Fine | 0-255 | Fine red |
| 6 | Green | 0-255 | Coarse green |
| 7 | Green Fine | 0-255 | Fine green |
| 8 | Blue | 0-255 | Coarse blue |
| 9 | Blue Fine | 0-255 | Fine blue |
| 10 | CTO | 0-255 | Color temperature high to low |
| 11 | Reserved | 0-255 | Reserved |
| 12 | Color Macro | 0-255 | Built-in color macro |
| 13 | Zoom | 0-255 | Narrow to wide |
| 14 | Zoom Fine | 0-255 | Fine zoom |
| 15 | Pan | 0-255 | Pan movement |
| 16 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 17 | Tilt | 0-255 | Tilt movement |
| 18 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 19 | Reset / Fan | 0-255 | 0-25 none; 26-76 effect reset; 77-127 pan/tilt reset; 128-129 none; 130-134 fan mute; 135-139 fan standard; 140-144 fan full; 145-191 none; 192-255 full reset |
| 20 | Frequency | 0-255 | Output/PWM frequency control |
| 21 | Reserved | 0-255 | Reserved |
| 22 | LED Built-in Effect | 0-255 | 0-4 none; 5-255 one effect per 5 values |
| 23 | LED Effect Speed | 0-255 | 0-127 CCW fast to slow; 128-255 CW slow to fast |
| 24 | Backlight Built-in | 0-255 | 0-1 none; 2-51 static colors, two values each; 52-255 dynamic effects, three values each |
| 25 | Backlight Effect Speed | 0-255 | Slow to fast |
| 26 | LED Effect Translation | 0-255 | Effect translation/offset |
| 27 | Backlight Strobe | 0-255 | 0-3 open; 4-250 strobe slow to fast; 251-255 open |
| 28 | Backlight Dimmer | 0-255 | Coarse backlight dimmer |
| 29 | Backlight Dimmer Fine | 0-255 | Fine backlight dimmer |
| 30 | Backlight Red | 0-255 | Red dimmer |
| 31 | Backlight Green | 0-255 | Green dimmer |
| 32 | Backlight Blue | 0-255 | Blue dimmer |
| 33 | Backlight CTO | 0-255 | 0-18 none; 19-255 color temperature low to high |
| 34 | Reserved | 0-255 | Reserved |
| 35 | Backlight Color Macro | 0-255 | Built-in backlight color macro |
| 36 | Reserved | 0-255 | Reserved |

6.4 57CH / 93CH Main-Pixel Extension

Both modes use Channels 37-57 for the seven RGB main pixels. The 57CH personality ends at Channel 57; the 93CH personality continues with the Aura extension below.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 37 | Main Pixel 1 Red | 0-255 | Main Pixel 1 red dimmer |
| 38 | Main Pixel 1 Green | 0-255 | Main Pixel 1 green dimmer |
| 39 | Main Pixel 1 Blue | 0-255 | Main Pixel 1 blue dimmer |
| 40 | Main Pixel 2 Red | 0-255 | Main Pixel 2 red dimmer |
| 41 | Main Pixel 2 Green | 0-255 | Main Pixel 2 green dimmer |
| 42 | Main Pixel 2 Blue | 0-255 | Main Pixel 2 blue dimmer |
| 43 | Main Pixel 3 Red | 0-255 | Main Pixel 3 red dimmer |
| 44 | Main Pixel 3 Green | 0-255 | Main Pixel 3 green dimmer |
| 45 | Main Pixel 3 Blue | 0-255 | Main Pixel 3 blue dimmer |
| 46 | Main Pixel 4 Red | 0-255 | Main Pixel 4 red dimmer |
| 47 | Main Pixel 4 Green | 0-255 | Main Pixel 4 green dimmer |
| 48 | Main Pixel 4 Blue | 0-255 | Main Pixel 4 blue dimmer |
| 49 | Main Pixel 5 Red | 0-255 | Main Pixel 5 red dimmer |
| 50 | Main Pixel 5 Green | 0-255 | Main Pixel 5 green dimmer |
| 51 | Main Pixel 5 Blue | 0-255 | Main Pixel 5 blue dimmer |
| 52 | Main Pixel 6 Red | 0-255 | Main Pixel 6 red dimmer |
| 53 | Main Pixel 6 Green | 0-255 | Main Pixel 6 green dimmer |
| 54 | Main Pixel 6 Blue | 0-255 | Main Pixel 6 blue dimmer |
| 55 | Main Pixel 7 Red | 0-255 | Main Pixel 7 red dimmer |
| 56 | Main Pixel 7 Green | 0-255 | Main Pixel 7 green dimmer |
| 57 | Main Pixel 7 Blue | 0-255 | Main Pixel 7 blue dimmer |

6.5 78CH Main-Pixel Extension - Channels 22-42

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22 | Main Pixel 1 Red | 0-255 | Main Pixel 1 red dimmer |
| 23 | Main Pixel 1 Green | 0-255 | Main Pixel 1 green dimmer |
| 24 | Main Pixel 1 Blue | 0-255 | Main Pixel 1 blue dimmer |
| 25 | Main Pixel 2 Red | 0-255 | Main Pixel 2 red dimmer |
| 26 | Main Pixel 2 Green | 0-255 | Main Pixel 2 green dimmer |
| 27 | Main Pixel 2 Blue | 0-255 | Main Pixel 2 blue dimmer |
| 28 | Main Pixel 3 Red | 0-255 | Main Pixel 3 red dimmer |
| 29 | Main Pixel 3 Green | 0-255 | Main Pixel 3 green dimmer |
| 30 | Main Pixel 3 Blue | 0-255 | Main Pixel 3 blue dimmer |
| 31 | Main Pixel 4 Red | 0-255 | Main Pixel 4 red dimmer |
| 32 | Main Pixel 4 Green | 0-255 | Main Pixel 4 green dimmer |
| 33 | Main Pixel 4 Blue | 0-255 | Main Pixel 4 blue dimmer |
| 34 | Main Pixel 5 Red | 0-255 | Main Pixel 5 red dimmer |
| 35 | Main Pixel 5 Green | 0-255 | Main Pixel 5 green dimmer |
| 36 | Main Pixel 5 Blue | 0-255 | Main Pixel 5 blue dimmer |
| 37 | Main Pixel 6 Red | 0-255 | Main Pixel 6 red dimmer |
| 38 | Main Pixel 6 Green | 0-255 | Main Pixel 6 green dimmer |
| 39 | Main Pixel 6 Blue | 0-255 | Main Pixel 6 blue dimmer |
| 40 | Main Pixel 7 Red | 0-255 | Main Pixel 7 red dimmer |
| 41 | Main Pixel 7 Green | 0-255 | Main Pixel 7 green dimmer |
| 42 | Main Pixel 7 Blue | 0-255 | Main Pixel 7 blue dimmer |

6.6 78CH Aura Extension - Channels 43-78

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 43 | Aura Filament 1 Red | 0-255 | Aura Filament 1 red dimmer |
| 44 | Aura Filament 1 Green | 0-255 | Aura Filament 1 green dimmer |
| 45 | Aura Filament 1 Blue | 0-255 | Aura Filament 1 blue dimmer |
| 46 | Aura Filament 2 Red | 0-255 | Aura Filament 2 red dimmer |
| 47 | Aura Filament 2 Green | 0-255 | Aura Filament 2 green dimmer |
| 48 | Aura Filament 2 Blue | 0-255 | Aura Filament 2 blue dimmer |
| 49 | Aura Filament 3 Red | 0-255 | Aura Filament 3 red dimmer |
| 50 | Aura Filament 3 Green | 0-255 | Aura Filament 3 green dimmer |
| 51 | Aura Filament 3 Blue | 0-255 | Aura Filament 3 blue dimmer |
| 52 | Aura Filament 4 Red | 0-255 | Aura Filament 4 red dimmer |
| 53 | Aura Filament 4 Green | 0-255 | Aura Filament 4 green dimmer |
| 54 | Aura Filament 4 Blue | 0-255 | Aura Filament 4 blue dimmer |
| 55 | Aura Filament 5 Red | 0-255 | Aura Filament 5 red dimmer |
| 56 | Aura Filament 5 Green | 0-255 | Aura Filament 5 green dimmer |
| 57 | Aura Filament 5 Blue | 0-255 | Aura Filament 5 blue dimmer |
| 58 | Aura Filament 6 Red | 0-255 | Aura Filament 6 red dimmer |
| 59 | Aura Filament 6 Green | 0-255 | Aura Filament 6 green dimmer |
| 60 | Aura Filament 6 Blue | 0-255 | Aura Filament 6 blue dimmer |
| 61 | Aura Filament 7 Red | 0-255 | Aura Filament 7 red dimmer |
| 62 | Aura Filament 7 Green | 0-255 | Aura Filament 7 green dimmer |
| 63 | Aura Filament 7 Blue | 0-255 | Aura Filament 7 blue dimmer |
| 64 | Aura Filament 8 Red | 0-255 | Aura Filament 8 red dimmer |
| 65 | Aura Filament 8 Green | 0-255 | Aura Filament 8 green dimmer |
| 66 | Aura Filament 8 Blue | 0-255 | Aura Filament 8 blue dimmer |
| 67 | Aura Filament 9 Red | 0-255 | Aura Filament 9 red dimmer |
| 68 | Aura Filament 9 Green | 0-255 | Aura Filament 9 green dimmer |
| 69 | Aura Filament 9 Blue | 0-255 | Aura Filament 9 blue dimmer |
| 70 | Aura Filament 10 Red | 0-255 | Aura Filament 10 red dimmer |
| 71 | Aura Filament 10 Green | 0-255 | Aura Filament 10 green dimmer |
| 72 | Aura Filament 10 Blue | 0-255 | Aura Filament 10 blue dimmer |
| 73 | Aura Filament 11 Red | 0-255 | Aura Filament 11 red dimmer |
| 74 | Aura Filament 11 Green | 0-255 | Aura Filament 11 green dimmer |
| 75 | Aura Filament 11 Blue | 0-255 | Aura Filament 11 blue dimmer |
| 76 | Aura Filament 12 Red | 0-255 | Aura Filament 12 red dimmer |
| 77 | Aura Filament 12 Green | 0-255 | Aura Filament 12 green dimmer |
| 78 | Aura Filament 12 Blue | 0-255 | Aura Filament 12 blue dimmer |

6.7 93CH Aura Extension - Channels 58-93

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 58 | Aura Filament 1 Red | 0-255 | Aura Filament 1 red dimmer |
| 59 | Aura Filament 1 Green | 0-255 | Aura Filament 1 green dimmer |
| 60 | Aura Filament 1 Blue | 0-255 | Aura Filament 1 blue dimmer |
| 61 | Aura Filament 2 Red | 0-255 | Aura Filament 2 red dimmer |
| 62 | Aura Filament 2 Green | 0-255 | Aura Filament 2 green dimmer |
| 63 | Aura Filament 2 Blue | 0-255 | Aura Filament 2 blue dimmer |
| 64 | Aura Filament 3 Red | 0-255 | Aura Filament 3 red dimmer |
| 65 | Aura Filament 3 Green | 0-255 | Aura Filament 3 green dimmer |
| 66 | Aura Filament 3 Blue | 0-255 | Aura Filament 3 blue dimmer |
| 67 | Aura Filament 4 Red | 0-255 | Aura Filament 4 red dimmer |
| 68 | Aura Filament 4 Green | 0-255 | Aura Filament 4 green dimmer |
| 69 | Aura Filament 4 Blue | 0-255 | Aura Filament 4 blue dimmer |
| 70 | Aura Filament 5 Red | 0-255 | Aura Filament 5 red dimmer |
| 71 | Aura Filament 5 Green | 0-255 | Aura Filament 5 green dimmer |
| 72 | Aura Filament 5 Blue | 0-255 | Aura Filament 5 blue dimmer |
| 73 | Aura Filament 6 Red | 0-255 | Aura Filament 6 red dimmer |
| 74 | Aura Filament 6 Green | 0-255 | Aura Filament 6 green dimmer |
| 75 | Aura Filament 6 Blue | 0-255 | Aura Filament 6 blue dimmer |
| 76 | Aura Filament 7 Red | 0-255 | Aura Filament 7 red dimmer |
| 77 | Aura Filament 7 Green | 0-255 | Aura Filament 7 green dimmer |
| 78 | Aura Filament 7 Blue | 0-255 | Aura Filament 7 blue dimmer |
| 79 | Aura Filament 8 Red | 0-255 | Aura Filament 8 red dimmer |
| 80 | Aura Filament 8 Green | 0-255 | Aura Filament 8 green dimmer |
| 81 | Aura Filament 8 Blue | 0-255 | Aura Filament 8 blue dimmer |
| 82 | Aura Filament 9 Red | 0-255 | Aura Filament 9 red dimmer |
| 83 | Aura Filament 9 Green | 0-255 | Aura Filament 9 green dimmer |
| 84 | Aura Filament 9 Blue | 0-255 | Aura Filament 9 blue dimmer |
| 85 | Aura Filament 10 Red | 0-255 | Aura Filament 10 red dimmer |
| 86 | Aura Filament 10 Green | 0-255 | Aura Filament 10 green dimmer |
| 87 | Aura Filament 10 Blue | 0-255 | Aura Filament 10 blue dimmer |
| 88 | Aura Filament 11 Red | 0-255 | Aura Filament 11 red dimmer |
| 89 | Aura Filament 11 Green | 0-255 | Aura Filament 11 green dimmer |
| 90 | Aura Filament 11 Blue | 0-255 | Aura Filament 11 blue dimmer |
| 91 | Aura Filament 12 Red | 0-255 | Aura Filament 12 red dimmer |
| 92 | Aura Filament 12 Green | 0-255 | Aura Filament 12 green dimmer |
| 93 | Aura Filament 12 Blue | 0-255 | Aura Filament 12 blue dimmer |

7  Routine Maintenance

ATTENTION  Disconnect power, allow the fixture to cool and ensure all connector interfaces are dry before inspection or cleaning.

Inspect mounting screws, clamps, brackets and safety attachments for tightness, corrosion or deformation.

Inspect the housing, lenses, seals, suspension points and moving parts for damage, wear or imbalance.

Inspect power and data cables for cuts, crushing, fatigue, overheating or contamination.

Clean internal and external optical surfaces regularly with a moist, lint-free cloth suitable for the coating. Never use alcohol or aggressive solvents.

Remove dust from fans and ventilation openings. The source recommends weekly cleaning in demanding environments.

Arrange a detailed electrical inspection by a qualified engineer approximately every three months, adjusted to usage and local requirements.

Cleaning frequency must reflect operating hours, dust, fog-fluid residue, airflow and outdoor exposure. Correct every safety issue before returning the fixture to service.

8  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| Fixture does not power on | Verify supply voltage, protective earth, circuit protection, cable, connector and product-label rating. |
| No DMX response | Confirm DMX mode, address, selected personality, controller patch, cable polarity and termination. |
| Unexpected channel behavior | Confirm the controller profile matches 21/30/36/57/78/93CH and verify firmware. |
| Movement or rotation error | Remove obstructions, run the correct reset and inspect pan, tilt, zoom and lens-rotation operation. |
| Thermistor warning | Stop operation. Inspect temperature, cooling and sensor status; refer open/short/hot faults to qualified service. |
| Reduced or unstable output | Check temperature, ventilation, fan operation, optical cleanliness and supply quality. |
| Water or condensation suspected | Disconnect power, move the fixture to a dry service area and have seals/connectors inspected before reuse. |

SERVICE  There are no user-serviceable high-voltage or sealed parts inside. Refer internal repair, seal replacement and calibration to qualified personnel, and use genuine replacement parts.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized modification, impact, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Include the model, serial number, firmware version, selected personality and a clear fault description when requesting support.

End of manual