AL1019WR

19 x 15 W LED Bee Eye
Moving Head

RGBW 4-IN-1 - ZOOM - ROTATING HONEYCOMB LENS - USER MANUAL

Beam, wash and individually controlled pixel effects in one moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview and Specifications |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Photometric Data |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is for model AL1019WR. Confirm the model, voltage, firmware and product-label ratings before installation or programming.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool before opening, cleaning, inspection or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Use only AC 100-240 V, 50/60 Hz. Connect to a protected, correctly grounded Class I supply.

Do not operate with a damaged housing, lens, connector, cable, mounting point or safety attachment.

After a large temperature change, keep the fixture disconnected until it reaches ambient temperature and condensation has evaporated.

Do not look directly into the LEDs at close range. Flashing light may affect susceptible people.

Keep combustible material at least 0.5 m away and keep every ventilation opening unobstructed.

For overhead installation, use rated clamps and an independently rated secondary safety cable.

Carry the fixture by its fixed handles or base. Never lift it by the moving head.

Do not modify the fixture or bypass protective devices. Refer internal repair to qualified service personnel.

IP20 / PROTECTED INDOOR USE  The source specifies IP20. Keep the fixture away from rain, splashing water, condensation and excessive dust.

2  Product Overview and Specifications

The AL1019WR combines nineteen individually controlled 15 W RGBW LEDs with a 4°-60° motorized zoom and a continuously rotating honeycomb lens. It produces narrow beams, broad washes, color mixing and animated kaleidoscopic pixel effects for stages, theatres, clubs, broadcast studios and rental productions.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Rated power | 450 W |
| Light source | 19 x 15 W high-power RGBW 4-in-1 LEDs; individual LED control |
| Optics | Motorized 4°-60° zoom; continuously rotating honeycomb lens |
| Dimming / strobe | 0-100% linear dimming; 0-20 Hz electronic strobe |
| Control | DMX512 / Auto / Sound-active / Master-slave |
| DMX personalities | 21CH / 23CH / 35CH / 78CH / 92CH / 97CH / 99CH |
| Pan / tilt | 540° pan and 270° tilt; 16-bit fine control |
| Display | Color LCD; English / Chinese; automatic screen inversion |
| Cooling / protection | Temperature-controlled fan; temperature sensing; position memory and correction |
| Ingress protection | IP20 |
| Fixture dimensions | 415 x 275 x 425 mm |
| Carton dimensions | 530 x 420 x 510 mm; 1 fixture per carton |
| Net / gross weight | 14.95 kg / 18.1 kg |
| Options | Rechargeable display battery; Art-Net |

2.2 Main Functions

RGBW color mixing with linear color-temperature and macro controls.

Individual LED pixel control and built-in static/dynamic effects.

Forward or reverse continuous lens rotation for kaleidoscopic effects.

Beam, wash and effect-light operation from one fixture.

Smooth 16-bit pan/tilt, position memory and automatic correction.

Seven DMX personalities for compact, pattern and full pixel programming.

2.3 Supplied Items

Package contents: AL1019WR fixture x 1; DMX signal cable x 1; hanging brackets x 2; power cable x 1; safety cable x 1; user manual x 1.

3  Setup and Installation

3.1 AC Power

Before connecting power, verify that the product-label voltage and frequency match the local supply. Complete mounting and data connections before applying power. Disconnect by holding the plug, not by pulling the cable.

3.2 Rigging

STRUCTURAL RATING  The support, mounting hardware and every attachment must be rated for at least ten times the fixture load or the higher factor required by local regulations.

Inspect the fixture, Omega brackets, rated clamps, M10 fasteners, mounting points, safety cable and support structure.

Attach rated clamps to the brackets and lock the brackets securely into the base.

Attach an independently rated safety cable to the designated point and the supporting structure.

Confirm unobstructed clearance for complete pan, tilt, zoom and lens-rotation movement.

Recheck every attachment before applying power and inspect the installation regularly.

3.3 DMX512 Connection

Use shielded twisted-pair cable designed for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Assign a unique start address to every independently controlled fixture.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the final fixture in a long or electrically noisy chain, connect a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use MENU to move back through the menu, UP/DOWN to change an item or value, and ENTER to open or save it. Menu wording may vary slightly with firmware revision.

| Menu | Item | Option / Range | Operation |
| --- | --- | --- | --- |
| Address | DMX Address | 001-512 | Set the fixture start address. |
| Run Mode | Operating Mode | DMX / Sound / Auto 1 / Auto 2 | Select controlled or standalone operation. |
| System Setup | Channel Mode | 21 / 23 / 35 / 78 / 92 / 97 / 99CH | Select the controller footprint. |
| System Setup | Invert Pan / Tilt | On / Off | Reverse pan or tilt direction. |
| System Setup | Hall Correction | On / Off | Enable or disable position-sensor correction. |
| System Setup | Encoder Correction | On / Off | Enable or disable encoder correction. |
| System Setup | Signal Keep | On / Off | Hold the last state when DMX is lost. |
| System Setup | Screen Saver | On / Off | Enable or disable the screen saver. |
| System Setup | Invert Screen | Auto / On / Off | Set display orientation. |
| System Setup | Update Slave | On / Off | Firmware/service function. |
| System Setup | Language | EN / Chinese | Select the display language. |
| System Setup | Load Default | Confirm | Restore factory settings. |
| Manual | Channel Control | Per channel | Control each channel manually. |
| Motor Calibration | Pan / Tilt / Zoom / Rotation / RGBW balance / Power / MIC | Calibration values | Service calibration; change only when necessary. |
| System Information | Note | Status information | View fixture information. |
| Reset | Effect / Scan / All | Confirm | Reset a subsystem or the complete fixture. |

CALIBRATION CAUTION  Do not change motor, color-balance, power or microphone calibration data unless instructed by qualified service personnel.

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. DMX values run from 000 to 255 unless a narrower range is shown. Fine channels provide 16-bit resolution when paired with their coarse channel.

5.1  21-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | 0-100% |
| 2 | Red Fine | 000-255 | 16-bit fine |
| 3 | Green | 000-255 | 0-100% |
| 4 | Green Fine | 000-255 | 16-bit fine |
| 5 | Blue | 000-255 | 0-100% |
| 6 | Blue Fine | 000-255 | 16-bit fine |
| 7 | White | 000-255 | 0-100% |
| 8 | White Fine | 000-255 | 16-bit fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized slow-fast / random |
| 12 | Master Dimmer | 000-255 | 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit fine |
| 14 | Pan | 000-255 | Position |
| 15 | Pan Fine | 000-255 | 16-bit fine |
| 16 | Tilt | 000-255 | Position |
| 17 | Tilt Fine | 000-255 | 16-bit fine |
| 18 | Reserved / Function | 000-255 | No source-defined action |
| 19 | Reset | 000-250 / 251-255 | No function / reset; hold 5 seconds |
| 20 | Zoom | 000-255 | 4° to 60° |
| 21 | Lens Rotation | 000-127 / 128-191 / 192-255 | 0°-60° position / CW fast-slow / CCW fast-slow |

5.2  23-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | Position |
| 2 | Pan Fine | 000-255 | 16-bit fine |
| 3 | Tilt | 000-255 | Position |
| 4 | Tilt Fine | 000-255 | 16-bit fine |
| 5 | Pan / Tilt Speed | 000-255 | Fast to slow |
| 6 | Zoom | 000-255 | 4° to 60° |
| 7 | Lens Rotation | 000-127 / 128-191 / 192-255 | 0°-60° position / CW fast-slow / CCW fast-slow |
| 8 | Master Dimmer | 000-255 | 0-100% |
| 9 | Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized slow-fast / random |
| 10 | Red | 000-255 | 0-100% |
| 11 | Green | 000-255 | 0-100% |
| 12 | Blue | 000-255 | 0-100% |
| 13 | White | 000-255 | 0-100% |
| 14 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 15 | Color Macro | 000-255 | Built-in color macro |
| 16 | Static Effect | 000-255 | Each 5-value block selects one static effect |
| 17 | Dynamic Effect | 000-255 | Each 5-value block selects one dynamic effect |
| 18 | Dynamic Speed | 000-255 | Slow to fast |
| 19 | Background Red | 000-255 | 0-100% |
| 20 | Background Green | 000-255 | 0-100% |
| 21 | Background Blue | 000-255 | 0-100% |
| 22 | Background White | 000-255 | 0-100% |
| 23 | Reset | 000-250 / 251-255 | No function / reset; hold 5 seconds |

5.3  35-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | 0-100% |
| 2 | Red Fine | 000-255 | 16-bit fine |
| 3 | Green | 000-255 | 0-100% |
| 4 | Green Fine | 000-255 | 16-bit fine |
| 5 | Blue | 000-255 | 0-100% |
| 6 | Blue Fine | 000-255 | 16-bit fine |
| 7 | White | 000-255 | 0-100% |
| 8 | White Fine | 000-255 | 16-bit fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized slow-fast / random |
| 12 | Master Dimmer | 000-255 | 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit fine |
| 14 | Pan | 000-255 | Position |
| 15 | Pan Fine | 000-255 | 16-bit fine |
| 16 | Tilt | 000-255 | Position |
| 17 | Tilt Fine | 000-255 | 16-bit fine |
| 18 | Reserved / Function | 000-255 | No source-defined action |
| 19 | Reset | 000-250 / 251-255 | No function / reset; hold 5 seconds |
| 20 | Zoom | 000-255 | 4° to 60° |
| 21 | Lens Rotation | 000-127 / 128-191 / 192-255 | 0°-60° position / CW fast-slow / CCW fast-slow |
| 22 | Shape Selection | 000-255 | Select built-in shape |
| 23 | Shape Speed | 000-255 | Shape speed |
| 24 | Shape Fade | 000-255 | Shape fade |
| 25 | Shape Red | 000-255 | 0-100% |
| 26 | Shape Green | 000-255 | 0-100% |
| 27 | Shape Blue | 000-255 | 0-100% |
| 28 | Shape White | 000-255 | 0-100% |
| 29 | Shape Dimmer | 000-255 | 0-100% |
| 30 | Background Dimmer | 000-255 | 0-100% |
| 31 | Shape Transition | 000-255 | Transition control |
| 32 | Shape Offset | 000-255 | Offset control |
| 33 | Foreground Strobe | 000-255 | Foreground strobe control |
| 34 | Background Strobe | 000-255 | Background strobe control |
| 35 | Background Selection | 000-255 | Background selection |

5.4  Pixel Personalities

The expanded personalities retain a complete base personality and then assign direct channels to each of the nineteen LED pixels. LEDs are numbered 1-19 in the fixture's internal order.

| Mode | Base channels | Pixel channels | Pixel components |
| --- | --- | --- | --- |
| 78CH | 1-21 = 21CH | 22-78 | 19 pixels x RGB |
| 92CH | 1-35 = 35CH | 36-92 | 19 pixels x RGB |
| 97CH | 1-21 = 21CH | 22-97 | 19 pixels x RGBW |
| 99CH | 1-23 = 23CH | 24-99 | 19 pixels x RGBW |

5.4.1  78CH Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-24 | LED 1 | 000-255 each | R / G / B; 0-100% per component |
| 25-27 | LED 2 | 000-255 each | R / G / B; 0-100% per component |
| 28-30 | LED 3 | 000-255 each | R / G / B; 0-100% per component |
| 31-33 | LED 4 | 000-255 each | R / G / B; 0-100% per component |
| 34-36 | LED 5 | 000-255 each | R / G / B; 0-100% per component |
| 37-39 | LED 6 | 000-255 each | R / G / B; 0-100% per component |
| 40-42 | LED 7 | 000-255 each | R / G / B; 0-100% per component |
| 43-45 | LED 8 | 000-255 each | R / G / B; 0-100% per component |
| 46-48 | LED 9 | 000-255 each | R / G / B; 0-100% per component |
| 49-51 | LED 10 | 000-255 each | R / G / B; 0-100% per component |
| 52-54 | LED 11 | 000-255 each | R / G / B; 0-100% per component |
| 55-57 | LED 12 | 000-255 each | R / G / B; 0-100% per component |
| 58-60 | LED 13 | 000-255 each | R / G / B; 0-100% per component |
| 61-63 | LED 14 | 000-255 each | R / G / B; 0-100% per component |
| 64-66 | LED 15 | 000-255 each | R / G / B; 0-100% per component |
| 67-69 | LED 16 | 000-255 each | R / G / B; 0-100% per component |
| 70-72 | LED 17 | 000-255 each | R / G / B; 0-100% per component |
| 73-75 | LED 18 | 000-255 each | R / G / B; 0-100% per component |
| 76-78 | LED 19 | 000-255 each | R / G / B; 0-100% per component |

5.4.2  92CH Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 36-38 | LED 1 | 000-255 each | R / G / B; 0-100% per component |
| 39-41 | LED 2 | 000-255 each | R / G / B; 0-100% per component |
| 42-44 | LED 3 | 000-255 each | R / G / B; 0-100% per component |
| 45-47 | LED 4 | 000-255 each | R / G / B; 0-100% per component |
| 48-50 | LED 5 | 000-255 each | R / G / B; 0-100% per component |
| 51-53 | LED 6 | 000-255 each | R / G / B; 0-100% per component |
| 54-56 | LED 7 | 000-255 each | R / G / B; 0-100% per component |
| 57-59 | LED 8 | 000-255 each | R / G / B; 0-100% per component |
| 60-62 | LED 9 | 000-255 each | R / G / B; 0-100% per component |
| 63-65 | LED 10 | 000-255 each | R / G / B; 0-100% per component |
| 66-68 | LED 11 | 000-255 each | R / G / B; 0-100% per component |
| 69-71 | LED 12 | 000-255 each | R / G / B; 0-100% per component |
| 72-74 | LED 13 | 000-255 each | R / G / B; 0-100% per component |
| 75-77 | LED 14 | 000-255 each | R / G / B; 0-100% per component |
| 78-80 | LED 15 | 000-255 each | R / G / B; 0-100% per component |
| 81-83 | LED 16 | 000-255 each | R / G / B; 0-100% per component |
| 84-86 | LED 17 | 000-255 each | R / G / B; 0-100% per component |
| 87-89 | LED 18 | 000-255 each | R / G / B; 0-100% per component |
| 90-92 | LED 19 | 000-255 each | R / G / B; 0-100% per component |

5.4.3  97CH Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-25 | LED 1 | 000-255 each | R / G / B / W; 0-100% per component |
| 26-29 | LED 2 | 000-255 each | R / G / B / W; 0-100% per component |
| 30-33 | LED 3 | 000-255 each | R / G / B / W; 0-100% per component |
| 34-37 | LED 4 | 000-255 each | R / G / B / W; 0-100% per component |
| 38-41 | LED 5 | 000-255 each | R / G / B / W; 0-100% per component |
| 42-45 | LED 6 | 000-255 each | R / G / B / W; 0-100% per component |
| 46-49 | LED 7 | 000-255 each | R / G / B / W; 0-100% per component |
| 50-53 | LED 8 | 000-255 each | R / G / B / W; 0-100% per component |
| 54-57 | LED 9 | 000-255 each | R / G / B / W; 0-100% per component |
| 58-61 | LED 10 | 000-255 each | R / G / B / W; 0-100% per component |
| 62-65 | LED 11 | 000-255 each | R / G / B / W; 0-100% per component |
| 66-69 | LED 12 | 000-255 each | R / G / B / W; 0-100% per component |
| 70-73 | LED 13 | 000-255 each | R / G / B / W; 0-100% per component |
| 74-77 | LED 14 | 000-255 each | R / G / B / W; 0-100% per component |
| 78-81 | LED 15 | 000-255 each | R / G / B / W; 0-100% per component |
| 82-85 | LED 16 | 000-255 each | R / G / B / W; 0-100% per component |
| 86-89 | LED 17 | 000-255 each | R / G / B / W; 0-100% per component |
| 90-93 | LED 18 | 000-255 each | R / G / B / W; 0-100% per component |
| 94-97 | LED 19 | 000-255 each | R / G / B / W; 0-100% per component |

5.4.4  99CH Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 24-27 | LED 1 | 000-255 each | R / G / B / W; 0-100% per component |
| 28-31 | LED 2 | 000-255 each | R / G / B / W; 0-100% per component |
| 32-35 | LED 3 | 000-255 each | R / G / B / W; 0-100% per component |
| 36-39 | LED 4 | 000-255 each | R / G / B / W; 0-100% per component |
| 40-43 | LED 5 | 000-255 each | R / G / B / W; 0-100% per component |
| 44-47 | LED 6 | 000-255 each | R / G / B / W; 0-100% per component |
| 48-51 | LED 7 | 000-255 each | R / G / B / W; 0-100% per component |
| 52-55 | LED 8 | 000-255 each | R / G / B / W; 0-100% per component |
| 56-59 | LED 9 | 000-255 each | R / G / B / W; 0-100% per component |
| 60-63 | LED 10 | 000-255 each | R / G / B / W; 0-100% per component |
| 64-67 | LED 11 | 000-255 each | R / G / B / W; 0-100% per component |
| 68-71 | LED 12 | 000-255 each | R / G / B / W; 0-100% per component |
| 72-75 | LED 13 | 000-255 each | R / G / B / W; 0-100% per component |
| 76-79 | LED 14 | 000-255 each | R / G / B / W; 0-100% per component |
| 80-83 | LED 15 | 000-255 each | R / G / B / W; 0-100% per component |
| 84-87 | LED 16 | 000-255 each | R / G / B / W; 0-100% per component |
| 88-91 | LED 17 | 000-255 each | R / G / B / W; 0-100% per component |
| 92-95 | LED 18 | 000-255 each | R / G / B / W; 0-100% per component |
| 96-99 | LED 19 | 000-255 each | R / G / B / W; 0-100% per component |

6  Photometric Data

Source illuminance measurements at the 4° beam angle:

| Color | 2.5 m | 5 m | 7.5 m |
| --- | --- | --- | --- |
| Red | 83,700 lx | 2,550 lx | 1,150 lx |
| Green | 13,800 lx | 3,800 lx | 1,780 lx |
| Blue | 15,300 lx | 4,460 lx | 1,970 lx |
| White | 18,000 lx | 5,300 lx | 2,320 lx |
| RGBW | 50,400 lx | 15,500 lx | 6,680 lx |

MEASUREMENT NOTE  Photometric values are source measurements and may vary with LED binning, calibration, mains conditions, optical cleanliness and measurement method.

7  Routine Maintenance

DISCONNECT POWER  Allow the fixture to cool before inspection or cleaning. There are no user-serviceable parts inside.

Inspect mounting screws, clamps, brackets, safety cable, housing, lenses and installation points for corrosion, wear, deformation or looseness.

Inspect moving parts for unusual wear, imbalance, noise or restricted movement.

Inspect power and data cables for damage, fatigue, overheating or contamination.

Clean internal and external optical surfaces regularly with a soft, lint-free cloth. Never use alcohol or solvents.

Keep fans, air inlets and outlets clean. Cleaning frequency must suit the dust level and operating environment.

Have the electrical system and internal connections inspected periodically by a qualified technician.

8  Troubleshooting and Service

| Symptom | Checks / corrective action |
| --- | --- |
| No power or display | Verify the supply, breaker, cable, connector and product-label voltage. Disconnect power before inspecting. |
| No DMX response | Confirm DMX mode and start address, cable pinout, controller output and chain termination. |
| Unexpected movement | Check pan/tilt inversion, free movement, Hall/encoder correction and reset the fixture. |
| Low or uneven output | Check dimmer/color channels, lens cleanliness, temperature status and airflow. |
| Fixture stops when hot | Disconnect power, clear airflow, clean the fans and allow the fixture to cool. Refer persistent temperature faults to service. |
| Lens rotation or zoom fault | Remove obstructions, reset the fixture and check calibration only if qualified to do so. |

SERVICE  Use genuine replacement parts. Refer internal repair, calibration and persistent faults to the dealer or a qualified service technician.