PROFESSIONAL LED EFFECT FIXTURE

AL120T

120W Tornado Moving Head

SINGLE-HEAD TORNADO EFFECT LIGHT

USER MANUAL

English Edition  |  Translated from the supplied Chinese manual

120 W RGBL main LED • 24-zone RGB halo • Zoom, beam and wash effects

| Ingress Protection | Rated Power | Input | DMX Modes |
| --- | --- | --- | --- |
| IP20 | 142 W | DC 36 V | 22 / 24 / 94 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Retain it for future reference.

Contents

1  Safety, Maintenance & Product Information

2  Display Panel & Menu Operation

3  Master/Slave Control

4  DMX512 Control & Channel Charts

5  Troubleshooting

TRANSLATION NOTE  This English edition reorganizes the supplied Chinese source for clarity while preserving its specifications, menu defaults, DMX values, and safety instructions.

1  Safety, Maintenance & Product Information

1.1 Statement

Thank you for choosing this product. It left the factory in good condition and with complete packaging. Read this manual fully before use. It contains important installation and operating information; follow all instructions and retain the manual for future reference.

Technical changes may be made without prior notice.

1.2 Maintenance & Safety

This fixture is intended for professional use only.

If the external flexible cable is damaged, replace it only with the dedicated cable supplied by the manufacturer or its service agent.

The light source may be replaced only by the manufacturer, its service agent, or similarly qualified personnel.

Connect the product to a protected circuit fitted with a circuit breaker or fuse and ensure proper protective grounding to reduce the risk of electric shock or fire.

Disconnect power completely at the breaker or plug when the fixture is not in use.

The enclosure may become hot during operation. Install the fixture in a well-ventilated location.

After moving the product between extreme temperatures, allow it to acclimatize before applying power so internal condensation cannot cause a malfunction.

Strobe lighting may trigger epileptic seizures. Comply with all local regulations and warning requirements for strobe use.

1.3 Product Precautions

Disconnect power before cleaning the product or replacing a fuse.

When ports are not in use, reinstall and secure the protective covers for power, data, USB, and other connections so the specified IP protection is maintained.

Use a rated safety cable for overhead installation.

Connect the product only to a grounded and protected circuit.

Do not look directly into the light source while the product is operating.

Keep flammable materials at least 20 cm from the product during operation and while power is connected.

Do not operate the product if the enclosure, lens, or cable is damaged.

Carry the product only by its handle or rigging/mounting bracket.

Stop using the product immediately if a serious operating problem occurs.

1.4 Product Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | DC 36 V |
| Rated power | 142 W |
| Light sources | 1 × 120 W RGBL (red, green, blue, lime) 1212 LED; 24 × RGB 5050 auxiliary LEDs |
| Control modes | DMX512, standalone/auto, master/slave, sound-active, RDM |
| DMX modes | 22 CH / 24 CH / 94 CH |
| Dimming | 32-bit, 0–100% linear dimming |
| Effects | Single moving head, wash, beam |
| Operating temperature | −30 °C to 50 °C |
| Strobe frequency | 1–30 Hz |
| Housing | Black metal |
| Connections | DMX512 input/output; power input/output |
| Ingress protection | IP20 — indoor use only |
| Fixture dimensions | 200 × 184 × 328 mm |
| Fixture weight | 5.1 kg |

Source dimension drawing; all dimensions are in millimetres.

2  Display Panel & Menu Operation

2.1 Control Panel

Control panel. The source image shows the Chinese user interface; key functions are translated below.

| Control | Function |
| --- | --- |
| MENU | Open the main menu or return to the previous menu. |
| UP | Move upward through menu items or increase a parameter value. |
| DOWN | Move downward through menu items or decrease a parameter value. |
| ENTER | Select a menu function; confirm and save a modified parameter. |

2.2 Menu Operation

After power-on and the automatic reset, press MENU to open the main menu. The main menu exits after 30 seconds without input. Use UP/DOWN to select an item, press ENTER to edit it, then press ENTER again to save. Saved values are retained after power loss.

| Menu | Option | Setting / Default |
| --- | --- | --- |
| System Settings | DMX Address | 001–512; default 001. |
| System Settings | Encoder | On / Off; default On. |
| System Settings | Pan Invert | On / Off; default Off. |
| System Settings | Tilt Invert | On / Off; default Off. |
| System Settings | Zoom Invert | On / Off; default Off. |
| System Settings | No Signal | Blackout / Hold; default Blackout. |
| System Settings | Reset | Yes / No; default No. |
| System Settings | Factory Reset | Yes / No; default No. |
| System Settings | Advanced Settings | Open the advanced-settings submenu. |
| Operating Mode | Run Mode | DMX / Sound-active / Auto; default DMX. |
| Operating Mode | Channel Mode | 22 CH / 24 CH / 94 CH; default 24 CH. |

2.3 Manual Control

| Item | Range | Default |
| --- | --- | --- |
| Pan | 000–255 | 000 |
| Pan Fine | 000–255 | 000 |
| Tilt | 000–255 | 000 |
| Tilt Fine | 000–255 | 000 |
| Zoom | 000–255 | 000 |
| Pan/Tilt Speed | 000–255 | 000 |
| Dimmer | 000–255 | 000 |
| Dimmer Fine | 000–255 | 000 |
| Main Strobe | 000–255 | 000 |
| Color Temperature | 000–255 | 000 |
| Color Macro | 000–255 | 000 |
| Red | 000–255 | 000 |
| Green | 000–255 | 000 |
| Blue | 000–255 | 000 |
| Lime | 000–255 | 000 |
| Halo Strobe | 000–255 | 000 |
| Halo Red | 000–255 | 000 |
| Halo Green | 000–255 | 000 |
| Halo Blue | 000–255 | 000 |
| Halo Mode | 000–255 | 000 |
| Halo Mode Speed | 000–255 | 000 |
| Halo Background Color | 000–255 | 000 |
| Halo Background Dimmer | 000–255 | 000 |
| Function | 000–255 | 000 |

2.4 Display & Information

| Menu | Option | Setting / Function |
| --- | --- | --- |
| Display Settings | Language | Chinese / English; default Chinese. |
| Display Settings | Display Reverse | Off / On; default Off. |
| Display Settings | Screen Saver | Off / On; default On. |
| Display Settings | Software Version | Program identification code D1TF; read-only. |
| Information | Sensors | View Hall-sensor and temperature-communication status. |
| Information | Error Information | View stored error information. |
| Information | Channel Information | View live DMX channel data. |
| Information | Time Information | View power-on time and total fixture operating time. |

2.5 Factory Settings

ACCESS  When prompted for the factory-settings password, change the value to 138 and press ENTER.

| Item | Range / Options | Default |
| --- | --- | --- |
| Pan Calibration | 000–255 | 128 |
| Tilt Calibration | 000–255 | 128 |
| Focus/Zoom Axis Calibration | 000–255 | 128 |
| Pan Travel | 000–255 | 128 |
| Tilt Travel | 000–255 | 128 |
| Focus/Zoom Travel | 000–255 | 128 |
| Red | 032–255 | 252 |
| Green | 032–255 | 252 |
| Blue | 032–255 | 252 |
| Lime | 032–255 | 252 |
| Current | 032–255 | 240 |
| Temperature | 040–070 | 065 |
| Temperature / Fan | Auto / Standard / Quiet | Auto |
| LED Selection | LEDA / LEDB / LEDC / LEDD | LEDA |
| LED PWM Frequency | 900, 1000, 1100, 1200, 1300, 1400, 1500, 2500, 4000, 5000, 6000, 10000, 15000, 20000, 25000 Hz | 4000 Hz |
| Dimmer Curve | Linear / Square / Inverse Square / S-Curve | Linear |
| Dimmer Mode | Fast / Smooth | Fast |
| Logo | 000 / 001 | 000 |
| Clear Time | Yes / No | No |

3  Master/Slave Control

Connect two or more identical fixtures with a 3-pin DMX cable. Set each fixture to any address from 001–512. Configure one fixture as the master and all others as slaves. The slave displays should remain steady. When gradient, pulse, jump, sound-active, or auto effects are selected on the master, all slave fixtures run the same effect in synchronization.

IMPORTANT  Use only one master in each fixture group. Multiple masters will cause unsynchronized flashing. Master/slave control operates only when the DMX512 console is disconnected or switched off.

4  DMX512 Control

4.1 DMX Address

Set the address of every fixture, then connect the fixtures to the DMX512 console using 3-pin DMX cables. When the address display stops flashing, the fixture is receiving a DMX signal. Use the channel charts below to control each function.

OUTPUT NOTE  To produce light output, set the applicable strobe channel to its full-open value.

4.2  22 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000–255 | Pan |
| 2 | 000–255 | Pan Fine |
| 3 | 000–255 | Tilt |
| 4 | 000–255 | Tilt Fine |
| 5 | 000–255 | Zoom |
| 6 | 000–255 | Main Light Red |
| 7 | 000–255 | Main Light Green |
| 8 | 000–255 | Main Light Blue |
| 9 | 000–255 | Main Light Lime |
| 10 | 000–255 | Main Light Color Temperature |
| 11 | 000–255 | Main Light Color Macro |
| 12 | 000–255 | Halo Red |
| 13 | 000–255 | Halo Green |
| 14 | 000–255 | Halo Blue |
| 15 | 000–255 | Halo Mode |
| 16 | 000–255 | Halo Mode Speed |
| 17 | 000–255 | Master Dimmer |
| 18 | 000–255 | Master Dimmer Fine |
| 19 | 000–255 | Main Light Strobe |
| 20 | 000–255 | Halo Strobe |
| 21 | 000–255 | Pan/Tilt Speed |
| 22 | 000–255 | Function |

4.3  24 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000–255 | Pan |
| 2 | 000–255 | Pan Fine |
| 3 | 000–255 | Tilt |
| 4 | 000–255 | Tilt Fine |
| 5 | 000–255 | Zoom |
| 6 | 000–255 | Pan/Tilt Speed |
| 7 | 000–255 | Master Dimmer |
| 8 | 000–255 | Master Dimmer Fine |
| 9 | 000–255 | Main Light Strobe |
| 10 | 000–255 | Main Light Color Temperature |
| 11 | 000–255 | Main Light Color Macro |
| 12 | 000–255 | Main Light Red |
| 13 | 000–255 | Main Light Green |
| 14 | 000–255 | Main Light Blue |
| 15 | 000–255 | Main Light Lime |
| 16 | 000–255 | Halo Strobe |
| 17 | 000–255 | Halo Red |
| 18 | 000–255 | Halo Green |
| 19 | 000–255 | Halo Blue |
| 20 | 000–255 | Halo Mode |
| 21 | 000–255 | Halo Mode Speed |
| 22 | 000–255 | Halo Background Color |
| 23 | 000–255 | Halo Background Dimmer |
| 24 | 000–255 | Function |

4.4  94 CH Mode

EXPANDED SEQUENCE  The source chart shows channels 22–24 for halo zone 1 and channels 91–93 for zone 24 with an arrow between them. The repeating RGB sequence for zones 2–23 is expanded below for easier programming.

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000–255 | Pan |
| 2 | 000–255 | Pan Fine |
| 3 | 000–255 | Tilt |
| 4 | 000–255 | Tilt Fine |
| 5 | 000–255 | Zoom |
| 6 | 000–255 | Pan/Tilt Speed |
| 7 | 000–255 | Main Light Dimmer |
| 8 | 000–255 | Main Light Dimmer Fine |
| 9 | 000–255 | Main Light Strobe |
| 10 | 000–255 | Main Light Color Temperature |
| 11 | 000–255 | Main Light Color Macro |
| 12 | 000–255 | Main Light Red |
| 13 | 000–255 | Main Light Green |
| 14 | 000–255 | Main Light Blue |
| 15 | 000–255 | Main Light Lime |
| 16 | 000–255 | Halo Dimmer |
| 17 | 000–255 | Halo Strobe |
| 18 | 000–255 | Halo Mode |
| 19 | 000–255 | Halo Mode Speed |
| 20 | 000–255 | Halo Background Color |
| 21 | 000–255 | Halo Background Dimmer |
| 22 | 000–255 | Halo Red 1 |
| 23 | 000–255 | Halo Green 1 |
| 24 | 000–255 | Halo Blue 1 |
| 25 | 000–255 | Halo Red 2 |
| 26 | 000–255 | Halo Green 2 |
| 27 | 000–255 | Halo Blue 2 |
| 28 | 000–255 | Halo Red 3 |
| 29 | 000–255 | Halo Green 3 |
| 30 | 000–255 | Halo Blue 3 |
| 31 | 000–255 | Halo Red 4 |
| 32 | 000–255 | Halo Green 4 |
| 33 | 000–255 | Halo Blue 4 |
| 34 | 000–255 | Halo Red 5 |
| 35 | 000–255 | Halo Green 5 |
| 36 | 000–255 | Halo Blue 5 |
| 37 | 000–255 | Halo Red 6 |
| 38 | 000–255 | Halo Green 6 |
| 39 | 000–255 | Halo Blue 6 |
| 40 | 000–255 | Halo Red 7 |
| 41 | 000–255 | Halo Green 7 |
| 42 | 000–255 | Halo Blue 7 |
| 43 | 000–255 | Halo Red 8 |
| 44 | 000–255 | Halo Green 8 |
| 45 | 000–255 | Halo Blue 8 |
| 46 | 000–255 | Halo Red 9 |
| 47 | 000–255 | Halo Green 9 |
| 48 | 000–255 | Halo Blue 9 |
| 49 | 000–255 | Halo Red 10 |
| 50 | 000–255 | Halo Green 10 |
| 51 | 000–255 | Halo Blue 10 |
| 52 | 000–255 | Halo Red 11 |
| 53 | 000–255 | Halo Green 11 |
| 54 | 000–255 | Halo Blue 11 |
| 55 | 000–255 | Halo Red 12 |
| 56 | 000–255 | Halo Green 12 |
| 57 | 000–255 | Halo Blue 12 |
| 58 | 000–255 | Halo Red 13 |
| 59 | 000–255 | Halo Green 13 |
| 60 | 000–255 | Halo Blue 13 |
| 61 | 000–255 | Halo Red 14 |
| 62 | 000–255 | Halo Green 14 |
| 63 | 000–255 | Halo Blue 14 |
| 64 | 000–255 | Halo Red 15 |
| 65 | 000–255 | Halo Green 15 |
| 66 | 000–255 | Halo Blue 15 |
| 67 | 000–255 | Halo Red 16 |
| 68 | 000–255 | Halo Green 16 |

| Channel | DMX Value | Function |
| --- | --- | --- |
| 69 | 000–255 | Halo Blue 16 |
| 70 | 000–255 | Halo Red 17 |
| 71 | 000–255 | Halo Green 17 |
| 72 | 000–255 | Halo Blue 17 |
| 73 | 000–255 | Halo Red 18 |
| 74 | 000–255 | Halo Green 18 |
| 75 | 000–255 | Halo Blue 18 |
| 76 | 000–255 | Halo Red 19 |
| 77 | 000–255 | Halo Green 19 |
| 78 | 000–255 | Halo Blue 19 |
| 79 | 000–255 | Halo Red 20 |
| 80 | 000–255 | Halo Green 20 |
| 81 | 000–255 | Halo Blue 20 |
| 82 | 000–255 | Halo Red 21 |
| 83 | 000–255 | Halo Green 21 |
| 84 | 000–255 | Halo Blue 21 |
| 85 | 000–255 | Halo Red 22 |
| 86 | 000–255 | Halo Green 22 |
| 87 | 000–255 | Halo Blue 22 |
| 88 | 000–255 | Halo Red 23 |
| 89 | 000–255 | Halo Green 23 |
| 90 | 000–255 | Halo Blue 23 |
| 91 | 000–255 | Halo Red 24 |
| 92 | 000–255 | Halo Green 24 |
| 93 | 000–255 | Halo Blue 24 |
| 94 | 000–255 | Function |

4.5  Function Channel Values

| Index | DMX Value | Function |
| --- | --- | --- |
| 1 | 000–005 | No function |
| 2 | 006–007 | No function |
| 3 | 008–009 | No function |
| 4 | 010–029 | No function |
| 5 | 030–039 | Dimmer curve: Linear |
| 6 | 040–049 | Dimmer curve: Square |
| 7 | 050–059 | Dimmer curve: Inverse Square |
| 8 | 060–069 | Dimmer curve: S-Curve |
| 9 | 070–079 | Fan mode: Auto |
| 10 | 080–089 | Fan mode: Standard |
| 11 | 090–099 | Fan mode: Quiet |
| 12 | 100–109 | No function |
| 13 | 110–119 | No function |
| 14 | 120–122 | No function |
| 15 | 123 | PWM frequency: 900 Hz |
| 16 | 124 | PWM frequency: 1000 Hz |
| 17 | 125 | PWM frequency: 1100 Hz |
| 18 | 126 | PWM frequency: 1200 Hz |
| 19 | 127 | PWM frequency: 1300 Hz |
| 20 | 128 | PWM frequency: 1400 Hz |
| 21 | 129 | PWM frequency: 1500 Hz |
| 22 | 130 | PWM frequency: 2500 Hz |
| 23 | 131 | PWM frequency: 4000 Hz |
| 24 | 132 | PWM frequency: 5000 Hz |
| 25 | 133 | PWM frequency: 6000 Hz |
| 26 | 134 | PWM frequency: 10 kHz |
| 27 | 135 | PWM frequency: 15 kHz |
| 28 | 136 | PWM frequency: 20 kHz |
| 29 | 137 | PWM frequency: 25 kHz |
| 30 | 138–139 | No function |
| 31 | 140–149 | Pan/Tilt reset |
| 32 | 150–159 | Head reset |
| 33 | 160–169 | No function |
| 34 | 170–179 | No function |
| 35 | 180–199 | No function |
| 36 | 200–209 | Reset all (MA fixture-library reset value: 80–82) |
| 37 | 210–219 | Dimmer speed: Fast |
| 38 | 220–229 | Dimmer speed: Smooth |
| 39 | 230–255 | No function |

5  Troubleshooting

The following remedies cover common faults. Disconnect power before maintenance. Any problem that cannot be resolved must be handled by qualified service personnel.

5.1 Light Source Does Not Illuminate

Verify that the supplied voltage matches the fixture rating.

Check the power connection and control switch for poor contact.

Confirm that the power supply has sufficient capacity.

Confirm that the DMX512 controller is transmitting the required command.

5.2 Fixture Resets but Does Not Respond to the Console

Verify the DMX start address and selected operating options.

Check the control-cable connections and look for an open circuit or excessive cable length.

Check the controller and any signal amplifier connected in series.

Reduce cable length, separate high- and low-voltage wiring, and improve cable routing.

Use a high-quality shielded twisted-pair data cable and add a signal amplifier when required.

Install a 120 Ω terminator at the final fixture.

5.3 Fixture Does Not Start

Verify that the power-supply parameters match the fixture.

Inspect for shipping deformation, vibration damage, moisture, loose parts, or disconnected internal components.

Check internal wiring and connectors for looseness or disconnection.

Have qualified personnel inspect electronic components, including the transformer, PCB, and motor-control board, for looseness, short circuits, or burn damage.

5.4 Abnormal Pan or Tilt Movement

Carry out the checks listed above.

Inspect the corresponding pan and tilt drive belts for detachment or breakage.

Inspect the pan/tilt position-feedback Hall sensors for damage.

Restart the fixture and allow it to reset again.

END OF MANUAL  Retain this manual with the fixture. Before operation, verify the product label, local electrical code, rigging requirements, and venue safety procedures.