AL1650WX

16 x 50 W LED Wash
Moving Head IP65

RGBL PIXELS - RGB HALO - 4° TO 60° ZOOM - USER MANUAL

Outdoor wash and effect fixture with independent pixel control

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Data Connections |
| 4 | Control Panel and Menu |
| 5 | DMX Channel Reference |
| 6 | Routine Maintenance |
| 7 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition is organized for model AL1650WX, 16 x 50 W LED Wash Moving Head IP65. Confirm the model, voltage rating and firmware shown on the actual fixture before installation or programming.

This edition uses the user-supplied AL1650WX model designation consistently throughout the manual.

1  Safety Information

DANGER  Disconnect AC power and wait at least 15 minutes for the fixture to cool before opening, cleaning, inspection or service. Do not open the unit within five minutes after switching it off.

Use the fixture only with the voltage and frequency marked on its product label. Connect the yellow/green protective conductor to earth.

Keep at least 0.5 m clearance from adjacent surfaces and combustible materials. Do not block ventilation openings.

Maximum ambient temperature is 40°C. The housing can reach approximately 85°C during operation; do not touch it with bare hands.

Inspect the housing, lenses and filters before use. Replace any visibly damaged optical or structural component.

Do not look directly into the output at close range. The LEDs can produce intense visible light.

Carry the fixture by its base, never by the moving head. Protect it from severe impact and vibration.

Installation and repair must be performed by qualified personnel. There are no user-serviceable parts inside.

The support structure must hold at least 10 times the fixture weight. Use a secondary safety cable rated for at least 12 times the fixture weight.

IP65 USE  IP65 protection depends on correctly fitted seals, closed connector caps and approved mating connectors. Do not immerse the fixture, open it while wet, or connect/disconnect power or data where water can enter the connector interface.

SERIOUS FAULT  Stop using the fixture immediately if a serious operating problem occurs. Disconnect power and contact an authorized service center.

2  Product Overview

The AL1650WX is an outdoor LED wash moving head with sixteen 50 W RGBL main emitters and a 204-pixel RGB halo system. Motorized zoom, individual pixel control and built-in main/halo effects support architectural, touring and live-event applications.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Power consumption | 900 W |
| Main light source | 16 x 50 W RGBL 4-in-1 LEDs |
| Halo light source | 204 x 0.3 W RGB 3-in-1 LEDs |
| Source lifetime | 20,000 hours (source specification) |
| Color system | RGBL mixing, rainbow effects, 2700-8000 K color-temperature range |
| Zoom | 4°-60°, motorized linear zoom |
| Dimmer | 0-100% linear dimming |
| Strobe | 1-30 flashes per second plus random strobe |
| Pan / tilt | 540° / 270° |
| DMX personalities | 24CH Basic / 44CH Standard / 48CH Extend / 113CH Extend 2 |
| Control | DMX512 / Auto / Master-Slave; RDM and Art-Net |
| Network protocol | Source lists 'Scan'; likely sACN - verify installed firmware |
| Display | OLED touch display |
| Ingress protection | IP65 |
| DMX / power | Seetronic waterproof 3-pin XLR in/out; waterproof power in/out |
| Network | Seetronic waterproof Art-Net in/out |
| Housing | Die-cast aluminum |
| Fixture size | 392 x 283 x 511 mm |
| Packing size | 470 x 320 x 620 mm |
| Net / gross weight | 21 kg / 23.7 kg |

PROTOCOL VERIFICATION  The source specification prints 'Scan' beside DMX512, RDM and Art-Net. This is likely a transcription of sACN, but the exact network protocol must be confirmed on the installed firmware before commissioning.

2.2 Main Functions

Sixteen individually controllable RGBL main emitters with coarse/fine color channels in the compact personalities.

Individually controllable RGB halo pixels for flowing color and chase effects.

Main-light effects with direction, speed, fade, length, foreground/background color and strobe controls.

Halo effects with dimmer, fine dimmer, effect selection, direction/speed and background color.

RDM bidirectional communication and Art-Net network control.

Temperature-controlled cooling and overheating protection.

3  Setup and Data Connections

3.1 Rigging

Inspect the fixture, bracket, fasteners, safety cable and support structure before installation.

Attach the fixture through the bracket screw holes using correctly rated hardware and clamps.

Secure the fixture to a structure rated for at least 10 times its weight.

Attach an independent safety cable rated for at least 12 times the fixture weight to the designated safety point.

Keep the fixture out of reach, confirm adequate movement clearance and recheck every fastener before applying power.

3.2 DMX512 Connection

Use shielded, twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Keep data cabling separated from high-voltage cables where practical.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Ground / shield |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  For long or electrically noisy DMX lines, connect a 120 ohm terminator between pins 2 and 3 at the DMX OUT of the final fixture.

3.3 RDM and Art-Net

RDM provides bidirectional communication over a compatible DMX line. Art-Net settings are available from the System Settings menu. Configure network addressing and universe parameters to match the controller and local network plan; the source manual does not define factory network values.

WATERPROOF CONNECTIONS  Use only compatible sealed connectors and close every unused cap. Inspect seals and mating surfaces before outdoor use.

4  Control Panel and Menu

Use MENU to select a programming function, UP/DOWN to change an option or value, and ENTER to confirm. MENU exits without saving. The display can rotate automatically according to fixture orientation.

| Menu | Options | Operation |
| --- | --- | --- |
| Address | 001-512 | Set the DMX start address. |
| Operation Mode | DMX / Voice / Auto 1 / Auto 2 | Select console, sound-active or automatic operation. |
| Channel Mode | 24CH / 44CH / 48CH / 113CH | Select Basic, Standard, Extend or Extend 2. |
| Art-Net Set | Network submenu | Configure supported Art-Net parameters. |
| Pan / Tilt Reverse | On / Off | Reverse movement direction. |
| Hall / Optocoupler Correction | On / Off | Enable movement sensor correction. |
| Signal Hold | On / Off | Retain or release the last state after signal loss. |
| Screen Saver | On / Off | Control automatic display blanking. |
| Screen Rotation | On / Off / Auto | Set normal, inverted or automatic orientation. |
| Synchronous Update | On / Off | Enable synchronized output updates. |
| Language | 中文 / EN | Choose the menu language. |
| Dimming Speed | Standard / Fast | Select dimmer response. |
| Dimming Curve | Curve 1 / 2 / 3 / 4 | Select the dimming curve. |
| Dimming Frequency | 6K / 4K / 1K / 25K / 20K / 15K / 10K | Select the source-listed PWM frequency label. |
| Screen Lock | On / Off | Enable or disable panel locking. |
| Factory Reset | Confirm | Restore factory data. |

4.1 Manual Control and Calibration

Manual Control displays the channels for the selected personality. Press ENTER to edit, use UP/DOWN to set 0-255, then press ENTER to save. MENU exits without saving.

| Calibration item | Range / purpose |
| --- | --- |
| Pan / Tilt / Focus home | 0-255 starting-position compensation; 127 = no adjustment |
| Pan / Tilt / Focus travel | 0-255 movement-range calibration; reference 127 |
| Motor dynamics | 0-255 pan, tilt and effect-motor strength; reference 127 |
| Other calibration | Microphone sensitivity and password modification |

CALIBRATION  Record every original value before adjustment. Incorrect home, travel or motor-strength settings can cause abnormal movement or mechanical stress.

4.2 Reset and System Information

| Item | Action |
| --- | --- |
| Effect Motor Reset | Reset effect motors except pan and tilt. |
| Scan Motor Reset | Reset pan and tilt. |
| All Motor Reset | Perform a complete fixture reset. |
| Reset Information | View a detailed error when the red ERR indicator is on. |
| DMX Monitoring | View received channel values numerically. |
| Sensor Information | Monitor Hall and optocoupler status. |
| Hardware / Software | View hardware and firmware versions. |

5  DMX Channel Reference

Select the matching personality on the fixture and controller. Channels 1-18 are common to all modes. Basic/Standard and Extend/Extend 2 use different Channel 19 crossfade ranges; use the table for the selected mode.

| Mode | Channel map |
| --- | --- |
| 24CH Basic | 1-18 common + Basic core 19-24 |
| 44CH Standard | 1-18 common + Basic core 19-24 + Standard extension 25-44 |
| 48CH Extend | 1-18 common + Extend core 19-24 + Extend extension 25-48 |
| 113CH Extend 2 | 48CH map + main-light pixel/ring control 49-113 |

SOURCE NORMALIZATION  The source 113CH table omits Channel 66 and then shifts the Ring 5-7 labels by one. This edition reconstructs Channels 66-105 from the continuous four-channel RGBW pattern and the source's correctly numbered Ring 15-16 ending. Verify this correction on the installed firmware before programming.

5.1 Common Channels 1-18 - All Modes

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan movement |
| 2 | Pan Fine | 0-255 | 16-bit pan fine adjustment |
| 3 | Tilt | 0-255 | Tilt movement |
| 4 | Tilt Fine | 0-255 | 16-bit tilt fine adjustment |
| 5 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 6 | Background Red | 0-255 | Coarse red, 0-100% |
| 7 | Background Red Fine | 0-255 | Fine red adjustment |
| 8 | Background Green | 0-255 | Coarse green, 0-100% |
| 9 | Background Green Fine | 0-255 | Fine green adjustment |
| 10 | Background Blue | 0-255 | Coarse blue, 0-100% |
| 11 | Background Blue Fine | 0-255 | Fine blue adjustment |
| 12 | Background White | 0-255 | Coarse white/lime channel as labeled by source DMX profile |
| 13 | Background White Fine | 0-255 | Fine white/lime adjustment |
| 14 | Strobe | 0-255 | 0-31 blackout; 32-63 open; 64-95 strobe slow to fast; 96-127 open; 128-159 pulse slow to fast; 160-191 open; 192-223 random strobe; 224-255 open |
| 15 | Master Dimmer | 0-255 | Linear dimming, 0-100% |
| 16 | Dimmer Fine | 0-255 | Fine dimmer adjustment |
| 17 | CTO | 0-255 | 0-42 no function; 43-84 7500 K; 85-126 6500 K; 127-168 5500 K; 169-210 4500 K; 211-255 3500 K |
| 18 | Color Presets | 0-255 | Color preset selection |

5.2 Basic / Standard Core - Channels 19-24

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 19 | Preset / RGBW Crossfade | 0-255 | 0 RGBW values; 1-254 blend RGBW and preset channels; 255 preset colors |
| 20 | Preset Crossfade Time | 0-255 | 0-4 no function; 5-255 two-color crossfade from 100 ms to 5 s |
| 21 | Zoom | 0-255 | Motorized zoom |
| 22 | Zoom Fine | 0-255 | Fine zoom adjustment |
| 23 | Reserved | 0-255 | Reserved |
| 24 | Reset / Internal Function | 0-255 | 0-4 none; 5-9 display off; 10-14 display on; 15-19 reverse off; 20-24 reverse on; 45-49 linear curve; 50-54 square curve; 55-57 1K; 58-60 2.5K; 61-63 6K; 64-69 10K; 80-84 all reset; 85-87 pan/tilt reset; 88-90 effect/zoom reset. Other values undocumented. |

5.3 Extend / Extend 2 Core - Channels 19-24

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 19 | Preset / RGBW Crossfade | 0-255 | 0-125 RGBW values; 126-130 blend RGBW and presets; 131-255 preset colors |
| 20 | Preset Crossfade Time | 0-255 | 0-4 no function; 5-255 two-color crossfade from 100 ms to 5 s |
| 21 | Zoom | 0-255 | Motorized zoom |
| 22 | Zoom Fine | 0-255 | Fine zoom adjustment |
| 23 | Reserved | 0-255 | Reserved |
| 24 | Reset / Internal Function | 0-255 | 0-4 none; 5-9 display off; 10-14 display on; 15-19 reverse off; 20-24 reverse on; 45-49 linear curve; 50-54 square curve; 55-57 1K; 58-60 2.5K; 61-63 6K; 64-69 10K; 80-84 all reset; 85-87 pan/tilt reset; 88-90 effect/zoom reset. Other values undocumented. |

5.4 Standard 44CH Extension - Channels 25-44

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 25 | Main Light Effect | 0-255 | 0-9 none; 10-109 effects 1-20 in 5-value steps; 110-255 reserved |
| 26 | Main Effect Speed | 0-255 | 0-125 forward fast to slow; 126-130 stop; 131-255 reverse slow to fast |
| 27 | Main Effect Fade | 0-255 | Fade amount, 0-100% |
| 28 | Main Effect Length | 0-255 | Effect length |
| 29 | Main Effect Red | 0-255 | Foreground red, 0-100% |
| 30 | Main Effect Green | 0-255 | Foreground green, 0-100% |
| 31 | Main Effect Blue | 0-255 | Foreground blue, 0-100% |
| 32 | Main Effect White | 0-255 | Foreground white/lime, 0-100% |
| 33 | Main Effect Dimmer | 0-255 | Foreground master dimmer, 0-100% |
| 34 | Main Background Dimmer | 0-255 | Background master dimmer, 0-100% |
| 35 | Main Effect Strobe | 0-255 | 0-31 blackout; 32-63 open; 64-95 strobe slow to fast; 96-127 open; 128-159 pulse slow to fast; 160-191 open; 192-223 random; 224-255 open |
| 36 | Main Light Control | 0-255 | Crossfade between RGBW settings and individual-color channels |
| 37 | Ring 1 Red | 0-255 | Ring 1 red dimmer |
| 38 | Ring 1 Green | 0-255 | Ring 1 green dimmer |
| 39 | Ring 1 Blue | 0-255 | Ring 1 blue dimmer |
| 40 | Ring 1 White | 0-255 | Ring 1 white/lime dimmer |
| 41 | Ring 2 Red | 0-255 | Ring 2 red dimmer |
| 42 | Ring 2 Green | 0-255 | Ring 2 green dimmer |
| 43 | Ring 2 Blue | 0-255 | Ring 2 blue dimmer |
| 44 | Ring 2 White | 0-255 | Ring 2 white/lime dimmer |

5.5 Extend 48CH / Extend 2 Core - Channels 25-48

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 25 | Main Light Effect | 0-255 | 0-9 none; 10-109 effects 1-20 in 5-value steps; 110-255 reserved |
| 26 | Main Effect Speed | 0-255 | 0-125 forward fast to slow; 126-130 stop; 131-255 reverse slow to fast |
| 27 | Main Effect Fade | 0-255 | Fade amount, 0-100% |
| 28 | Main Effect Length | 0-255 | Effect length |
| 29 | Main Effect Red | 0-255 | Foreground red, 0-100% |
| 30 | Main Effect Green | 0-255 | Foreground green, 0-100% |
| 31 | Main Effect Blue | 0-255 | Foreground blue, 0-100% |
| 32 | Main Effect White | 0-255 | Foreground white/lime, 0-100% |
| 33 | Main Effect Dimmer | 0-255 | Foreground master dimmer, 0-100% |
| 34 | Main Background Dimmer | 0-255 | Background master dimmer, 0-100% |
| 35 | Main Effect Strobe | 0-255 | 0-31 blackout; 32-63 open; 64-95 strobe slow to fast; 96-127 open; 128-159 pulse slow to fast; 160-191 open; 192-223 random; 224-255 open |
| 36 | Halo Control | 0-255 | Source lists no function |
| 37 | Halo Dimmer | 0-255 | Halo master dimmer |
| 38 | Halo Dimmer Fine | 0-255 | Halo fine dimmer |
| 39 | Halo Effect Red | 0-255 | Halo effect red |
| 40 | Halo Effect Green | 0-255 | Halo effect green |
| 41 | Halo Effect Blue | 0-255 | Halo effect blue |
| 42 | Halo Effect | 0-255 | 0-9 none; 10-205 effects 1-97, two values each; 206-255 effect 98 |
| 43 | Halo Effect Speed | 0-255 | 0-130 forward fast to slow; 131-255 reverse slow to fast |
| 44 | Reserved | 0-255 | No function |
| 45 | Halo Background Dimmer | 0-255 | Halo effect background dimmer |
| 46 | Halo Background Red | 0-255 | Halo background red |
| 47 | Halo Background Green | 0-255 | Halo background green |
| 48 | Halo Background Blue | 0-255 | Halo background blue |

5.6 Extend 2 113CH Pixel Extension - Channels 49-113

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 49 | Main Light Control | 0-255 | Crossfade between RGBW settings and individual-color channels |
| 50 | Main Ring 1 Red | 0-255 | Main-light ring/pixel 1 red dimmer |
| 51 | Main Ring 1 Green | 0-255 | Main-light ring/pixel 1 green dimmer |
| 52 | Main Ring 1 Blue | 0-255 | Main-light ring/pixel 1 blue dimmer |
| 53 | Main Ring 1 White | 0-255 | Main-light ring/pixel 1 white dimmer |
| 54 | Main Ring 2 Red | 0-255 | Main-light ring/pixel 2 red dimmer |
| 55 | Main Ring 2 Green | 0-255 | Main-light ring/pixel 2 green dimmer |
| 56 | Main Ring 2 Blue | 0-255 | Main-light ring/pixel 2 blue dimmer |
| 57 | Main Ring 2 White | 0-255 | Main-light ring/pixel 2 white dimmer |
| 58 | Main Ring 3 Red | 0-255 | Main-light ring/pixel 3 red dimmer |
| 59 | Main Ring 3 Green | 0-255 | Main-light ring/pixel 3 green dimmer |
| 60 | Main Ring 3 Blue | 0-255 | Main-light ring/pixel 3 blue dimmer |
| 61 | Main Ring 3 White | 0-255 | Main-light ring/pixel 3 white dimmer |
| 62 | Main Ring 4 Red | 0-255 | Main-light ring/pixel 4 red dimmer |
| 63 | Main Ring 4 Green | 0-255 | Main-light ring/pixel 4 green dimmer |
| 64 | Main Ring 4 Blue | 0-255 | Main-light ring/pixel 4 blue dimmer |
| 65 | Main Ring 4 White | 0-255 | Main-light ring/pixel 4 white dimmer |
| 66 | Main Ring 5 Red | 0-255 | Main-light ring/pixel 5 red dimmer |
| 67 | Main Ring 5 Green | 0-255 | Main-light ring/pixel 5 green dimmer |
| 68 | Main Ring 5 Blue | 0-255 | Main-light ring/pixel 5 blue dimmer |
| 69 | Main Ring 5 White | 0-255 | Main-light ring/pixel 5 white dimmer |
| 70 | Main Ring 6 Red | 0-255 | Main-light ring/pixel 6 red dimmer |
| 71 | Main Ring 6 Green | 0-255 | Main-light ring/pixel 6 green dimmer |
| 72 | Main Ring 6 Blue | 0-255 | Main-light ring/pixel 6 blue dimmer |
| 73 | Main Ring 6 White | 0-255 | Main-light ring/pixel 6 white dimmer |
| 74 | Main Ring 7 Red | 0-255 | Main-light ring/pixel 7 red dimmer |
| 75 | Main Ring 7 Green | 0-255 | Main-light ring/pixel 7 green dimmer |
| 76 | Main Ring 7 Blue | 0-255 | Main-light ring/pixel 7 blue dimmer |
| 77 | Main Ring 7 White | 0-255 | Main-light ring/pixel 7 white dimmer |
| 78 | Main Ring 8 Red | 0-255 | Main-light ring/pixel 8 red dimmer |
| 79 | Main Ring 8 Green | 0-255 | Main-light ring/pixel 8 green dimmer |
| 80 | Main Ring 8 Blue | 0-255 | Main-light ring/pixel 8 blue dimmer |
| 81 | Main Ring 8 White | 0-255 | Main-light ring/pixel 8 white dimmer |
| 82 | Main Ring 9 Red | 0-255 | Main-light ring/pixel 9 red dimmer |
| 83 | Main Ring 9 Green | 0-255 | Main-light ring/pixel 9 green dimmer |
| 84 | Main Ring 9 Blue | 0-255 | Main-light ring/pixel 9 blue dimmer |
| 85 | Main Ring 9 White | 0-255 | Main-light ring/pixel 9 white dimmer |
| 86 | Main Ring 10 Red | 0-255 | Main-light ring/pixel 10 red dimmer |
| 87 | Main Ring 10 Green | 0-255 | Main-light ring/pixel 10 green dimmer |
| 88 | Main Ring 10 Blue | 0-255 | Main-light ring/pixel 10 blue dimmer |
| 89 | Main Ring 10 White | 0-255 | Main-light ring/pixel 10 white dimmer |
| 90 | Main Ring 11 Red | 0-255 | Main-light ring/pixel 11 red dimmer |
| 91 | Main Ring 11 Green | 0-255 | Main-light ring/pixel 11 green dimmer |
| 92 | Main Ring 11 Blue | 0-255 | Main-light ring/pixel 11 blue dimmer |
| 93 | Main Ring 11 White | 0-255 | Main-light ring/pixel 11 white dimmer |
| 94 | Main Ring 12 Red | 0-255 | Main-light ring/pixel 12 red dimmer |
| 95 | Main Ring 12 Green | 0-255 | Main-light ring/pixel 12 green dimmer |
| 96 | Main Ring 12 Blue | 0-255 | Main-light ring/pixel 12 blue dimmer |
| 97 | Main Ring 12 White | 0-255 | Main-light ring/pixel 12 white dimmer |
| 98 | Main Ring 13 Red | 0-255 | Main-light ring/pixel 13 red dimmer |
| 99 | Main Ring 13 Green | 0-255 | Main-light ring/pixel 13 green dimmer |
| 100 | Main Ring 13 Blue | 0-255 | Main-light ring/pixel 13 blue dimmer |
| 101 | Main Ring 13 White | 0-255 | Main-light ring/pixel 13 white dimmer |
| 102 | Main Ring 14 Red | 0-255 | Main-light ring/pixel 14 red dimmer |
| 103 | Main Ring 14 Green | 0-255 | Main-light ring/pixel 14 green dimmer |
| 104 | Main Ring 14 Blue | 0-255 | Main-light ring/pixel 14 blue dimmer |
| 105 | Main Ring 14 White | 0-255 | Main-light ring/pixel 14 white dimmer |
| 106 | Main Ring 15 Red | 0-255 | Main-light ring/pixel 15 red dimmer |
| 107 | Main Ring 15 Green | 0-255 | Main-light ring/pixel 15 green dimmer |
| 108 | Main Ring 15 Blue | 0-255 | Main-light ring/pixel 15 blue dimmer |
| 109 | Main Ring 15 White | 0-255 | Main-light ring/pixel 15 white dimmer |
| 110 | Main Ring 16 Red | 0-255 | Main-light ring/pixel 16 red dimmer |
| 111 | Main Ring 16 Green | 0-255 | Main-light ring/pixel 16 green dimmer |
| 112 | Main Ring 16 Blue | 0-255 | Main-light ring/pixel 16 blue dimmer |
| 113 | Main Ring 16 White | 0-255 | Main-light ring/pixel 16 white dimmer |

6  Routine Maintenance

ATTENTION  Disconnect power, allow the fixture to cool and ensure all connector interfaces are dry before inspection or cleaning.

Inspect the housing, lenses, seals, bracket, mounting points, safety cable, power cable and waterproof connectors.

Clean external optical surfaces with a soft cloth and normal glass-cleaning fluid, then dry every part carefully.

The source recommends cleaning external optics at least every 20 days and internal optics every 30-60 days; adjust the interval for dust, smoke, moisture and operating hours.

Do not open the sealed housing unless qualified and authorized to restore its IP65 protection correctly.

Replace visibly damaged lenses, filters, seals or structural parts before returning the fixture to service.

Cleaning frequency depends on the installation environment. Damp, smoky or dirty surroundings can cause faster optical contamination and require more frequent inspection.

7  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| No power, light or fan | Check the power connection, supply voltage, main fuse and power indicator. |
| No DMX response | Confirm DMX mode, address, personality, data polarity, connectors, cables and controller output. |
| Intermittent DMX | Inspect connector pins and the previous fixture, add termination and separate DMX from high-voltage cabling. |
| Unexpected channels | Confirm 24CH/44CH/48CH/113CH selection and verify the reconstructed 113CH pixel mapping against firmware. |
| Movement error | Remove obstructions, run Scan Motor Reset and inspect Hall/optocoupler correction and sensor information. |
| Network control unavailable | Verify Art-Net settings, cabling, controller universe/network configuration and supported firmware protocol. |
| Reduced or unstable output | Check temperature, cooling, optical cleanliness, supply quality and recorded error information. |

SERVICE  There are no user-serviceable parts inside. Refer internal repair, seal replacement and calibration to qualified personnel, and use matching replacement parts.

7.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair or incorrect installation may be excluded from warranty coverage. Include the model, serial number, firmware version, active personality and a clear fault description when requesting support.

End of manual