AL1940WR

19 x 40 W LED Bee Eye
Moving Head Mini

RGBW 4-IN-1 - 4°-60° ZOOM - CONTINUOUS FRONT-LENS ROTATION

Beam, wash, pixel-graphics and vortex effects in a compact moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation, rigging and service must be performed by qualified personnel.

English Edition  |  Professionally translated and technically standardized

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview and Confirmed Specifications |
| 3 | Setup, Rigging and Data Connections |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Shape Engine and Source Value Anchors |
| 7 | Inspection, Maintenance and Cleaning |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition is organized for model AL1940WR. Before installation, confirm the model, mains rating, connector configuration and firmware shown on the actual fixture.

EDITORIAL NORMALIZATION  The legacy source contains inconsistent family names and a dimension drawing that does not match the supplied product photograph. Editable text in this edition uses AL1940WR only, and unverified dimensions are omitted. The supplied photograph retains legacy badge artwork; confirm the final product-label artwork before publication. Confirmed technical and DMX data are retained.

Quick Reference

| Item | Confirmed value |
| --- | --- |
| Input | AC 110-240 V, 50/60 Hz |
| Rated power | 500 W |
| Light source | 19 x 40 W RGBW 4-in-1 LEDs |
| DMX modes | 21 / 35 / 78 / 92 / 97 channels |
| Movement | Pan 540° / Tilt 270° |

1  Safety Information

DANGER  Disconnect the fixture from AC power and allow it to cool before opening, cleaning, inspection or service.

This is a professional entertainment-lighting fixture. Installation, operation and service must be performed by trained, qualified personnel.

Use indoors in a dry location. The source does not state an ingress-protection rating; do not expose the fixture to rain, splashing water, condensation or corrosive environments.

Connect only to AC 110-240 V, 50/60 Hz through a protected, correctly grounded Class I supply. Never defeat the protective earth connection.

After a major temperature change, leave the fixture disconnected until it reaches room temperature and all condensation has evaporated.

Do not operate the fixture with a damaged housing, lens, cable, connector, mounting point, bracket or secondary attachment.

Do not look directly into the LEDs at close range. Intense light and strobe effects can cause eye discomfort and may affect photosensitive people.

Keep illuminated combustible surfaces at least 0.5 m from the light output. Maintain at least 0.5 m of free space at ventilation openings and nearby walls.

Keep cables away from sharp edges, hot surfaces, moving parts and public traffic. Disconnect power by holding the plug or connector, never by pulling the cable.

Do not modify the fixture or bypass protective devices. Internal repair, calibration and component replacement require qualified service personnel.

1.1 Suspended Installation

The supporting structure and every load-bearing component must be capable of supporting at least ten times the fixture load, or the higher factor required by local regulations.

Use the two quick-lock/Omega brackets with correctly rated clamps and fully tightened M10 clamp fasteners.

Lock every quarter-turn fastener completely and attach an independently rated safety cable to the designated safety point.

Keep people clear of the area below the fixture during installation, focusing, inspection and removal.

Allow unobstructed clearance for the full pan, tilt, zoom and front-lens rotation movement before applying power.

HOT SURFACES  The housing and optical assembly can become hot during use. Allow adequate cooling time before handling.

2  Product Overview and Confirmed Specifications

The AL1940WR is a compact bee-eye moving head with nineteen individually controllable 40 W RGBW emitters, motorized zoom and continuous front-lens rotation. It combines narrow beams, broad washes, pixel graphics, color effects and vortex/kaleidoscope looks for stages, theatres, studios, clubs and rental productions.

2.1 Technical Specifications

| Parameter | Confirmed specification |
| --- | --- |
| Input voltage | AC 110-240 V, 50/60 Hz |
| Rated power | 500 W |
| Light source | 19 x 40 W RGBW 4-in-1 LEDs; individual LED/pixel control |
| Nominal LED life | 50,000 hours (source specification) |
| Color temperature | 2,500-8,000 K |
| Zoom | Motorized 4°-60° |
| Front lens | Continuous motorized rotation; vortex/kaleidoscope effects |
| Dimming | 0-100%; four selectable dimming curves (curve names not stated) |
| Strobe | 1-25 Hz, including pulse and random effects |
| Color system | RGBW and virtual CMY color mixing |
| Pan / Tilt | 540° / 270° |
| Control | DMX512; Auto, Sound and Art-Net options appear in the source menu |
| DMX personalities | 21 CH / 35 CH / 78 CH / 92 CH / 97 CH |
| Data connection | 3-pin DMX wiring documented; some versions may use 5-pin XLR |
| Electrical class / use | Protection Class I / indoor use |

SOURCE LIMITATIONS  The supplied source does not confirm fixture weight, shipping data, photometric output, cooling specification, ambient-temperature rating, fuse/current rating or ingress-protection rating. These values are omitted rather than estimated. The legacy dimension drawing is also omitted because its chassis details do not match the supplied photograph.

2.2 Package Contents

| Item | Quantity / note |
| --- | --- |
| AL1940WR fixture | 1 |
| Power input cable | 1 |
| 3-pin DMX cable | 1 |
| Quick-lock/Omega mounting brackets | 2 |
| Safety cable | 1 |
| English user manual | 1 |
| Flight case and clamps | Optional / package-dependent |

3  Setup, Rigging and Data Connections

3.1 Unpacking and Inspection

- Unpack carefully and inspect the fixture, lenses, mounting points, cables and accessories for transport damage.

- Confirm that the product-label voltage and frequency match the local mains supply.

- Record the model, firmware version and connector configuration for the controller profile and service records.

- Do not power a fixture that appears damaged or incomplete; retain the packaging and contact the supplier.

3.2 Rigging Procedure

- Verify the load rating of the structure, clamps, fasteners, brackets and safety cable.

- Attach rated clamps to the two Omega brackets using suitable M10 fasteners.

- Insert the quarter-turn fasteners into the base and rotate each one fully to lock.

- Attach the independently rated safety cable to the fixture's designated safety point and the supporting structure.

- Check clearances and every attachment before applying power; inspect the installation regularly.

3.3 DMX512 Connection

Use shielded twisted-pair cable designed for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Avoid star wiring unless an approved DMX splitter is used.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the final fixture in a long or electrically noisy DMX line, connect a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

3.4 Addressing

| Personality | Example consecutive start addresses |
| --- | --- |
| 21 CH | 001, 022, 043 |
| 35 CH | 001, 036, 071 |
| 78 CH | 001, 079, 157 |
| 92 CH | 001, 093, 185 |
| 97 CH | 001, 098, 195 |

ART-NET  Art-Net appears as an operating option in the source menu, but the source does not document connector assignment, addressing workflow or network topology. Confirm the production hardware and firmware before network deployment.

4  Control Panel and Menu

Use the navigation keys or encoder to highlight an item, adjust its value and confirm/save the selection. Use Back/Left or the menu key to return to the previous level. The display can rotate automatically; button legends and menu order may vary with firmware.

| Menu group | Item | Option / range | Purpose |
| --- | --- | --- | --- |
| Address | DMX Address | 001-512 | Set the DMX start address. |
| Operating Mode | Protocol / source | Art-Net / DMX / Sound / Auto | Select the operating source. |
| Channel Mode | 21 | 21 CH | Core control. |
| Channel Mode | 35 | 35 CH | Core plus shape engine. |
| Channel Mode | 21+RGB | 78 CH | Core plus 19-pixel RGB. |
| Channel Mode | 35+RGB | 92 CH | Shape engine plus 19-pixel RGB. |
| Channel Mode | 21+RGBW | 97 CH | Core plus 19-pixel RGBW. |
| System | Pan Default Position | 000-255 | Set the pan startup position. |
| System | Tilt Default Position | 000-255 | Set the tilt startup position. |
| System | Pan Invert | Off / On | Reverse pan direction. |
| System | Tilt Invert | Off / On | Reverse tilt direction. |
| System | Pan/Tilt Swap | Off / On | Exchange pan and tilt control. |
| System | Fast Pan/Tilt | Off / On | Select faster movement response. |
| System | Signal Hold | Off / On | Hold the last state after control loss. |
| System | Optical Sensor Correction | Off / On | Enable optical position correction. |
| System | Hall Sensor Correction | Off / On | Enable Hall-sensor correction. |
| System | Synchronized Update | Off / On | Apply received parameters together. |
| Display | Backlight | 15 / 30 / 60 s / Always On | Set the display timeout. |
| Reset | Fixture Reset | Confirm / Cancel | Reinitialize the fixture. |

SIGNAL HOLD  Choose the loss-of-signal behavior required by the production safety plan. Test it before the fixture is placed in service.

4.1 Calibration, Diagnostics and Service Menus

| Menu | Item | Range / credential | Purpose / note |
| --- | --- | --- | --- |
| Manual Test | Current channel | 000-255 | Manually test one channel at a time. |
| Calibration | Password | 0000 | Access source-documented calibration. |
| Calibration | Pan / Tilt / Zoom / Front-Lens Rotation | 000-255; 127 neutral | Home-position offset. |
| Calibration | LED 1-19 Output | 000-255; 255 maximum | Per-emitter maximum-output calibration. |
| Calibration | Overall RGBW White Balance | 000-255; 255 maximum | Global color balance. |
| Calibration | LED 1-19 RGBW White Balance | 000-255; 255 maximum | Per-emitter color balance. |
| Calibration | Sound Sensitivity / Password | Source service parameters | Adjust sensitivity or change the calibration password. |
| Advanced | Password | 988 | Access source-documented advanced menu. |
| Network | IP-MS | Source label; MS undefined | Network parameter shown by the source. |
| Network | Data Packet | Net 000-127 / Sub-Uni | Art-Net universe-related parameters; workflow not documented. |
| Reset | Effect Motor Reset | Execute | Reset motors except pan and tilt. |
| Reset | Pan/Tilt Reset | Execute | Reset pan and tilt motors. |
| Reset | Reset All | Execute | Reset all fixture systems. |
| Diagnostics | Error Log | View / clear | Review stored fault information. |
| Diagnostics | DMX Monitor | Live values | View received channel data. |
| Diagnostics | Sensors | Hall / X optical / Y optical | View source-listed sensor states. |
| Information | Versions / Operating Hours | Hardware / firmware / total hours | Record before service or profile work. |

CALIBRATION WARNING  Record every original value before adjustment. Incorrect offsets, output limits or white-balance values can cause abnormal movement, reduced output or uneven color. Calibration should be performed only by qualified service personnel.

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. Each channel accepts an 8-bit DMX value from 000 to 255. The source provides the channel names but does not provide semantic sub-ranges for several functions; do not import value bands from a related fixture profile.

5.1  21-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | Red intensity |
| 2 | Red Fine | 000-255 | 16-bit red fine |
| 3 | Green | 000-255 | Green intensity |
| 4 | Green Fine | 000-255 | 16-bit green fine |
| 5 | Blue | 000-255 | Blue intensity |
| 6 | Blue Fine | 000-255 | 16-bit blue fine |
| 7 | White | 000-255 | White intensity |
| 8 | White Fine | 000-255 | 16-bit white fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature control; bands not supplied |
| 10 | Color Macro | 000-255 | Built-in colors; map not supplied |
| 11 | Strobe | 000-255 | Strobe control; detailed bands not supplied |
| 12 | Master Dimmer | 000-255 | Overall intensity |
| 13 | Dimmer Fine | 000-255 | 16-bit master-dimmer fine |
| 14 | Pan | 000-255 | Pan position |
| 15 | Pan Fine | 000-255 | 16-bit pan fine |
| 16 | Tilt | 000-255 | Tilt position |
| 17 | Tilt Fine | 000-255 | 16-bit tilt fine |
| 18 | Function | 000-255 | Function bands not supplied |
| 19 | Reset | 000-255 | Reset bands / hold time not supplied |
| 20 | Zoom | 000-255 | Motorized zoom; direction not specified |
| 21 | Front-Lens Rotation | 000-255 | Rotation behavior / direction bands not supplied |

5.2  35-Channel Personality

Channels 1-21 are identical to the 21-channel personality. Channels 22-35 add the shape engine.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | Red intensity |
| 2 | Red Fine | 000-255 | 16-bit red fine |
| 3 | Green | 000-255 | Green intensity |
| 4 | Green Fine | 000-255 | 16-bit green fine |
| 5 | Blue | 000-255 | Blue intensity |
| 6 | Blue Fine | 000-255 | 16-bit blue fine |
| 7 | White | 000-255 | White intensity |
| 8 | White Fine | 000-255 | 16-bit white fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature control; bands not supplied |
| 10 | Color Macro | 000-255 | Built-in colors; map not supplied |
| 11 | Strobe | 000-255 | Strobe control; detailed bands not supplied |
| 12 | Master Dimmer | 000-255 | Overall intensity |
| 13 | Dimmer Fine | 000-255 | 16-bit master-dimmer fine |
| 14 | Pan | 000-255 | Pan position |
| 15 | Pan Fine | 000-255 | 16-bit pan fine |
| 16 | Tilt | 000-255 | Tilt position |
| 17 | Tilt Fine | 000-255 | 16-bit tilt fine |
| 18 | Function | 000-255 | Function bands not supplied |
| 19 | Reset | 000-255 | Reset bands / hold time not supplied |
| 20 | Zoom | 000-255 | Motorized zoom; direction not specified |
| 21 | Front-Lens Rotation | 000-255 | Rotation behavior / direction bands not supplied |
| 22 | Shape Selection | 000-255 | Select a built-in shape; complete map not supplied |
| 23 | Shape Speed | 000-255 | Shape animation speed |
| 24 | Shape Fade | 000-255 | Shape fade / gamma; selected anchors in 6.2 |
| 25 | Shape Red | 000-255 | Foreground red |
| 26 | Shape Green | 000-255 | Foreground green |
| 27 | Shape Blue | 000-255 | Foreground blue |
| 28 | Shape White | 000-255 | Foreground white |
| 29 | Shape Dimmer | 000-255 | Foreground intensity |
| 30 | Background Dimmer | 000-255 | Background intensity |
| 31 | Shape Transition | 000-255 | Internal transition time; anchors in 6.2 |
| 32 | Shape Offset | 000-255 | Pattern density / fixture distribution offset |
| 33 | Foreground Strobe | 000-255 | Foreground strobe |
| 34 | Background Strobe | 000-255 | Background strobe |
| 35 | Background Selection | 000-255 | Background group selection; map in 6.2 |

5.3  78-Channel Personality - 21 CH + RGB Pixel

Channels 1-21 use the 21-channel core. Channels 22-78 provide RGB control of all 19 pixels. Formula for pixel n: R = 22 + 3(n-1), G = 23 + 3(n-1), B = 24 + 3(n-1).

| CH Range | Pixel | Component Channels | DMX Value |
| --- | --- | --- | --- |
| 22-24 | LED 1 | R 22, G 23, B 24 | 000-255 per component |
| 25-27 | LED 2 | R 25, G 26, B 27 | 000-255 per component |
| 28-30 | LED 3 | R 28, G 29, B 30 | 000-255 per component |
| 31-33 | LED 4 | R 31, G 32, B 33 | 000-255 per component |
| 34-36 | LED 5 | R 34, G 35, B 36 | 000-255 per component |
| 37-39 | LED 6 | R 37, G 38, B 39 | 000-255 per component |
| 40-42 | LED 7 | R 40, G 41, B 42 | 000-255 per component |
| 43-45 | LED 8 | R 43, G 44, B 45 | 000-255 per component |
| 46-48 | LED 9 | R 46, G 47, B 48 | 000-255 per component |
| 49-51 | LED 10 | R 49, G 50, B 51 | 000-255 per component |
| 52-54 | LED 11 | R 52, G 53, B 54 | 000-255 per component |
| 55-57 | LED 12 | R 55, G 56, B 57 | 000-255 per component |
| 58-60 | LED 13 | R 58, G 59, B 60 | 000-255 per component |
| 61-63 | LED 14 | R 61, G 62, B 63 | 000-255 per component |
| 64-66 | LED 15 | R 64, G 65, B 66 | 000-255 per component |
| 67-69 | LED 16 | R 67, G 68, B 69 | 000-255 per component |
| 70-72 | LED 17 | R 70, G 71, B 72 | 000-255 per component |
| 73-75 | LED 18 | R 73, G 74, B 75 | 000-255 per component |
| 76-78 | LED 19 | R 76, G 77, B 78 | 000-255 per component |

PIXEL ORDER  Confirm physical LED numbering with a low-level test scene before creating position-dependent graphics.

5.4  92-Channel Personality - 35 CH + RGB Pixel

Channels 1-35 use the 35-channel core and shape engine. Channels 36-92 provide RGB control of all 19 pixels. Formula for pixel n: R = 36 + 3(n-1), G = 37 + 3(n-1), B = 38 + 3(n-1).

| CH Range | Pixel | Component Channels | DMX Value |
| --- | --- | --- | --- |
| 36-38 | LED 1 | R 36, G 37, B 38 | 000-255 per component |
| 39-41 | LED 2 | R 39, G 40, B 41 | 000-255 per component |
| 42-44 | LED 3 | R 42, G 43, B 44 | 000-255 per component |
| 45-47 | LED 4 | R 45, G 46, B 47 | 000-255 per component |
| 48-50 | LED 5 | R 48, G 49, B 50 | 000-255 per component |
| 51-53 | LED 6 | R 51, G 52, B 53 | 000-255 per component |
| 54-56 | LED 7 | R 54, G 55, B 56 | 000-255 per component |
| 57-59 | LED 8 | R 57, G 58, B 59 | 000-255 per component |
| 60-62 | LED 9 | R 60, G 61, B 62 | 000-255 per component |
| 63-65 | LED 10 | R 63, G 64, B 65 | 000-255 per component |
| 66-68 | LED 11 | R 66, G 67, B 68 | 000-255 per component |
| 69-71 | LED 12 | R 69, G 70, B 71 | 000-255 per component |
| 72-74 | LED 13 | R 72, G 73, B 74 | 000-255 per component |
| 75-77 | LED 14 | R 75, G 76, B 77 | 000-255 per component |
| 78-80 | LED 15 | R 78, G 79, B 80 | 000-255 per component |
| 81-83 | LED 16 | R 81, G 82, B 83 | 000-255 per component |
| 84-86 | LED 17 | R 84, G 85, B 86 | 000-255 per component |
| 87-89 | LED 18 | R 87, G 88, B 89 | 000-255 per component |
| 90-92 | LED 19 | R 90, G 91, B 92 | 000-255 per component |

CORRECTED LABEL  The legacy table incorrectly labels this footprint as RGBW. Its 57 pixel channels provide 19 x RGB, so this edition identifies it as 35 CH + RGB Pixel.

5.5  97-Channel Personality - 21 CH + RGBW Pixel

Channels 1-21 use the 21-channel core. Channels 22-97 provide RGBW control of all 19 pixels. Formula for pixel n: R = 22 + 4(n-1), G = 23 + 4(n-1), B = 24 + 4(n-1), W = 25 + 4(n-1).

| CH Range | Pixel | Component Channels | DMX Value |
| --- | --- | --- | --- |
| 22-25 | LED 1 | R 22, G 23, B 24, W 25 | 000-255 per component |
| 26-29 | LED 2 | R 26, G 27, B 28, W 29 | 000-255 per component |
| 30-33 | LED 3 | R 30, G 31, B 32, W 33 | 000-255 per component |
| 34-37 | LED 4 | R 34, G 35, B 36, W 37 | 000-255 per component |
| 38-41 | LED 5 | R 38, G 39, B 40, W 41 | 000-255 per component |
| 42-45 | LED 6 | R 42, G 43, B 44, W 45 | 000-255 per component |
| 46-49 | LED 7 | R 46, G 47, B 48, W 49 | 000-255 per component |
| 50-53 | LED 8 | R 50, G 51, B 52, W 53 | 000-255 per component |
| 54-57 | LED 9 | R 54, G 55, B 56, W 57 | 000-255 per component |
| 58-61 | LED 10 | R 58, G 59, B 60, W 61 | 000-255 per component |
| 62-65 | LED 11 | R 62, G 63, B 64, W 65 | 000-255 per component |
| 66-69 | LED 12 | R 66, G 67, B 68, W 69 | 000-255 per component |
| 70-73 | LED 13 | R 70, G 71, B 72, W 73 | 000-255 per component |
| 74-77 | LED 14 | R 74, G 75, B 76, W 77 | 000-255 per component |
| 78-81 | LED 15 | R 78, G 79, B 80, W 81 | 000-255 per component |
| 82-85 | LED 16 | R 82, G 83, B 84, W 85 | 000-255 per component |
| 86-89 | LED 17 | R 86, G 87, B 88, W 89 | 000-255 per component |
| 90-93 | LED 18 | R 90, G 91, B 92, W 93 | 000-255 per component |
| 94-97 | LED 19 | R 94, G 95, B 96, W 97 | 000-255 per component |

OUTPUT BALANCE  Per-pixel RGBW values are affected by the master dimmer and any stored calibration/output-limit settings.

6  Shape Engine

FIRMWARE VERIFICATION  The legacy shape-engine prose was written for another fixture family. The channel names below match the AL1940WR source table, but shape selection, numbering and animation behavior must be verified on the production firmware before show programming.

The selected shape divides the LED face into foreground pixels and background pixels. The source states that 63 shapes are available, but it does not provide a complete shape-to-value map.

| Layer / task | Relevant channels | Programming guidance |
| --- | --- | --- |
| Overall output | CH 12 Master Dimmer; CH 11 Strobe | Open the common output before evaluating foreground or background layers. |
| Foreground color | CH 25-28 Shape RGBW | Set the color of pixels inside the selected shape. |
| Foreground level | CH 29 Shape Dimmer; CH 33 Foreground Strobe | Control foreground intensity and strobe independently. |
| Shape selection | CH 22 Shape Selection | Select the built-in shape; complete value map is not supplied. |
| Animation | CH 23 Shape Speed; CH 24 Shape Fade | Set motion speed and the source-defined fade/gamma response. |
| Background color | CH 1 / 3 / 5 / 7 RGBW | The common RGBW coarse channels set background color in the source workflow. |
| Background level | CH 30 Background Dimmer; CH 34 Background Strobe | Control the pixels outside the foreground shape. |
| Background group | CH 35 Background Selection | Select the center pixel and/or LED rings using the documented values. |
| Transition | CH 31 Shape Transition | Sets the internal crossfade between shapes; set console fades to zero when using this function. |
| Offset | CH 32 Shape Offset | Adjust pattern density or distribute the effect across fixtures with different start addresses. |
| Vortex example | CH 22 value 010 + CH 21 Front-Lens Rotation | Source example only; verify the selected shape and rotation behavior on the fixture. |

6.1 Basic Programming Sequence

- Select the 35 CH or 92 CH personality and set CH 12 Master Dimmer above zero.

- Choose a shape on CH 22, then set CH 25-29 for foreground color and level.

- Set CH 1, 3, 5 and 7 for background color, then raise CH 30 Background Dimmer.

- Add shape speed, fade, transition, offset and layer strobes as required.

- Test the complete cue on every installed firmware version before public operation.

6.2 Source-Documented Value Anchors

Only the values explicitly printed in the supplied source are listed. Unprinted intermediate mappings are identified rather than reconstructed.

| CH 24 Shape Fade | Documented behavior |
| --- | --- |
| 000-015 | Snap / no fade |
| 016 | Gamma 0.500 |
| 017 | Gamma 0.506 |
| 018 | Gamma 0.513 |
| 019-242 | Progressive gamma; exact intermediate value map not fully printed |
| 243 | Gamma 1.986 |
| 244 | Gamma 1.993 |
| 245 | Gamma 2.000 |
| 246-255 | Automatic gamma |

| CH 31 Shape Transition | Documented time anchor |
| --- | --- |
| 000-004 | No fade |
| 005 | 100 ms |
| 073 | 0.5 s |
| 113 | 1 s |
| 171 | 2 s |
| 216 | 3 s |
| 255 | 4 s |

| CH 35 Background Selection | Selected background group |
| --- | --- |
| 000-007 | Not documented |
| 008 | None |
| 009 | Pixel 1 |
| 010 | Ring 2 |
| 011 | Ring 3 |
| 012 | Pixel 1 + Ring 3 |
| 013 | Pixel 1 + Ring 2 |
| 014 | Pixel 1 + Ring 2 + Ring 3 |
| 015 | Ring 2 + Ring 3 |
| 016-255 | None |

LEVEL CONVENTION  For Shape RGBW, Shape Dimmer and Background Dimmer, the source uses 000 = off and 255 = full output. Intermediate values provide proportional control.

7  Inspection, Maintenance and Cleaning

DISCONNECT POWER  Isolate the fixture from AC power, prevent reconnection and allow all parts to cool before inspection or cleaning.

7.1 Condition-Based Inspection

Inspection frequency depends on operating hours, environment, mounting method, transport and venue conditions. Increase the frequency for touring, dusty, smoky or high-vibration use.

Check the power and data cables, connectors, housing, lens, vents and control panel for damage or contamination.

Inspect clamps, M10 fasteners, Omega brackets, quarter-turn locks, mounting points and the safety cable for wear, deformation or corrosion.

Check that pan, tilt, zoom and front-lens rotation move smoothly without impact, grinding noise or obstruction.

Verify that all 19 LEDs produce consistent RGBW output at matched test values.

Review the error log, sensor status, total operating hours and firmware version when diagnosing recurrent faults.

7.2 External Cleaning

- Remove loose dust with clean, dry, low-pressure air or a soft brush while preventing debris from entering the fixture.

- Wipe the housing with a soft, lint-free cloth lightly dampened with water or a mild suitable cleaner.

- Clean the optical surface gently with a clean optical-grade cloth. Do not use abrasive materials, aggressive solvents or excessive liquid.

- Allow the fixture to dry completely, then inspect all connectors and attachments before reconnecting power.

7.3 Service and Calibration

There are no user-serviceable internal parts documented in the supplied source. Internal cleaning, fan or power-supply service, motor work, firmware service and calibration must be performed by qualified personnel. Record original calibration values and service actions.

RETURN TO SERVICE  Do not return the fixture to use until every cover, connector, mounting component and safety attachment has been correctly reinstalled and function-tested.

8  Troubleshooting and Service

| Symptom | Checks / corrective action |
| --- | --- |
| No power or display | Verify the branch supply, product-label rating, power connector seating and cable condition. Disconnect immediately if a cable or connector is damaged. |
| No DMX response | Confirm DMX mode, start address and personality; inspect polarity and cable continuity; test directly from the controller; terminate the final fixture when required. |
| Incorrect channels | Confirm that the controller profile matches 21, 35, 78, 92 or 97 CH. Do not use an unverified profile from another fixture family. |
| Output is dark | Raise Master Dimmer, check Strobe and color levels, then review Signal Hold, manual-test values and stored output calibration. |
| Uneven pixel color | Send equal RGBW test values; inspect the lens; review per-LED output limits and white-balance calibration. Record values before adjustment. |
| Pan / tilt fault | Remove mechanical obstructions, verify invert/swap settings, execute Pan/Tilt Reset and review Hall/optical sensor and error-log information. |
| Zoom or front lens does not move | Remove obstructions, execute Effect Motor Reset, review calibration offsets and refer persistent mechanical faults to qualified service. |
| Unexpected shape behavior | Verify 35 or 92 CH mode, foreground/background dimmers, console fades and firmware-specific shape mapping. |
| Art-Net does not respond | Confirm that the production fixture has the required network hardware and obtain the firmware-specific IP, Net and Sub-Uni workflow from the supplier. |
| Fixture overheats or shuts down | Disconnect power, allow cooling, clear external airflow paths and refer persistent temperature or fan faults to qualified service. |

8.1 Information to Record Before Service

Model and serial number from the actual product label.

Mains voltage, firmware and hardware versions, operating hours and selected personality.

Exact error-log message, sensor status and the steps required to reproduce the fault.

Controller model/profile, start address, cabling layout and a copy of the affected cue values.

PRODUCTION-VERSION VERIFICATION  Before publication or sale, confirm the final product-label artwork, dimensions, net weight, connector configuration, ingress-protection claim (if any), Art-Net hardware/workflow and firmware-specific DMX sub-ranges. These items are not established by the supplied source package.