AL1940WZ

19 x 40 W LED Bee Eye
Moving Head Halo

RGBW 4-IN-1 - ROTATING HONEYCOMB LENS - USER MANUAL

Beam, wash, pixel-pattern and halo effects in one moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview and Specifications |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Photometric Data |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is for model AL1940WZ. Confirm the model, voltage, firmware and product-label ratings before installation or programming.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool before opening, cleaning, inspection or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Use only AC 100-240 V, 50/60 Hz. Connect to a protected, correctly grounded Class I supply.

Do not operate the fixture with a damaged housing, lens, connector, cable, mounting point or safety attachment.

After a large temperature change, leave the fixture disconnected until it reaches ambient temperature and all condensation has evaporated.

Do not look directly into the LEDs at close range. Flashing light may trigger seizures in susceptible people.

Keep combustible material at least 0.5 m from the fixture and keep every ventilation opening unobstructed.

For overhead installation, use rated clamps and an independently rated secondary safety cable.

Carry the fixture by its fixed handles or base. Never lift it by the moving head.

Do not modify the fixture or bypass protective devices. Refer internal repair to qualified service personnel.

IP20 / PROTECTED INDOOR USE  The specification identifies the fixture as IP20. Keep it away from rain, splashing water, condensation and excessive dust. Do not use it outdoors without a suitable rated enclosure.

2  Product Overview and Specifications

The AL1940WZ combines nineteen individually controlled 40 W RGBW LEDs, a motorized 7°-45° zoom, a rotating honeycomb lens and a pixel-controlled halo ring. It can produce narrow beams, broad washes, animated patterns, color effects and synchronized halo looks for stages, theatres, broadcast studios, clubs and rental productions.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Rated power | 750 W (Chinese source specification) |
| Light source | 19 x 40 W RGBW 4-in-1 LEDs; individual LED control |
| Optics | Motorized 7°-45° zoom; rotating honeycomb lens |
| Dimming / strobe | 0-100% dimming; 1-25 Hz electronic strobe with effects |
| Control | DMX512 / Auto / Sound-active / RDM |
| DMX personalities | 21CH / 26CH / 35CH / 43CH / 78CH / 92CH / 97CH / 119CH |
| Pan | 540° with 16-bit fine control |
| Tilt | 250° with 16-bit fine control (specification page) |
| Display | Chinese / English menu display |
| Protection | Position memory, automatic reset, fan and temperature protection |
| Ingress protection | IP20 |
| Fixture dimensions | 434 x 260 x 577 mm |
| Carton dimensions | 480 x 380 x 510 mm |
| Net / gross weight | 16.2 kg / 18.7 kg |
| Two-unit flight case | 990 x 430 x 610 mm; case 22 kg; loaded gross 56 kg |

TILT VALUES IN SOURCE  The source specification lists 250° tilt, while its setup menu lists selectable 270° / 180° / 90° limits and one channel page contains a different figure. This manual preserves those source values. Verify the actual firmware and mechanical limit before programming.

2.2 Main Functions

RGBW color mixing with color-temperature and macro controls.

Individual LED pixel control for animated static and dynamic patterns.

Rotating honeycomb lens for beam, wash and kaleidoscopic effects.

Independent pixel halo ring with dimmer, strobe, color macro and speed controls.

Smooth 16-bit pan and tilt, position feedback and automatic correction.

Eight DMX personalities for compact, pattern and full pixel programming.

2.3 Supplied Items

Package contents: AL1940WZ fixture x 1; power cable x 1; DMX signal cable x 1; quick-lock brackets x 2; safety cable x 1; user manual x 1.

3  Setup and Installation

3.1 AC Power

Before connecting power, verify that the product-label voltage and frequency match the local supply. Complete mounting and data connections before applying power. Disconnect by holding the plug, not by pulling the cable.

3.2 Rigging

STRUCTURAL RATING  The support, mounting hardware and every attachment must be rated for at least ten times the fixture load or the higher factor required by local regulations. Keep people clear of the area below during installation and service.

Inspect the fixture, quick-lock brackets, clamps, M8 fasteners, mounting points, safety cable and support structure.

Attach rated clamps to the supplied brackets and lock the brackets securely into the base.

Attach an independently rated safety cable to the designated point and the supporting structure.

Confirm unobstructed clearance for the complete pan, tilt, zoom and rotating-lens movement.

Recheck every attachment before applying power. Inspect the installation regularly.

3.3 DMX512 Connection

Use shielded twisted-pair cable designed for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Assign a unique start address to every independently controlled fixture.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the final fixture in a long or electrically noisy DMX chain, connect a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use MENU to return or move through the menu, UP/DOWN to change a value, and ENTER to open or save a setting. Hold MENU for approximately 3 seconds to activate the display lock. Labels may vary slightly with firmware revision.

| Menu | Item | Option / Range | Operation |
| --- | --- | --- | --- |
| Address | DMX Address | 001-512 | Set the fixture start address. |
| Run Mode | DMX Personality | 21 / 26 / 35 / 43 / 78 / 92 / 97 / 119CH | Select the controller footprint. |
| Run Mode | Test / Manual | Auto Test / Manual | Run a test or control functions manually. |
| Run Mode | Standalone | Auto / Sound | Select automatic or sound-active operation. |
| Setup | Pan Reverse | Off / On | Reverse pan direction. |
| Setup | Tilt Reverse | Off / On | Reverse tilt direction. |
| Setup | Screen Reverse | Off / On | Rotate the display. |
| Setup | No DMX Signal | Hold | Hold the last state when signal is lost. |
| Setup | Pan Angle | 540° / 360° / 180° | Limit the pan travel. |
| Setup | Tilt Angle | 270° / 180° / 90° | Limit the tilt travel as listed by the source menu. |
| Setup | Dehumidification Time | 085 | Service/environment setting; use only as directed. |
| Setup | Language | English / Chinese | Select the display language. |
| Setup | Dimmer Response | Smooth / Fast | Select dimmer response behavior. |
| Setup | Dimmer Curve | Linear / Square / Inverse Square / S Curve | Select the output curve. |
| Setup | Reset | Confirm | Reset the fixture. |
| Setup | Factory Set | Confirm | Restore factory settings. |
| System Info | Status | Version / Mode / Address / Temperature / Hours | View fixture information. |

TEMPERATURE MESSAGES  Open = thermistor circuit open; Short = thermistor circuit shorted; Hot = over-temperature. Disconnect power and refer persistent sensor or temperature faults to qualified service.

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. DMX values run from 000 to 255 unless a narrower range is shown. Fine channels provide 16-bit resolution when paired with their coarse channel.

5.1  21-Channel Personality (K10)

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | 0-100% |
| 2 | Red Fine | 000-255 | 16-bit fine |
| 3 | Green | 000-255 | 0-100% |
| 4 | Green Fine | 000-255 | 16-bit fine |
| 5 | Blue | 000-255 | 0-100% |
| 6 | Blue Fine | 000-255 | 16-bit fine |
| 7 | White | 000-255 | 0-100% |
| 8 | White Fine | 000-255 | 16-bit fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-103 / 104-107 / 108-207 / 208-212 / 213-251 / 252-255 | Blackout / synchronized 1-25 Hz / open / pulse fast-slow / open / random slow-fast / open |
| 12 | Master Dimmer | 000-255 | 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit fine |
| 14 | Pan | 000-255 | Position |
| 15 | Pan Fine | 000-255 | 16-bit fine |
| 16 | Tilt | 000-255 | Position |
| 17 | Tilt Fine | 000-255 | 16-bit fine |
| 18 | Reserved | 000-255 | No function |
| 19 | Reset | 000-250 / 251-255 | No function / Reset; hold 5 seconds |
| 20 | Zoom | 000-255 | Narrow to wide |
| 21 | Lens Rotation | 000-127 / 128-189 / 190-191 / 192-255 | 0-60° position / CCW fast-slow / stop / CW slow-fast |

5.2  26-Channel Personality (Standard)

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | Position |
| 2 | Pan Fine | 000-255 | 16-bit fine |
| 3 | Tilt | 000-255 | Position |
| 4 | Tilt Fine | 000-255 | 16-bit fine |
| 5 | Pan / Tilt Speed | 000-255 | Fast to slow |
| 6 | Zoom | 000-255 | Narrow to wide |
| 7 | Lens Rotation | 000-127 / 128-189 / 190-191 / 192-255 | 0-60° position / CCW fast-slow / stop / CW slow-fast |
| 8 | Master Dimmer | 000-255 | 0-100% |
| 9 | Strobe | 000-003 / 004-200 / 201-255 | Open / synchronized fast-slow / random slow-fast |
| 10 | Red | 000-255 | 0-100% |
| 11 | Green | 000-255 | 0-100% |
| 12 | Blue | 000-255 | 0-100% |
| 13 | White | 000-255 | 0-100% |
| 14 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 15 | Color Macro | 000-255 | Built-in color macro |
| 16 | Static Effect | 000-009 / 010-184 / 185-194 / 195-255 | None / Effects 1-35 (5 values each) / Effect 36 / Effect 37 |
| 17 | Dynamic Effect | 000-004 / 005-016 / 017-255 | None / Effect 1 / Effects 2-80 (3 values each) |
| 18 | Dynamic Speed | 000-127 / 128-255 | Forward fast-slow / reverse slow-fast |
| 19 | Background Red | 000-255 | 0-100% |
| 20 | Background Green | 000-255 | 0-100% |
| 21 | Background Blue | 000-255 | 0-100% |
| 22 | Background White | 000-255 | 0-100% |
| 23 | Halo Strobe | 000-003 / 004-250 / 251-255 | Blackout / synchronized slow-fast / open |
| 24 | Halo Macro | 000-001 / 002-051 / 052-054 / 055-250 / 251-255 | None / fixed colors / dynamic 1 / dynamic effects 2-50 / effect 51 |
| 25 | Halo Effect Speed | 000-255 | Slow to fast |
| 26 | Reset | 000-239 / 240-255 | No function / Reset; hold 5 seconds |

5.3  35-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | 0-100% |
| 2 | Red Fine | 000-255 | 16-bit fine |
| 3 | Green | 000-255 | 0-100% |
| 4 | Green Fine | 000-255 | 16-bit fine |
| 5 | Blue | 000-255 | 0-100% |
| 6 | Blue Fine | 000-255 | 16-bit fine |
| 7 | White | 000-255 | 0-100% |
| 8 | White Fine | 000-255 | 16-bit fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized 1-25 Hz / random |
| 12 | Master Dimmer | 000-255 | 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit fine |
| 14 | Pan | 000-255 | Position |
| 15 | Pan Fine | 000-255 | 16-bit fine |
| 16 | Tilt | 000-255 | Position |
| 17 | Tilt Fine | 000-255 | 16-bit fine |
| 18 | Reserved | 000-255 | No function |
| 19 | Reset | 000-250 / 251-255 | No function / Reset; hold 5 seconds |
| 20 | Zoom | 000-255 | Narrow to wide |
| 21 | Lens Zoom / Rotation | 000-127 / 128-191 / 192-255 | 0-60° position / CW rotation / CCW rotation |
| 22 | Pattern Selection | 000-255 | Pattern selection |
| 23 | Pattern Speed | 000-255 | Effect speed |
| 24 | Pattern Fade | 000-255 | Fade amount |
| 25 | Pattern Red | 000-255 | 0-100% |
| 26 | Pattern Green | 000-255 | 0-100% |
| 27 | Pattern Blue | 000-255 | 0-100% |
| 28 | Pattern White | 000-255 | 0-100% |
| 29 | Pattern Dimmer | 000-255 | 0-100% |
| 30 | Background Dimmer | 000-255 | 0-100% |
| 31 | Pattern Transition | 000-255 | Transition control |
| 32 | Pattern Angle / Position | 000-255 | Angle or position |
| 33 | Foreground Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized / random |
| 34 | Background Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized / random |
| 35 | Background Selection | 000-255 | Background selection |

5.4  43-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | 0-100% |
| 2 | Red Fine | 000-255 | 16-bit fine |
| 3 | Green | 000-255 | 0-100% |
| 4 | Green Fine | 000-255 | 16-bit fine |
| 5 | Blue | 000-255 | 0-100% |
| 6 | Blue Fine | 000-255 | 16-bit fine |
| 7 | White | 000-255 | 0-100% |
| 8 | White Fine | 000-255 | 16-bit fine |
| 9 | Linear CTO | 000-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized 1-25 Hz / random |
| 12 | Master Dimmer | 000-255 | 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit fine |
| 14 | Pan | 000-255 | Position |
| 15 | Pan Fine | 000-255 | 16-bit fine |
| 16 | Tilt | 000-255 | Position |
| 17 | Tilt Fine | 000-255 | 16-bit fine |
| 18 | Reserved | 000-255 | No function |
| 19 | Reset | 000-239 / 240-255 | No function / Reset; hold 5 seconds |
| 20 | Zoom | 000-255 | Narrow to wide |
| 21 | Lens Rotation | 000-127 / 128-189 / 190-191 / 192-255 | 0-60° position / CCW fast-slow / stop / CW slow-fast |
| 22 | Pattern Select | 000-007 / 008-014 / 015-087 / 088-255 | None / static pattern / dynamic pattern / none |
| 23 | Pattern Speed | 000-127 / 128-255 | CCW slow-fast / dynamic slow-fast |
| 24 | Pattern Fade | 000-255 | Fade amount |
| 25 | Pattern Red | 000-255 | 0-100% |
| 26 | Pattern Green | 000-255 | 0-100% |
| 27 | Pattern Blue | 000-255 | 0-100% |
| 28 | Pattern White | 000-255 | 0-100% |
| 29 | Pattern Dimmer | 000-255 | 0-100% |
| 30 | Background Dimmer | 000-255 | 0-100% |
| 31 | Pattern Transition | 000-255 | Transition control |
| 32 | Pattern Angle / Position | 000-255 | Angle or position |
| 33 | Foreground Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized / random |
| 34 | Background Strobe | 000-003 / 004-203 / 204-255 | Open / synchronized / random |
| 35 | Background Select | 000-008 / 009-015 / 016-255 | None / background choices / none |
| 36 | Shape Frequency | 000-255 | Shape frequency |
| 37 | Halo Dimmer | 000-255 | 0-100% |
| 38 | Halo Strobe | 000-003 / 004-250 / 251-255 | Open / synchronized slow-fast / open |
| 39 | Halo Red | 000-255 | 0-100% |
| 40 | Halo Green | 000-255 | 0-100% |
| 41 | Halo Blue | 000-255 | 0-100% |
| 42 | Halo Macro | 000-001 / 002-051 / 052-054 / 055-250 / 251-255 | None / fixed colors / dynamic 1 / dynamic effects 2-50 / effect 51 |
| 43 | Halo Speed | 000-255 | Slow to fast |

5.5  78-Channel Personality (K10)

Channels 1-21 use the same functions and values as the 21-channel personality. Channels 22-78 provide direct RGB control of all nineteen LEDs.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-24 | LED 1 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 25-27 | LED 2 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 28-30 | LED 3 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 31-33 | LED 4 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 34-36 | LED 5 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 37-39 | LED 6 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 40-42 | LED 7 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 43-45 | LED 8 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 46-48 | LED 9 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 49-51 | LED 10 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 52-54 | LED 11 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 55-57 | LED 12 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 58-60 | LED 13 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 61-63 | LED 14 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 64-66 | LED 15 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 67-69 | LED 16 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 70-72 | LED 17 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 73-75 | LED 18 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 76-78 | LED 19 | 000-255 each | Red / Green / Blue; 0-100% per component |

5.6  92-Channel Personality

Channels 1-35 use the same functions and values as the 35-channel personality. Channels 36-92 provide direct RGB control of all nineteen LEDs.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 36-38 | LED 1 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 39-41 | LED 2 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 42-44 | LED 3 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 45-47 | LED 4 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 48-50 | LED 5 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 51-53 | LED 6 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 54-56 | LED 7 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 57-59 | LED 8 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 60-62 | LED 9 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 63-65 | LED 10 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 66-68 | LED 11 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 69-71 | LED 12 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 72-74 | LED 13 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 75-77 | LED 14 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 78-80 | LED 15 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 81-83 | LED 16 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 84-86 | LED 17 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 87-89 | LED 18 | 000-255 each | Red / Green / Blue; 0-100% per component |
| 90-92 | LED 19 | 000-255 each | Red / Green / Blue; 0-100% per component |

5.7  97-Channel Personality

Channels 1-21 use the same functions and values as the 21-channel personality. Channels 22-97 provide direct RGBW control of all nineteen LEDs.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-25 | LED 1 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 26-29 | LED 2 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 30-33 | LED 3 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 34-37 | LED 4 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 38-41 | LED 5 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 42-45 | LED 6 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 46-49 | LED 7 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 50-53 | LED 8 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 54-57 | LED 9 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 58-61 | LED 10 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 62-65 | LED 11 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 66-69 | LED 12 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 70-73 | LED 13 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 74-77 | LED 14 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 78-81 | LED 15 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 82-85 | LED 16 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 86-89 | LED 17 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 90-93 | LED 18 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 94-97 | LED 19 | 000-255 each | Red / Green / Blue / White; 0-100% per component |

5.8  119-Channel Personality (K10)

Channels 1-43 use the same functions and values as the 43-channel personality. Channels 44-119 provide direct RGBW control of all nineteen LEDs.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 44-47 | LED 1 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 48-51 | LED 2 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 52-55 | LED 3 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 56-59 | LED 4 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 60-63 | LED 5 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 64-67 | LED 6 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 68-71 | LED 7 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 72-75 | LED 8 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 76-79 | LED 9 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 80-83 | LED 10 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 84-87 | LED 11 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 88-91 | LED 12 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 92-95 | LED 13 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 96-99 | LED 14 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 100-103 | LED 15 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 104-107 | LED 16 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 108-111 | LED 17 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 112-115 | LED 18 | 000-255 each | Red / Green / Blue / White; 0-100% per component |
| 116-119 | LED 19 | 000-255 each | Red / Green / Blue / White; 0-100% per component |

6  Photometric Data

The source lists the following illuminance values for a 7° beam. Actual output varies with production tolerance, optical condition, mains voltage and measurement method.

| Color | 3 m | 5 m | 6 m | 8 m | 10 m |
| --- | --- | --- | --- | --- | --- |
| Red | 20,000 lux | 7,650 lux | 5,160 lux | 2,860 lux | 1,840 lux |
| Green | 40,000 lux | 15,400 lux | 10,300 lux | 5,780 lux | 3,720 lux |
| Blue | 72,000 lux | 29,200 lux | 119,400 lux | 10,860 lux | 1,060 lux |
| White | 64,600 lux | 25,600 lux | 17,100 lux | 9,550 lux | 6,150 lux |
| RGBW | 129,000 lux | 68,000 lux | 47,400 lux | 26,500 lux | 17,000 lux |

SOURCE DATA  The blue row is non-monotonic: the source lists 119,400 lux at 6 m and 1,060 lux at 10 m. These values are reproduced without correction; verify them before publication or photometric planning.

7  Routine Maintenance

ATTENTION  Disconnect AC power and allow the fixture to cool completely before inspection or cleaning.

Inspect screws, brackets, clamps, safety attachments and the support structure for tightness, corrosion or deformation.

Inspect the housing, lenses, mounting points, power cable and data connectors for damage or heat stress.

Inspect pan, tilt, zoom and lens rotation for wear, obstruction, imbalance, resistance or abnormal noise.

Clean optical surfaces with a soft, lint-free cloth. Do not use alcohol, solvents or abrasive materials on optics or plastic parts.

Remove dust from fans and ventilation openings regularly. Cleaning frequency depends on the operating environment.

Have internal electrical, thermal and mechanical systems inspected periodically by qualified service personnel.

8  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| No light output | Verify AC supply, power connection, circuit protection, master dimmer, strobe/blackout values, active mode and controller output. |
| No DMX response | Confirm the start address, selected personality, controller patch, cable polarity, connectors and the controller's DMX output. |
| Unstable DMX | Use shielded twisted-pair cable, separate data from power wiring, inspect connectors and terminate the final fixture with 120 ohms. |
| Abnormal pan or tilt | Remove obstructions, check reverse/angle settings, run a reset and refer persistent encoder or drive faults to qualified service. |
| Zoom or lens does not move | Check the active channel values, remove obstructions, reset the fixture and refer persistent motor faults to service. |
| Reduced or unstable output | Check ventilation, fans, optical cleanliness, mains quality, temperature messages and dimmer settings. |
| Thermistor Open / Short / Hot | Disconnect power. Allow the fixture to cool and have the temperature sensor, wiring and cooling system inspected. |

SERVICE  There are no user-serviceable high-voltage parts inside. Refer internal repair, LED-module replacement, motor service and calibration to the manufacturer, its service agent or similarly qualified personnel.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Include the model, serial number, firmware, active personality and a clear fault description when requesting support.

End of manual