AL1960WR

# 19 x 60 W LED Super Bee Eye
Moving Head

RGBW 4-IN-1 - MOTORIZED ZOOM - CONTINUOUS HONEYCOMB-LENS ROTATION

High-output beam, wash, pixel-graphics and shape effects in one professional moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation, rigging and service must be performed by qualified personnel.

English Edition  |  Professionally translated and technically standardized

# Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview and Specifications |
| 3 | Setup, Rigging and Data Connections |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Shape Engine |
| 7 | Photometric Data |
| 8 | Inspection, Maintenance and Cleaning |
| 9 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition is organized for model AL1960WR. Before installation, confirm the model, mains rating, connector configuration, firmware and protection marking shown on the actual fixture.

SOURCE RECONCILIATION  The supplied source contains conflicting voltage, zoom-angle and dimension statements. This edition preserves each documented value, flags the conflicts and treats the product label and production firmware as the final authority.

## Quick Reference

| Item | Source value |
| --- | --- |
| Input | AC 90-240 V or AC 100-240 V, 50/60 Hz; verify product label |
| Rated power | 1,350 W |
| Light source | 19 x 60 W RGBW 4-in-1 LEDs |
| DMX modes | 21 / 35 / 78 / 92 / 97 channels |
| Movement | Pan 540° / Tilt 250°; 16-bit fine control |
| Protection | IP20 / protected indoor use |

# 1  Safety Information

DANGER  Disconnect the fixture from AC power and allow it to cool before opening, cleaning, inspection or service.

This is a professional entertainment-lighting fixture. Installation, operation and service must be performed by trained, qualified personnel.

Use indoors in a dry, protected location. The final specification states IP20; do not expose the fixture to rain, splashing water, condensation, icing or corrosive environments.

Connect only to a protected, correctly grounded Class I supply within the rating printed on the product label. Never defeat the protective earth connection.

After a major temperature change, leave the fixture disconnected until it reaches room temperature and all condensation or ice has disappeared.

Do not operate the fixture with a damaged housing, lens, cable, connector, mounting point, bracket or secondary attachment.

Do not look directly into the LEDs at close range. Intense light and strobe effects can cause eye discomfort and may affect photosensitive people.

Keep combustible materials at least 0.5 m from the fixture and keep every air inlet and outlet unobstructed.

Keep cables away from sharp edges, hot surfaces, moving parts and public traffic. Disconnect power by holding the plug or connector, never by pulling the cable.

Do not modify the fixture or bypass protective devices. Internal repair, calibration and component replacement require qualified service personnel.

## 1.1 Suspended Installation

The supporting structure and every load-bearing component must support at least ten times the fixture load, or the higher factor required by local regulations.

Use the two supplied quick-lock brackets with correctly rated clamps and suitable M8 clamp fasteners.

Lock every quick-lock fastener completely and attach an independently rated safety cable to the designated safety point, not to a carrying handle.

Keep people clear of the area below the fixture during installation, focusing, inspection and removal.

Allow unobstructed clearance for the full pan, tilt, zoom and front-lens rotation movement before applying power.

HOT SURFACES  The housing and optical assembly can become hot during use. Allow adequate cooling time before handling.

# 2  Product Overview and Specifications

The AL1960WR combines nineteen individually controllable 60 W RGBW emitters with motorized zoom and a continuously rotating honeycomb lens. It produces high-output beams, broad washes, RGBW pixel graphics, shape animations and kaleidoscopic effects for concerts, theatres, studios, clubs, attractions and rental productions.

## 2.1 Technical Specifications

| Parameter | Source specification |
| --- | --- |
| Input voltage | AC 90-240 V, 50/60 Hz on the specification page; AC 100-240 V in the safety section |
| Rated power | 1,350 W |
| Light source | 19 x 60 W high-output RGBW 4-in-1 LEDs; individual LED/pixel control |
| Zoom | Motorized 7°-45° on the specification page; photometric page labels 11° and 55° test angles |
| Front lens | Continuously rotating honeycomb lens for beam, wash and kaleidoscopic effects |
| Dimming | 0-100% linear dimming; Linear, Inverse Square, Square and S-Curve menu options |
| Strobe | 1-25 Hz with multiple strobe effects |
| Pan / tilt | 540° pan / 250° tilt with 16-bit fine control and position memory |
| Control | DMX512 / Auto / Sound-active / RDM |
| DMX personalities | 21 CH / 35 CH / 78 CH / 92 CH / 97 CH |
| Display | Chinese / English; normal or inverted orientation; 60-second timeout or always on |
| Cooling / protection | Fan cooling, automatic temperature detection and over-temperature shutdown |
| Ingress protection | IP20 / protected indoor use |
| Fixture dimensions | 465 x 300 x 510 mm in the specification text; see drawing and discrepancy note |
| Carton | 620 x 570 x 490 mm; 1 fixture per carton |
| Net / gross weight | 21.4 kg / 25.5 kg |
| Two-unit flight case | 780 x 520 x 790 mm; loaded gross weight 74.5 kg |

VERIFY BEFORE PUBLICATION OR INSTALLATION  Use the rating label on the production fixture for the permitted mains range. Confirm the actual zoom endpoints and overall dimensions before preparing photometric claims, road-case foam or installation drawings.

## 2.2 Product Dimensions

DIMENSION DISCREPANCY  The drawing shows maxima of approximately 463.33 mm wide, 290.37 mm deep and 462.4 mm high in the illustrated head position, while the specification text states 465 x 300 x 510 mm. Preserve installation clearance for full movement and confirm the production chassis.

## 2.3 Package Contents

| Item | Quantity |
| --- | --- |
| AL1960WR fixture | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |
| Quick-lock mounting brackets | 2 |
| Safety cable | 1 |
| User manual | 1 |

# 3  Setup, Rigging and Data Connections

## 3.1 Unpacking and Inspection

- Unpack carefully and inspect the fixture, lenses, mounting points, cables and accessories for transport damage.

- Confirm that the product-label voltage and frequency match the local mains supply.

- Record the model, firmware version and connector configuration for the controller profile and service records.

- Do not power a fixture that appears damaged or incomplete; retain the packaging and contact the supplier.

## 3.2 Rigging Procedure

- Verify the load rating of the structure, clamps, fasteners, brackets and safety cable.

- Attach rated clamps to the two quick-lock brackets using suitable M8 fasteners.

- Insert and fully lock the brackets in the fixture base.

- Attach the independently rated safety cable to the designated fixture point and the supporting structure.

- Check movement clearances and every attachment before applying power; inspect the installation regularly.

## 3.3 DMX512 Connection

Use shielded twisted-pair cable designed for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Avoid star wiring unless an approved DMX splitter is used.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the final fixture in a long or electrically noisy DMX line, connect a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

## 3.4 Addressing

| Personality | Example consecutive start addresses |
| --- | --- |
| 21 CH | 001, 022, 043 |
| 35 CH | 001, 036, 071 |
| 78 CH | 001, 079, 157 |
| 92 CH | 001, 093, 185 |
| 97 CH | 001, 098, 195 |

# 4  Control Panel and Menu

Use the menu/navigation keys to highlight an item, adjust its value and confirm/save the selection. Menu order and English wording may vary slightly with firmware revision.

| Menu group | Item | Option / range | Purpose |
| --- | --- | --- | --- |
| Address | DMX Address | 001-512 | Set the DMX start address. |
| Fixture Setup | Channel Mode | 21 / 35 / 78 / 97 / 92 CH | Select the controller footprint. |
| Fixture Setup | Run Mode | DMX / Sound / Auto | Select controlled or standalone operation. |
| Fixture Setup | Pan Reverse | On / Off | Reverse pan direction. |
| Fixture Setup | Tilt Reverse | On / Off | Reverse tilt direction. |
| Fixture Setup | Fan Control | Silent / Low / Medium / High | Select the cooling-fan mode. |
| Fixture Setup | Dimmer Curve | Linear / Inverse Square / Square / S-Curve | Select the dimming response. |
| Fixture Setup | Fast Blackout | On / Off | Select rapid output cutoff behavior. |
| Fixture Setup | No DMX State | Blackout / Hold | Choose the response to signal loss. |
| Manual Control | Channel Values | 000-255 per channel | Test fixture functions manually. |
| Display | Language | Chinese / English | Select the display language. |
| Display | Orientation | Normal / Reverse | Rotate the display for the installation. |
| Display | Display Mode | 60 s / Always On | Set the display timeout. |
| Reset | Reset All | Execute / Cancel | Reinitialize the fixture. |
| Reset | Factory Default | Execute / Cancel | Restore factory settings. |

LOSS OF SIGNAL  Choose Blackout or Hold according to the production safety plan and test the selected behavior before the fixture is placed in service.

## 4.1 Information, Sensors and Calibration

| Menu | Item | Range / status | Purpose / note |
| --- | --- | --- | --- |
| Fixture Information | Power-On Time / Total Time / Power-On Count | Status values | Review operating history. |
| Sensor Check | Pan / Tilt / Zoom / Rotation Sensor | Status values | Review motion-sensor states. |
| Software Version | Display / Motor / Dimmer / Network Version | Version values | Record before profile or service work. |
| Factory Setup | Password | 000-255 | Access source-listed service settings. |
| Calibration | Pan / Tilt / Zoom / Zoom Rotation | 000-255 | Adjust motor home-position offsets. |
| Calibration | Red and further output channels | 000-255 | Adjust source-listed output calibration values. |

CALIBRATION WARNING  Record every original value before adjustment. Incorrect offsets or output values can cause abnormal movement, reduced output or uneven color. Calibration should be performed only by qualified service personnel.

# 5  DMX512 Channel Reference

Select the same personality on the fixture and controller. DMX values run from 000 to 255 unless a narrower range is shown. Fine channels provide 16-bit resolution when paired with their coarse channel.

## 5.1  21-Channel Personality

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | Red intensity |
| 2 | Red Fine | 000-255 | 16-bit red fine |
| 3 | Green | 000-255 | Green intensity |
| 4 | Green Fine | 000-255 | 16-bit green fine |
| 5 | Blue | 000-255 | Blue intensity |
| 6 | Blue Fine | 000-255 | 16-bit blue fine |
| 7 | White | 000-255 | White intensity |
| 8 | White Fine | 000-255 | 16-bit white fine |
| 9 | Linear CTO | 000-255 | Color temperature, high to low |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-103 / 104-107 / 108-207 / 208-212 / 213-251 / 252-255 | Blackout / strobe slow-fast / open / reverse strobe / open / random strobe / open |
| 12 | Master Dimmer | 000-255 | Overall intensity |
| 13 | Dimmer Fine | 000-255 | 16-bit master-dimmer fine |
| 14 | Pan | 000-255 | Pan position |
| 15 | Pan Fine | 000-255 | 16-bit pan fine |
| 16 | Tilt | 000-255 | Tilt position |
| 17 | Tilt Fine | 000-255 | 16-bit tilt fine |
| 18 | Function | 000-255 | Reserved |
| 19 | Reset | 000-127 / 128-255 | Reserved / fixture reset |
| 20 | Zoom | 000-255 | Narrow to wide |
| 21 | Lens Rotation | 000-127 / 128-190 / 191-192 / 193-255 | 0°-60° position / CCW fast-slow / stop / CW slow-fast |

## 5.2  35-Channel Personality

Channels 1-21 are identical to the 21-channel personality. Channels 22-35 add the foreground/background shape engine.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | Red intensity |
| 2 | Red Fine | 000-255 | 16-bit red fine |
| 3 | Green | 000-255 | Green intensity |
| 4 | Green Fine | 000-255 | 16-bit green fine |
| 5 | Blue | 000-255 | Blue intensity |
| 6 | Blue Fine | 000-255 | 16-bit blue fine |
| 7 | White | 000-255 | White intensity |
| 8 | White Fine | 000-255 | 16-bit white fine |
| 9 | Linear CTO | 000-255 | Color temperature, high to low |
| 10 | Color Macro | 000-255 | Built-in color macro |
| 11 | Strobe | 000-003 / 004-103 / 104-107 / 108-207 / 208-212 / 213-251 / 252-255 | Blackout / strobe slow-fast / open / reverse strobe / open / random strobe / open |
| 12 | Master Dimmer | 000-255 | Overall intensity |
| 13 | Dimmer Fine | 000-255 | 16-bit master-dimmer fine |
| 14 | Pan | 000-255 | Pan position |
| 15 | Pan Fine | 000-255 | 16-bit pan fine |
| 16 | Tilt | 000-255 | Tilt position |
| 17 | Tilt Fine | 000-255 | 16-bit tilt fine |
| 18 | Function | 000-255 | Reserved |
| 19 | Reset | 000-127 / 128-255 | Reserved / fixture reset |
| 20 | Zoom | 000-255 | Narrow to wide |
| 21 | Lens Rotation | 000-127 / 128-190 / 191-192 / 193-255 | 0°-60° position / CCW fast-slow / stop / CW slow-fast |
| 22 | Shape Selection | 000-255 | Select a built-in foreground shape; map in Section 6 |
| 23 | Shape Speed | 000-063 / 064-158 / 159-160 / 161-255 | Static / fast-slow / stop / slow-fast |
| 24 | Shape Fade | 000-015 / 016-255 | No fade / fade enabled with curve selection |
| 25 | Shape Red | 000-255 | Foreground red intensity |
| 26 | Shape Green | 000-255 | Foreground green intensity |
| 27 | Shape Blue | 000-255 | Foreground blue intensity |
| 28 | Shape White | 000-255 | Foreground white intensity |
| 29 | Shape Dimmer | 000-255 | Foreground master intensity |
| 30 | Background Dimmer | 000-255 | Background master intensity |
| 31 | Shape Transition | 000-004 / 005-255 | No transition delay / 100 ms to 4 s |
| 32 | Shape Offset | 000-255 | Shape angle or density |
| 33 | Foreground Strobe | 000-003 / 004-103 / 104-107 / 108-207 / 208-212 / 203-251 / 252-255 | Blackout / pulse slow-fast / open / even strobe slow-fast / open / random slow-fast / open |
| 34 | Background Strobe | 000-003 / 004-103 / 104-107 / 108-207 / 208-212 / 203-251 / 252-255 | Blackout / pulse slow-fast / open / even strobe slow-fast / open / random slow-fast / open |
| 35 | Background Selection | 000-008 / 009-023 / 024-255 | No function / select background pattern / no selection |

PRINTED OVERLAP IN CH 33 AND CH 34  The source prints the random-strobe band as 203-251, which overlaps the 208-212 open band. The value is retained here rather than silently changed; verify the firmware behavior before programming.

## 5.3  Pixel Personalities

| Mode | Base channels | Pixel channels | Pixel components |
| --- | --- | --- | --- |
| 78 CH | 1-21 = 21 CH | 22-78 | 19 pixels x RGB |
| 92 CH | 1-35 = 35 CH | 36-92 | 19 pixels x RGB |
| 97 CH | 1-21 = 21 CH | 22-97 | 19 pixels x RGBW |

PIXEL ORDER  Confirm physical LED numbering with a low-level test scene before creating position-dependent graphics.

## 5.3.1  78-Channel Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-24 | LED 1 | 000-255 each | R / G / B; 0-100% per component |
| 25-27 | LED 2 | 000-255 each | R / G / B; 0-100% per component |
| 28-30 | LED 3 | 000-255 each | R / G / B; 0-100% per component |
| 31-33 | LED 4 | 000-255 each | R / G / B; 0-100% per component |
| 34-36 | LED 5 | 000-255 each | R / G / B; 0-100% per component |
| 37-39 | LED 6 | 000-255 each | R / G / B; 0-100% per component |
| 40-42 | LED 7 | 000-255 each | R / G / B; 0-100% per component |
| 43-45 | LED 8 | 000-255 each | R / G / B; 0-100% per component |
| 46-48 | LED 9 | 000-255 each | R / G / B; 0-100% per component |
| 49-51 | LED 10 | 000-255 each | R / G / B; 0-100% per component |
| 52-54 | LED 11 | 000-255 each | R / G / B; 0-100% per component |
| 55-57 | LED 12 | 000-255 each | R / G / B; 0-100% per component |
| 58-60 | LED 13 | 000-255 each | R / G / B; 0-100% per component |
| 61-63 | LED 14 | 000-255 each | R / G / B; 0-100% per component |
| 64-66 | LED 15 | 000-255 each | R / G / B; 0-100% per component |
| 67-69 | LED 16 | 000-255 each | R / G / B; 0-100% per component |
| 70-72 | LED 17 | 000-255 each | R / G / B; 0-100% per component |
| 73-75 | LED 18 | 000-255 each | R / G / B; 0-100% per component |
| 76-78 | LED 19 | 000-255 each | R / G / B; 0-100% per component |

## 5.3.2  92-Channel Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 36-38 | LED 1 | 000-255 each | R / G / B; 0-100% per component |
| 39-41 | LED 2 | 000-255 each | R / G / B; 0-100% per component |
| 42-44 | LED 3 | 000-255 each | R / G / B; 0-100% per component |
| 45-47 | LED 4 | 000-255 each | R / G / B; 0-100% per component |
| 48-50 | LED 5 | 000-255 each | R / G / B; 0-100% per component |
| 51-53 | LED 6 | 000-255 each | R / G / B; 0-100% per component |
| 54-56 | LED 7 | 000-255 each | R / G / B; 0-100% per component |
| 57-59 | LED 8 | 000-255 each | R / G / B; 0-100% per component |
| 60-62 | LED 9 | 000-255 each | R / G / B; 0-100% per component |
| 63-65 | LED 10 | 000-255 each | R / G / B; 0-100% per component |
| 66-68 | LED 11 | 000-255 each | R / G / B; 0-100% per component |
| 69-71 | LED 12 | 000-255 each | R / G / B; 0-100% per component |
| 72-74 | LED 13 | 000-255 each | R / G / B; 0-100% per component |
| 75-77 | LED 14 | 000-255 each | R / G / B; 0-100% per component |
| 78-80 | LED 15 | 000-255 each | R / G / B; 0-100% per component |
| 81-83 | LED 16 | 000-255 each | R / G / B; 0-100% per component |
| 84-86 | LED 17 | 000-255 each | R / G / B; 0-100% per component |
| 87-89 | LED 18 | 000-255 each | R / G / B; 0-100% per component |
| 90-92 | LED 19 | 000-255 each | R / G / B; 0-100% per component |

92 CH STRUCTURE  This footprint contains 35 base/shape channels plus 57 RGB pixel channels: 19 pixels x RGB.

## 5.3.3  97-Channel Pixel Map

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22-25 | LED 1 | 000-255 each | R / G / B / W; 0-100% per component |
| 26-29 | LED 2 | 000-255 each | R / G / B / W; 0-100% per component |
| 30-33 | LED 3 | 000-255 each | R / G / B / W; 0-100% per component |
| 34-37 | LED 4 | 000-255 each | R / G / B / W; 0-100% per component |
| 38-41 | LED 5 | 000-255 each | R / G / B / W; 0-100% per component |
| 42-45 | LED 6 | 000-255 each | R / G / B / W; 0-100% per component |
| 46-49 | LED 7 | 000-255 each | R / G / B / W; 0-100% per component |
| 50-53 | LED 8 | 000-255 each | R / G / B / W; 0-100% per component |
| 54-57 | LED 9 | 000-255 each | R / G / B / W; 0-100% per component |
| 58-61 | LED 10 | 000-255 each | R / G / B / W; 0-100% per component |
| 62-65 | LED 11 | 000-255 each | R / G / B / W; 0-100% per component |
| 66-69 | LED 12 | 000-255 each | R / G / B / W; 0-100% per component |
| 70-73 | LED 13 | 000-255 each | R / G / B / W; 0-100% per component |
| 74-77 | LED 14 | 000-255 each | R / G / B / W; 0-100% per component |
| 78-81 | LED 15 | 000-255 each | R / G / B / W; 0-100% per component |
| 82-85 | LED 16 | 000-255 each | R / G / B / W; 0-100% per component |
| 86-89 | LED 17 | 000-255 each | R / G / B / W; 0-100% per component |
| 90-93 | LED 18 | 000-255 each | R / G / B / W; 0-100% per component |
| 94-97 | LED 19 | 000-255 each | R / G / B / W; 0-100% per component |

# 6  Shape Engine

The 35 CH and 92 CH personalities divide the 19-pixel face into a selected foreground shape and the remaining background. The center LED is Ring 1, the next six LEDs form Ring 2, and the outer twelve LEDs form Ring 3.

| Layer / task | Channels | Programming guidance |
| --- | --- | --- |
| Overall output | CH 11 Strobe; CH 12 Master Dimmer | Open the common output before evaluating either layer. |
| Foreground shape | CH 22 | Select the built-in foreground pattern. |
| Foreground motion | CH 23 / 24 / 31 / 32 | Set speed, fade, transition and angle/density. |
| Foreground color / level | CH 25-29; CH 33 | Set RGBW, foreground dimmer and foreground strobe. |
| Background color | CH 1 / 3 / 5 / 7 | Use the common RGBW coarse channels for background color. |
| Background level / group | CH 30 / 34 / 35 | Set background dimmer, strobe and selected ring group. |
| Lens effect | CH 21 | Add continuous honeycomb-lens rotation as required. |

## 6.1 Basic Programming Sequence

- Select the 35 CH or 92 CH personality, open CH 11 Strobe and raise CH 12 Master Dimmer.

- Raise CH 29 Shape Dimmer, open CH 33 Foreground Strobe and set CH 25-28 foreground RGBW.

- Select a shape with CH 22 and adjust CH 23, 24, 31 and 32.

- For a background layer, set CH 1, 3, 5 and 7, then raise CH 30 and open CH 34; select a group with CH 35.

- Add CH 21 lens rotation and verify the complete cue on the production firmware.

SOURCE COUNT DISCREPANCY  The prose says 54 foreground shapes are available, while the printed table continues through Shape 64 and leaves the description for DMX value 66 blank. The table below follows the printed value map.

## 6.2 Foreground Shape Value Map

| DMX Value | Shape | Description | Source note |
| --- | --- | --- | --- |
| 000-007 | 0 | Off |  |
| 008 | 1 | Inner ring |  |
| 009 | 2 | Middle ring |  |
| 010 | 3 | Outer ring |  |
| 011 | 4 | Reserved / no function | K20 reserved |
| 012 | 5 | Inner + middle rings |  |
| 013 | 6 | Inner + outer rings |  |
| 014 | 7 | Reserved / no function | K20 reserved |
| 015 | 8 | Inner-middle-outer single-ring sequence | Random color when Shape RGBW = 0 |
| 016 | 9 | Inner + middle + outer full-ring sequence | Random color when Shape RGBW = 0 |
| 017 | 10 | Single-ring sequence with on/off steps | Random color when Shape RGBW = 0 |
| 018 | 11 | Full-ring sequence with on/off steps | Random color when Shape RGBW = 0 |
| 019 | 12 | Random single pixel | Random color when Shape RGBW = 0 |
| 020 | 13 | Random two pixels | Random color when Shape RGBW = 0 |
| 021 | 14 | Rainbow 1 | Dynamic speed adjustable |
| 022 | 15 | Rainbow 2 | Fixed motion speed; color-change speed adjustable |
| 023 | 16 | Fan |  |
| 024 | 17 | Bar 1 |  |
| 025 | 18 | Half moon |  |
| 026 | 19 | Large triangle chase |  |
| 027 | 20 | Split 1 |  |
| 028 | 21 | Arc 1 |  |
| 029 | 22 | Arc 2 |  |
| 030 | 23 | Bar 2 | Size variation |
| 031 | 24 | Random burst | Random color when Shape RGBW = 0 |
| 032 | 25 | Split 2 |  |
| 033 | 26 | Reserved / no function | K20 reserved |
| 034 | 27 | Reserved / no function | K20 reserved |
| 035 | 28 | Small triangle switch |  |
| 036 | 29 | Diagonal, 6 pixels |  |
| 037 | 30 | Diagonal, 4 pixels |  |
| 038 | 31 | Bar left/right switch |  |
| 039 | 32 | Bar up/down switch |  |
| 040 | 33 | Bar 3 | Spiral switch |
| 041 | 34 | Reserved / no function | K20 reserved |
| 042 | 35 | Arc left/right switch |  |
| 043 | 36 | Reserved / no function | K20 reserved |
| 044 | 37 | Arc up/down switch |  |
| 045 | 38 | Mirror |  |
| 046 | 39 | Pixel animation 1 |  |
| 047 | 40 | Pixel animation 2 |  |
| 048 | 41 | Pixel animation 3 |  |
| 049 | 42 | Pixel animation 4 |  |
| 050 | 43 | Pixel animation 5 |  |
| 051 | 44 | Half arc |  |
| 052 | 45 | Concave/convex arc |  |
| 053 | 46 | Pixel animation 6 |  |
| 054 | 47 | Vertical two-pixel switch |  |
| 055 | 48 | Two-pixel segmented jump |  |
| 056 | 49 | Outer-ring pair + middle-ring single-pixel split |  |
| 057 | 50 | Concave/convex 1 |  |
| 058 | 51 | Concave/convex 2 |  |
| 059 | 52 | Concave/convex 3 |  |
| 060 | 53 | Vertical single-pixel jump |  |
| 061 | 54 | Vertical random jump |  |
| 062 | 55 | Horizontal random jump | Random color when Shape RGBW = 0 |
| 063 | 56 | Outer-ring random jump | Random color when Shape RGBW = 0 |
| 064 | 57 | Reserved / no function | K20 reserved |
| 065 | 58 | Outer + middle rings random |  |
| 066 | 59 | Undocumented in source | Description cell is blank |
| 067 | 60 | Middle-ring single-pixel switch |  |
| 068 | 61 | Outer-ring single-pixel switch |  |
| 069 | 62 | Reserved / no function | K20 reserved |
| 070 | 63 | Outer-middle-inner single-pixel spiral |  |
| 071-255 | 64 | Reserved |  |

# 7  Photometric Data

Source illuminance values for the page labeled 11° beam angle:

| Color | 3 m | 4 m | 5 m | 6 m | 8 m | 10 m |
| --- | --- | --- | --- | --- | --- | --- |
| Red | 22,000 lx | 21,500 lx | 14,620 lx | 10,000 lx | 5,900 lx | 3,400 lx |
| Green | 43,000 lx | 39,500 lx | 27,200 lx | 18,700 lx | 10,900 lx | 6,730 lx |
| Blue | 77,000 lx | 62,800 lx | 45,100 lx | 31,300 lx | 18,200 lx | 11,350 lx |
| White | 72,000 lx | 60,500 lx | 43,000 lx | 29,500 lx | 17,200 lx | 10,600 lx |
| RGBW | 132,100 lx | 124,600 lx | 99,200 lx | 77,600 lx | 49,000 lx | 30,600 lx |

Source illuminance values for the page labeled 55° beam angle:

| Color | 3 m | 4 m | 5 m |
| --- | --- | --- | --- |
| Red | 1,190 lx | 680 lx | 460 lx |
| Green | 2,170 lx | 1,250 lx | 830 lx |
| Blue | 3,940 lx | 2,270 lx | 1,500 lx |
| White | 3,610 lx | 2,080 lx | 1,370 lx |
| RGBW | 10,400 lx | 6,000 lx | 3,960 lx |

MEASUREMENT NOTE  The photometric page labels the tests as 11° and 55°, while the specification page states a 7°-45° zoom. Values are reproduced from the source and may vary with LED binning, calibration, mains conditions, optical cleanliness and measurement method.

# 8  Inspection, Maintenance and Cleaning

DISCONNECT POWER  Isolate the fixture from AC power, prevent reconnection and allow all parts to cool before inspection or cleaning.

## 8.1 Condition-Based Inspection

Check all installation screws, clamps, brackets, mounting points and the safety cable for looseness, wear, deformation or corrosion.

Inspect the housing, lens, connectors and supporting structure for damage or deformation.

Check that pan, tilt, zoom and honeycomb-lens rotation move smoothly without imbalance, impact, grinding noise or obstruction.

Inspect the power cable for cuts, abrasion, overheating or damaged insulation.

Verify that all 19 LEDs produce consistent RGBW output at matched test values and review any stored sensor or temperature fault.

## 8.2 Cleaning Schedule

- Clean the external optical surface approximately weekly, or more often in dusty, smoky or touring environments.

- Clean the rear fan and air inlets approximately monthly and remove any blockage from the dust-cover intake.

- Have a qualified electrical technician inspect internal connections approximately every three months, adjusted for operating conditions.

- Use a soft, lint-free cloth lightly dampened with water or a mild suitable cleaner. Do not use abrasive material, alcohol, aggressive solvent or excessive liquid.

- Allow the fixture to dry completely and inspect every connector and attachment before reconnecting power.

## 8.3 Service

There are no user-serviceable internal parts documented in the source. Internal cleaning, fan or power-supply service, motor work, firmware service and calibration must be performed by qualified personnel. Obtain replacement parts from the supplier or authorized dealer.

# 9  Troubleshooting and Service

| Symptom | Checks / corrective action |
| --- | --- |
| No power or display | Verify the protected branch supply, product-label rating, power connector seating and cable condition. Disconnect immediately if a cable or connector is damaged. |
| No DMX response | Confirm DMX mode, start address and personality; inspect polarity and cable continuity; test directly from the controller; terminate the final fixture when required. |
| Incorrect channels | Confirm that the controller profile matches 21, 35, 78, 92 or 97 CH. Do not use an unverified profile from another fixture family. |
| Output is dark | Raise Master Dimmer, open Strobe and set RGBW levels; review the selected No DMX state and any manual-control values. |
| Uneven pixel color | Send equal RGBW test values; inspect the lens and review output calibration. Record values before adjustment. |
| Pan / tilt fault | Remove mechanical obstructions, verify invert settings, execute Reset All and review the pan/tilt sensor information. |
| Zoom or lens does not move | Remove obstructions, reset the fixture, review zoom/rotation sensor status and refer persistent mechanical faults to qualified service. |
| Unexpected shape behavior | Verify 35 or 92 CH mode, foreground/background dimmers, source strobe overlap and firmware-specific shape mapping. |
| Fixture overheats or shuts down | Disconnect power, allow cooling, clean the fan and external airflow paths, select a suitable fan mode and refer persistent faults to qualified service. |

## 9.1 Information to Record Before Service

Model and serial number from the actual product label.

Mains voltage, firmware versions, operating hours and selected personality.

Exact sensor/error indication and the steps required to reproduce the fault.

Controller model/profile, start address, cabling layout and a copy of the affected cue values.

PRODUCTION VERIFICATION  Before publication or sale, confirm the final product-label mains range, zoom endpoints, dimensions, connector configuration, RDM behavior and firmware-specific response for the overlapping strobe values and undocumented Shape 59.