PROFESSIONAL LED WASH FIXTURE

AL3715WZ

37 x 15 W LED WASH MOVING HEAD

USER MANUAL

English Edition  |  Professionally edited from the supplied manual

37 x 15 W RGBW LEDs | 5-60 degree motorized zoom | Pixel-area effects

| Application | Maximum Power | Input | DMX Modes |
| --- | --- | --- | --- |
| Indoor | 600 W | AC 90-240 V, 50/60 Hz | 14 / 17 / 20 / 26 / 48 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Retain it for future reference.

Contents

1  Product Information & Safety

2  Installation & DMX Connections

3  Display Menu & Operation

4  DMX512 Control & Channel Charts

5  Inspection, Maintenance & Cleaning

EDITORIAL NOTE  Grammar, terminology, menu labels, and channel descriptions have been standardized for professional lighting use. Source values are retained; repeated RGBW area blocks are expanded for clear programming.

1  Product Information & Safety

1.1 Intended Use

The AL3715WZ is a professional indoor RGBW wash moving head with 37 individually lensed 15 W four-color LEDs, motorized zoom, built-in effects, and selectable pixel-area control. It supports DMX512, automatic programs, sound-active operation, and master/slave operation. Typical applications include live productions, theatres, studios, clubs, and entertainment venues.

Inspect the fixture and accessories immediately after unpacking. Do not operate a product that appears damaged in transit.

1.2 Package Contents

| Item | Quantity |
| --- | --- |
| AL3715WZ fixture | 1 |
| Power cable | 1 |
| 3-pin DMX cable | 1 |
| Safety cable | 1 |
| Omega bracket | 2 |
| User manual | 1 |

1.3 Safety Instructions

Installation, operation, and service must be performed by qualified personnel familiar with professional entertainment lighting.

Connect only to the rated AC supply and a properly grounded Protection Class I circuit.

Disconnect mains power before cleaning, servicing, or when the fixture is not in use. Pull the plug, never the cable.

After a severe temperature change, allow the fixture to reach room temperature before applying power so condensation can evaporate.

Do not operate the fixture if the power cable, housing, lens, bracket, or safety attachment is damaged.

Keep ventilation openings unobstructed and maintain at least 0.5 m clearance from walls or objects that restrict airflow.

Keep illuminated objects at least 0.2 m from the light output and never project the beam onto combustible materials.

Do not lift or carry the fixture by the moving head. Protect it from impact, excessive vibration, moisture, and heavy dust.

Strobe lighting may trigger photosensitive seizures. Follow venue and local warning requirements.

THERMAL LIMITATION  The supplied manual instructs operators not to run all RGBW emitters at full output continuously for more than 10 minutes. Observe this limitation and ensure unrestricted cooling airflow.

1.4 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 90-240 V, 50/60 Hz |
| Maximum power | 600 W |
| Light source | 37 x 15 W high-brightness RGBW 4-in-1 LEDs |
| Color system | RGBW additive color mixing |
| Zoom range | 5-60 degrees, motorized |
| Pan / Tilt | Pan: 540 degrees; Tilt: 270 degrees; 16-bit fine control |
| Dimming | 0-100% continuous linear dimming |
| Strobe | 1-25 Hz; synchronized, pulse, lightning, and random modes |
| Control modes | DMX512, master/slave, auto programs, sound-active |
| DMX personalities | 14 CH / 17 CH / 20 CH / 26 CH / 48 CH |
| Display | LCD control display |
| Rated LED life | Average 50,000 hours |
| Application | Indoor use |
| Listed fixture dimensions | 386 x 423 x 263 mm |
| Dimension drawing | Approx. 385.88 x 462.84 x 263.04 mm maximum envelope |
| Net / gross weight | 13.5 kg / 15 kg |
| Protection | Fan-assisted cooling with automatic temperature control |

DIMENSION NOTE  The source specification and supplied dimension drawing show different overall heights. Both values are recorded above; use the dimension drawing when checking installation clearance.

Supplied dimension drawing. Values are in millimetres.

2  Installation & DMX Connections

2.1 Rigging

Before mounting, verify that the supporting structure and all rigging hardware can support at least ten times the fixture weight. Fit appropriately rated clamps to both omega brackets with suitable M10 fasteners. Complete all rigging before connecting mains power.

Install the fixture outside normal walking and seating areas and beyond unauthorized reach.

Use both omega brackets for overhead suspension and fully engage every quarter-turn fastener.

Attach at least one correctly rated safety cable to a designated safety point on the base.

Never attach the safety cable to a carrying handle.

Inspect clamps, fasteners, brackets, safety cables, and the supporting structure before every use.

Rigging arrangement shown in the supplied manual.

2.2 DMX512 Data Connection

Connect the controller DMX output to the fixture DMX input with a shielded twisted-pair DMX cable. Connect each fixture output to the next fixture input. The supplied diagram uses 3-pin XLR connectors; 5-pin wiring follows the same DMX512 data principles when fitted.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Ground / shield |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  For long cable runs or electrically noisy environments, install a 120 ohm resistor between pins 2 and 3 in a male XLR plug and connect it to the DMX output of the final fixture.

3  Display Menu & Operation

3.1 Menu Map

| Menu | Option | Range / Function |
| --- | --- | --- |
| DMX | DMX Address | 001-512 |
| DMX | Run Mode | DMX / Auto 1 / Auto 2 / Sound Control |
| DMX | Channel Mode | 14 / 17 / 20 / 26 / 48 CH |
| DMX | Pan Invert | No / Yes |
| DMX | Tilt Invert | No / Yes |
| DMX | Hall Sensor Correction | No / Yes |
| DMX | Encoder Correction | No / Yes |
| DMX | Hold on DMX Loss | No / Yes |
| Display | Screen Saver | No / Yes |
| Display | Display Orientation | Auto / Normal / Inverted |
| System | Update Slave | No / Yes |
| Display | Language | Chinese / English |
| System | Restore Settings | Confirm / Cancel |
| Manual | Pan / Tilt | 001-255 |
| Calibration | Focus (Zoom) / Rotation | 001-255 |
| Calibration | Red / Green / Blue / White Balance | 001-255 |
| Calibration | LED Power | 010-255 |
| Sound | Microphone Sensitivity | 001-255 |
| System Reset | Password / Effect / Movement / All | Select and confirm |
| Information | DMX Data / Sensors / Software | View status information |

FIRMWARE NOTE  Menu wording and available items may vary slightly with installed firmware. “Focus” in the source menu refers to zoom calibration.

4  DMX512 Control

Set a unique DMX start address for each independently controlled fixture. The highest usable start address depends on the selected personality. Reset requires the specified value to be held for 3 seconds.

PROGRAMMING NOTE  The source charts do not provide sub-ranges for color, static, dynamic, or background effect macros. These channels are therefore documented as continuous 000-255 selectors without invented boundaries.

4.1  14 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine (16-bit) |
| 3 | 000-255 | Tilt, 0-270 degrees |
| 4 | 000-255 | Tilt Fine (16-bit) |
| 5 | 000-255 | Pan/Tilt Speed, fast to slow |
| 6 | 000-255 | Motorized Zoom, 5-60 degrees |
| 7 | 000-255 | Master Dimmer, 0-100% |
| 8 | 000-255 | Strobe; see 4.7 |
| 9 | 000-255 | Red, 0-100% |
| 10 | 000-255 | Green, 0-100% |
| 11 | 000-255 | Blue, 0-100% |
| 12 | 000-255 | White, 0-100% |
| 13 | 000-255 | Auto Program; see 4.7 |
| 14 | 000-255 | Reset; see 4.7 |

4.2  17 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine (16-bit) |
| 3 | 000-255 | Tilt, 0-270 degrees |
| 4 | 000-255 | Tilt Fine (16-bit) |
| 5 | 000-255 | Pan/Tilt Speed, fast to slow |
| 6 | 000-255 | Motorized Zoom, 5-60 degrees |
| 7 | 000-255 | Master Dimmer, 0-100% |
| 8 | 000-255 | Strobe; see 4.7 |
| 9 | 000-255 | Red, 0-100% |
| 10 | 000-255 | Green, 0-100% |
| 11 | 000-255 | Blue, 0-100% |
| 12 | 000-255 | White, 0-100% |
| 13 | 000-255 | Auto Program; see 4.7 |
| 14 | 000-255 | Color Temperature; see 4.7 |
| 15 | 000-255 | Static Effect Macro |
| 16 | 000-255 | Dynamic Effect Macro |
| 17 | 000-255 | Reset; see 4.7 |

4.3  20 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine (16-bit) |
| 3 | 000-255 | Tilt, 0-270 degrees |
| 4 | 000-255 | Tilt Fine (16-bit) |
| 5 | 000-255 | Pan/Tilt Speed, fast to slow |
| 6 | 000-255 | Motorized Zoom, 5-60 degrees |
| 7 | 000-255 | Master Dimmer, 0-100% |
| 8 | 000-255 | Strobe; see 4.7 |
| 9 | 000-255 | Red, 0-100% |
| 10 | 000-255 | Green, 0-100% |
| 11 | 000-255 | Blue, 0-100% |
| 12 | 000-255 | White, 0-100% |
| 13 | 000-255 | Color Temperature; see 4.7 |
| 14 | 000-255 | Color Macro |
| 15 | 000-255 | Static Effect Macro |
| 16 | 000-255 | Dynamic Effect Macro |
| 17 | 000-255 | Background Effect |
| 18 | 000-255 | Effect Speed |
| 19 | 000-255 | Auto Program; see 4.7 |
| 20 | 000-255 | Reset; see 4.7 |

4.4  26 CH - Four-Area RGBW Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine (16-bit) |
| 3 | 000-255 | Tilt, 0-270 degrees |
| 4 | 000-255 | Tilt Fine (16-bit) |
| 5 | 000-255 | Pan/Tilt Speed, fast to slow |
| 6 | 000-255 | Motorized Zoom, 5-60 degrees |
| 7 | 000-255 | Master Dimmer, 0-100% |
| 8 | 000-255 | Strobe; see 4.7 |
| 9 | 000-255 | Area 1 Red, 0-100% |
| 10 | 000-255 | Area 1 Green, 0-100% |
| 11 | 000-255 | Area 1 Blue, 0-100% |
| 12 | 000-255 | Area 1 White, 0-100% |
| 13 | 000-255 | Area 2 Red, 0-100% |
| 14 | 000-255 | Area 2 Green, 0-100% |
| 15 | 000-255 | Area 2 Blue, 0-100% |
| 16 | 000-255 | Area 2 White, 0-100% |
| 17 | 000-255 | Area 3 Red, 0-100% |
| 18 | 000-255 | Area 3 Green, 0-100% |
| 19 | 000-255 | Area 3 Blue, 0-100% |
| 20 | 000-255 | Area 3 White, 0-100% |
| 21 | 000-255 | Area 4 Red, 0-100% |
| 22 | 000-255 | Area 4 Green, 0-100% |
| 23 | 000-255 | Area 4 Blue, 0-100% |
| 24 | 000-255 | Area 4 White, 0-100% |
| 25 | 000-255 | Auto Program; see 4.7 |
| 26 | 000-255 | Reset; see 4.7 |

AREA MAPPING  Channels 9-24 form four consecutive RGBW control blocks: Area 1 = 9-12, Area 2 = 13-16, Area 3 = 17-20, and Area 4 = 21-24.

4.5  48 CH - Seven-Area RGBW Effect Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-540 degrees |
| 2 | 000-255 | Pan Fine (16-bit) |
| 3 | 000-255 | Tilt, 0-270 degrees |
| 4 | 000-255 | Tilt Fine (16-bit) |
| 5 | 000-255 | Pan/Tilt Speed, fast to slow |
| 6 | 000-255 | Motorized Zoom, 5-60 degrees |
| 7 | 000-255 | Master Dimmer, 0-100% |
| 8 | 000-255 | Strobe; see 4.7 |
| 9 | 000-255 | Area 1 Red, 0-100% |
| 10 | 000-255 | Area 1 Green, 0-100% |
| 11 | 000-255 | Area 1 Blue, 0-100% |
| 12 | 000-255 | Area 1 White, 0-100% |
| 13 | 000-255 | Area 2 Red, 0-100% |
| 14 | 000-255 | Area 2 Green, 0-100% |
| 15 | 000-255 | Area 2 Blue, 0-100% |
| 16 | 000-255 | Area 2 White, 0-100% |
| 17 | 000-255 | Area 3 Red, 0-100% |
| 18 | 000-255 | Area 3 Green, 0-100% |
| 19 | 000-255 | Area 3 Blue, 0-100% |
| 20 | 000-255 | Area 3 White, 0-100% |
| 21 | 000-255 | Area 4 Red, 0-100% |
| 22 | 000-255 | Area 4 Green, 0-100% |
| 23 | 000-255 | Area 4 Blue, 0-100% |
| 24 | 000-255 | Area 4 White, 0-100% |
| 25 | 000-255 | Area 5 Red, 0-100% |
| 26 | 000-255 | Area 5 Green, 0-100% |
| 27 | 000-255 | Area 5 Blue, 0-100% |
| 28 | 000-255 | Area 5 White, 0-100% |
| 29 | 000-255 | Area 6 Red, 0-100% |
| 30 | 000-255 | Area 6 Green, 0-100% |
| 31 | 000-255 | Area 6 Blue, 0-100% |
| 32 | 000-255 | Area 6 White, 0-100% |
| 33 | 000-255 | Area 7 Red, 0-100% |
| 34 | 000-255 | Area 7 Green, 0-100% |
| 35 | 000-255 | Area 7 Blue, 0-100% |
| 36 | 000-255 | Area 7 White, 0-100% |
| 37 | 000-255 | Color Temperature; see 4.7 |
| 38 | 000-255 | Color Macro |
| 39 | 000-255 | Static Effect Macro |
| 40 | 000-255 | Dynamic Effect Macro |
| 41 | 000-255 | Background Red, 0-100% |
| 42 | 000-255 | Background Green, 0-100% |
| 43 | 000-255 | Background Blue, 0-100% |
| 44 | 000-255 | Background White, 0-100% |
| 45 | 000-127 | Effect Direction: forward |
| 45 | 128-255 | Effect Direction: reverse |
| 46 | 000-255 | Effect Speed |
| 47 | 000-255 | Auto Program; see 4.7 |
| 48 | 000-255 | Reset; see 4.7 |

BACKGROUND COLOR  Use channels 41-44 with the Dynamic Effect Macro on channel 40. The source chart labels these as background-color channels.

4.6  Personality Summary

| Mode | Control Resolution |
| --- | --- |
| 14 CH | Basic movement, zoom, master dimmer/strobe, RGBW mixing, auto program, and reset. |
| 17 CH | Adds color-temperature, static-effect, and dynamic-effect macro controls. |
| 20 CH | Adds color macro, background effect, and effect-speed controls. |
| 26 CH | Four independent RGBW areas, plus shared movement, zoom, dimmer/strobe, auto program, and reset. |
| 48 CH | Seven independent RGBW areas, effect macros, background RGBW, effect direction/speed, auto program, and reset. |

SOURCE CONSISTENCY NOTE  The source feature list mentions 10 controllable areas, but the documented DMX personalities define four RGBW areas in 26 CH and seven RGBW areas in 48 CH. This manual follows the channel charts and does not invent undocumented channels.

4.7  Shared Channel Behavior

| Parameter | DMX Value | Function |
| --- | --- | --- |
| Strobe | 000-063 | Synchronized Strobe |
| Strobe | 064-126 | Pulse Strobe |
| Strobe | 127-189 | Lightning Strobe |
| Strobe | 190-255 | Random Strobe |
| Color Temperature | 000-009 | No color-temperature correction |
| Color Temperature | 010-255 | Variable color temperature, approximately 2500-10000 K |
| Auto Program | 000-009 | No Function |
| Auto Program | 010-255 | Auto Program selection / run |
| Reset | 000-239 | No Function |
| Reset | 240-250 | Reset; hold for 3 seconds |
| Reset | 251-255 | No Function |

STROBE NOTE  The supplied channel charts identify four strobe types but do not define a separate open or blackout range. Values are retained exactly as supplied.

5  Inspection, Maintenance & Cleaning

Disconnect mains power and allow the fixture to cool before inspection or cleaning. There are no user-serviceable parts inside the fixture. Internal service must be performed by qualified personnel using genuine replacement parts.

5.1 Inspection

Confirm that all installation screws and fasteners are tight and free from corrosion.

Check the housing, lenses, brackets, suspension points, and rigging hardware for deformation, cracks, or damage.

Check moving parts for excessive wear, imbalance, abnormal noise, or restricted travel.

Inspect power and data cables for cuts, crushing, heat damage, material fatigue, or contamination.

Correct every safety issue before returning the fixture to service.

5.2 Cleaning

Clean inner and outer lens surfaces regularly to prevent output loss caused by dust accumulation.

Remove dust from fans and ventilation openings regularly. Cleaning frequency depends on the environment.

Use a soft, lint-free cloth lightly moistened with water. Do not use alcohol, solvents, abrasives, or pressurized liquid.

Arrange a detailed electrical inspection by a qualified technician at least every three months, or more often in demanding environments.

END OF MANUAL  Retain this manual with the fixture. Before operation, verify the product label, local electrical code, rigging requirements, and venue safety procedures.