PROFESSIONAL LED EFFECT FIXTURE

AL5120T

5 x 120 W LED TORNADO MOVING HEAD

USER MANUAL

English Edition  |  Professionally edited from the supplied manual

5 independent RGBL heads | 5 RGB pixel halos | Motorized zoom

| Ingress Protection | Rated Power | Input | DMX Modes |
| --- | --- | --- | --- |
| IP20 | 750 W | AC 110-240 V, 50/60 Hz | 21 / 41 / 71 / 75 / 85 / 170 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Retain it for future reference.

Contents

1  Product Information & Safety

2  Installation & DMX Connections

3  Display Menu & Operation

4  DMX512 Control & Channel Charts

5  Inspection, Maintenance & Cleaning

EDITORIAL NOTE  Terminology, grammar, and control labels have been standardized for professional lighting use. Source specifications and DMX values are retained; ambiguous repeated channel structures are clarified with deterministic mapping tables.

1  Product Information & Safety

1.1 Intended Use

The AL5120T is a professional indoor moving-head effect fixture with five independently moving RGBL heads, motorized zoom, and five RGB pixel halos. It supports DMX512, automatic programs, sound-active operation, and standalone use. Typical applications include live productions, theatres, studios, clubs, and entertainment venues.

Inspect the fixture and accessories immediately after unpacking. Do not operate a product that appears damaged in transit.

1.2 Package Contents

| Item | Quantity |
| --- | --- |
| AL5120T fixture | 1 |
| Power cable | 1 |
| 3-pin DMX cable | 1 |
| Safety cable | 1 |
| Omega bracket | 2 |
| User manual | 1 |

1.3 Safety Instructions

This fixture is intended for professional use and must be installed and serviced by qualified personnel.

Connect only to the rated AC supply and a properly grounded Protection Class I circuit.

Disconnect the fixture from mains power before cleaning, servicing, or when it is not in use. Pull the plug, never the cable.

Do not power the fixture immediately after a severe temperature change. Allow it to reach room temperature so condensation can evaporate.

Do not use the fixture if the power cable, housing, lens, bracket, or safety attachment is damaged.

Keep ventilation openings unobstructed and maintain at least 0.5 m clearance from walls or other objects that could restrict airflow.

Keep illuminated objects at least 0.2 m from the light output, as indicated by the product marking.

Do not project the beam onto combustible materials and do not install the fixture directly on combustible surfaces.

Strobe lighting may trigger photosensitive seizures. Follow venue and local warning requirements.

The external flexible cable may be replaced only by the manufacturer, its service agent, or similarly qualified personnel.

WARNING  Never stand directly below the fixture during installation, removal, or service. Always use a correctly rated secondary safety attachment.

1.4 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 110-240 V, 50/60 Hz |
| Rated power | 750 W |
| Main light sources | 5 x 120 W RGBL LEDs (red, green, blue, lime) |
| Halo light sources | 120 x 0.5 W RGB LEDs; 24 LEDs per head |
| Rated LED life | Average 20,000 hours |
| Beam angle | 3 degrees-30 degrees |
| Beam divergence | 4 degrees-35 degrees |
| Pan / Tilt | Pan: 60 degrees; Tilt: 220 degrees |
| Dimming | 0-100% continuous; four selectable dimmer curves |
| Strobe | 1-25 Hz |
| Color functions | RGBL mixing, continuous CTO adjustment, LEE-referenced color macros |
| Control modes | DMX512, auto programs, sound-active, standalone |
| DMX personalities | 21 CH / 41 CH / 71 CH / 75 CH / 85 CH / 170 CH |
| Ingress protection | IP20 - indoor use only |
| Fixture dimensions | 1002 x 196 x 334 mm |
| Net / gross weight | 27.6 kg / 30.6 kg |
| Carton, 1 fixture | 1170 x 395 x 305 mm |
| Flight case, 2 fixtures | 1070 x 670 x 580 mm; case N.W. 38 kg; loaded G.W. 96 kg |

Front dimensions. All values are in millimetres.

Side dimensions. All values are in millimetres.

2  Installation & DMX Connections

2.1 Rigging

Before mounting, verify that the supporting structure and all rigging hardware can support at least ten times the fixture weight. Fit appropriately rated clamps to the two omega brackets using M10 fasteners. Complete all rigging before connecting mains power.

Use both omega brackets for overhead suspension.

Attach at least one correctly rated safety cable to a designated safety attachment point on the fixture.

Never attach the safety cable to a carrying handle.

Inspect clamps, fasteners, brackets, safety cables, and the supporting structure before every use.

Permitted mounting orientations shown in the supplied manual.

2.2 DMX512 Data Connection

Connect the controller DMX output to the fixture DMX input with a shielded twisted-pair DMX cable. Connect each fixture output to the next fixture input. The source fixture provides 3-pin XLR connectors; 5-pin wiring follows the same DMX512 data principles when fitted.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Ground / shield |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  For long cable runs or electrically noisy environments, install a 120 ohm resistor between pins 2 and 3 in a male XLR plug, then connect it to the DMX output of the final fixture.

3  Display Menu & Operation

3.1 Menu Map

| Menu | Option | Range / Function |
| --- | --- | --- |
| DMX Address | Address | 001-512 |
| Fixture | Channel Mode | 21 / 41 / 71 / 75 / 85 / 170 CH |
| Fixture | Run Mode | DMX / Auto |
| Fixture | Pan Invert | Off / On |
| Fixture | Tilt Invert | Off / On |
| Fixture | Fan Mode | Normal / Low / Quiet / High |
| Fixture | Dimmer Curve | Linear / Square / Inverse Square / S-Curve |
| Fixture | Dimmer Response | Smooth / Fast |
| Fixture | On DMX Loss | Blackout / Hold last value |
| Fixture | LED PWM Frequency | 900, 1000, 1100, 1200, 1300, 1400, 1500, 2500, 4000, 5000, 6000, 10000, 15000, 20000, or 25000 Hz |
| Manual | Manual Control | Control each channel manually |
| Information | Time | Current operating time / total operating time / power-cycle count |
| Information | Sensors | Pan, tilt, zoom, temperature, and additional sensor status |
| Information | Temperature | Head 1-5 temperature |
| Information | Software Version | Display, motor, and dimmer firmware versions |
| Factory | Password | 000-255 |
| Factory | Calibration | Pan, tilt, and zoom calibration for heads 1-5; 000-255 |
| Reset | Reset All | Off / Run |
| Reset | Factory Reset | Off / Run |
| Display | Language | Chinese / English |
| Display | Display Reverse | Yes / No |
| Display | Display Mode | Screen off after 60 s / always on |

FIRMWARE NOTE  The feature list also identifies sound-active control. Menu labels and available items may vary with installed firmware.

4  DMX512 Control

Set a unique DMX start address for each independently controlled fixture. The highest usable start address depends on the selected personality. Function-channel commands must remain in the specified value range for at least 3 seconds before activation.

OUTPUT NOTE  To produce light output, set the relevant dimmer above zero and set the applicable strobe channel to an open value.

4.1  21 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Pan, 0-60 degrees |
| 2 | 000-255 | Pan Fine |
| 3 | 000-255 | Tilt, 0-220 degrees |
| 4 | 000-255 | Tilt Fine |
| 5 | 000-255 | Zoom, wide to narrow |
| 6 | 000-255 | Main LED Red, 0-100% |
| 7 | 000-255 | Main LED Green, 0-100% |
| 8 | 000-255 | Main LED Blue, 0-100% |
| 9 | 000-255 | Main LED Lime, 0-100% |
| 10 | 000-255 | Main LED CTO; see 4.7 |
| 11 | 000-255 | Main LED color macro; see 4.7 |
| 12 | 000-255 | Halo Red, 0-100% |
| 13 | 000-255 | Halo Green, 0-100% |
| 14 | 000-255 | Halo Blue, 0-100% |
| 15 | 000-255 | Halo macro and speed; see 4.7 |
| 16 | 000-255 | Master Dimmer, 0-100% |
| 17 | 000-255 | Master Dimmer Fine |
| 18 | 000-255 | Main LED Strobe; see 4.7 |
| 19 | 000-255 | Halo Strobe; see 4.7 |
| 20 | 000-255 | Pan/Tilt Speed, fast to slow |
| 21 | 000-255 | Function; see 4.8 |

4.2  41 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Head 1 Pan, 0-60 degrees |
| 2 | 000-255 | Head 1 Pan Fine |
| 3 | 000-255 | Head 1 Tilt, 0-220 degrees |
| 4 | 000-255 | Head 1 Tilt Fine |
| 5 | 000-255 | Head 1 Zoom, wide to narrow |
| 6 | 000-255 | Head 2 Pan, 0-60 degrees |
| 7 | 000-255 | Head 2 Pan Fine |
| 8 | 000-255 | Head 2 Tilt, 0-220 degrees |
| 9 | 000-255 | Head 2 Tilt Fine |
| 10 | 000-255 | Head 2 Zoom, wide to narrow |
| 11 | 000-255 | Head 3 Pan, 0-60 degrees |
| 12 | 000-255 | Head 3 Pan Fine |
| 13 | 000-255 | Head 3 Tilt, 0-220 degrees |
| 14 | 000-255 | Head 3 Tilt Fine |
| 15 | 000-255 | Head 3 Zoom, wide to narrow |
| 16 | 000-255 | Head 4 Pan, 0-60 degrees |
| 17 | 000-255 | Head 4 Pan Fine |
| 18 | 000-255 | Head 4 Tilt, 0-220 degrees |
| 19 | 000-255 | Head 4 Tilt Fine |
| 20 | 000-255 | Head 4 Zoom, wide to narrow |
| 21 | 000-255 | Head 5 Pan, 0-60 degrees |
| 22 | 000-255 | Head 5 Pan Fine |
| 23 | 000-255 | Head 5 Tilt, 0-220 degrees |
| 24 | 000-255 | Head 5 Tilt Fine |
| 25 | 000-255 | Head 5 Zoom, wide to narrow |
| 26 | 000-255 | Main LEDs Red, 0-100% |
| 27 | 000-255 | Main LEDs Green, 0-100% |
| 28 | 000-255 | Main LEDs Blue, 0-100% |
| 29 | 000-255 | Main LEDs Lime, 0-100% |
| 30 | 000-255 | Main LED CTO; see 4.7 |
| 31 | 000-255 | Main LED color macro; see 4.7 |
| 32 | 000-255 | Halo Red, 0-100% |
| 33 | 000-255 | Halo Green, 0-100% |
| 34 | 000-255 | Halo Blue, 0-100% |
| 35 | 000-255 | Halo macro and speed; see 4.7 |
| 36 | 000-255 | Master Dimmer, 0-100% |
| 37 | 000-255 | Master Dimmer Fine |
| 38 | 000-255 | Main LED Strobe; see 4.7 |
| 39 | 000-255 | Halo Strobe; see 4.7 |
| 40 | 000-255 | Pan/Tilt Speed, fast to slow |
| 41 | 000-255 | Function; see 4.8 |

4.3  71 / 75 / 85 CH Modes

These three personalities use the same five-head sequence. The 75 CH mode adds an individual strobe channel to each head. The 85 CH mode adds individual dimmer, fine dimmer, and strobe channels to each head.

| Head | 71 CH Range | 75 CH Range | 85 CH Range |
| --- | --- | --- | --- |
| Head 1 | 1-13 | 1-14 | 1-16 |
| Head 2 | 14-26 | 15-28 | 17-32 |
| Head 3 | 27-39 | 29-42 | 33-48 |
| Head 4 | 40-52 | 43-56 | 49-64 |
| Head 5 | 53-65 | 57-70 | 65-80 |

| Relative Position | 71 CH Function | 75 CH Function | 85 CH Function |
| --- | --- | --- | --- |
| 1 | Pan | Pan | Pan |
| 2 | Pan Fine | Pan Fine | Pan Fine |
| 3 | Tilt | Tilt | Tilt |
| 4 | Tilt Fine | Tilt Fine | Tilt Fine |
| 5 | Red | Red | Red |
| 6 | Green | Green | Green |
| 7 | Blue | Blue | Blue |
| 8 | Lime | Lime | Lime |
| 9 | Zoom | Head Strobe | Head Dimmer |
| 10 | Halo Red | Zoom | Head Dimmer Fine |
| 11 | Halo Green | Halo Red | Head Strobe |
| 12 | Halo Blue | Halo Green | Zoom |
| 13 | Halo Macro / Speed | Halo Blue | Halo Red |
| 14 | - | Halo Macro / Speed | Halo Green |
| 15 | - | - | Halo Blue |
| 16 | - | - | Halo Macro / Speed |

MAPPING RULE  Within each head range, add the relative position minus 1 to that head range's first channel. All listed parameter channels use 000-255.

| Function | 71 CH | 75 CH | 85 CH |
| --- | --- | --- | --- |
| Master Dimmer | 66 | 71 | 81 |
| Master Dimmer Fine | 67 | 72 | 82 |
| Global Main LED Strobe | 68 | - | - |
| Global Halo Strobe | 69 | 73 | 83 |
| Pan/Tilt Speed | 70 | 74 | 84 |
| Function | 71 | 75 | 85 |

4.4  170 CH Pixel Mode

The 170 CH personality provides individual RGB control of eight halo blocks on every head. Each head occupies 33 consecutive channels.

| Head | Channel Range | Start Channel |
| --- | --- | --- |
| Head 1 | 1-33 | 1 |
| Head 2 | 34-66 | 34 |
| Head 3 | 67-99 | 67 |
| Head 4 | 100-132 | 100 |
| Head 5 | 133-165 | 133 |

| Relative Position | Function |
| --- | --- |
| 1 | Pan |
| 2 | Pan Fine |
| 3 | Tilt |
| 4 | Tilt Fine |
| 5 | Main LED Red |
| 6 | Main LED Green |
| 7 | Main LED Blue |
| 8 | Main LED Lime |
| 9 | Zoom |
| 10-12 | Halo Block 1 Red / Green / Blue |
| 13-15 | Halo Block 2 Red / Green / Blue |
| 16-18 | Halo Block 3 Red / Green / Blue |
| 19-21 | Halo Block 4 Red / Green / Blue |
| 22-24 | Halo Block 5 Red / Green / Blue |
| 25-27 | Halo Block 6 Red / Green / Blue |
| 28-30 | Halo Block 7 Red / Green / Blue |
| 31-33 | Halo Block 8 Red / Green / Blue |

MAPPING RULE  Add the relative position minus 1 to the head start channel. Every parameter uses 000-255. For halo block n, RGB begins at head start + 9 + 3 x (n - 1).

| Channel | DMX Value | Function |
| --- | --- | --- |
| 166 | 000-255 | Master Dimmer, 0-100% |
| 167 | 000-255 | Master Dimmer Fine |
| 168 | 000-255 | Main LED Strobe; see 4.7 |
| 169 | 000-255 | Pan/Tilt Speed, fast to slow |
| 170 | 000-255 | Function; see 4.8 |

4.5  Personality Summary

| Mode | Control Resolution |
| --- | --- |
| 21 CH | All heads move together; shared RGBL, CTO, color macro, halo, dimmer, and strobe control. |
| 41 CH | Independent pan, tilt, and zoom per head; shared RGBL, CTO, color macro, halo, dimmer, and strobe control. |
| 71 CH | Independent pan, tilt, RGBL, zoom, and halo per head; global dimmer and main/halo strobe. |
| 75 CH | As 71 CH, plus individual main-LED strobe per head; global dimmer and halo strobe. |
| 85 CH | As 75 CH, plus individual dimmer and fine dimmer per head; global dimmer and halo strobe. |
| 170 CH | Independent movement, RGBL, zoom, and eight RGB halo blocks per head; global dimmer and main-LED strobe. |

4.6  Common Parameter Behavior

| Parameter | DMX Behavior |
| --- | --- |
| Pan | 000-255: 0-60 degrees |
| Tilt | 000-255: 0-220 degrees |
| Zoom | 000-255: wide to narrow |
| Red / Green / Blue / Lime | 000-255: 0-100% |
| Dimmer | 000-255: 0-100% |
| Pan/Tilt Speed | 000-255: fast to slow |

4.7  Shared Macro & Strobe Tables

CTO Channel

| DMX Value | Nominal Color Temperature |
| --- | --- |
| 000 | No CTO correction |
| 001-004 | 8000 K |
| 005-009 | 7900 K |
| 010-013 | 7800 K |
| 014-018 | 7700 K |
| 019-022 | 7600 K |
| 023-027 | 7500 K |
| 028-225 | Continues in approximately 100 K steps |
| 226-229 | 3000 K |
| 230-234 | 2900 K |
| 235-238 | 2800 K |
| 239-243 | 2700 K |
| 244-247 | 2600 K |
| 248-255 | 2500 K |

SOURCE NOTE  The supplied chart omits the middle CTO bands with an ellipsis. Endpoint bands and the stated continuous 8000 K-to-2500 K progression are retained here without inventing omitted boundaries.

Color Macro Channel

| DMX Value | Color Macro |
| --- | --- |
| 000-009 | No color macro |
| 010-016 | LEE 790 - Moroccan Pink |
| 017-023 | LEE 157 - Pink |
| 024-030 | LEE 332 - Special Rose Pink |
| 031-037 | LEE 328 - Follies Pink |
| 038-044 | LEE 345 - Fuchsia Pink |
| 045-051 | LEE 194 - Surprise Pink |
| 052-058 | LEE 181 - Congo Blue |
| 059-065 | LEE 071 - Tokyo Blue |
| 066-072 | LEE 120 - Deep Blue |
| 073-079 | LEE 079 - Just Blue |
| 080-086 | LEE 132 - Medium Blue |
| 087-093 | LEE 200 - Double CT Blue |
| 094-100 | LEE 161 - Slate Blue |
| 101-107 | LEE 201 - Full CT Blue |
| 108-114 | LEE 202 - Half CT Blue |
| 115-121 | LEE 117 - Steel Blue |
| 122-128 | LEE 353 - Lighter Blue |
| 129-135 | LEE 118 - Light Blue |
| 136-142 | LEE 116 - Medium Blue-Green |
| 143-149 | LEE 124 - Dark Green |
| 150-156 | LEE 139 - Primary Green |
| 157-163 | LEE 089 - Moss Green |
| 164-170 | LEE 122 - Fern Green |
| 171-177 | LEE 738 - JAS Green |
| 178-184 | LEE 088 - Lime Green |
| 185-191 | LEE 100 - Spring Yellow |
| 192-198 | LEE 104 - Deep Amber |
| 199-205 | LEE 179 - Chrome Orange |
| 206-212 | LEE 105 - Orange |
| 213-219 | LEE 021 - Gold Amber |
| 220-226 | LEE 778 - Millennium Gold |
| 227-233 | LEE 135 - Deep Golden Amber |
| 234-255 | LEE 164 - Flame Red |

Halo Macro & Speed Channel

| DMX Value | Function |
| --- | --- |
| 000-015 | No halo macro |
| 016-031 | Macro 1, fast to slow within the band |
| 032-047 | Macro 2, fast to slow within the band |
| 048-223 | Macros 3-13 in consecutive 16-value bands, fast to slow |
| 224-239 | Macro 14, fast to slow within the band |
| 240-255 | Macro 15, fast to slow within the band |

Main LED / Halo Strobe Channel

| DMX Value | Strobe Function |
| --- | --- |
| 000-007 | Closed (blackout) |
| 008-015 | Open |
| 016-131 | Standard strobe, slow to fast |
| 132-139 | Open |
| 140-181 | Pulse: fast open / slow close, slow to fast |
| 182-189 | Open |
| 190-231 | Pulse: slow open / fast close, slow to fast |
| 232-239 | Open |
| 240-247 | Random strobe, slow to fast |
| 248-255 | Open |

4.8  Function Channel

ACTIVATION  Hold the selected function value for at least 3 seconds. Reset commands may interrupt output and movement.

| DMX Value | Function |
| --- | --- |
| 000-029 | No function |
| 030-039 | Dimmer curve: Linear |
| 040-049 | Dimmer curve: Square |
| 050-059 | Dimmer curve: Inverse Square |
| 060-069 | Dimmer curve: S-Curve |
| 070-079 | Fan mode: Standard |
| 080-089 | Fan mode: Quiet |
| 090-099 | No function |
| 100-109 | Enable LED PWM-frequency setting |
| 110-119 | Disable LED PWM-frequency setting |
| 120-122 | No function |
| 123 | LED PWM frequency: 900 Hz |
| 124 | LED PWM frequency: 1000 Hz |
| 125 | LED PWM frequency: 1100 Hz |
| 126 | LED PWM frequency: 1200 Hz |
| 127 | LED PWM frequency: 1300 Hz |
| 128 | LED PWM frequency: 1400 Hz |
| 129 | LED PWM frequency: 1500 Hz |
| 130 | LED PWM frequency: 2500 Hz |
| 131 | LED PWM frequency: 4000 Hz |
| 132 | LED PWM frequency: 5000 Hz |
| 133 | LED PWM frequency: 6000 Hz |
| 134 | LED PWM frequency: 10 kHz |
| 135 | LED PWM frequency: 15 kHz |
| 136 | LED PWM frequency: 20 kHz |
| 137 | LED PWM frequency: 25 kHz |
| 138-139 | No function |
| 140-149 | Pan/Tilt reset |
| 150-159 | Effect reset |
| 160-169 | Head inversion: On |
| 170-179 | Head inversion: Off |
| 180-189 | CCI calibration: On |
| 190-199 | CCI calibration: Off |
| 200-209 | Reset all |
| 210-219 | Dimmer response: Fast |
| 220-229 | Dimmer response: Smooth |
| 230 | Dimming start mode: High |
| 231 | Dimming start mode: Low |
| 232-255 | No function |

5  Inspection, Maintenance & Cleaning

Disconnect mains power and allow the fixture to cool before inspection or cleaning. There are no user-serviceable parts inside the fixture. Internal service must be performed by qualified personnel using genuine replacement parts.

5.1 Inspection

Confirm that all installation screws and fasteners are tight and free from corrosion.

Check the housing, lenses, brackets, suspension points, and rigging hardware for deformation, cracks, or other damage.

Check all moving parts for excessive wear, imbalance, abnormal noise, or restricted travel.

Inspect power and data cables for cuts, crushing, heat damage, material fatigue, or contamination.

Correct every safety issue before returning the fixture to service.

5.2 Cleaning

Clean the inner and outer lens surfaces regularly to prevent output loss caused by dust accumulation.

Remove dust from fans and ventilation openings regularly. Cleaning frequency depends on the operating environment.

Use a soft, lint-free cloth lightly moistened with water. Do not use alcohol, solvents, abrasives, or pressurized liquid.

Arrange a detailed electrical inspection by a qualified technician at least every three months, or more often in demanding environments.

END OF MANUAL  Retain this manual with the fixture. Before operation, verify the product label, local electrical code, rigging requirements, and venue safety procedures.