PROFESSIONAL LED EFFECT FIXTURE

5x120W Led Tornado IP65

USER MANUAL

English Edition  |  Based on Chinese edition dated 10 November 2025

Five independently controlled moving heads • RGB + Lime main LEDs • RGB halo effects

| Ingress Protection | Rated Power | Zoom | DMX Modes |
| --- | --- | --- | --- |
| IP65 | 800 W | 3.5°–45° | 41 / 71 / 75 / 85 / 170 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Keep it available for future reference.

Contents

1  Safety, Maintenance & Packing List

2  DMX Connection & Installation

3  Control Panel Operation

4  System Settings, Calibration & Information

5  DMX Channel Charts

6  Technical Specifications

7  Troubleshooting & Operating Precautions

8  RDM Precautions

TRANSLATION NOTE  This English manual reorganizes and edits the supplied Chinese source for clarity while preserving its technical values and operating instructions. Values in the DMX charts are reproduced from the source document.

1  Safety, Maintenance & Packing List

1.1 Maintenance

Keep the fixture dry and prevent water from entering connectors or opened housings.

Intermittent operation can help extend the service life of the fixture.

Clean fans, ventilation grilles, and lenses regularly to maintain airflow and optical performance.

Do not clean the enclosure with alcohol or other organic solvents; they may damage the finish or plastics.

Disconnect power before cleaning or servicing.

1.2 Safety Statement

For correct and safe use, read this manual before installation and operation. Follow all installation and operating procedures and retain the manual.

The product leaves the factory in good condition and with complete packaging. Damage caused by misuse or by failure to follow this manual is not covered by the warranty. Technical specifications may change without prior notice.

WARNING  Do not modify or disassemble the fixture unless you are a qualified service technician. Unqualified work can cause short circuits, burns, electric shock, fire, equipment damage, or personal injury.

1.3 Packing List

Unpack carefully and inspect for transport damage. Confirm that the following items are present:

| Item | Quantity |
| --- | --- |
| TB5 IP effect light | 1 |
| DMX signal cable | 1 |
| Power cable | 1 |
| User manual | 1 |

1.4 Product Safety Precautions

Do not operate the fixture in temperatures above 60 °C. After exposure to extreme temperature changes, allow it to return to normal temperature before applying power; condensation may damage the fixture.

Use only within the AC 100–240 V range. Connect the protective-earth conductor to a properly grounded Class I outlet.

Inspect the fixture and power cable regularly. Do not use a folded, pinched, abraded, or damaged cable, and do not bundle it carelessly with other wiring.

Do not install the fixture on a loose or vibration-prone support.

Familiarize yourself with all controls before use. Avoid shock, vibration, and excessive force during installation or operation.

Only trained personnel may operate, install, or service the fixture.

After power-off, allow adequate cooling before powering the fixture again. The source manual specifies a 20-minute cooling period.

2  DMX Connection & Installation

2.1 DMX / RS-485 Cabling

Use a standards-compliant RS-485 cable with shielding, 120 Ω characteristic impedance, 22–24 AWG conductors, and low capacitance. Do not use microphone cable. Use 3-pin or 5-pin XLR connectors as appropriate.

IMPORTANT  Signal conductors must not touch one another or the metal connector shell.

For long cable runs or electrically noisy environments, install a DMX terminator at the final fixture. Connect a 120 Ω, minimum 1/4 W resistor between XLR pins 2 and 3, then plug the terminator into the final DMX output.

2.2 Rigging and Positioning

The fixture may be placed horizontally, mounted at an angle, or inverted. Angled and inverted installations require special attention.

Install away from walkways, seating areas, and places where people can reach the fixture.

The mounting point and hardware must support at least 10 times the fixture weight.

Always use an independent secondary safety attachment such as a rated safety cable.

No person may stand under the fixture during rigging, removal, adjustment, or service.

Keep the fixture at least 0.5 m from combustible materials during installation.

Use suitable quick-lock clamps secured with M10 hardware. Route the safety cable through the designated safety point and secure it independently to the truss.

Before applying power, verify that every required mounting and safety step is complete.

Figure 1. Typical inverted mounting configurations (illustrative only).

INSPECTION  Check safety cables for wear and clamps, hooks, and screws for looseness at regular intervals. Keep people clear of the area below the fixture during setup and testing.

3  Control Panel Operation

3.1 Four-button Display Panel

Figure 2. Four-button control panel.

| Control | Function |
| --- | --- |
| MENU | Return to the previous screen or exit the current item. |
| ENTER | Open the selected item or confirm a value. |
| UP | Move upward or increase the selected value. |
| DOWN | Move downward or decrease the selected value. |

Temperature: the example display shows 27 °C. If --- °C appears, the temperature-sensor circuit may be open. If *** °C appears or the value is clearly unrealistic, the circuit may be shorted. Stop use and have the fixture serviced.

DMX address: selectable from 001 to 512; the example shows 001.

DMX mode: the selected channel mode is shown on the display.

DISPLAY CARE  Do not press the display with a pointed or sharp object.

3.2 Home Menu

Figure 3. Home menu (Chinese UI shown in source; select Language > EN for English menus).

| Menu | Purpose |
| --- | --- |
| Address | Set the DMX start address (001–512). |
| Settings | Configure operating mode, channel mode, movement, display, dimming, and other parameters. |
| Manual | Operate fixture functions manually; refer to the DMX charts for parameter behavior. |
| Calibration | Enter password 0000 to adjust power, motor travel, and other calibration values. |
| Reset | Run effect-light, movement, or full reset functions. |
| Information | View reset status, DMX monitoring, sensor status, and hardware/software versions. |

4  System Settings, Calibration & Information

4.1 Menu Structure

| Main Menu | Submenu | Third-level Menu / Parameter |
| --- | --- | --- |
| Address | 001 - 512 | Increment by channel count; decrement normally. |
| System Settings | Operating Mode | DMX / Auto 1 / Auto 2 / Sound-active |
| System Settings | Channel Mode | 41CH/71CH/75CH/85CH/170CH |
| System Settings | Network Settings | Ethernet / Universe |
| System Settings | Pan Invert | On / Off |
| System Settings | Tilt Invert | On / Off |
| System Settings | Hall Sensor Correction | On / Off |
| System Settings | Optical Sensor Correction | On / Off |
| System Settings | DMX Hold | On / Off |
| System Settings | Screen Saver | On / Off |
| System Settings | Display Orientation | On / Off / Auto |
| System Settings | Synchronized Update | On / Off |
| System Settings | Language | Chinese / English |
| System Settings | Screen Lock | On / Off |
| System Settings | Silent Mode | On / Off |
| System Settings | Dimmer Speed | Standard / Fast |
| System Settings | Dimmer Curve | Linear / Inverse Square / Square / S-Curve |
| System Settings | PWM Frequency | 10 kHz/15 kHz/20 kHz/25 kHz/900 Hz/1000 Hz/1100 Hz/1200 Hz/1300 Hz/1400 Hz/1500 Hz/2500 Hz/4000 Hz/5000 Hz/6000 Hz |
| System Settings | LED Head ID | Off / 1–5 |
| System Settings | Factory Reset | Confirm / Cancel |
| Manual Mode | Channels in the Current Mode | 0-255 |
| System Calibration | Enter Password | Fixture Calibration |
| System Reset | Effect Motor Reset | Reset Effect Motors Except Pan/Tilt |
| System Reset | Movement Reset | Pan/Tilt Reset Only |
| System Reset | Full Reset | Fixture Reset |
| System Information | Reset Status | Device Error Information |
| System Information | DMX Data Monitor | Receives Channel Values from the Console |
| System Information | Sensor Information | Sensor Status Information |
| System Information | Hardware Version | Display Hardware Version |
| System Information | Software Version | Display Software Version |

4.2 System Settings Reference

| Option | Description | Description |
| --- | --- | --- |
| Operating Mode | Fixture operating mode: DMX / Sound-active / Auto 1 / Auto 2 | Fixture operating mode: DMX / Sound-active / Auto 1 / Auto 2 |
| Operating Mode | DMX Mode | Console mode; receives DMX data. |
| Operating Mode | Auto Program 1 | Runs built-in auto program 1. |
| Operating Mode | Auto Program 2 | Runs built-in auto program 2. |
| Operating Mode | Sound-active Mode | When a strong sound is detected, the fixture runs one scene from its built-in program; otherwise it holds the last scene. |
| Channel Mode | 41CH/71CH/75CH/85CH/170CH | 41CH/71CH/75CH/85CH/170CH |
| Network Settings | Ethernet | Set the fixture IP address and subnet mask. |
| Network Settings | Universe | Set Art-Net Net and SubUni values. |
| Pan Invert | Set the X-axis movement direction. | Set the X-axis movement direction. |
| Pan Invert | Off | Normal direction. |
| Pan Invert | On | Reverse direction. |
| Tilt Invert | Set the Y-axis movement direction. | Set the Y-axis movement direction. |
| Tilt Invert | Off | Normal direction. |
| Tilt Invert | On | Reverse direction. |
| Hall Sensor Correction | Detect and correct lost steps on the X/Y axes. | Detect and correct lost steps on the X/Y axes. |
| Hall Sensor Correction | Off | Do not correct position after a lost step. |
| Hall Sensor Correction | On | Automatically correct position after a lost step. |
| Optical Sensor Correction | Detect and correct X/Y lost steps. | Detect and correct X/Y lost steps. |
| Optical Sensor Correction | Off | Do not correct position after a lost step. |
| Optical Sensor Correction | On | Automatically correct position after a lost step. |
| DMX Hold | Fixture output behavior when DMX is lost. | Fixture output behavior when DMX is lost. |
| DMX Hold | Off | On signal loss, motors and light sources return to their post-reset positions and states. |
| DMX Hold | On | On signal loss, hold the last received DMX frame. |
| Screen Saver | Display illumination duration. | Display illumination duration. |
| Screen Saver | Off | Always on. |
| Screen Saver | On | Turns the display off after a timeout when idle. |
| Display Orientation | Set the display orientation. | Set the display orientation. |
| Display Orientation | Off | Normal display. |
| Display Orientation | On | Inverted display. |
| Display Orientation | Auto | Automatically rotates the screen according to gravity. |
| Synchronized Update | Synchronize setup or calibration parameters across multiple fixtures. | Synchronize setup or calibration parameters across multiple fixtures. |
| Synchronized Update | Off | Synchronized update is disabled. |
| Synchronized Update | On | When enabled, connect multiple fixtures with DMX cable to synchronize setup and calibration data. Disconnect the DMX cable from the console first. |
| Language | Switch between Chinese and English menus. | Switch between Chinese and English menus. |
| Language | Chinese | Chinese |
| Language | English | English menu. |
| Screen Lock | Off | Screen lock is disabled. |
| Screen Lock | On | Prevents accidental input after the display turns off; unlock before use. |
| Silent Mode | Off | Fans run at full speed. |
| Silent Mode | On | Fan speed follows temperature. |
| Dimmer Speed | Fast | Fast dimming. |
| Dimmer Speed | Standard | Smooth/slow dimming. |
| Dimmer Curve | Linear | Linear curve. |
| Dimmer Curve | Square | Square-law curve. |
| Dimmer Curve | Inverse Square | Inverse-square curve. |
| Dimmer Curve | S-Curve | S-Curve |
| PWM Frequency | 10 kHz/15 kHz/20 kHz/25 kHz/900 Hz/1000 Hz/1100 Hz/1200 Hz/1300 Hz/1400 Hz/1500 Hz/2500 Hz/4000 Hz/5000 Hz/6000 Hz | 10 kHz/15 kHz/20 kHz/25 kHz/900 Hz/1000 Hz/1100 Hz/1200 Hz/1300 Hz/1400 Hz/1500 Hz/2500 Hz/4000 Hz/5000 Hz/6000 Hz |
| LED Head ID | Off | Head ID assignment is disabled. |
| LED Head ID | 1–5 | Assign the X-axis MCU head ID used to receive the corresponding channel data. Facing the display, heads are numbered 1 to 5 from left to right. |
| Factory Reset | Return fixture parameters to factory defaults. | Return fixture parameters to factory defaults. |
| Factory Reset | Cancel | Keep current settings. |
| Factory Reset | Confirm | Restore factory settings. |

4.3 Calibration

Enter password 0000. Use UP and DOWN to change a value and ENTER to confirm. Calibration changes affect output power, motor travel, sensor values, and related operating parameters; adjustment should be performed only by trained personnel.

| Option | Description |
| --- | --- |
| Home Position | In the submenu, adjust the home positions of the X-axis, Y-axis, and focus motors 1–5. Range: 0–255; 127 means no offset. |
| Travel Calibration | In the submenu, adjust X-axis, Y-axis, focus-motor travel, and RGBL LED 1–5 white balance. Range: 0–255; 127 means no offset. |
| Power | In the submenu, set the fixture's maximum power; 255 is maximum output. |
| Microphone Sensitivity | Sound-sensitivity adjustment; default 127. Lower values increase sensitivity and higher values reduce sensitivity. |
| Change Password | Set System Calibration Password |

4.4 Reset

| Option | Description |
| --- | --- |
| Effect Motor Reset | Reset color-wheel, fixed-gobo, focus, and related motors. |
| Movement Reset | Pan/Tilt Reset Only |
| Full Reset | Fixture Reset |

4.5 System Information

| Option | Description |
| --- | --- |
| Reset Status | A continuously lit red ERR indicator means an operating fault: X1–X5 communication fault (X-axis motor/display board); Y1–Y5 communication fault (Y-axis motor/display board); Z1–Z5 communication fault (LED driver/display board); X/Y optical-sensor error; or X/Y/Z Hall-sensor error for heads 1–5. |
| DMX Data Monitor | Open the submenu to view channel values numerically. |
| Sensor Information | Hall sensors: X1–X5. Sensor status: X/Y optical sensors: XXXX–XXXX. |
| Hardware Version Number | Fixture hardware information: XX.XX.XX.XX — display-board version / motor version. |
| Software Version Number | Fixture software information: XX.XX.XX.XX — display-board version / motor version. |

5  DMX Channel Charts

The following tables reproduce the channel ranges and functions from the supplied Chinese manual. DMX values are decimal (0–255). Verify that the selected fixture channel mode matches the console profile.

5.1  41CH Mode

| 41CH | Channel Name | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | Pan 1 | 0-255 | 0°–540° |
| 2 | Pan 1 Fine | 0-255 | 0°–2° |
| 3 | Tilt 1 | 0-255 | 0°–540° |
| 4 | Tilt 1 Fine | 0-255 | 0°–2° |
| 5 | Zoom 1 | 0-255 | 0%-100% |
| 6 | Pan 2 | 0-255 | 0°–540° |
| 7 | Pan 2 Fine | 0-255 | 0°–2° |
| 8 | Tilt 2 | 0-255 | 0°–540° |
| 9 | Tilt 2 Fine | 0-255 | 0°–2° |
| 10 | Zoom 2 | 0-255 | 0%-100% |
| 11 | Pan 3 | 0-255 | 0°–540° |
| 12 | Pan 3 Fine | 0-255 | 0°–2° |
| 13 | Tilt 3 | 0-255 | 0°–540° |
| 14 | Tilt 3 Fine | 0-255 | 0°–2° |
| 15 | Zoom 3 | 0-255 | 0%-100% |
| 16 | Pan 4 | 0-255 | 0°–540° |
| 17 | Pan 4 Fine | 0-255 | 0°–2° |
| 18 | Tilt 4 | 0-255 | 0°–540° |
| 19 | Tilt 4 Fine | 0-255 | 0°–2° |
| 20 | Zoom 4 | 0-255 | 0%-100% |
| 21 | Pan 5 | 0-255 | 0°–540° |
| 22 | Pan 5 Fine | 0-255 | 0°–2° |
| 23 | Tilt 5 | 0-255 | 0°–540° |
| 24 | Tilt 5 Fine | 0-255 | 0°–2° |
| 25 | Zoom 5 | 0-255 | 0%-100% |
| 26 | Main LED Red | 0-255 | Red 0%-100% |
| 27 | Main LED Green | 0-255 | Green 0%-100% |
| 28 | Main LED Blue | 0-255 | Blue 0%-100% |
| 29 | Main LED Lime | 0-255 | Lime 0%-100% |
| 30 | Main LED Color Temperature | 0 | No function |
| 30 | Main LED Color Temperature | 1-255 | 8000K-2500K |
| 31 | Main LED Color | 0-9 | No function |
| 31 | Main LED Color | 10-16 | LEE 790 - Moroccan Pink |
| 31 | Main LED Color | 17-23 | LEE 157 - Pink |
| 31 | Main LED Color | 24-30 | LEE 332 - Special Rose Pink |
| 31 | Main LED Color | 31-37 | LEE 328 - Follies Pink |
| 31 | Main LED Color | 38-44 | LEE 345 - Fuchsia Pink |
| 31 | Main LED Color | 45-51 | LEE 194 - Surprise Pink |
| 31 | Main LED Color | 52-58 | LEE 181 - Congo Blue |
| 31 | Main LED Color | 59-65 | LEE 071 - Tokyo Blue |
| 31 | Main LED Color | 66-72 | LEE 120 - Deep Blue |
| 31 | Main LED Color | 73-79 | LEE 079 - Just Blue |
| 31 | Main LED Color | 80-86 | LEE 132 - Medium Blue |
| 31 | Main LED Color | 87-93 | LEE 200 - Double CT Blue |
| 31 | Main LED Color | 94-100 | LEE 161 - State Blue |
| 31 | Main LED Color | 101-107 | LEE 201 - Full CT Blue |
| 31 | Main LED Color | 108-114 | LEE 202 - Half CT Blue |
| 31 | Main LED Color | 115-121 | LEE 117 - Steel Blue |
| 31 | Main LED Color | 122-128 | LEE 353 - Lighter Blue |
| 31 | Main LED Color | 129-135 | LEE 118 - Light Blue |
| 31 | Main LED Color | 136-142 | LEE 116 - Medium Blue Green |
| 31 | Main LED Color | 143-149 | LEE 124 - Dark Green |
| 31 | Main LED Color | 150-156 | LEE 139 - Primary Green |
| 31 | Main LED Color | 157-163 | LEE 089 - Moss Green |
| 31 | Main LED Color | 164-170 | LEE 122 - Fern Green |
| 31 | Main LED Color | 171-177 | LEE 738 - JAS Green |
| 31 | Main LED Color | 178-184 | LEE 088 - Lime Green |
| 31 | Main LED Color | 185-191 | LEE 100 - Spring Yellow |
| 31 | Main LED Color | 192-198 | LEE 104 - Deep Amber |
| 31 | Main LED Color | 199-205 | LEE 179 - Chrome Orange |
| 31 | Main LED Color | 206-212 | LEE 105 - Orange |
| 31 | Main LED Color | 213-219 | LEE 021 - Gold Amber |
| 31 | Main LED Color | 220-226 | LEE 778 - Millennium Gold |
| 31 | Main LED Color | 227-233 | LEE 135 - Deep Gold Amber |
| 31 | Main LED Color | 234-255 | LEE 164 - Flame Red |
| 32 | Halo Red | 0-255 | Red 0%-100% |
| 33 | Halo Green | 0-255 | Green 0%-100% |
| 34 | Halo Blue | 0-255 | Blue 0%-100% |
| 35 | Halo Effect & Speed | 0-15 | No function |
| 35 | Halo Effect & Speed | 16-31 | Effect 1，fast to slow |
| 35 | Halo Effect & Speed | 32-47 | Effect 2，fast to slow |
| 35 | Halo Effect & Speed | 48-63 | Effect 3，fast to slow |
| 35 | Halo Effect & Speed | 64-79 | Effect 4，fast to slow |
| 35 | Halo Effect & Speed | 80-95 | Effect 5，fast to slow |
| 35 | Halo Effect & Speed | 96-111 | Effect 6，fast to slow |
| 35 | Halo Effect & Speed | 112-127 | Effect 7，fast to slow |
| 35 | Halo Effect & Speed | 128-143 | Effect 8，fast to slow |
| 35 | Halo Effect & Speed | 144-159 | Effect 9，fast to slow |
| 35 | Halo Effect & Speed | 160-175 | Effect 10，fast to slow |
| 35 | Halo Effect & Speed | 176-191 | Effect 11，fast to slow |
| 35 | Halo Effect & Speed | 192-207 | Effect 12，fast to slow |
| 35 | Halo Effect & Speed | 208-223 | Effect 13，fast to slow |
| 35 | Halo Effect & Speed | 224-239 | Effect 14，fast to slow |
| 35 | Halo Effect & Speed | 240-255 | Effect 15，fast to slow |
| 36 | Master Dimmer | 0-255 | 0%-100% |
| 37 | Master Dimmer Fine | 0-255 | 16bit |
| 38 | Main LED Strobe | 0-7 | Blackout |
| 38 | Main LED Strobe | 8-15 | Open |
| 38 | Main LED Strobe | 16-131 | Synchronous strobe, slow to fast |
| 38 | Main LED Strobe | 132-139 | Open |
| 38 | Main LED Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 38 | Main LED Strobe | 182-189 | Open |
| 38 | Main LED Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 38 | Main LED Strobe | 232-239 | Open |
| 38 | Main LED Strobe | 240-247 | Random strobe, slow to fast |
| 38 | Main LED Strobe | 248-255 | Open |
| 39 | Halo Strobe | 0-7 | Blackout |
| 39 | Halo Strobe | 8-15 | Open |
| 39 | Halo Strobe | 16-131 | Synchronous strobe, slow to fast |
| 39 | Halo Strobe | 132-139 | Open |
| 39 | Halo Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 39 | Halo Strobe | 182-189 | Open |
| 39 | Halo Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 39 | Halo Strobe | 232-239 | Open |
| 39 | Halo Strobe | 240-247 | Random strobe, slow to fast |
| 39 | Halo Strobe | 248-255 | Open |
| 40 | Pan/Tilt Speed | 0-255 | fast to slow |
| 41 | Function | 0-29 | No function |
| 41 | Function | 30-39 | Dimmer curve: linear |
| 41 | Function | 40-49 | Dimmer curve: square |
| 41 | Function | 50-59 | Dimmer curve: inverse square |
| 41 | Function | 60-69 | Dimmer curve: S-curve |
| 41 | Function | 70-122 | No function |
| 41 | Function | 123 | 900 Hz |
| 41 | Function | 124 | 1000 Hz |
| 41 | Function | 125 | 1100 Hz |
| 41 | Function | 126 | 1200 Hz |
| 41 | Function | 127 | 1300 Hz |
| 41 | Function | 128 | 1400 Hz |
| 41 | Function | 129 | 1500 Hz |
| 41 | Function | 130 | 2500 Hz |
| 41 | Function | 131 | 4000 Hz |
| 41 | Function | 132 | 5000 Hz |
| 41 | Function | 133 | 6000 Hz |
| 41 | Function | 134 | 10 kHz |
| 41 | Function | 135 | 15 kHz |
| 41 | Function | 136 | 20 kHz |
| 41 | Function | 137 | 25 kHz |
| 41 | Function | 138-139 | No function |
| 41 | Function | 140-149 | Pan/Tilt Reset |
| 41 | Function | 150-159 | Head Reset |
| 41 | Function | 160-199 | No function |
| 41 | Function | 200-209 | Reset All |
| 41 | Function | 210-255 | No function |

5.2  71CH Mode

| 71CH | Channel Name | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | Pan 1 | 0-255 | 0° → 540° |
| 2 | Pan 1 Fine | 0-255 | 0°–2° |
| 3 | Tilt 1 | 0-255 | 0° → 540° |
| 4 | Tilt 1 Fine | 0-255 | 0°–2° |
| 5 | Main LED 1 Red | 0-255 | Red 0% → 100% |
| 6 | Main LED 1 Green | 0-255 | Green 0% → 100% |
| 7 | Main LED 1 Blue | 0-255 | Blue 0% → 100% |
| 8 | Main LED 1 Lime | 0-255 | Lime 0% → 100% |
| 9 | Zoom 1 | 0-255 | 0% → 100% |
| 10 | Halo 1 Red | 0-255 | Red 0% → 100% |
| 11 | Halo 1 Green | 0-255 | Green 0% → 100% |
| 12 | Halo 1 Blue | 0-255 | Blue 0% → 100% |
| 13 | Halo 1 Effect & Speed | 0-15 | No function |
| 13 | Halo 1 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 13 | Halo 1 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 13 | Halo 1 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 13 | Halo 1 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 13 | Halo 1 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 13 | Halo 1 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 13 | Halo 1 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 13 | Halo 1 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 13 | Halo 1 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 13 | Halo 1 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 13 | Halo 1 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 13 | Halo 1 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 13 | Halo 1 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 13 | Halo 1 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 13 | Halo 1 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 14 | Pan 2 | 0-255 | 0° → 540° |
| 15 | Pan 2 Fine | 0-255 | 0°–2° |
| 16 | Tilt 2 | 0-255 | 0° → 540° |
| 17 | Tilt 2 Fine | 0-255 | 0°–2° |
| 18 | Main LED 2 Red | 0-255 | Red 0% → 100% |
| 19 | Main LED 2 Green | 0-255 | Green 0% → 100% |
| 20 | Main LED 2 Blue | 0-255 | Blue 0% → 100% |
| 21 | Main LED 2 Lime | 0-255 | Lime 0% → 100% |
| 22 | Zoom 2 | 0-255 | 0% → 100% |
| 23 | Halo 2 Red | 0-255 | Red 0% → 100% |
| 24 | Halo 2 Green | 0-255 | Green 0% → 100% |
| 25 | Halo 2 Blue | 0-255 | Blue 0% → 100% |
| 26 | Halo 2 Effect & Speed | 0-15 | No function |
| 26 | Halo 2 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 26 | Halo 2 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 26 | Halo 2 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 26 | Halo 2 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 26 | Halo 2 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 26 | Halo 2 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 26 | Halo 2 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 26 | Halo 2 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 26 | Halo 2 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 26 | Halo 2 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 26 | Halo 2 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 26 | Halo 2 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 26 | Halo 2 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 26 | Halo 2 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 26 | Halo 2 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 27 | Pan 3 | 0-255 | 0° → 540° |
| 28 | Pan 3 Fine | 0-255 | 0°–2° |
| 29 | Tilt 3 | 0-255 | 0° → 540° |
| 30 | Tilt 3 Fine | 0-255 | 0°–2° |
| 31 | Main LED 3 Red | 0-255 | Red 0% → 100% |
| 32 | Main LED 3 Green | 0-255 | Green 0% → 100% |
| 33 | Main LED 3 Blue | 0-255 | Blue 0% → 100% |
| 34 | Main LED 3 Lime | 0-255 | Lime 0% → 100% |
| 35 | Zoom 3 | 0-255 | 0% → 100% |
| 36 | Halo 3 Red | 0-255 | Red 0% → 100% |
| 37 | Halo 3 Green | 0-255 | Green 0% → 100% |
| 38 | Halo 3 Blue | 0-255 | Blue 0% → 100% |
| 39 | Halo 3 Effect & Speed | 0-15 | No function |
| 39 | Halo 3 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 39 | Halo 3 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 39 | Halo 3 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 39 | Halo 3 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 39 | Halo 3 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 39 | Halo 3 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 39 | Halo 3 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 39 | Halo 3 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 39 | Halo 3 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 39 | Halo 3 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 39 | Halo 3 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 39 | Halo 3 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 39 | Halo 3 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 39 | Halo 3 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 39 | Halo 3 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 40 | Pan 4 | 0-255 | 0° → 540° |
| 41 | Pan 4 Fine | 0-255 | 0°–2° |
| 42 | Tilt 4 | 0-255 | 0° → 540° |
| 43 | Tilt 4 Fine | 0-255 | 0°–2° |
| 44 | Main LED 4 Red | 0-255 | Red 0% → 100% |
| 45 | Main LED 4 Green | 0-255 | Green 0% → 100% |
| 46 | Main LED 4 Blue | 0-255 | Blue 0% → 100% |
| 47 | Main LED 4 Lime | 0-255 | Lime 0% → 100% |
| 48 | Zoom 4 | 0-255 | 0% → 100% |
| 49 | Halo 4 Red | 0-255 | Red 0% → 100% |
| 50 | Halo 4 Green | 0-255 | Green 0% → 100% |
| 51 | Halo 4 Blue | 0-255 | Blue 0% → 100% |
| 52 | Halo 4 Effect & Speed | 0-15 | No function |
| 52 | Halo 4 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 52 | Halo 4 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 52 | Halo 4 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 52 | Halo 4 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 52 | Halo 4 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 52 | Halo 4 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 52 | Halo 4 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 52 | Halo 4 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 52 | Halo 4 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 52 | Halo 4 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 52 | Halo 4 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 52 | Halo 4 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 52 | Halo 4 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 52 | Halo 4 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 52 | Halo 4 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 53 | Pan 5 | 0-255 | 0° → 540° |
| 54 | Pan 5 Fine | 0-255 | 0°–2° |
| 55 | Tilt 5 | 0-255 | 0° → 540° |
| 56 | Tilt 5 Fine | 0-255 | 0°–2° |
| 57 | Main LED 5 Red | 0-255 | Red 0% → 100% |
| 58 | Main LED 5 Green | 0-255 | Green 0% → 100% |
| 59 | Main LED 5 Blue | 0-255 | Blue 0% → 100% |
| 60 | Main LED 5 Lime | 0-255 | Lime 0% → 100% |
| 61 | Zoom 5 | 0-255 | 0% → 100% |
| 62 | Halo 5 Red | 0-255 | Red 0% → 100% |
| 63 | Halo 5 Green | 0-255 | Green 0% → 100% |
| 64 | Halo 5 Blue | 0-255 | Blue 0% → 100% |
| 65 | Halo 5 Effect & Speed | 0-15 | No function |
| 65 | Halo 5 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 65 | Halo 5 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 65 | Halo 5 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 65 | Halo 5 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 65 | Halo 5 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 65 | Halo 5 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 65 | Halo 5 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 65 | Halo 5 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 65 | Halo 5 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 65 | Halo 5 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 65 | Halo 5 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 65 | Halo 5 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 65 | Halo 5 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 65 | Halo 5 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 65 | Halo 5 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 66 | Master Dimmer | 0-255 | 0% → 100% |
| 67 | Master Dimmer Fine | 0-255 | Dimmer Fine |
| 68 | Main LED Strobe | 0-7 | Blackout |
| 68 | Main LED Strobe | 8-15 | Open |
| 68 | Main LED Strobe | 16-131 | Synchronous strobe, slow to fast |
| 68 | Main LED Strobe | 132-139 | Open |
| 68 | Main LED Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 68 | Main LED Strobe | 182-189 | Open |
| 68 | Main LED Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 68 | Main LED Strobe | 232-239 | Open |
| 68 | Main LED Strobe | 240-247 | Random strobe, slow to fast |
| 68 | Main LED Strobe | 248-255 | Open |
| 69 | Halo Strobe | 0-7 | Blackout |
| 69 | Halo Strobe | 8-15 | Open |
| 69 | Halo Strobe | 16-131 | Synchronous strobe, slow to fast |
| 69 | Halo Strobe | 132-139 | Open |
| 69 | Halo Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 69 | Halo Strobe | 182-189 | Open |
| 69 | Halo Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 69 | Halo Strobe | 232-239 | Open |
| 69 | Halo Strobe | 240-247 | Random strobe, slow to fast |
| 69 | Halo Strobe | 248-255 | Open |
| 70 | Pan/Tilt Speed | 0-255 | fast to slow |
| 71 | Function | 0-29 | No function |
| 71 | Function | 30-39 | Dimmer curve: linear |
| 71 | Function | 40-49 | Dimmer curve: square |
| 71 | Function | 50-59 | Dimmer curve: inverse square |
| 71 | Function | 60-69 | Dimmer curve: S-curve |
| 71 | Function | 70-122 | No function |
| 71 | Function | 123 | 900 Hz |
| 71 | Function | 124 | 1000 Hz |
| 71 | Function | 125 | 1100 Hz |
| 71 | Function | 126 | 1200 Hz |
| 71 | Function | 127 | 1300 Hz |
| 71 | Function | 128 | 1400 Hz |
| 71 | Function | 129 | 1500 Hz |
| 71 | Function | 130 | 2500 Hz |
| 71 | Function | 131 | 4000 Hz |
| 71 | Function | 132 | 5000 Hz |
| 71 | Function | 133 | 6000 Hz |
| 71 | Function | 134 | 10 kHz |
| 71 | Function | 135 | 15 kHz |
| 71 | Function | 136 | 20 kHz |
| 71 | Function | 137 | 25 kHz |
| 71 | Function | 138-139 | No function |
| 71 | Function | 140-149 | Pan/Tilt Reset |
| 71 | Function | 150-159 | Head Reset |
| 71 | Function | 160-199 | No function |
| 71 | Function | 200-209 | Reset All |
| 71 | Function | 210-255 | No function |

5.3  75CH Mode

| 75CH | Channel Name | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | Pan 1 | 0-255 | 0° → 540° |
| 2 | Pan 1 Fine | 0-255 | 0°–2° |
| 3 | Tilt 1 | 0-255 | 0° → 540° |
| 4 | Tilt 1 Fine | 0-255 | 0°–2° |
| 5 | Main LED 1 Red | 0-255 | Red 0% → 100% |
| 6 | Main LED 1 Green | 0-255 | Green 0% → 100% |
| 7 | Main LED 1 Blue | 0-255 | Blue 0% → 100% |
| 8 | Main LED 1 Lime | 0-255 | Lime 0% → 100% |
| 9 | Main LED 1 Strobe | 0-7 | Blackout |
| 9 | Main LED 1 Strobe | 8-15 | Open |
| 9 | Main LED 1 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 9 | Main LED 1 Strobe | 132-139 | Open |
| 9 | Main LED 1 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 9 | Main LED 1 Strobe | 182-189 | Open |
| 9 | Main LED 1 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 9 | Main LED 1 Strobe | 232-239 | Open |
| 9 | Main LED 1 Strobe | 240-247 | Random strobe, slow to fast |
| 9 | Main LED 1 Strobe | 248-255 | Open |
| 10 | Zoom 1 | 0-255 | 0% → 100% |
| 11 | Halo 1 Red | 0-255 | Red 0% → 100% |
| 12 | Halo 1 Green | 0-255 | Green 0% → 100% |
| 13 | Halo 1 Blue | 0-255 | Blue 0% → 100% |
| 14 | Halo 1 Effect & Speed | 0-15 | No function |
| 14 | Halo 1 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 14 | Halo 1 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 14 | Halo 1 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 14 | Halo 1 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 14 | Halo 1 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 14 | Halo 1 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 14 | Halo 1 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 14 | Halo 1 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 14 | Halo 1 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 14 | Halo 1 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 14 | Halo 1 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 14 | Halo 1 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 14 | Halo 1 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 14 | Halo 1 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 14 | Halo 1 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 15 | Pan 2 | 0-255 | 0° → 540° |
| 16 | Pan 2 Fine | 0-255 | 0°–2° |
| 17 | Tilt 2 | 0-255 | 0° → 540° |
| 18 | Tilt 2 Fine | 0-255 | 0°–2° |
| 19 | Main LED 2 Red | 0-255 | Red 0% → 100% |
| 20 | Main LED 2 Green | 0-255 | Green 0% → 100% |
| 21 | Main LED 2 Blue | 0-255 | Blue 0% → 100% |
| 22 | Main LED 2 Lime | 0-255 | Lime 0% → 100% |
| 23 | Main LED 2 Strobe | 0-7 | Blackout |
| 23 | Main LED 2 Strobe | 8-15 | Open |
| 23 | Main LED 2 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 23 | Main LED 2 Strobe | 132-139 | Open |
| 23 | Main LED 2 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 23 | Main LED 2 Strobe | 182-189 | Open |
| 23 | Main LED 2 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 23 | Main LED 2 Strobe | 232-239 | Open |
| 23 | Main LED 2 Strobe | 240-247 | Random strobe, slow to fast |
| 23 | Main LED 2 Strobe | 248-255 | Open |
| 24 | Zoom 2 | 0-255 | 0% → 100% |
| 25 | Halo 2 Red | 0-255 | Red 0% → 100% |
| 26 | Halo 2 Green | 0-255 | Green 0% → 100% |
| 27 | Halo 2 Blue | 0-255 | Blue 0% → 100% |
| 28 | Halo 2 Effect & Speed | 0-15 | No function |
| 28 | Halo 2 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 28 | Halo 2 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 28 | Halo 2 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 28 | Halo 2 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 28 | Halo 2 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 28 | Halo 2 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 28 | Halo 2 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 28 | Halo 2 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 28 | Halo 2 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 28 | Halo 2 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 28 | Halo 2 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 28 | Halo 2 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 28 | Halo 2 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 28 | Halo 2 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 28 | Halo 2 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 29 | Pan 3 | 0-255 | 0° → 540° |
| 30 | Pan 3 Fine | 0-255 | 0°–2° |
| 31 | Tilt 3 | 0-255 | 0° → 540° |
| 32 | Tilt 3 Fine | 0-255 | 0°–2° |
| 33 | Main LED 3 Red | 0-255 | Red 0% → 100% |
| 34 | Main LED 3 Green | 0-255 | Green 0% → 100% |
| 35 | Main LED 3 Blue | 0-255 | Blue 0% → 100% |
| 36 | Main LED 3 Lime | 0-255 | Lime 0% → 100% |
| 37 | Main LED 3 Strobe | 0-7 | Blackout |
| 37 | Main LED 3 Strobe | 8-15 | Open |
| 37 | Main LED 3 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 37 | Main LED 3 Strobe | 132-139 | Open |
| 37 | Main LED 3 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 37 | Main LED 3 Strobe | 182-189 | Open |
| 37 | Main LED 3 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 37 | Main LED 3 Strobe | 232-239 | Open |
| 37 | Main LED 3 Strobe | 240-247 | Random strobe, slow to fast |
| 37 | Main LED 3 Strobe | 248-255 | Open |
| 38 | Zoom 3 | 0-255 | 0% → 100% |
| 39 | Halo 3 Red | 0-255 | Red 0% → 100% |
| 40 | Halo 3 Green | 0-255 | Green 0% → 100% |
| 41 | Halo 3 Blue | 0-255 | Blue 0% → 100% |
| 42 | Halo 3 Effect & Speed | 0-15 | No function |
| 42 | Halo 3 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 42 | Halo 3 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 42 | Halo 3 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 42 | Halo 3 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 42 | Halo 3 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 42 | Halo 3 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 42 | Halo 3 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 42 | Halo 3 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 42 | Halo 3 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 42 | Halo 3 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 42 | Halo 3 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 42 | Halo 3 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 42 | Halo 3 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 42 | Halo 3 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 42 | Halo 3 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 43 | Pan 4 | 0-255 | 0° → 540° |
| 44 | Pan 4 Fine | 0-255 | 0°–2° |
| 45 | Tilt 4 | 0-255 | 0° → 540° |
| 46 | Tilt 4 Fine | 0-255 | 0°–2° |
| 47 | Main LED 4 Red | 0-255 | Red 0% → 100% |
| 48 | Main LED 4 Green | 0-255 | Green 0% → 100% |
| 49 | Main LED 4 Blue | 0-255 | Blue 0% → 100% |
| 50 | Main LED 4 Lime | 0-255 | Lime 0% → 100% |
| 51 | Main LED 4 Strobe | 0-7 | Blackout |
| 51 | Main LED 4 Strobe | 8-15 | Open |
| 51 | Main LED 4 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 51 | Main LED 4 Strobe | 132-139 | Open |
| 51 | Main LED 4 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 51 | Main LED 4 Strobe | 182-189 | Open |
| 51 | Main LED 4 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 51 | Main LED 4 Strobe | 232-239 | Open |
| 51 | Main LED 4 Strobe | 240-247 | Random strobe, slow to fast |
| 51 | Main LED 4 Strobe | 248-255 | Open |
| 52 | Zoom 4 | 0-255 | 0% → 100% |
| 53 | Halo 4 Red | 0-255 | Red 0% → 100% |
| 54 | Halo 4 Green | 0-255 | Green 0% → 100% |
| 55 | Halo 4 Blue | 0-255 | Blue 0% → 100% |
| 56 | Halo 4 Effect & Speed | 0-15 | No function |
| 56 | Halo 4 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 56 | Halo 4 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 56 | Halo 4 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 56 | Halo 4 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 56 | Halo 4 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 56 | Halo 4 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 56 | Halo 4 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 56 | Halo 4 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 56 | Halo 4 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 56 | Halo 4 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 56 | Halo 4 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 56 | Halo 4 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 56 | Halo 4 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 56 | Halo 4 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 56 | Halo 4 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 57 | Pan 5 | 0-255 | 0° → 540° |
| 58 | Pan 5 Fine | 0-255 | 0°–2° |
| 59 | Tilt 5 | 0-255 | 0° → 540° |
| 60 | Tilt 5 Fine | 0-255 | 0°–2° |
| 61 | Main LED 5 Red | 0-255 | Red 0% → 100% |
| 62 | Main LED 5 Green | 0-255 | Green 0% → 100% |
| 63 | Main LED 5 Blue | 0-255 | Blue 0% → 100% |
| 64 | Main LED 5 Lime | 0-255 | Lime 0% → 100% |
| 65 | Main LED 5 Strobe | 0-7 | Blackout |
| 65 | Main LED 5 Strobe | 8-15 | Open |
| 65 | Main LED 5 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 65 | Main LED 5 Strobe | 132-139 | Open |
| 65 | Main LED 5 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 65 | Main LED 5 Strobe | 182-189 | Open |
| 65 | Main LED 5 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 65 | Main LED 5 Strobe | 232-239 | Open |
| 65 | Main LED 5 Strobe | 240-247 | Random strobe, slow to fast |
| 65 | Main LED 5 Strobe | 248-255 | Open |
| 66 | Zoom 5 | 0-255 | 0% → 100% |
| 67 | Halo 5 Red | 0-255 | Red 0% → 100% |
| 68 | Halo 5 Green | 0-255 | Green 0% → 100% |
| 69 | Halo 5 Blue | 0-255 | Blue 0% → 100% |
| 70 | Halo 5 Effect & Speed | 0-15 | No function |
| 70 | Halo 5 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 70 | Halo 5 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 70 | Halo 5 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 70 | Halo 5 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 70 | Halo 5 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 70 | Halo 5 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 70 | Halo 5 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 70 | Halo 5 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 70 | Halo 5 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 70 | Halo 5 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 70 | Halo 5 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 70 | Halo 5 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 70 | Halo 5 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 70 | Halo 5 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 70 | Halo 5 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 71 | Master Dimmer | 0-255 | 0% → 100% |
| 72 | Master Dimmer Fine | 0-255 | Dimmer Fine |
| 73 | Halo Strobe | 0-7 | Blackout |
| 73 | Halo Strobe | 8-15 | Open |
| 73 | Halo Strobe | 16-131 | Synchronous strobe, slow to fast |
| 73 | Halo Strobe | 132-139 | Open |
| 73 | Halo Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 73 | Halo Strobe | 182-189 | Open |
| 73 | Halo Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 73 | Halo Strobe | 232-239 | Open |
| 73 | Halo Strobe | 240-247 | Random strobe, slow to fast |
| 73 | Halo Strobe | 248-255 | Open |
| 74 | Pan/Tilt Speed | 0-255 | fast to slow |
| 75 | Function | 0-29 | No function |
| 75 | Function | 30-39 | Dimmer curve: linear |
| 75 | Function | 40-49 | Dimmer curve: square |
| 75 | Function | 50-59 | Dimmer curve: inverse square |
| 75 | Function | 60-69 | Dimmer curve: S-curve |
| 75 | Function | 70-122 | No function |
| 75 | Function | 123 | 900 Hz |
| 75 | Function | 124 | 1000 Hz |
| 75 | Function | 125 | 1100 Hz |
| 75 | Function | 126 | 1200 Hz |
| 75 | Function | 127 | 1300 Hz |
| 75 | Function | 128 | 1400 Hz |
| 75 | Function | 129 | 1500 Hz |
| 75 | Function | 130 | 2500 Hz |
| 75 | Function | 131 | 4000 Hz |
| 75 | Function | 132 | 5000 Hz |
| 75 | Function | 133 | 6000 Hz |
| 75 | Function | 134 | 10 kHz |
| 75 | Function | 135 | 15 kHz |
| 75 | Function | 136 | 20 kHz |
| 75 | Function | 137 | 25 kHz |
| 75 | Function | 138-139 | No function |
| 75 | Function | 140-149 | Pan/Tilt Reset |
| 75 | Function | 150-159 | Head Reset |
| 75 | Function | 160-199 | No function |
| 75 | Function | 200-209 | Reset All |
| 75 | Function | 210-255 | No function |

5.4  85CH Mode

| 85CH | Channel Name | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | Pan 1 | 0-255 | 0° → 540° |
| 2 | Pan 1 Fine | 0-255 | 0°–2° |
| 3 | Tilt 1 | 0-255 | 0° → 540° |
| 4 | Tilt 1 Fine | 0-255 | 0°–2° |
| 5 | Main LED 1 Red | 0-255 | Red 0% → 100% |
| 6 | Main LED 1 Green | 0-255 | Green 0% → 100% |
| 7 | Main LED 1 Blue | 0-255 | Blue 0% → 100% |
| 8 | Main LED 1 Lime | 0-255 | Lime 0% → 100% |
| 9 | Main LED 1 Dimmer | 0-255 | Main LED 1 Dimmer |
| 10 | Main LED 1 Dimmer Fine | 0-255 | Main LED 1 Dimmer Fine |
| 11 | Main LED 1 Strobe | 0-7 | Blackout |
| 11 | Main LED 1 Strobe | 8-15 | Open |
| 11 | Main LED 1 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 11 | Main LED 1 Strobe | 132-139 | Open |
| 11 | Main LED 1 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 11 | Main LED 1 Strobe | 182-189 | Open |
| 11 | Main LED 1 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 11 | Main LED 1 Strobe | 232-239 | Open |
| 11 | Main LED 1 Strobe | 240-247 | Random strobe, slow to fast |
| 11 | Main LED 1 Strobe | 248-255 | Open |
| 12 | Zoom 1 | 0-255 | 0% → 100% |
| 13 | Halo 1 Red | 0-255 | Red 0% → 100% |
| 14 | Halo 1 Green | 0-255 | Green 0% → 100% |
| 15 | Halo 1 Blue | 0-255 | Blue 0% → 100% |
| 16 | Halo 1 Effect & Speed | 0-15 | No function |
| 16 | Halo 1 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 16 | Halo 1 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 16 | Halo 1 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 16 | Halo 1 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 16 | Halo 1 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 16 | Halo 1 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 16 | Halo 1 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 16 | Halo 1 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 16 | Halo 1 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 16 | Halo 1 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 16 | Halo 1 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 16 | Halo 1 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 16 | Halo 1 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 16 | Halo 1 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 16 | Halo 1 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 17 | Pan 2 | 0-255 | 0° → 540° |
| 18 | Pan 2 Fine | 0-255 | 0°–2° |
| 19 | Tilt 2 | 0-255 | 0° → 540° |
| 20 | Tilt 2 Fine | 0-255 | 0°–2° |
| 21 | Main LED 2 Red | 0-255 | Red 0% → 100% |
| 22 | Main LED 2 Green | 0-255 | Green 0% → 100% |
| 23 | Main LED 2 Blue | 0-255 | Blue 0% → 100% |
| 24 | Main LED 2 Lime | 0-255 | Lime 0% → 100% |
| 25 | Main LED 2 Dimmer | 0-255 | Main LED 2 Dimmer |
| 26 | Main LED 2 Dimmer Fine | 0-255 | Main LED 2 Dimmer Fine |
| 27 | Main LED 2 Strobe | 0-7 | Blackout |
| 27 | Main LED 2 Strobe | 8-15 | Open |
| 27 | Main LED 2 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 27 | Main LED 2 Strobe | 132-139 | Open |
| 27 | Main LED 2 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 27 | Main LED 2 Strobe | 182-189 | Open |
| 27 | Main LED 2 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 27 | Main LED 2 Strobe | 232-239 | Open |
| 27 | Main LED 2 Strobe | 240-247 | Random strobe, slow to fast |
| 27 | Main LED 2 Strobe | 248-255 | Open |
| 28 | Zoom 2 | 0-255 | 0% → 100% |
| 29 | Halo 2 Red | 0-255 | Red 0% → 100% |
| 30 | Halo 2 Green | 0-255 | Green 0% → 100% |
| 31 | Halo 2 Blue | 0-255 | Blue 0% → 100% |
| 32 | Halo 2 Effect & Speed | 0-15 | No function |
| 32 | Halo 2 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 32 | Halo 2 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 32 | Halo 2 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 32 | Halo 2 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 32 | Halo 2 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 32 | Halo 2 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 32 | Halo 2 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 32 | Halo 2 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 32 | Halo 2 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 32 | Halo 2 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 32 | Halo 2 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 32 | Halo 2 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 32 | Halo 2 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 32 | Halo 2 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 32 | Halo 2 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 33 | Pan 3 | 0-255 | 0° → 540° |
| 34 | Pan 3 Fine | 0-255 | 0°–2° |
| 35 | Tilt 3 | 0-255 | 0° → 540° |
| 36 | Tilt 3 Fine | 0-255 | 0°–2° |
| 37 | Main LED 3 Red | 0-255 | Red 0% → 100% |
| 38 | Main LED 3 Green | 0-255 | Green 0% → 100% |
| 39 | Main LED 3 Blue | 0-255 | Blue 0% → 100% |
| 40 | Main LED 3 Lime | 0-255 | Lime 0% → 100% |
| 41 | Main LED 3 Dimmer | 0-255 | Main LED 3 Dimmer |
| 42 | Main LED 3 Dimmer Fine | 0-255 | Main LED 3 Dimmer Fine |
| 43 | Main LED 3 Strobe | 0-7 | Blackout |
| 43 | Main LED 3 Strobe | 8-15 | Open |
| 43 | Main LED 3 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 43 | Main LED 3 Strobe | 132-139 | Open |
| 43 | Main LED 3 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 43 | Main LED 3 Strobe | 182-189 | Open |
| 43 | Main LED 3 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 43 | Main LED 3 Strobe | 232-239 | Open |
| 43 | Main LED 3 Strobe | 240-247 | Random strobe, slow to fast |
| 43 | Main LED 3 Strobe | 248-255 | Open |
| 44 | Zoom 3 | 0-255 | 0% → 100% |
| 45 | Halo 3 Red | 0-255 | Red 0% → 100% |
| 46 | Halo 3 Green | 0-255 | Green 0% → 100% |
| 47 | Halo 3 Blue | 0-255 | Blue 0% → 100% |
| 48 | Halo 3 Effect & Speed | 0-15 | No function |
| 48 | Halo 3 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 48 | Halo 3 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 48 | Halo 3 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 48 | Halo 3 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 48 | Halo 3 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 48 | Halo 3 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 48 | Halo 3 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 48 | Halo 3 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 48 | Halo 3 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 48 | Halo 3 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 48 | Halo 3 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 48 | Halo 3 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 48 | Halo 3 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 48 | Halo 3 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 48 | Halo 3 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 49 | Pan 4 | 0-255 | 0° → 540° |
| 50 | Pan 4 Fine | 0-255 | 0°–2° |
| 51 | Tilt 4 | 0-255 | 0° → 540° |
| 52 | Tilt 4 Fine | 0-255 | 0°–2° |
| 53 | Main LED 4 Red | 0-255 | Red 0% → 100% |
| 54 | Main LED 4 Green | 0-255 | Green 0% → 100% |
| 55 | Main LED 4 Blue | 0-255 | Blue 0% → 100% |
| 56 | Main LED 4 Lime | 0-255 | Lime 0% → 100% |
| 57 | Main LED 4 Dimmer | 0-255 | Main LED 4 Dimmer |
| 58 | Main LED 4 Dimmer Fine | 0-255 | Main LED 4 Dimmer Fine |
| 59 | Main LED 4 Strobe | 0-7 | Blackout |
| 59 | Main LED 4 Strobe | 8-15 | Open |
| 59 | Main LED 4 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 59 | Main LED 4 Strobe | 132-139 | Open |
| 59 | Main LED 4 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 59 | Main LED 4 Strobe | 182-189 | Open |
| 59 | Main LED 4 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 59 | Main LED 4 Strobe | 232-239 | Open |
| 59 | Main LED 4 Strobe | 240-247 | Random strobe, slow to fast |
| 59 | Main LED 4 Strobe | 248-255 | Open |
| 60 | Zoom 4 | 0-255 | 0% → 100% |
| 61 | Halo 4 Red | 0-255 | Red 0% → 100% |
| 62 | Halo 4 Green | 0-255 | Green 0% → 100% |
| 63 | Halo 4 Blue | 0-255 | Blue 0% → 100% |
| 64 | Halo 4 Effect & Speed | 0-15 | No function |
| 64 | Halo 4 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 64 | Halo 4 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 64 | Halo 4 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 64 | Halo 4 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 64 | Halo 4 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 64 | Halo 4 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 64 | Halo 4 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 64 | Halo 4 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 64 | Halo 4 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 64 | Halo 4 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 64 | Halo 4 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 64 | Halo 4 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 64 | Halo 4 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 64 | Halo 4 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 64 | Halo 4 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 65 | Pan 5 | 0-255 | 0° → 540° |
| 66 | Pan 5 Fine | 0-255 | 0°–2° |
| 67 | Tilt 5 | 0-255 | 0° → 540° |
| 68 | Tilt 5 Fine | 0-255 | 0°–2° |
| 69 | Main LED 5 Red | 0-255 | Red 0% → 100% |
| 70 | Main LED 5 Green | 0-255 | Green 0% → 100% |
| 71 | Main LED 5 Blue | 0-255 | Blue 0% → 100% |
| 72 | Main LED 5 Lime | 0-255 | Lime 0% → 100% |
| 73 | Main LED 5 Dimmer | 0-255 | Main LED 5 Dimmer |
| 74 | Main LED 5 Dimmer Fine | 0-255 | Main LED 5 Dimmer Fine |
| 75 | Main LED 5 Strobe | 0-7 | Blackout |
| 75 | Main LED 5 Strobe | 8-15 | Open |
| 75 | Main LED 5 Strobe | 16-131 | Synchronous strobe, slow to fast |
| 75 | Main LED 5 Strobe | 132-139 | Open |
| 75 | Main LED 5 Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 75 | Main LED 5 Strobe | 182-189 | Open |
| 75 | Main LED 5 Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 75 | Main LED 5 Strobe | 232-239 | Open |
| 75 | Main LED 5 Strobe | 240-247 | Random strobe, slow to fast |
| 75 | Main LED 5 Strobe | 248-255 | Open |
| 76 | Zoom 5 | 0-255 | 0% → 100% |
| 77 | Halo 5 Red | 0-255 | Red 0% → 100% |
| 78 | Halo 5 Green | 0-255 | Green 0% → 100% |
| 79 | Halo 5 Blue | 0-255 | Blue 0% → 100% |
| 80 | Halo 5 Effect & Speed | 0-15 | No function |
| 80 | Halo 5 Effect & Speed | 16-31 | Effect 1，fast to slow |
| 80 | Halo 5 Effect & Speed | 32-47 | Effect 2，fast to slow |
| 80 | Halo 5 Effect & Speed | 48-63 | Effect 3，fast to slow |
| 80 | Halo 5 Effect & Speed | 64-79 | Effect 4，fast to slow |
| 80 | Halo 5 Effect & Speed | 80-95 | Effect 5，fast to slow |
| 80 | Halo 5 Effect & Speed | 96-111 | Effect 6，fast to slow |
| 80 | Halo 5 Effect & Speed | 112-127 | Effect 7，fast to slow |
| 80 | Halo 5 Effect & Speed | 128-143 | Effect 8，fast to slow |
| 80 | Halo 5 Effect & Speed | 144-159 | Effect 9，fast to slow |
| 80 | Halo 5 Effect & Speed | 160-175 | Effect 10，fast to slow |
| 80 | Halo 5 Effect & Speed | 176-191 | Effect 11，fast to slow |
| 80 | Halo 5 Effect & Speed | 192-207 | Effect 12，fast to slow |
| 80 | Halo 5 Effect & Speed | 208-223 | Effect 13，fast to slow |
| 80 | Halo 5 Effect & Speed | 224-239 | Effect 14，fast to slow |
| 80 | Halo 5 Effect & Speed | 240-255 | Effect 15，fast to slow |
| 81 | Master Dimmer | 0-255 | 0% → 100% |
| 82 | Master Dimmer Fine | 0-255 | Dimmer Fine |
| 83 | Halo Strobe | 0-7 | Blackout |
| 83 | Halo Strobe | 8-15 | Open |
| 83 | Halo Strobe | 16-131 | Synchronous strobe, slow to fast |
| 83 | Halo Strobe | 132-139 | Open |
| 83 | Halo Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 83 | Halo Strobe | 182-189 | Open |
| 83 | Halo Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 83 | Halo Strobe | 232-239 | Open |
| 83 | Halo Strobe | 240-247 | Random strobe, slow to fast |
| 83 | Halo Strobe | 248-255 | Open |
| 84 | Pan/Tilt Speed | 0-255 | fast to slow |
| 85 | Function | 0-29 | No function |
| 85 | Function | 30-39 | Dimmer curve: linear |
| 85 | Function | 40-49 | Dimmer curve: square |
| 85 | Function | 50-59 | Dimmer curve: inverse square |
| 85 | Function | 60-69 | Dimmer curve: S-curve |
| 85 | Function | 70-122 | No function |
| 85 | Function | 123 | 900 Hz |
| 85 | Function | 124 | 1000 Hz |
| 85 | Function | 125 | 1100 Hz |
| 85 | Function | 126 | 1200 Hz |
| 85 | Function | 127 | 1300 Hz |
| 85 | Function | 128 | 1400 Hz |
| 85 | Function | 129 | 1500 Hz |
| 85 | Function | 130 | 2500 Hz |
| 85 | Function | 131 | 4000 Hz |
| 85 | Function | 132 | 5000 Hz |
| 85 | Function | 133 | 6000 Hz |
| 85 | Function | 134 | 10 kHz |
| 85 | Function | 135 | 15 kHz |
| 85 | Function | 136 | 20 kHz |
| 85 | Function | 137 | 25 kHz |
| 85 | Function | 138-139 | No function |
| 85 | Function | 140-149 | Pan/Tilt Reset |
| 85 | Function | 150-159 | Head Reset |
| 85 | Function | 160-199 | No function |
| 85 | Function | 200-209 | Reset All |
| 85 | Function | 210-255 | No function |

5.5  170CH Mode

| 170CH | Channel Name | DMX Value | Function |
| --- | --- | --- | --- |
| 1 | Pan 1 | 0-255 | 0° → 540° |
| 2 | Pan 1 Fine | 0-255 | 0°–2° |
| 3 | Tilt 1 | 0-255 | 0° → 540° |
| 4 | Tilt 1 Fine | 0-255 | 0°–2° |
| 5 | Main LED 1 Red | 0-255 | Red 0% → 100% |
| 6 | Main LED 1 Green | 0-255 | Green 0% → 100% |
| 7 | Main LED 1 Blue | 0-255 | Blue 0% → 100% |
| 8 | Main LED 1 Lime | 0-255 | Lime 0% → 100% |
| 9 | Zoom 1 | 0-255 | 0% → 100% |
| 10 | Halo 1 Zone 1 Red | 0-255 | Red 0% → 100% |
| 11 | Halo 1 Zone 1 Green | 0-255 | Green 0% → 100% |
| 12 | Halo 1 Zone 1 Blue | 0-255 | Blue 0% → 100% |
| 13 | Halo 1 Zone 2 Red | 0-255 | Red 0% → 100% |
| 14 | Halo 1 Zone 2 Green | 0-255 | Green 0% → 100% |
| 15 | Halo 1 Zone 2 Blue | 0-255 | Blue 0% → 100% |
| 16 | Halo 1 Zone 3 Red | 0-255 | Red 0% → 100% |
| 17 | Halo 1 Zone 3 Green | 0-255 | Green 0% → 100% |
| 18 | Halo 1 Zone 3 Blue | 0-255 | Blue 0% → 100% |
| 19 | Halo 1 Zone 4 Red | 0-255 | Red 0% → 100% |
| 20 | Halo 1 Zone 4 Green | 0-255 | Green 0% → 100% |
| 21 | Halo 1 Zone 4 Blue | 0-255 | Blue 0% → 100% |
| 22 | Halo 1 Zone 5 Red | 0-255 | Red 0% → 100% |
| 23 | Halo 1 Zone 5 Green | 0-255 | Green 0% → 100% |
| 24 | Halo 1 Zone 5 Blue | 0-255 | Blue 0% → 100% |
| 25 | Halo 1 Zone 6 Red | 0-255 | Red 0% → 100% |
| 26 | Halo 1 Zone 6 Green | 0-255 | Green 0% → 100% |
| 27 | Halo 1 Zone 6 Blue | 0-255 | Blue 0% → 100% |
| 28 | Halo 1 Zone 7 Red | 0-255 | Red 0% → 100% |
| 29 | Halo 1 Zone 7 Green | 0-255 | Green 0% → 100% |
| 30 | Halo 1 Zone 7 Blue | 0-255 | Blue 0% → 100% |
| 31 | Halo 1 Zone 8 Red | 0-255 | Red 0% → 100% |
| 32 | Halo 1 Zone 8 Green | 0-255 | Green 0% → 100% |
| 33 | Halo 1 Zone 8 Blue | 0-255 | Blue 0% → 100% |
| 34 | Pan 2 | 0-255 | 0° → 540° |
| 35 | Pan 2 Fine | 0-255 | 0°–2° |
| 36 | Tilt 2 | 0-255 | 0° → 540° |
| 37 | Tilt 2 Fine | 0-255 | 0°–2° |
| 38 | Main LED 2 Red | 0-255 | Red 0% → 100% |
| 39 | Main LED 2 Green | 0-255 | Green 0% → 100% |
| 40 | Main LED 2 Blue | 0-255 | Blue 0% → 100% |
| 41 | Main LED 2 Lime | 0-255 | Lime 0% → 100% |
| 42 | Zoom 2 | 0-255 | 0% → 100% |
| 43 | Halo 2 Zone 1 Red | 0-255 | Red 0% → 100% |
| 44 | Halo 2 Zone 1 Green | 0-255 | Green 0% → 100% |
| 45 | Halo 2 Zone 1 Blue | 0-255 | Blue 0% → 100% |
| 46 | Halo 2 Zone 2 Red | 0-255 | Red 0% → 100% |
| 47 | Halo 2 Zone 2 Green | 0-255 | Green 0% → 100% |
| 48 | Halo 2 Zone 2 Blue | 0-255 | Blue 0% → 100% |
| 49 | Halo 2 Zone 3 Red | 0-255 | Red 0% → 100% |
| 50 | Halo 2 Zone 3 Green | 0-255 | Green 0% → 100% |
| 51 | Halo 2 Zone 3 Blue | 0-255 | Blue 0% → 100% |
| 52 | Halo 2 Zone 4 Red | 0-255 | Red 0% → 100% |
| 53 | Halo 2 Zone 4 Green | 0-255 | Green 0% → 100% |
| 54 | Halo 2 Zone 4 Blue | 0-255 | Blue 0% → 100% |
| 55 | Halo 2 Zone 5 Red | 0-255 | Red 0% → 100% |
| 56 | Halo 2 Zone 5 Green | 0-255 | Green 0% → 100% |
| 57 | Halo 2 Zone 5 Blue | 0-255 | Blue 0% → 100% |
| 58 | Halo 2 Zone 6 Red | 0-255 | Red 0% → 100% |
| 59 | Halo 2 Zone 6 Green | 0-255 | Green 0% → 100% |
| 60 | Halo 2 Zone 6 Blue | 0-255 | Blue 0% → 100% |
| 61 | Halo 2 Zone 7 Red | 0-255 | Red 0% → 100% |
| 62 | Halo 2 Zone 7 Green | 0-255 | Green 0% → 100% |
| 63 | Halo 2 Zone 7 Blue | 0-255 | Blue 0% → 100% |
| 64 | Halo 2 Zone 8 Red | 0-255 | Red 0% → 100% |
| 65 | Halo 2 Zone 8 Green | 0-255 | Green 0% → 100% |
| 66 | Halo 2 Zone 8 Blue | 0-255 | Blue 0% → 100% |
| 67 | Pan 3 | 0-255 | 0° → 540° |
| 68 | Pan 3 Fine | 0-255 | 0°–2° |
| 69 | Tilt 3 | 0-255 | 0° → 540° |
| 70 | Tilt 3 Fine | 0-255 | 0°–2° |
| 71 | Main LED 3 Red | 0-255 | Red 0% → 100% |
| 72 | Main LED 3 Green | 0-255 | Green 0% → 100% |
| 73 | Main LED 3 Blue | 0-255 | Blue 0% → 100% |
| 74 | Main LED 3 Lime | 0-255 | Lime 0% → 100% |
| 75 | Zoom 3 | 0-255 | 0% → 100% |
| 76 | Halo 3 Zone 1 Red | 0-255 | Red 0% → 100% |
| 77 | Halo 3 Zone 1 Green | 0-255 | Green 0% → 100% |
| 78 | Halo 3 Zone 1 Blue | 0-255 | Blue 0% → 100% |
| 79 | Halo 3 Zone 2 Red | 0-255 | Red 0% → 100% |
| 80 | Halo 3 Zone 2 Green | 0-255 | Green 0% → 100% |
| 81 | Halo 3 Zone 2 Blue | 0-255 | Blue 0% → 100% |
| 82 | Halo 3 Zone 3 Red | 0-255 | Red 0% → 100% |
| 83 | Halo 3 Zone 3 Green | 0-255 | Green 0% → 100% |
| 84 | Halo 3 Zone 3 Blue | 0-255 | Blue 0% → 100% |
| 85 | Halo 3 Zone 4 Red | 0-255 | Red 0% → 100% |
| 86 | Halo 3 Zone 4 Green | 0-255 | Green 0% → 100% |
| 87 | Halo 3 Zone 4 Blue | 0-255 | Blue 0% → 100% |
| 88 | Halo 3 Zone 5 Red | 0-255 | Red 0% → 100% |
| 89 | Halo 3 Zone 5 Green | 0-255 | Green 0% → 100% |
| 90 | Halo 3 Zone 5 Blue | 0-255 | Blue 0% → 100% |
| 91 | Halo 3 Zone 6 Red | 0-255 | Red 0% → 100% |
| 92 | Halo 3 Zone 6 Green | 0-255 | Green 0% → 100% |
| 93 | Halo 3 Zone 6 Blue | 0-255 | Blue 0% → 100% |
| 94 | Halo 3 Zone 7 Red | 0-255 | Red 0% → 100% |
| 95 | Halo 3 Zone 7 Green | 0-255 | Green 0% → 100% |
| 96 | Halo 3 Zone 7 Blue | 0-255 | Blue 0% → 100% |
| 97 | Halo 3 Zone 8 Red | 0-255 | Red 0% → 100% |
| 98 | Halo 3 Zone 8 Green | 0-255 | Green 0% → 100% |
| 99 | Halo 3 Zone 8 Blue | 0-255 | Blue 0% → 100% |
| 100 | Pan 4 | 0-255 | 0° → 540° |
| 101 | Pan 4 Fine | 0-255 | 0°–2° |
| 102 | Tilt 4 | 0-255 | 0° → 540° |
| 103 | Tilt 4 Fine | 0-255 | 0°–2° |
| 104 | Main LED 4 Red | 0-255 | Red 0% → 100% |
| 105 | Main LED 4 Green | 0-255 | Green 0% → 100% |
| 106 | Main LED 4 Blue | 0-255 | Blue 0% → 100% |
| 107 | Main LED 4 Lime | 0-255 | Lime 0% → 100% |
| 108 | Zoom 4 | 0-255 | 0% → 100% |
| 109 | Halo 4 Zone 1 Red | 0-255 | Red 0% → 100% |
| 110 | Halo 4 Zone 1 Green | 0-255 | Green 0% → 100% |
| 111 | Halo 4 Zone 1 Blue | 0-255 | Blue 0% → 100% |
| 112 | Halo 4 Zone 2 Red | 0-255 | Red 0% → 100% |
| 113 | Halo 4 Zone 2 Green | 0-255 | Green 0% → 100% |
| 114 | Halo 4 Zone 2 Blue | 0-255 | Blue 0% → 100% |
| 115 | Halo 4 Zone 3 Red | 0-255 | Red 0% → 100% |
| 116 | Halo 4 Zone 3 Green | 0-255 | Green 0% → 100% |
| 117 | Halo 4 Zone 3 Blue | 0-255 | Blue 0% → 100% |
| 118 | Halo 4 Zone 4 Red | 0-255 | Red 0% → 100% |
| 119 | Halo 4 Zone 4 Green | 0-255 | Green 0% → 100% |
| 120 | Halo 4 Zone 4 Blue | 0-255 | Blue 0% → 100% |
| 121 | Halo 4 Zone 5 Red | 0-255 | Red 0% → 100% |
| 122 | Halo 4 Zone 5 Green | 0-255 | Green 0% → 100% |
| 123 | Halo 4 Zone 5 Blue | 0-255 | Blue 0% → 100% |
| 124 | Halo 4 Zone 6 Red | 0-255 | Red 0% → 100% |
| 125 | Halo 4 Zone 6 Green | 0-255 | Green 0% → 100% |
| 126 | Halo 4 Zone 6 Blue | 0-255 | Blue 0% → 100% |
| 127 | Halo 4 Zone 7 Red | 0-255 | Red 0% → 100% |
| 128 | Halo 4 Zone 7 Green | 0-255 | Green 0% → 100% |
| 129 | Halo 4 Zone 7 Blue | 0-255 | Blue 0% → 100% |
| 130 | Halo 4 Zone 8 Red | 0-255 | Red 0% → 100% |
| 131 | Halo 4 Zone 8 Green | 0-255 | Green 0% → 100% |
| 132 | Halo 4 Zone 8 Blue | 0-255 | Blue 0% → 100% |
| 133 | Pan 5 | 0-255 | 0° → 540° |
| 134 | Pan 5 Fine | 0-255 | 0°–2° |
| 135 | Tilt 5 | 0-255 | 0° → 540° |
| 136 | Tilt 5 Fine | 0-255 | 0°–2° |
| 137 | Main LED 5 Red | 0-255 | Red 0% → 100% |
| 138 | Main LED 5 Green | 0-255 | Green 0% → 100% |
| 139 | Main LED 5 Blue | 0-255 | Blue 0% → 100% |
| 140 | Main LED 5 Lime | 0-255 | Lime 0% → 100% |
| 141 | Zoom 5 | 0-255 | 0% → 100% |
| 142 | Halo 5 Zone 1 Red | 0-255 | Red 0% → 100% |
| 143 | Halo 5 Zone 1 Green | 0-255 | Green 0% → 100% |
| 144 | Halo 5 Zone 1 Blue | 0-255 | Blue 0% → 100% |
| 145 | Halo 5 Zone 2 Red | 0-255 | Red 0% → 100% |
| 146 | Halo 5 Zone 2 Green | 0-255 | Green 0% → 100% |
| 147 | Halo 5 Zone 2 Blue | 0-255 | Blue 0% → 100% |
| 148 | Halo 5 Zone 3 Red | 0-255 | Red 0% → 100% |
| 149 | Halo 5 Zone 3 Green | 0-255 | Green 0% → 100% |
| 150 | Halo 5 Zone 3 Blue | 0-255 | Blue 0% → 100% |
| 151 | Halo 5 Zone 4 Red | 0-255 | Red 0% → 100% |
| 152 | Halo 5 Zone 4 Green | 0-255 | Green 0% → 100% |
| 153 | Halo 5 Zone 4 Blue | 0-255 | Blue 0% → 100% |
| 154 | Halo 5 Zone 5 Red | 0-255 | Red 0% → 100% |
| 155 | Halo 5 Zone 5 Green | 0-255 | Green 0% → 100% |
| 156 | Halo 5 Zone 5 Blue | 0-255 | Blue 0% → 100% |
| 157 | Halo 5 Zone 6 Red | 0-255 | Red 0% → 100% |
| 158 | Halo 5 Zone 6 Green | 0-255 | Green 0% → 100% |
| 159 | Halo 5 Zone 6 Blue | 0-255 | Blue 0% → 100% |
| 160 | Halo 5 Zone 7 Red | 0-255 | Red 0% → 100% |
| 161 | Halo 5 Zone 7 Green | 0-255 | Green 0% → 100% |
| 162 | Halo 5 Zone 7 Blue | 0-255 | Blue 0% → 100% |
| 163 | Halo 5 Zone 8 Red | 0-255 | Red 0% → 100% |
| 164 | Halo 5 Zone 8 Green | 0-255 | Green 0% → 100% |
| 165 | Halo 5 Zone 8 Blue | 0-255 | Blue 0% → 100% |
| 166 | Main LED Dimmer | 0-255 | Main LED Dimmer |
| 167 | Main LED Strobe | 0-7 | Blackout |
| 167 | Main LED Strobe | 8-15 | Open |
| 167 | Main LED Strobe | 16-131 | Synchronous strobe, slow to fast |
| 167 | Main LED Strobe | 132-139 | Open |
| 167 | Main LED Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 167 | Main LED Strobe | 182-189 | Open |
| 167 | Main LED Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 167 | Main LED Strobe | 232-239 | Open |
| 167 | Main LED Strobe | 240-247 | Random strobe, slow to fast |
| 167 | Main LED Strobe | 248-255 | Open |
| 168 | Halo Dimmer | 0-255 | 0% → 100% |
| 169 | Halo Strobe | 0-7 | Blackout |
| 169 | Halo Strobe | 8-15 | Open |
| 169 | Halo Strobe | 16-131 | Synchronous strobe, slow to fast |
| 169 | Halo Strobe | 132-139 | Open |
| 169 | Halo Strobe | 140-181 | Fast-open / slow-close strobe, slow to fast |
| 169 | Halo Strobe | 182-189 | Open |
| 169 | Halo Strobe | 190-231 | Slow-open / fast-close strobe, slow to fast |
| 169 | Halo Strobe | 232-239 | Open |
| 169 | Halo Strobe | 240-247 | Random strobe, slow to fast |
| 169 | Halo Strobe | 248-255 | Open |
| 170 | Function | 0-29 | No function |
| 170 | Function | 30-39 | Dimmer curve: linear |
| 170 | Function | 40-49 | Dimmer curve: square |
| 170 | Function | 50-59 | Dimmer curve: inverse square |
| 170 | Function | 60-69 | Dimmer curve: S-curve |
| 170 | Function | 70-122 | No function |
| 170 | Function | 123 | 900 Hz |
| 170 | Function | 124 | 1000 Hz |
| 170 | Function | 125 | 1100 Hz |
| 170 | Function | 126 | 1200 Hz |
| 170 | Function | 127 | 1300 Hz |
| 170 | Function | 128 | 1400 Hz |
| 170 | Function | 129 | 1500 Hz |
| 170 | Function | 130 | 2500 Hz |
| 170 | Function | 131 | 4000 Hz |
| 170 | Function | 132 | 5000 Hz |
| 170 | Function | 133 | 6000 Hz |
| 170 | Function | 134 | 10 kHz |
| 170 | Function | 135 | 15 kHz |
| 170 | Function | 136 | 20 kHz |
| 170 | Function | 137 | 25 kHz |
| 170 | Function | 138-139 | No function |
| 170 | Function | 140-149 | Pan/Tilt Reset |
| 170 | Function | 150-159 | Head Reset |
| 170 | Function | 160-199 | No function |
| 170 | Function | 200-209 | Reset All |
| 170 | Function | 210-255 | No function |

6  Technical Specifications

| Category | Specification |
| --- | --- |
| Input power | AC 100–240 V, 50/60 Hz |
| Rated fixture power | 800 W |
| Main light source | 5 × 120 W RGB + Lime LED modules (source specifies ‘1212 LED package’) plus 120 × 0.5 W RGB auxiliary LEDs |
| Halo / auxiliary light | 24 × 0.2 W RGB LED module around each head |
| Head type | 120 W RGB + Lime LED module per head |
| Average source life | >20,000 hours |
| Connections | DMX512 input/output; power input/output |
| Zoom / focus | Electronic linear zoom, 3.5°–45°; specified focus distance >3 m |
| Dimming / strobe | 0–100% linear electronic dimming; high-speed electronic strobe |
| Display | LCD; Chinese/English menu selection; fixture information display |
| DMX modes | 41 CH / 71 CH / 75 CH / 85 CH / 170 CH |
| Control protocols | DMX512 and RDM; firmware upgrade through DMX signal cable |
| Cooling | Temperature sensing with adaptive fan-speed control |
| Movement | Five independently controlled heads with smooth X/Y-axis movement |
| Pan range | 60° with 16-bit resolution |
| Tilt range | 220° with 16-bit resolution |
| Position correction | Automatic error-correcting reset |
| Housing / finish | Metal; black |
| Ingress protection | IP65 |

7  Troubleshooting & Operating Precautions

SERVICE SAFETY  The fixture contains professional electronic assemblies and high-voltage power components. Do not open or repair it unless you are qualified to do so.

Fixture does not start

Check the mains supply and input connector.

Check whether the input fuse has opened; replace only with the specified type and rating.

Inspect internal plug-in connections and control-board connectors if the fixture has been exposed to transport vibration; qualified service personnel only.

Fixture resets normally but does not respond to the console

Verify the DMX start address and selected channel mode.

Inspect the DMX cable and XLR connectors for damage or looseness.

Add a compatible signal amplifier where required and terminate the final fixture with 120 Ω.

Fixture operates intermittently

Confirm that every fan is running and that air inlets/outlets are clean.

Check whether internal temperature protection has activated.

Allow the fixture to cool and correct the airflow problem before further use.

Output appears dim

Clean lenses and optical components; accumulated dust reduces output.

Check dimmer, strobe, color, zoom, and master channel values.

If output remains abnormal, have the LED modules and driver circuits inspected by qualified personnel.

Projection or beam appears unfocused

Adjust the zoom/focus channel for the current throw distance.

Clean the lenses and verify that no optical component is loose or damaged.

7.1 Operating Precautions

Confirm that the local supply voltage matches the rating and that leakage-current and over-current protection devices are suitable for the connected load.

Never use a power cable with damaged insulation and do not splice the power cable into other conductors.

Clean forced-air cooling paths at least monthly, and more often in dusty environments.

Tighten all mounting screws, fit an independent safety cable, and inspect both regularly.

Maintain the minimum clearances stated in the source manual: 10 m from flammable or explosive materials and 2.5 m from illuminated objects. Do not install directly on a combustible surface.

The source manual recommends no more than 10 hours of continuous operation and at least 10 minutes between consecutive starts.

Avoid leaving multiple fixtures indefinitely between scenes; complete scene transitions promptly to maintain synchronization.

Stop using the fixture immediately if abnormal behavior, smell, sound, heat, or visible damage is detected.

8  RDM Precautions

RDM (Remote Device Management) extends DMX512-A with bidirectional communication over the RS-485 bus. Because the bus is half-duplex, only one controller may transmit as master at a time.

Use a console or controller that supports RDM.

Use bidirectional RDM-compatible signal amplifiers. Traditional one-way amplifiers block return data and prevent fixture discovery.

Set all fixtures to DMX mode and ensure that only one RDM controller is active on the signal line.

Install a 120 Ω termination resistor between pins 2 and 3 at the end of long signal runs to reduce reflections and improve communication quality.

If DMX control works but RDM discovery fails, first check the signal amplifier, then inspect conductors 2 and 3 and their connector contacts.

END OF MANUAL  Retain this document with the fixture. Before operation, always verify the latest product labeling, local electrical code, rigging requirements, and venue safety procedures.