AL640WX

6 x 40 W Laser Bee Eye
Moving Head

RGBW PIXELS · RGB HALO · ANIMATION LASER — USER MANUAL

Beam, wash, vortex, kaleidoscope and animated-laser fixture

LASER SAFETY  Avoid direct eye or skin exposure to laser radiation. Never aim the beam at people, aircraft or vehicles. Operate only where applicable laser-safety requirements are satisfied.

English Edition — Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX Channel Reference |
| 6 | Routine Maintenance |
| 7 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is organized for model AL640WX. Confirm the model, voltage rating, laser labeling and firmware shown on the actual fixture before installation or programming.

The source document identifies the product as a six-emitter 40 W laser bee-eye fixture. This edition uses the complete model designation AL640WX supplied with the product files.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool completely before opening the housing, cleaning, inspection or service.

Use the fixture only with the voltage and frequency marked on its product label. Connect it to a properly grounded supply.

This is a Class I electrical appliance. Never defeat the protective earth connection.

Protect power and data cables from crushing, sharp edges, heat and other damage. Replace damaged cables before use.

Use the fixture indoors in a dry, well-ventilated location. Keep ventilation openings clear and protect the product from moisture and condensation.

Do not look into the LED or laser output. Do not use optical instruments to view the beam and do not expose people to the output path.

Never aim laser output at people, animals, aircraft, vehicles, reflective surfaces or public areas where exposure cannot be controlled.

Installation, operation, calibration and service must be performed by trained personnel. Do not modify the fixture.

For suspended installation, use suitable clamps and an independently rated safety cable. Keep the area below the fixture clear during installation and service.

CONDENSATION  If the fixture is moved between environments with a large temperature difference, leave it disconnected until it reaches room temperature and all condensation has evaporated.

LASER COMPLIANCE  The source manual does not state a laser class, wavelength, divergence or accessible-emission limit. Check the product label and comply with all local licensing, operator, audience-scanning and controlled-area requirements before use.

2  Product Overview

The AL640WX combines six 40 W RGBW bee-eye emitters, an RGB animation laser and an RGB halo ring in a moving-head platform. Its continuously rotating front lens supports beam, wash, vortex and kaleidoscope looks, while independent pixel and laser controls provide layered effects.

2.1 Package Contents

| Item | Quantity |
| --- | --- |
| AL640WX fixture | 1 |
| User manual | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |

2.2 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100–240 V, 50/60 Hz |
| Rated power | 350 W |
| Main light source | 6 × 40 W OSRAM RGBW 4-in-1 LEDs |
| Laser source | 8 W RGB laser |
| Rated LED life | 50,000 hours |
| Color system | RGBW color mixing; linear CTO |
| Zoom range | 4.5°–45° |
| Pan / tilt | 540° / 270°, 16-bit control |
| DMX personalities | 49CH / 73CH |
| Control modes | DMX512 / RDM / Sound / Auto 1 / Auto 2 |
| Connections | 3-pin DMX in/out; optional 5-pin; power input/output daisy chain |
| Display | 1.77-inch LCD; Chinese/English menu; error and temperature display |
| Ingress protection | IP20 — indoor use |

SOURCE NORMALIZATION  The specification states 270° tilt, while the source DMX table describes Channel 3 as 0–205°. This edition preserves both values and flags the channel-table entry for firmware verification. Use the actual fixture behavior when creating a console profile.

2.3 Main Functions

Beam, wash, vortex and kaleidoscope effects from a continuously rotating front lens.

Six independently controllable RGBW pixels in the 73-channel personality.

Animated RGB laser with pattern, color, scan, rotation, movement, zoom and wave controls.

RGB halo dimming, strobe, built-in color/effect selection and effect speed.

DMX512 and RDM support with sound-active and automatic programs.

3  Setup and Installation

3.1 AC Power

Before connecting power, check that the product-label rating matches the local supply. Connect the fixture to protective earth. When linking power between fixtures, do not exceed the connector, cable or circuit rating.

3.2 DMX Data Connection

Use shielded, two-conductor twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Keep data wiring separate from mains wiring where practical.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data − |
| Pin 3 | Data + |

TERMINATION  At the last fixture in a long or unstable DMX chain, connect a 120 Ω, 1 W resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

3.3 DMX Addressing

Set each fixture's start address so that its channel range does not overlap another fixture unless synchronized control is intended. The next start address equals the current address plus the selected channel count.

| 49CH fixture | Start address |
| --- | --- |
| Fixture 1 | A001 |
| Fixture 2 | A050 |
| Fixture 3 | A099 |

3.4 Rigging

Inspect the fixture, mounting hardware and support structure before installation.

Attach an appropriately rated clamp to the fixture's mounting point and secure it to the structure.

Attach an independently rated safety cable through the designated safety point; do not use handles or cables as safety anchors.

Confirm that the moving head and laser output have adequate clearance throughout the programmed pan and tilt range.

After installation, recheck every fastener and establish a periodic inspection schedule.

4  Control Panel and Menu

Use ENTER to open or save the selected item, UP and DOWN to change a value, and MENU/EXIT to return without saving. The display can rotate automatically to match the fixture orientation. Menu wording may vary with firmware revision.

| Menu | Options | Operation |
| --- | --- | --- |
| Address | 001–512 | Set the DMX start address. |
| Run Mode | DMX / Sound / Auto 1 / Auto 2 | Select console control, sound-active or an automatic program. |
| Channel Mode | 49CH / 73CH | Select the DMX personality used by the controller profile. |
| Pan / Tilt Invert | Off / On | Reverse movement direction to suit the installation. |
| Hall / Optical Correction | Off / On | Enable sensor-based position correction. |
| Signal Hold | Off / On | Choose whether the last DMX state is retained after signal loss. |
| Screen Saver | Off / On | Control automatic display blanking. |
| Screen Flip | Off / On / Auto | Set normal, inverted or automatic display orientation. |
| Synchronized Update | Off / On | Apply supported output changes synchronously. |
| Language | 中文 / English | Choose the display language. |
| Factory Reset | Confirm | Restore default settings. |

4.1 Manual Control and Calibration

In Manual Control, select a channel, press ENTER to edit, use UP/DOWN to set 0–255, and press ENTER again to save. EXIT cancels the unsaved change.

| Calibration item | Range | Meaning |
| --- | --- | --- |
| Pan / Tilt / Zoom home | 0–255 | Home-position offset; 127 = no offset |
| Pan / Tilt / Zoom travel | 0–255 | Movement-range calibration; factory reference 127 |
| Pan / Tilt motor strength | 0–255 | Motor drive strength; factory reference 127 |
| Other motor strength | 0–255 | Effect-motor drive strength; factory reference 127 |
| Microphone sensitivity | Firmware value | Sound-active input sensitivity |
| Change password | Protected | Update access password for calibration settings |

CALIBRATION  Record original values before adjustment. Incorrect home, travel or motor-strength values can cause abnormal positioning or mechanical stress.

4.2 Reset and Information

| Item | Action |
| --- | --- |
| Effect motor reset | Reset effect motors without resetting pan/tilt. |
| Movement motor reset | Reset pan and tilt motors. |
| Reset all | Perform a complete fixture reset. |
| Reset errors | Clear stored error information after the cause has been corrected. |
| DMX monitor | View received channel values. |
| Sensor status | View Hall, X-axis and Y-axis sensor state. |
| Hardware / software | View hardware and firmware versions. |

5  DMX Channel Reference

Select 49CH or 73CH on the fixture and use the matching controller personality. The first 29 channels are common to both modes. The 49CH personality then adds 20 laser channels; the 73CH personality adds 24 RGBW pixel channels followed by 20 laser channels.

| Mode | Channel map |
| --- | --- |
| 49CH | 1–29 common fixture/halo + 30–49 laser |
| 73CH | 1–29 common fixture/halo + 30–53 six RGBW pixels + 54–73 laser |

PROFILE VERIFICATION  The source laser table contains an overlapping halo-strobe range and misaligned scan rows. This edition normalizes halo strobe to 4–250 and reconstructs laser scan as 0–127 line scan / 128–255 point scan. Verify both behaviors on the installed firmware before show programming.

5.1 Common Channels 1–29 — 49CH and 73CH

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | 0–540° |
| 2 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 3 | Tilt | 0-255 | 0–205° (source channel table) |
| 4 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 5 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 6 | Zoom | 0-255 | Narrow to wide (4.5°–45° nominal) |
| 7 | Rotating Lens | 0-255 | 0–127: 0–60° position; 128–188: CCW fast→slow; 189–193: stop; 194–255: CW slow→fast |
| 8 | Master Dimmer | 0-255 | Linear dimming, dark to full |
| 9 | Strobe | 0-255 | 0–19 blackout; 20–24 open; 25–69 standard; 70–89 fast-open/slow-close; 90–109 slow-open/fast-close; 110–129 random; 130–149 random fast-open/slow-close; 150–169 random slow-open/fast-close; 170–189 pulse; 190–209 random pulse; 210–229 fade; 230–244 pulse 2; 245–255 open |
| 10 | Red | 0-255 | Main red dimmer |
| 11 | Green | 0-255 | Main green dimmer |
| 12 | Blue | 0-255 | Main blue dimmer |
| 13 | White | 0-255 | Main white dimmer |
| 14 | Linear CTO | 0-255 | Linear color-temperature control |
| 15 | Color Macro | 0-255 | Built-in color macros |
| 16 | Built-in Effect | 0-255 | 0–19 dimmer mode; 20–255 effects 1–13, allocated in 20-value steps |
| 17 | Effect Speed / Direction | 0-255 | 0–127 forward fast→slow; 128 stop; 129–255 reverse slow→fast |
| 18 | Background Red | 0-255 | Background red dimmer |
| 19 | Background Green | 0-255 | Background green dimmer |
| 20 | Background Blue | 0-255 | Background blue dimmer |
| 21 | Background White | 0-255 | Background white dimmer |
| 22 | Reset | 0-255 | 0–25 none; 26–76 effect-motor reset (hold 5 s); 77–127 pan/tilt reset (hold 5 s); 128–255 full reset (hold 5 s) |
| 23 | Halo Dimmer | 0-255 | Halo-ring master dimmer |
| 24 | Halo Strobe | 0-255 | 0–3 blackout; 4–250 synchronized strobe; 251–255 open |
| 25 | Halo Red | 0-255 | Halo red dimmer |
| 26 | Halo Green | 0-255 | Halo green dimmer |
| 27 | Halo Blue | 0-255 | Halo blue dimmer |
| 28 | Halo Effect | 0-255 | 0–51 built-in colors; 52–55 effect 1; 56–59 effect 2; 60–251 effects 3–50 (four values each); 252–255 effect 51 |
| 29 | Halo Effect Speed | 0-255 | Slow to fast |

5.2 49CH Laser Extension — Channels 30–49

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 30 | Laser Dimmer | 0-255 | 0 off; 1–255 intensity 1–100% |
| 31 | Laser Strobe | 0-255 | 1–255 strobe-speed selection |
| 32 | Horizontal Position | 0-255 | 0–255 position; 128 center; effective with laser output enabled |
| 33 | Vertical Position | 0-255 | 0–255 position; 128 center; effective with laser output enabled |
| 34 | Pattern File | 0-255 | Select a pattern from the fixture library |
| 35 | Laser Color | 0-255 | 0–63 fixed colors; 64–95 fixed-color/change speed; 96–127 color-change speed; 128–159 color-change/water-flow speed; 160–223 water-flow speed; 224–255 gradient speed |
| 36 | Scan Speed | 0-255 | 0–127 line-scan speed; 128–255 point-scan speed (reconstructed from source table) |
| 37 | Pattern Effect | 0-255 | Select pattern/effect library in 8-value steps |
| 38 | Trigger | 0-255 | 0–26 default auto speed; 27–127 auto speed; 128–255 sound sensitivity |
| 39 | Z Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 40 | X Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 41 | Y Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 42 | Horizontal Move | 0-255 | 1–127 position; 128–255 movement speed |
| 43 | Vertical Move | 0-255 | 1–127 position; 128–255 movement speed |
| 44 | Laser Zoom | 0-255 | 1–127 size; 128–255 zoom speed |
| 45 | Gradual Draw | 0-255 | 1–255 gradient/drawing speed |
| 46 | XY Wave | 0-255 | 1–127 X-wave speed; 128–255 Y-wave speed |
| 47 | Laser Red | 0-255 | Red laser intensity |
| 48 | Laser Green | 0-255 | Green laser intensity |
| 49 | Laser Blue | 0-255 | Blue laser intensity |

5.3 73CH Pixel Extension — Channels 30–53

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 30 | Pixel 1 Red | 0-255 | Pixel 1 red dimmer |
| 31 | Pixel 1 Green | 0-255 | Pixel 1 green dimmer |
| 32 | Pixel 1 Blue | 0-255 | Pixel 1 blue dimmer |
| 33 | Pixel 1 White | 0-255 | Pixel 1 white dimmer |
| 34 | Pixel 2 Red | 0-255 | Pixel 2 red dimmer |
| 35 | Pixel 2 Green | 0-255 | Pixel 2 green dimmer |
| 36 | Pixel 2 Blue | 0-255 | Pixel 2 blue dimmer |
| 37 | Pixel 2 White | 0-255 | Pixel 2 white dimmer |
| 38 | Pixel 3 Red | 0-255 | Pixel 3 red dimmer |
| 39 | Pixel 3 Green | 0-255 | Pixel 3 green dimmer |
| 40 | Pixel 3 Blue | 0-255 | Pixel 3 blue dimmer |
| 41 | Pixel 3 White | 0-255 | Pixel 3 white dimmer |
| 42 | Pixel 4 Red | 0-255 | Pixel 4 red dimmer |
| 43 | Pixel 4 Green | 0-255 | Pixel 4 green dimmer |
| 44 | Pixel 4 Blue | 0-255 | Pixel 4 blue dimmer |
| 45 | Pixel 4 White | 0-255 | Pixel 4 white dimmer |
| 46 | Pixel 5 Red | 0-255 | Pixel 5 red dimmer |
| 47 | Pixel 5 Green | 0-255 | Pixel 5 green dimmer |
| 48 | Pixel 5 Blue | 0-255 | Pixel 5 blue dimmer |
| 49 | Pixel 5 White | 0-255 | Pixel 5 white dimmer |
| 50 | Pixel 6 Red | 0-255 | Pixel 6 red dimmer |
| 51 | Pixel 6 Green | 0-255 | Pixel 6 green dimmer |
| 52 | Pixel 6 Blue | 0-255 | Pixel 6 blue dimmer |
| 53 | Pixel 6 White | 0-255 | Pixel 6 white dimmer |

5.4 73CH Laser Extension — Channels 54–73

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 54 | Laser Dimmer | 0-255 | 0 off; 1–255 intensity 1–100% |
| 55 | Laser Strobe | 0-255 | 1–255 strobe-speed selection |
| 56 | Horizontal Position | 0-255 | 0–255 position; 128 center; effective with laser output enabled |
| 57 | Vertical Position | 0-255 | 0–255 position; 128 center; effective with laser output enabled |
| 58 | Pattern File | 0-255 | Select a pattern from the fixture library |
| 59 | Laser Color | 0-255 | 0–63 fixed colors; 64–95 fixed-color/change speed; 96–127 color-change speed; 128–159 color-change/water-flow speed; 160–223 water-flow speed; 224–255 gradient speed |
| 60 | Scan Speed | 0-255 | 0–127 line-scan speed; 128–255 point-scan speed (reconstructed from source table) |
| 61 | Pattern Effect | 0-255 | Select pattern/effect library in 8-value steps |
| 62 | Trigger | 0-255 | 0–26 default auto speed; 27–127 auto speed; 128–255 sound sensitivity |
| 63 | Z Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 64 | X Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 65 | Y Rotation | 0-255 | 1–127 angle; 128–255 rotation speed |
| 66 | Horizontal Move | 0-255 | 1–127 position; 128–255 movement speed |
| 67 | Vertical Move | 0-255 | 1–127 position; 128–255 movement speed |
| 68 | Laser Zoom | 0-255 | 1–127 size; 128–255 zoom speed |
| 69 | Gradual Draw | 0-255 | 1–255 gradient/drawing speed |
| 70 | XY Wave | 0-255 | 1–127 X-wave speed; 128–255 Y-wave speed |
| 71 | Laser Red | 0-255 | Red laser intensity |
| 72 | Laser Green | 0-255 | Green laser intensity |
| 73 | Laser Blue | 0-255 | Blue laser intensity |

6  Routine Maintenance

ATTENTION  Disconnect AC power, allow the fixture to cool and ensure the laser cannot emit before inspection or cleaning.

Inspect the housing, mounting points, safety cable, power cable, connectors and ventilation openings.

Remove loose dust with a soft brush, lint-free cloth, vacuum cleaner or clean dry compressed air.

Clean optical surfaces with lint-free optical tissue lightly moistened with purified water or a cleaner suitable for the surface coating. Do not scratch lenses or coatings.

Check that fans turn freely and that vents are not obstructed. Do not force debris deeper into the fixture.

After service, refit all covers and safety devices before reconnecting power.

Cleaning frequency depends on dust, fog-fluid residue, airflow and operating hours. Establish a recurring interval based on inspection results and the installation environment.

7  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| Fixture does not power on | Verify the supply, protective earth, circuit protection, cable, connector and product-label rating. |
| No DMX response | Confirm DMX mode, start address, selected 49CH/73CH personality, controller patch, cable polarity and termination. |
| Unexpected channel behavior | Confirm that the controller profile matches the selected personality and verify the installed firmware. |
| Movement error | Remove obstructions, run the appropriate reset and inspect the Hall/X/Y sensor status. |
| Laser effect differs from profile | Verify the pattern library and firmware; test the reconstructed scan ranges before programming a show. |
| Output is reduced or unstable | Check temperature, ventilation, fan operation, optical cleanliness and power quality. |
| Red ERR indicator | Open the information/error page, record the message, correct the cause and then clear the stored error. |

SERVICE  There are no user-serviceable laser or high-voltage parts inside. Refer internal repair, optical alignment, laser service and calibration to qualified personnel.

7.1 Warranty and Support

Damage caused by misuse, unauthorized modification, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Keep the purchase record and warranty documentation, and include the model, serial number, firmware version and a clear fault description when requesting service.

End of manual