AL740WZ

7 x 40 W LED Wash
Moving Head

PIXEL & HALO RING - USER MANUAL

RGBW wash, zoom and pixel-effect moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Control Panel and Menu |
| 4 | DMX Channel Reference |
| 5 | Routine Maintenance |
| 6 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is organized for model AL740WZ. Confirm the model, voltage rating and firmware shown on the actual fixture before use.

The source manual identifies the product as 740WZ. This edition uses the complete model designation AL740WZ supplied with the product files.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool completely before opening the housing, cleaning, inspection or service.

Use the fixture only with the voltage and frequency marked on its product label.

Connect the fixture to a properly grounded supply and keep power and data cables protected from damage.

Keep ventilation openings clear. Do not operate the fixture where airflow is restricted or dust accumulation is excessive.

Do not look directly into the light output at close range. The LEDs can produce intense visible light.

Do not modify the fixture. Internal repair and calibration must be performed by qualified personnel.

Use suitable clamps, fasteners and an independently rated safety cable for suspended installation.

OPTICAL SURFACES  The optical coating is fragile and easily scratched. Work in a clean, bright area and do not use cleaners that can damage plastic, coatings or colored surfaces.

2  Product Overview

The AL740WZ is a compact LED wash moving head with seven individually controllable 40 W RGBW emitters, motorized zoom and an RGB halo ring. It supports DMX512, master/slave, sound-active, automatic and manual operation.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 90-240 V, 50/60 Hz |
| Rated power | 350 W |
| Main light source | 7 x 40 W RGBW 4-in-1 LEDs |
| Halo ring | RGB ring LEDs |
| Zoom angle | 3-60 degrees |
| DMX personalities | 18CH / 29CH / 41CH / 57CH |
| Control modes | DMX512 / Master-Slave / Sound / Auto / Manual |
| Pan movement | 540 degrees |
| Tilt movement | 270 degrees nominal |
| Strobe | 1-25 Hz or random strobe |
| Display | LCD |
| Net weight | 6.5 kg |
| Dimensions | 266 x 207 x 258 mm |

SOURCE NORMALIZATION  The detailed source tables define 18CH, 29CH, 41CH and 57CH modes; these values are used in this edition. The source dimension unit has been normalized from cm to mm. The source channel tables describe a 205-degree tilt range, while the specification states 270 degrees; verify the active firmware before profile programming.

2.2 Main Functions

RGBW color mixing with coarse and fine control in the 41-channel personality.

Motorized zoom from narrow beam to wide wash.

Built-in static, dynamic, foreground and background effects.

Independent RGBW pixel control in the 57-channel personality.

RGB halo-ring strobe, color and dynamic-effect controls.

3  Control Panel and Menu

Use ENTER to open or confirm a menu item, UP and DOWN to change the selected option or value, and the return/menu key to leave the current screen. Menu names may vary slightly with firmware revision.

| Menu | Options | Operation |
| --- | --- | --- |
| DMX Settings | Address and channel mode | Set the DMX start address and select 18CH, 29CH, 41CH or 57CH. Press ENTER to save. |
| Run Mode | DMX512 / Slow / Fast / Sound / Manual | Select console control, slow auto, fast auto, sound-active or manual scene operation. |
| Invert Pan/Tilt | Yes / No | Reverse pan and/or tilt direction as required by the installation. |
| Display Set | Invert / English-Chinese / Backlight | Rotate the display, choose the interface language and set the backlight behavior. |
| Manual Set | Channel values | Create and save a local scene. RUN MODE must be set to Manual. |
| Sound Sense | 0-100 | Adjust sound-active sensitivity. |
| System Set | Version / Defaults / Code Settings | View system data, restore defaults and access protected calibration parameters. |
| Reset | Fixture reset | Select Reset and wait approximately 3 seconds for initialization. |

3.1 Run Modes

| Mode | Description |
| --- | --- |
| DMX512 | Control from a compatible DMX512 console. |
| Slow | Slow automatic program. |
| Fast | Fast automatic program. |
| Sound | Sound-active automatic program. |
| Manual | Manual scene mode; saved values can be restored after power cycling. |

3.2 Display Settings

When LCD Backlight is set to ON, the display remains illuminated. When set to OFF, it turns off after approximately 20 seconds without key input and lights again when a key is pressed.

3.3 Protected Calibration

Code Settings protects pan, tilt and zoom home-position adjustment and RGBW balance parameters. The factory key sequence in the source manual is DOWN, DOWN, UP, UP.

| Parameter | Purpose |
| --- | --- |
| PAN_INIT | Pan home-position adjustment |
| TILT_INIT | Tilt home-position adjustment |
| ZOOM_INIT | Zoom home-position adjustment |
| SET_Red / Green / Blue / White | LED white-balance calibration |

CALIBRATION  Record the original values before adjustment. Incorrect settings can cause uneven color or abnormal mechanical positioning.

4  DMX Channel Reference

Select the personality in DMX Settings and configure the controller with the matching channel layout. Channel functions can vary with firmware; compare the fixture display and controller profile before programming.

| Mode | Use |
| --- | --- |
| 18CH | Compact movement, main RGBW and halo-ring control |
| 29CH | Movement, main effects, background color and halo-ring control |
| 41CH | 16-bit RGBW, movement, shape effects and halo-ring control |
| 57CH | 29CH-style core plus seven-pixel RGBW control and halo ring |

4.1 18-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | 0-540 degrees |
| 2 | Tilt | 0-255 | 0-205 degrees (source table) |
| 3 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 4 | Zoom | 0-255 | Narrow to wide |
| 5 | Rotating Lens | 0-255 | Lens rotation |
| 6 | Master Dimmer | 0-255 | Main output linear dimming, dark to full |
| 7 | Strobe | 0-255 | Main-light strobe |
| 8 | Red | 0-255 | Linear dimming, dark to full |
| 9 | Green | 0-255 | Linear dimming, dark to full |
| 10 | Blue | 0-255 | Linear dimming, dark to full |
| 11 | White | 0-255 | Linear dimming, dark to full |
| 12 | Reset | 0-255 | 0-250: No function; 251-255: Fixture reset after value is held for 5 seconds |
| 13 | Halo Strobe | 0-255 | Halo-ring strobe |
| 14 | Halo Red | 0-255 | Halo-ring red dimmer |
| 15 | Halo Green | 0-255 | Halo-ring green dimmer |
| 16 | Halo Blue | 0-255 | Halo-ring blue dimmer |
| 17 | Halo Dynamic Effect | 0-255 | Halo-ring dynamic effect |
| 18 | Halo Effect Speed | 0-255 | Halo-ring dynamic-effect speed |

4.2 29-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | 0-540 degrees |
| 2 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 3 | Tilt | 0-255 | 0-205 degrees (source table) |
| 4 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 5 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 6 | Zoom | 0-255 | Narrow to wide |
| 7 | Rotating Lens | 0-255 | Lens rotation |
| 8 | Master Dimmer | 0-255 | Linear dimming, dark to full |
| 9 | Strobe | 0-255 | Main-light strobe |
| 10 | Red | 0-255 | Linear dimming, dark to full |
| 11 | Green | 0-255 | Linear dimming, dark to full |
| 12 | Blue | 0-255 | Linear dimming, dark to full |
| 13 | White | 0-255 | Linear dimming, dark to full |
| 14 | Color Temperature | 0-255 | Linear color-temperature adjustment |
| 15 | Color Macro | 0-255 | Built-in color gradient |
| 16 | Static Effect | 0-255 | Built-in static effect selection |
| 17 | Dynamic Effect | 0-255 | Built-in dynamic effect selection |
| 18 | Dynamic Effect Speed | 0-255 | Slow to fast |
| 19 | Background Red | 0-255 | Linear dimming, dark to full |
| 20 | Background Green | 0-255 | Linear dimming, dark to full |
| 21 | Background Blue | 0-255 | Linear dimming, dark to full |
| 22 | Background White | 0-255 | Linear dimming, dark to full |
| 23 | Reset | 0-255 | 0-250: No function; 251-255: Fixture reset after value is held for 5 seconds |
| 24 | Halo Strobe | 0-255 | Halo-ring strobe |
| 25 | Halo Red | 0-255 | Halo-ring red dimmer |
| 26 | Halo Green | 0-255 | Halo-ring green dimmer |
| 27 | Halo Blue | 0-255 | Halo-ring blue dimmer |
| 28 | Halo Dynamic Effect | 0-255 | Halo-ring dynamic effect |
| 29 | Halo Effect Speed | 0-255 | Halo-ring dynamic-effect speed |

4.3 41-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Red dimmer |
| 2 | Red Fine | 0-255 | Red fine adjustment |
| 3 | Green | 0-255 | Green dimmer |
| 4 | Green Fine | 0-255 | Green fine adjustment |
| 5 | Blue | 0-255 | Blue dimmer |
| 6 | Blue Fine | 0-255 | Blue fine adjustment |
| 7 | White | 0-255 | White dimmer |
| 8 | White Fine | 0-255 | White fine adjustment |
| 9 | Linear CTO | 0-255 | Linear color-temperature adjustment |
| 10 | Color Macro | 0-255 | Built-in color macros |
| 11 | Strobe | 0-255 | Main-light strobe |
| 12 | Master Dimmer | 0-255 | Main-light master dimmer |
| 13 | Master Dimmer Fine | 0-255 | Fine master-dimmer adjustment |
| 14 | Pan | 0-255 | Pan movement |
| 15 | Pan Fine | 0-255 | Pan fine adjustment |
| 16 | Tilt | 0-255 | Tilt movement |
| 17 | Tilt Fine | 0-255 | Tilt fine adjustment |
| 18 | Function | 0-255 | Fixture function channel |
| 19 | Reset | 0-255 | Fixture reset |
| 20 | Zoom | 0-255 | Motorized zoom |
| 21 | Rotating Lens | 0-255 | Lens rotation |
| 22 | Shape Selection | 0-255 | Built-in effect selection |
| 23 | Shape Speed | 0-255 | Built-in effect speed |
| 24 | Shape Fade | 0-255 | Built-in effect fade |
| 25 | Shape Red | 0-255 | Foreground red dimmer |
| 26 | Shape Green | 0-255 | Foreground green dimmer |
| 27 | Shape Blue | 0-255 | Foreground blue dimmer |
| 28 | Shape White | 0-255 | Foreground white dimmer |
| 29 | Shape Dimmer | 0-255 | Foreground master dimmer |
| 30 | Background Dimmer | 0-255 | Background master dimmer |
| 31 | Shape Transition | 0-255 | Built-in effect transformation |
| 32 | Shape Offset | 0-255 | Built-in effect offset / disable |
| 33 | Foreground Strobe | 0-255 | Foreground strobe |
| 34 | Background Strobe | 0-255 | Background strobe |
| 35 | Background Select | 0-255 | Background selection |
| 36 | Halo Strobe | 0-255 | Halo-ring strobe |
| 37 | Halo Red | 0-255 | Halo-ring red dimmer |
| 38 | Halo Green | 0-255 | Halo-ring green dimmer |
| 39 | Halo Blue | 0-255 | Halo-ring blue dimmer |
| 40 | Halo Dynamic Effect | 0-255 | Halo-ring dynamic effect |
| 41 | Halo Effect Speed | 0-255 | Halo-ring dynamic-effect speed |

4.4 57-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | 0-540 degrees |
| 2 | Pan Fine | 0-255 | 16-bit fine adjustment |
| 3 | Tilt | 0-255 | 0-205 degrees (source table) |
| 4 | Tilt Fine | 0-255 | 16-bit fine adjustment |
| 5 | Pan/Tilt Speed | 0-255 | Fast to slow |
| 6 | Zoom | 0-255 | Narrow to wide |
| 7 | Rotating Lens | 0-255 | Lens rotation |
| 8 | Master Dimmer | 0-255 | Linear dimming, dark to full |
| 9 | Strobe | 0-255 | Main-light strobe |
| 10 | Red | 0-255 | Linear dimming, dark to full |
| 11 | Green | 0-255 | Linear dimming, dark to full |
| 12 | Blue | 0-255 | Linear dimming, dark to full |
| 13 | White | 0-255 | Linear dimming, dark to full |
| 14 | Color Temperature | 0-255 | Linear color-temperature adjustment |
| 15 | Color Macro | 0-255 | Built-in color gradient |
| 16 | Static Effect | 0-255 | Built-in static effect selection |
| 17 | Dynamic Effect | 0-255 | Built-in dynamic effect selection |
| 18 | Dynamic Effect Speed | 0-255 | Slow to fast |
| 19 | Background Red | 0-255 | Linear dimming, dark to full |
| 20 | Background Green | 0-255 | Linear dimming, dark to full |
| 21 | Background Blue | 0-255 | Linear dimming, dark to full |
| 22 | Background White | 0-255 | Linear dimming, dark to full |
| 23 | Reset | 0-255 | 0-250: No function; 251-255: Fixture reset after value is held for 5 seconds |
| 24 | Pixel 1 Red | 0-255 | Pixel 1 red dimmer |
| 25 | Pixel 1 Green | 0-255 | Pixel 1 green dimmer |
| 26 | Pixel 1 Blue | 0-255 | Pixel 1 blue dimmer |
| 27 | Pixel 1 White | 0-255 | Pixel 1 white dimmer |
| 28 | Pixel 2 Red | 0-255 | Pixel 2 red dimmer |
| 29 | Pixel 2 Green | 0-255 | Pixel 2 green dimmer |
| 30 | Pixel 2 Blue | 0-255 | Pixel 2 blue dimmer |
| 31 | Pixel 2 White | 0-255 | Pixel 2 white dimmer |
| 32 | Pixel 3 Red | 0-255 | Pixel 3 red dimmer |
| 33 | Pixel 3 Green | 0-255 | Pixel 3 green dimmer |
| 34 | Pixel 3 Blue | 0-255 | Pixel 3 blue dimmer |
| 35 | Pixel 3 White | 0-255 | Pixel 3 white dimmer |
| 36 | Pixel 4 Red | 0-255 | Pixel 4 red dimmer |
| 37 | Pixel 4 Green | 0-255 | Pixel 4 green dimmer |
| 38 | Pixel 4 Blue | 0-255 | Pixel 4 blue dimmer |
| 39 | Pixel 4 White | 0-255 | Pixel 4 white dimmer |
| 40 | Pixel 5 Red | 0-255 | Pixel 5 red dimmer |
| 41 | Pixel 5 Green | 0-255 | Pixel 5 green dimmer |
| 42 | Pixel 5 Blue | 0-255 | Pixel 5 blue dimmer |
| 43 | Pixel 5 White | 0-255 | Pixel 5 white dimmer |
| 44 | Pixel 6 Red | 0-255 | Pixel 6 red dimmer |
| 45 | Pixel 6 Green | 0-255 | Pixel 6 green dimmer |
| 46 | Pixel 6 Blue | 0-255 | Pixel 6 blue dimmer |
| 47 | Pixel 6 White | 0-255 | Pixel 6 white dimmer |
| 48 | Pixel 7 Red | 0-255 | Pixel 7 red dimmer |
| 49 | Pixel 7 Green | 0-255 | Pixel 7 green dimmer |
| 50 | Pixel 7 Blue | 0-255 | Pixel 7 blue dimmer |
| 51 | Pixel 7 White | 0-255 | Pixel 7 white dimmer |
| 52 | Halo Strobe | 0-255 | Halo-ring strobe |
| 53 | Halo Red | 0-255 | Halo-ring red dimmer |
| 54 | Halo Green | 0-255 | Halo-ring green dimmer |
| 55 | Halo Blue | 0-255 | Halo-ring blue dimmer |
| 56 | Halo Dynamic Effect | 0-255 | Halo-ring dynamic effect |
| 57 | Halo Effect Speed | 0-255 | Halo-ring dynamic-effect speed |

5  Routine Maintenance

Cleaning frequency depends on operating conditions. Inspect the fixture after approximately 100 hours when any of the following factors are present, then establish a suitable recurring interval.

Frequent use of smoke or fog machines.

High airflow, such as operation near an air inlet or outlet.

Dust from stage effects, building structures or simulated outdoor environments.

ATTENTION  Disconnect AC power and allow the fixture to cool completely before opening the housing or cleaning any component.

5.1 Cleaning Optical Components

Disconnect power, allow the fixture to cool completely and open the housing only if qualified to do so.

Use clean, dry compressed air to remove loose dust.

For stubborn marks, use lint-free optical tissue lightly moistened with purified water or a suitable glass cleaner. Wipe gently without scratching.

For oily residue, use an alcohol-moistened lint-free cloth. Remove any cleaner residue with purified water, working slowly in circular motions from the center outward.

Dry with a clean, soft cloth or clean compressed air.

5.2 Cleaning Fans and Vents

Remove dust from fans and ventilation openings with a soft brush, lint-free cloth, vacuum cleaner or clean compressed air. Do not force debris deeper into the fixture.

6  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| Fixture does not power on | Verify the supply, power cable, plug and fixture rating. |
| Fixture does not respond to DMX | Confirm DMX mode, start address, controller profile and data-cable connections. |
| Unexpected channel behavior | Verify that the controller personality matches 18CH, 29CH, 41CH or 57CH and confirm the fixture firmware. |
| Pan or tilt is reversed | Change the Pan/Tilt Invert setting. |
| Sound mode is unresponsive | Increase Sound Sense and confirm that RUN MODE is set to Sound. |
| Reduced output or overheating | Allow the fixture to cool, then clean fans, vents and optical components. |

SERVICE  All repairs not described in this manual must be referred to a qualified technician. Do not operate a fixture that has damaged wiring, abnormal movement, persistent errors or signs of overheating.

END OF MANUAL  Retain this manual with the fixture and verify the product label, controller profile, local electrical requirements and rigging procedures before each installation.