PROFESSIONAL IP65 LED PIXEL BAR

ALP1060WB

10 x 60 W LED MOVING BAR IP65

USER MANUAL

English Edition  |  Professionally edited from the supplied manual

10 x 60 W RGBW main LEDs | Pixel-controlled line effects | Motorized zoom

| Ingress Protection | Rated Power | Input | DMX Modes |
| --- | --- | --- | --- |
| IP65 | 800 W | AC 110-240 V, 50/60 Hz | 17 / 27 / 99 / 108 / 168 CH |

IMPORTANT  Read this manual completely before installing, powering, operating, or servicing the fixture. Retain it for future reference.

Contents

1  Product Information & Safety

2  Installation & DMX Connections

3  Display Menu & Operation

4  DMX512 Control & Channel Charts

5  Inspection, Maintenance & Cleaning

EDITORIAL NOTE  Terminology, grammar, units, and control labels have been standardized for professional lighting use. The requested model ALP1060WB is used throughout; old model names in the supplied source files are not carried into the manual.

1  Product Information & Safety

1.1 Intended Use

The ALP1060WB is an IP65-rated professional moving LED pixel bar with ten 60 W RGBW main emitters, motorized zoom, separate cool-white and warm-white line LEDs, and an RGB line-effect system. It supports DMX512, automatic programs, sound-active operation, and high-resolution pixel control. Typical applications include touring productions, concerts, theatres, architectural events, and outdoor entertainment installations.

Inspect the fixture and accessories immediately after unpacking. Do not operate a product that appears damaged in transit.

1.2 Package Contents

| Item | Quantity |
| --- | --- |
| ALP1060WB fixture | 1 |
| Power cable | 1 |
| 3-pin DMX cable | 1 |
| Safety cable | 1 |
| User manual | 1 |

1.3 Safety Instructions

Installation, operation, and service must be performed by qualified personnel familiar with professional entertainment lighting.

Connect only to the rated AC supply and a properly grounded Protection Class I circuit.

Disconnect mains power before cleaning, servicing, connecting, or when the fixture is not in use. Pull the plug, never the cable.

After a severe temperature change, allow the fixture to reach room temperature before applying power so condensation can evaporate.

Do not operate the fixture if a cable, connector, housing seal, lens, bracket, or safety attachment is damaged.

Maintain at least 0.5 m clearance around ventilation areas. Never project the beam onto combustible materials.

Do not lift the fixture by the moving light bar. Protect the mechanism from impact and excessive vibration.

Strobe lighting may trigger photosensitive seizures. Follow venue and local warning requirements.

IP65 PROTECTION  IP65 protection applies only when all covers, seals, and compatible weatherproof power/data connectors are correctly fitted. Do not immerse the fixture or expose open connectors to water. Disconnect power before opening any service cover.

THERMAL LIMITATION  The supplied manual instructs operators not to run all RGBW emitters at full output continuously for more than 10 minutes. Observe this limitation and maintain unrestricted cooling airflow.

1.4 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 110-240 V, 50/60 Hz |
| Rated power | 800 W |
| Main LEDs | 10 x 60 W RGBW 4-in-1 LEDs |
| Cool-white line LEDs | 96 x 1 W LEDs |
| Warm-white line LEDs | 96 x 1 W LEDs |
| RGB line LEDs | 96 x 0.5 W RGB 3-in-1 LEDs |
| Zoom / beam angle | Motorized 7:1 zoom; 5-35 degrees |
| Tilt | 220 degrees with 16-bit fine control |
| Dimming | 0-100% continuous; four selectable curves |
| Strobe | 0-25 Hz; standard, pulse, and random effects |
| Control modes | DMX512, auto programs, sound-active |
| DMX personalities | 17 CH / 27 CH / 99 CH / 108 CH / 168 CH |
| Pixel control | Main RGBW pixels plus white-line and RGB-line pixel modes |
| Cooling | Fan-assisted forced-air cooling |
| Ingress protection | IP65 |
| Rated LED life | Average more than 20,000 hours |
| Fixture dimensions | 1002 x 131 x 369.95 mm maximum envelope |
| Net weight | 25.95 kg |

UNIT CORRECTION  The source feature list labels the dimensions as centimetres. The supplied engineering drawing is explicitly in millimetres and is authoritative here.

Supplied engineering drawing. Dimensions are in millimetres.

2  Installation & DMX Connections

2.1 Rigging

Before mounting, verify that the supporting structure and every rigging component can support at least ten times the fixture weight. Use two appropriately rated clamps and complete all rigging before connecting mains power.

Install outside normal walking and seating areas and beyond unauthorized reach.

Use both mounting points for overhead suspension and fully tighten all fasteners.

Attach at least one correctly rated safety cable to the designated point on the fixture.

Never attach the safety cable to a moving section or carrying handle.

Inspect clamps, fasteners, safety cables, seals, and the supporting structure before every use.

Mounting orientations shown in the supplied manual.

2.2 DMX512 Data Connection

Connect the controller DMX output to the fixture DMX input with a shielded twisted-pair DMX cable. Link the fixture output to the next fixture input. Preserve IP65 protection by using compatible weatherproof connectors and closing unused connector caps.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Ground / shield |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  For long cable runs or electrically noisy environments, connect a 120 ohm terminator between pins 2 and 3 at the DMX output of the final fixture.

3  Display Menu & Operation

3.1 Menu Map

| Menu | Option | Range / Function |
| --- | --- | --- |
| DMX Address | Address | 001-512 |
| Fixture | Channel Mode | 17 / 27 / 99 / 108 / 168 CH |
| Fixture | Run Mode | DMX / Host Auto Run |
| Fixture | Tilt Invert | Off / On |
| Fixture | Fan Control | Normal / High / Low / Quiet |
| Fixture | Dimmer Curve | Linear / S-Curve / Square / Inverse Square |
| Fixture | Dimmer Response | Fast / Smooth |
| Fixture | Power Mode | Normal / Quiet |
| Fixture | On DMX Loss | Hold Last Value / Blackout |
| Fixture | Key Lock | Off / On |
| Manual | Manual Channel Control | 000-255 per channel |
| Information | Operating Time | Current / Total / Power-cycle count |
| Information | Sensors | Tilt / Zoom 1 / Zoom 2 / Temperature |
| Information | Software Version | Display panel / LED driver |
| Factory | Password | 000-255 |
| Factory | Calibration | Tilt / Zoom 1 / Zoom 2 / color calibration |
| Reset | Motor Reset | Run / Off |
| Reset | Factory Reset | Run / Off |
| Display | Language | English / Chinese |
| Display | Display Flip | Normal / 180-degree reverse |
| Display | Display Timeout | Always on / 60 seconds |

FIRMWARE NOTE  Menu wording and available calibration items may vary slightly with installed firmware.

4  DMX512 Control

Set a valid DMX start address and select the required personality. All five personalities share channels 1-8; higher-resolution personalities then add main-LED and line-LED pixel control.

OUTPUT NOTE  Set the Master Dimmer above zero and place the relevant strobe channel in an Open range to obtain steady output.

4.1  Common Channels 1-8

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Tilt, 0-220 degrees |
| 2 | 000-255 | Tilt Fine (16-bit) |
| 3 | 000-255 | Motorized Zoom, 5-35 degrees |
| 4 | 000-255 | Master Dimmer, 0-100% |
| 5 | 000-255 | Main LED Strobe; see 4.6 |
| 6 | 000-255 | White-Line Strobe; see 4.6 |
| 7 | 000-255 | RGB-Line Strobe; see 4.6 |
| 8 | 000-255 | Function; see 4.7 |

4.2  17 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 9 | 000-255 | Main Red, 0-100% |
| 10 | 000-255 | Main Green, 0-100% |
| 11 | 000-255 | Main Blue, 0-100% |
| 12 | 000-255 | Main White, 0-100% |
| 13 | 000-255 | White Line: Cool White, 0-100% |
| 14 | 000-255 | White Line: Warm White, 0-100% |
| 15 | 000-255 | RGB Line: Red, 0-100% |
| 16 | 000-255 | RGB Line: Green, 0-100% |
| 17 | 000-255 | RGB Line: Blue, 0-100% |

4.3  27 CH Mode

| Channel | DMX Value | Function |
| --- | --- | --- |
| 9 | 000-255 | Main Red, 0-100% |
| 10 | 000-255 | Main Green, 0-100% |
| 11 | 000-255 | Main Blue, 0-100% |
| 12 | 000-255 | Main White, 0-100% |
| 13 | 000-255 | Main LED CTO |
| 14 | 000-255 | Main LED Color Macro; see 4.6 |
| 15 | 000-255 | Main LED Effect Macro; see 4.6 |
| 16 | 000-255 | Main LED Effect Speed / Fade; see 4.6 |
| 17 | 000-255 | White Line: Cool White, 0-100% |
| 18 | 000-255 | White Line: Warm White, 0-100% |
| 19 | 000-255 | White-Line CTO; see 4.6 |
| 20 | 000-255 | White-Line Effect Macro; see 4.6 |
| 21 | 000-255 | White-Line Effect Speed / Fade; see 4.6 |
| 22 | 000-255 | RGB Line: Red, 0-100% |
| 23 | 000-255 | RGB Line: Green, 0-100% |
| 24 | 000-255 | RGB Line: Blue, 0-100% |
| 25 | 000-255 | RGB-Line Color Macro; see 4.6 |
| 26 | 000-255 | RGB-Line Effect Macro; see 4.6 |
| 27 | 000-255 | RGB-Line Effect Speed / Fade; see 4.6 |

LABEL CORRECTION  Channel 25 is labeled as a CTO channel in the source, but its value bands select 31 colors. It is therefore standardized here as RGB-Line Color Macro.

4.4  Pixel-Control Personalities

| Mode | Main RGBW Pixels | White-Line Pixels | RGB-Line Pixels |
| --- | --- | --- | --- |
| 99 CH | 10 pixels: CH 9-48 | 24 CW/WW pixels: CH 49-96 | Global RGB: CH 97-99 |
| 108 CH | 10 pixels: CH 9-48 | 12 CW/WW pixels: CH 49-72 | 12 RGB pixels: CH 73-108 |
| 168 CH | 10 pixels: CH 9-48 | 24 CW/WW pixels: CH 49-96 | 24 RGB pixels: CH 97-168 |

All pixel intensity channels use 000-255 for 0-100% output. The formulas below expand every ellipsis in the supplied source without listing hundreds of repetitive rows.

Main RGBW Pixel Formula - 99 / 108 / 168 CH

| Pixel n | Red | Green | Blue | White |
| --- | --- | --- | --- | --- |
| n = 1-10 | 9 + 4(n-1) | 10 + 4(n-1) | 11 + 4(n-1) | 12 + 4(n-1) |
| Pixel 1 | 9 | 10 | 11 | 12 |
| Pixel 10 | 45 | 46 | 47 | 48 |

White-Line CW/WW Pixel Formula

| Mode | Pixel Count | Cool White for pixel n | Warm White for pixel n |
| --- | --- | --- | --- |
| 99 CH | 24 | 49 + 2(n-1) | 50 + 2(n-1) |
| 108 CH | 12 | 49 + 2(n-1) | 50 + 2(n-1) |
| 168 CH | 24 | 49 + 2(n-1) | 50 + 2(n-1) |

RGB-Line Pixel Formula

| Mode | Pixel Count | Red for pixel n | Green for pixel n | Blue for pixel n |
| --- | --- | --- | --- | --- |
| 99 CH | Global | 97 | 98 | 99 |
| 108 CH | 12 | 73 + 3(n-1) | 74 + 3(n-1) | 75 + 3(n-1) |
| 168 CH | 24 | 97 + 3(n-1) | 98 + 3(n-1) | 99 + 3(n-1) |

4.5  Personality Summary

| Mode | Control Resolution |
| --- | --- |
| 17 CH | Global main RGBW, global cool/warm-white line, and global RGB line control. |
| 27 CH | Adds CTO, color macros, effect macros, and speed/fade controls. |
| 99 CH | Ten main RGBW pixels, 24 cool/warm-white pixels, and global RGB line control. |
| 108 CH | Ten main RGBW pixels, 12 cool/warm-white pixels, and 12 RGB line pixels. |
| 168 CH | Ten main RGBW pixels, 24 cool/warm-white pixels, and 24 RGB line pixels. |

4.6  Shared Strobe & Macro Behavior

Strobe Channels 5-7

| DMX Value | Function |
| --- | --- |
| 000-007 | Closed / No output |
| 008-015 | Open |
| 016-131 | Standard Strobe, slow to fast |
| 132-139 | Open |
| 140-181 | Pulse: slow open / fast close, slow to fast |
| 182-189 | Open |
| 190-231 | Pulse: fast open / slow close, slow to fast |
| 232-239 | Open |
| 240-247 | Random Strobe, slow to fast |
| 248-255 | Open |

27 CH Macro Channels

| Parameter | DMX Value | Function |
| --- | --- | --- |
| Main / RGB-Line Color Macro | 000-007 | No color macro |
| Main / RGB-Line Color Macro | 008-247 | Macros 1-30 in consecutive 8-value bands |
| Main / RGB-Line Color Macro | 248-255 | Macro 31 |
| Main / White-Line / RGB-Line Effect Macro | 000-007 | No effect macro |
| Main / White-Line / RGB-Line Effect Macro | 008-191 | Effects 1-23 in consecutive 8-value bands |
| Main / White-Line / RGB-Line Effect Macro | 192-255 | No effect macro |
| White-Line CTO | 000-067 | No CTO |
| White-Line CTO | 068-247 | Variable CTO |
| White-Line CTO | 248-255 | No CTO |
| Effect Speed / Fade | 000-127 | Slow to fast, without fade |
| Effect Speed / Fade | 128-255 | Slow to fast, with fade |

MACRO MAPPING  For an 8-value macro band, macro n begins at 8n. The source defines macro numbers but does not name their visual content; no unsupported effect names are introduced here.

4.7  Function Channel

| DMX Value | Function |
| --- | --- |
| 000-029 | No Function |
| 030-039 | Dimmer Curve: Linear |
| 040-049 | Dimmer Curve: Square |
| 050-059 | Dimmer Curve: Inverse Square |
| 060-069 | Dimmer Curve: S-Curve |
| 070-079 | Power Mode: Standard |
| 080-089 | No Function |
| 090-099 | Power Mode: Quiet |
| 100-109 | Enable LED PWM-frequency setting |
| 110-119 | Disable LED PWM-frequency setting |
| 120-122 | No Function |
| 123 | LED PWM Frequency: 900 Hz |
| 124 | LED PWM Frequency: 1000 Hz |
| 125 | LED PWM Frequency: 1100 Hz |
| 126 | LED PWM Frequency: 1200 Hz |
| 127 | LED PWM Frequency: 1300 Hz |
| 128 | LED PWM Frequency: 1400 Hz |
| 129 | LED PWM Frequency: 1500 Hz |
| 130 | LED PWM Frequency: 2500 Hz |
| 131 | LED PWM Frequency: 4000 Hz |
| 132 | LED PWM Frequency: 5000 Hz |
| 133 | LED PWM Frequency: 6000 Hz |
| 134 | LED PWM Frequency: 10 kHz |
| 135 | LED PWM Frequency: 15 kHz |
| 136 | LED PWM Frequency: 20 kHz |
| 137 | LED PWM Frequency: 25 kHz |
| 138-139 | No Function |
| 140-149 | Tilt Reset |
| 150-159 | Effect Reset |
| 160-169 | CCT Calibration: On |
| 170-179 | CCT Calibration: Off |
| 180-199 | No Function |
| 200-209 | Reset All |
| 210-219 | Dimmer Response: Fast |
| 220-229 | Dimmer Response: Smooth |
| 230-239 | Line LED Power: Off |
| 240-249 | Line LED Power: On |
| 250-255 | No Function |

FREQUENCY CORRECTION  The source contains isolated unit and digit errors. The corrected sequence is 900, 1000, 1100, 1200, 1300, 1400, 1500, 2500, 4000, 5000, 6000 Hz, followed by 10, 15, 20, and 25 kHz.

5  Inspection, Maintenance & Cleaning

Disconnect mains power and allow the fixture to cool before inspection or cleaning. There are no user-serviceable parts inside. Internal service and seal replacement must be performed by qualified personnel using genuine parts.

5.1 Inspection

Confirm that installation screws, clamps, and safety attachments are tight and free from corrosion.

Inspect the housing, lenses, seals, connector caps, mounting points, and rigging hardware for damage or deformation.

Check moving parts for excessive wear, imbalance, abnormal noise, or restricted tilt travel.

Inspect power and data cables for cuts, crushing, heat damage, fatigue, contamination, or damaged weather seals.

Correct every safety or weatherproofing issue before returning the fixture to service.

5.2 Cleaning

Clean lens surfaces regularly to prevent output loss caused by dust or residue.

Remove dust from cooling surfaces and ventilation paths regularly; frequency depends on the environment.

Use a soft, lint-free cloth lightly moistened with clean water. Do not use alcohol, solvents, abrasives, or high-pressure spray.

Before outdoor use, confirm that every gasket, cap, and compatible weatherproof connector is correctly seated.

Arrange a detailed electrical and mechanical inspection by a qualified technician at least every three months.

END OF MANUAL  Retain this manual with the fixture. Before operation, verify the product label, local electrical code, rigging requirements, weather conditions, and venue safety procedures.