ALP1080WV 10-HEAD WATERPROOF EFFECT LIGHT

10 x 80 W RGBW IP67
Motorized LED Bar

INDIVIDUAL TILT HEADS - DUAL-WHITE AUXILIARY ARRAYS - USER MANUAL

Professional beam, wash and synchronized 'Thousand Hands' effect fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Control Panel and Menu |
| 4 | Master/Slave Operation |
| 5 | DMX512 Channel Reference |
| 6 | Function-Channel Reference |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  No model code is shown in the supplied source. This edition therefore uses the descriptive designation '10 x 80 W RGBW IP67 Motorized LED Bar'. Confirm the product label, voltage rating and firmware before installation or programming.

1  Safety Information

DANGER  Disconnect power and allow the fixture to cool before cleaning, inspection, fuse replacement or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Connect the fixture only to a protected, properly grounded circuit that matches its product-label rating.

If an external flexible cable is damaged, have it replaced by the manufacturer, its service agent or similarly qualified personnel.

Do not operate the fixture with a damaged housing, lens, seal, connector or cable.

After a large temperature change, leave the fixture disconnected until it reaches ambient temperature and all condensation has evaporated.

Keep flammable materials at least 0.2 m from the fixture and provide adequate ventilation.

Do not look directly into the LEDs at close range. Flashing light may trigger seizures in susceptible people; follow local restrictions.

For overhead installation, use rated mounting hardware and an independently rated safety cable.

Carry the fixture only by its handles or mounting structure. Do not lift it by a moving head.

IP67 PROTECTION  IP67 protection depends on intact seals, secured protective caps and correctly mated waterproof connectors. Replace every cap after use and never open the housing while wet.

2  Product Overview

This ten-head outdoor effect bar combines individually motorized RGBW beam/wash heads with warm-white and cool-white auxiliary LED arrays. Independent tilt, color, pattern and pixel control create synchronized fan, wave and 'Thousand Hands' effects for stages, events and architectural entertainment applications.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Source power-supply notation | 36 V / 18 A + 24 V / 15 A (verify against the product label before connection) |
| Power consumption | 880 W |
| Main light source | 10 x 80 W RGBW 4-in-1 LEDs |
| Auxiliary light source | 360 x 2016 warm-white LEDs + 360 x 2016 cool-white LEDs |
| Control | DMX512 / Sound-active / Auto / Master-Slave |
| DMX personalities | 29CH / 56CH / 144CH / 216CH / 262CH / 282CH |
| Dimming | 32-bit, 0-100% linear dimming |
| Primary effects | Individual head tilt, wash, beam, patterns and synchronized movement |
| Operating temperature | -30 °C to 50 °C (source specification) |
| Strobe | 1-30 Hz |
| Connections | DMX512 input/output; power input/output |
| Ingress protection | IP67 |
| Construction / finish | Metal, black |
| Dimensions | 1009 x 153 x 331 mm |
| Net weight | 28.46 kg |

2.2 Main Functions

Ten independently controlled tilt heads with 16-bit positioning in the extended personalities.

RGBW color mixing plus warm-white and cool-white auxiliary output.

Built-in main-light, white-light and background patterns with speed and fade/trailing controls.

Multiple dimming, strobe, color-macro and synchronized movement options.

Six DMX personalities from compact whole-fixture control to detailed per-head and pixel control.

3  Control Panel and Menu

Press MENU to enter or return to the main menu, use UP/DOWN to select an item or change a value, and press ENTER to open an item or save the edited value. The main menu closes after approximately 30 seconds without input. Saved settings are retained after power-off.

PROTECTED SETTINGS  Factory calibration is password-protected (source password: 138). Record every original value and restrict changes to qualified service personnel; incorrect calibration can damage movement, optics or output.

| Menu | Item | Operation |
| --- | --- | --- |
| 1. System settings | 1. DMX address | Set the DMX start address from 1-512 with UP/DOWN; press ENTER to save. Default: 002. |
| 1. System settings | 2. Encoder | Select On/Off with UP/DOWN; press ENTER to save. Default: On. |
| 1. System settings | 3. Tilt Reverse | Select On/Off with UP/DOWN; press ENTER to save. Default: Off. |
| 1. System settings | 4. Zoom Reverse | Select On/Off with UP/DOWN; press ENTER to save. Default: Off. |
| 1. System settings | 5. No signal | Select Blackout or Hold with UP/DOWN; press ENTER to save. Default: Blackout. |
| 1. System settings | 6. Sound-active sensitivity | Set sound sensitivity from 0-99 with UP/DOWN; press ENTER to save. Default: 009. |
| 1. System settings | 7. Reset | Select Yes/No with UP/DOWN; press ENTER to execute. Default: No. |
| 1. System settings | 8. Restore factory settings | Select Yes/No with UP/DOWN; press ENTER to execute. Default: No. |
| 1. System settings | 9. Advanced settings |  |
| 2. Operation mode | 1. Operation mode | Select DMX, Sound-active or Auto with UP/DOWN; press ENTER to save. Default: DMX. |
| 2. Operation mode | 2. Channel selection | Select CH29, CH56, CH144, CH216, CH262 or CH282 with UP/DOWN; press ENTER to save. Default: CH29. |
| 2. Operation mode | 3. tilt effect amplitude | Set the tilt-effect amplitude from 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 1、Y | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 2. Y Fine | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 3. Tilt Speed | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 4. Tilt Effect | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 5、CTO | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 6. White CTC | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 7. Color macro | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 8. Pattern | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 9. White light pattern | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 10. LED mode | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 11. LED speed | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 12. LED tailing | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 13. White light mode | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 14. White light speed | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 15. White light trailing | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 16. Background color | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 17. Background color light | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 18. Dimming | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 19. White Master Dimmer | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 20. Strobe | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 21. White light strobe | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 22. Zoom | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 23. Functions | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 24. Red | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 25. Green | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 26. Blue | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 27. White | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 28. Cool white dimming | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 3. Manual control | 29. Warm white dimming | Set 0-255 with UP/DOWN; press ENTER to save. Default: 0. |
| 4. Display settings | 1. Language selection | Select Chinese or English with UP/DOWN; press ENTER to save. Default: Chinese. |
| 4. Display settings | 2. Inverted display settings | Select On/Off with UP/DOWN; press ENTER to save. Default: On. |
| 4. Display settings | 3. Screen protection | Select On/Off with UP/DOWN; press ENTER to save. Default: On. |
| 4. Display settings | 4. Software version | Displays the installed firmware identifier; read-only. |
| 5. Information settings | sensor | Used to view Hall status and temperature communication status |
| 5. Information settings | error message | Used to display error messages |
| 5. Information settings | Channel information | Used to display DMX channel data |
| 5. Information settings | Time information | Lights on time Fixture time |
| 6. Factory setting | 1. Head 1 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 2. Head 1 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 3. Head 2 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 4. Head 2 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 5. Head 3 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 6. Head 3 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 7. Head 4 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 8. Head 4 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 9. Head 5 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 10. Head 5 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 11. Head 6 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 12. Head 6 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 13. Head 7 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 14. Head 7 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 15. Head 8 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 16. Head 8 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 17. Head 9 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 18. Head 9 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 19. Head 10 Tilt Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 20. Head 10 Zoom Calibration | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 21, Y stroke | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 22. Focusing stroke | Service calibration, 0-255. Press ENTER to save. Default: 128. |
| 6. Factory setting | 23, red 1 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 24, green 1 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 25, blue 1 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 26, white 1 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 27, red 2 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 28, green 2 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 29, blue 2 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 30, white 2 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 31, red 3 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 32, green 3 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 33, blue 3 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 34, white 3 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 35, red 4 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 36, green 4 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 37, blue 4 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 38, white 4 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 39, red 5 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 40, green 5 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 41, blue 5 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 42, white 5 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 43, red 6 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 44, green 6 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 45, blue 6 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 46, white 6 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 47, red 7 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 48, green 7 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 49, blue 7 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 50, white 7 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 51, red 8 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 52, green 8 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 53, blue 8 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 54, white 8 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 55, red 9 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 56, green 9 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 57, blue 9 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 58, white 9 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 59, red 10 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 60, green 10 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 61, blue 10 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 62, white 10 | Service color calibration, 0-255. Press ENTER to save. Default: 252. |
| 6. Factory setting | 63. LED Y/F Direction | Select 000/001 with UP/DOWN; press ENTER to save. Default: 000. |
| 6. Factory setting | 64. LED Current | Service output-current setting, 32-255. Press ENTER to save. Default: 225. |
| 6. Factory setting | 65、Logo | Select 000 /001 with UP/DOWN, press ENTER to save, and 000 is the default. |
| 6. Factory setting | 66. Temperature | Set the temperature parameter from 40-70 with UP/DOWN; press ENTER to save. Default: 55. |
| 6. Factory setting | 67. Fan Temperature Mode | Select Off, Standard or Quiet with UP/DOWN; press ENTER to save. Default: Off. |
| 6. Factory setting | 68、LED | Modify LEDA/LEDB with UP/DOWN, confirm that the key is saved, and LEDA is the default. |
| 6. Factory setting | 69. Color mixing mode | Select RGBW or CMY with UP/DOWN; press ENTER to save. Default: RGBW. |
| 6. Factory setting | 70. PWM frequency | Select 600, 1200, 2000, 4000, 6000 or 15000 Hz; press ENTER to save. Default: 1200 Hz. |
| 6. Factory setting | 71. Dimmer Curve | Select Linear, Square, Inverse Square or S-curve; press ENTER to save. Default: Linear. |
| 6. Factory setting | 72. Dimming mode | Select Fast or Smooth with UP/DOWN; press ENTER to save. Default: Fast. |
| 6. Factory setting | 73. Movement Blackout | Select On/Off with UP/DOWN; press ENTER to save. Default: Off. |
| 6. Factory setting | 74. Clear Operating Time | Select Yes/No with UP/DOWN; press ENTER to execute. Default: No. |

4  Master/Slave Operation

Connect two or more identical fixtures with shielded DMX cable. Set every fixture to any valid DMX address from 1 to 512, designate only one fixture as the master and set all others as slaves. When the master runs fade, pulse, jump, sound-active or automatic programs, the slaves follow synchronously.

ONE MASTER ONLY  A linked group must contain only one master. Multiple masters will cause unsynchronized flashing. Master/slave operation is available only when the DMX512 controller is disconnected or switched off.

5  DMX512 Channel Reference

Set the fixture address and select the same personality on the fixture and controller. Link DMX OUT to DMX IN using shielded RS-485 cable. When a valid controller signal is received, the address display stops flashing. Channel wording can vary slightly with firmware; keep channel numbers and DMX ranges matched to the installed fixture.

SOURCE NOTATION  Ellipsis rows in the extended personalities reproduce repeated channel blocks that are abbreviated in the supplied source. Use the shown start/end channels and verify the complete controller profile against the installed firmware.

| Personality | Coverage |
| --- | --- |
| 29CH | Compact whole-fixture tilt, color, effects, dimming, strobe and auxiliary-white control |
| 56CH | Individual tilt/fine control for all ten heads plus shared light functions |
| 144CH | Shared movement with detailed RGBW control for each head and auxiliary LED groups |
| 216CH | Individual movement plus expanded main and auxiliary pixel control |
| 262CH | Extended individual-head and auxiliary control |
| 282CH | Maximum channel personality with the most detailed independent control |

5.1 29-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Tilt (tilt direction) |
| 2 | 0-255 | Tilt Fine (tilt effect amplitude) |
| 3 | 0-255 | Tilt Speed |
| 4 | 0-255 | Tilt Effect (4 values and 1 effect for motors). 0-3 no function 4-255 (63 effects total) |
| 5 | 0-255 | CTC |
| 6 | 0-255 | White light CTC |
| 7 | 0-255 | Color Macro |
| 8 | 0-255 | Pattern |
| 9 | 0-255 | White light pattern |
| 10 | 0-255 | LED built-in effect (main light, 2 values, 1 effect) 0-1 no function 2-255 (127 effects total) |
| 11 | 0-255 | LED built-in effect speed |
| 12 | 0-255 | LED built-in effect trailing time |
| 13 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light) 0-7 no function 8-255 (31 effects total) |
| 14 | 0-255 | White light built-in effect speed |
| 15 | 0-255 | White light built-in effect trailing time |
| 16 | 0-255 | LED background color |
| 17 | 0-255 | LED background tint light |
| 18 | 0-255 | Master Dimmer |
| 19 | 0-255 | White Master Dimmer |
| 20 | 0-255 | Strobe |
| 21 | 0-255 | White Strobe |
| 22 | 0-255 | Master Zoom |
| 23 | 0-255 | Function |
| 24 | 0-255 | Red (color selection for the effect) |
| 25 | 0-255 | Green (effect color selection) |
| 26 | 0-255 | Blue (effect color selection) |
| 27 | 0-255 | White (color selection for effect) |
| 28 | 0-255 | Cool white master dimmer |
| 29 | 0-255 | Warm White LED Group master dimmer |

5.2 56-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Head 1 Tilt (tilt direction) |
| 2 | 0-255 | Head 1 Tilt Fine |
| 3 | 0-255 | Head 2 Tilt (tilt effect amplitude) |
| 4 | 0-255 | Head 2 Tilt Fine |
| 5 | 0-255 | Head 3 Tilt |
| 6 | 0-255 | Head 3 Tilt Fine |
| 7 | 0-255 | Head 4 Tilt |
| 8 | 0-255 | Head 4 Tilt Fine |
| 9 | 0-255 | Head 5 Tilt |
| 10 | 0-255 | Head 5 Tilt Fine |
| 11 | 0-255 | Head 6 Tilt |
| 12 | 0-255 | Head 6 Tilt Fine |
| 13 | 0-255 | Head 7 Tilt |
| 14 | 0-255 | Head 7 Tilt Fine |
| 15 | 0-255 | Head 8 Tilt |
| 16 | 0-255 | Head 8 Tilt Fine |
| 17 | 0-255 | Head 9 Tilt |
| 18 | 0-255 | Head 9 Tilt Fine |
| 19 | 0-255 | Head 10 Tilt |
| 20 | 0-255 | Head 10 Tilt Fine |
| 21 | 0-255 | Tilt Speed |
| 22 | 0-255 | Tilt Effect (4 values and 1 effect for motors). 0-3 no function 4-255 (63 effects total) |
| 23 | 0-255 | CTC |
| 24 | 0-255 | White light CTC |
| 25 | 0-255 | Color Macro |
| 26 | 0-255 | Pattern |
| 27 | 0-255 | White light pattern |
| 28 | 0-255 | LED built-in effects (2 values for the main light, 1 effect). 0-1 no function 2-255 (127 effects total) |
| 29 | 0-255 | LED built-in effect speed |
| 30 | 0-255 | LED built-in effect trailing time |
| 31 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light). 0-7 no function 8-255 (31 effects total) |
| 32 | 0-255 | White light built-in effect speed |
| 33 | 0-255 | White light built-in effect trailing time |
| 34 | 0-255 | LED background color |
| 35 | 0-255 | LED background color light coarse adjustment |
| 36 | 0-255 | Master Dimmer |
| 37 | 0-255 | White Master Dimmer |
| 38 | 0-255 | Strobe |
| 39 | 0-255 | White Strobe |
| 40 | 0-255 | Head 1 Zoom |
| 41 | 0-255 | Head 2 Zoom |
| 42 | 0-255 | Head 3 Zoom |
| 43 | 0-255 | Head 4 Zoom |
| 44 | 0-255 | Head 5 Zoom |
| 45 | 0-255 | Head 6 Zoom |
| 46 | 0-255 | Head 7 Zoom |
| 47 | 0-255 | Head 8 Zoom |
| 48 | 0-255 | Head 9 Zoom |
| 49 | 0-255 | Head 10 Zoom |
| 50 | 0-255 | Function |
| 51 | 0-255 | Red (color selection for the effect) |
| 52 | 0-255 | Green (effect color selection) |
| 53 | 0-255 | Blue (effect color selection) |
| 54 | 0-255 | White (color selection for effect) |
| 55 | 0-255 | Cool white master dimmer |
| 56 | 0-255 | Warm White LED Group master dimmer |

5.3 144-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Master Dimmer |
| 2 | 0-255 | Master Dimmer Fine |
| 3 | 0-255 | Strobe |
| 4 | 0-255 | White Strobe |
| 5 | 0-255 | CTC |
| 6 | 0-255 | Pattern |
| 7 | 0-255 | White light pattern |
| 8 | 0-255 | Color Macro |
| 9 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light). 0-7 no function 8-255 (31 effects total) |
| 10 | 0-255 | White light built-in effect speed |
| 11 | 0-255 | White light built-in effect trailing time |
| 12 | 0-255 | Function |
| 13 | 0-255 | Head 1 Tilt (tilt direction) |
| 14 | 0-255 | Head 1 Tilt Fine |
| 15 | 0-255 | Head 1 Zoom |
| 16 | 0-255 | Head 1 Red coarse |
| 17 | 0-255 | Head 1 Red Fine |
| 18 | 0-255 | Head 1 Green coarse |
| 19 | 0-255 | Head 1 Green Fine |
| 20 | 0-255 | Head 1 Blue coarse |
| 21 | 0-255 | Head 1 Blue Fine |
| 22 | 0-255 | Head 1 White coarse |
| 23 | 0-255 | Head 1 White Fine |
| 24 | 0-255 | Cool white 1-2-3-58-59-60 |
| 25 | 0-255 | Warm White LED Group 1-2-3-58-59-60 |
| 26 | 0-255 | Head 2 Tilt (tilt effect amplitude) |
| 27 | 0-255 | Head 2 Tilt Fine |
| 28 | 0-255 | Head 2 Zoom |
| 29 | 0-255 | Head 2 Red coarse |
| 30 | 0-255 | Head 2 Red Fine |
| 31 | 0-255 | Head 2 Green coarse |
| 32 | 0-255 | Head 2 Green Fine |
| 33 | 0-255 | Head 2 Blue coarse |
| 34 | 0-255 | Head 2 Blue Fine |
| 35 | 0-255 | Head 2 White coarse |
| 36 | 0-255 | Head 2 White Fine |
| 37 | 0-255 | Cool white 4-5-6-55-56-57 |
| 38 | 0-255 | Warm White LED Group 4-5-6-55-56-57 |
| 39 | 0-255 | Head 3 Tilt |
| 40 | 0-255 | Head 3 Tilt Fine |
| 41 | 0-255 | Head 3 Zoom |
| 42 | 0-255 | Head 3 Red coarse |
| 43 | 0-255 | Head 3 Red Fine |
| 44 | 0-255 | Head 3 Green coarse |
| 45 | 0-255 | Head 3 Green Fine |
| 46 | 0-255 | Head 3 Blue coarse |
| 47 | 0-255 | Head 3 Blue Fine |
| 48 | 0-255 | Head 3 White coarse |
| 49 | 0-255 | Head 3 White Fine |
| 50 | 0-255 | Cool white 7-8-9-52-53-54 |
| 51 | 0-255 | Warm White LED Group 7-8-9-52-53-54 |
| 52 | 0-255 | Head 4 Tilt |
| 53 | 0-255 | Head 4 Tilt Fine |
| 54 | 0-255 | Head 4 Zoom |
| 55 | 0-255 | Head 4 Red coarse |
| 56 | 0-255 | Head 4 Red Fine |
| 57 | 0-255 | Head 4 Green coarse |
| 58 | 0-255 | Head 4 Green Fine |
| 59 | 0-255 | Head 4 Blue Coarse |
| 60 | 0-255 | Head 4 Blue Fine |
| 61 | 0-255 | Head 4 White coarse |
| 62 | 0-255 | Head 4 White Fine |
| 63 | 0-255 | Cool White LED Group 10-11-12-49-50-51 |
| 64 | 0-255 | Warm White LED Group 10-11-12-49-50-51 |
| 65 | 0-255 | Head 5 Tilt |
| 66 | 0-255 | Head 5 Tilt Fine |
| 67 | 0-255 | Head 5 Zoom |
| 68 | 0-255 | Head 5 Red coarse |
| 69 | 0-255 | Head 5 Red Fine |
| 70 | 0-255 | Head 5 Green coarse |
| 71 | 0-255 | Head 5 Green Fine |
| 72 | 0-255 | Head 5 Blue coarse |
| 73 | 0-255 | Head 5 Blue Fine |
| 74 | 0-255 | Head 5 White coarse |
| 75 | 0-255 | Head 5 White Fine |
| 76 | 0-255 | Cool White LED Group 13-14-15-46-47-48 |
| 77 | 0-255 | Warm White LED Group 13-14-15-46-47-48 |
| 78 | 0-255 | Head 6 Tilt |
| 79 | 0-255 | Head 6 Tilt Fine |
| 80 | 0-255 | Head 6 Zoom |
| 81 | 0-255 | Head 6 Red coarse |
| 82 | 0-255 | Head 6 Red Fine |
| 83 | 0-255 | Head 6 Green coarse |
| 84 | 0-255 | Head 6 Green Fine |
| 85 | 0-255 | Head 6 Blue Coarse |
| 86 | 0-255 | Head 6 Blue Fine |
| 87 | 0-255 | Head 6 White coarse |
| 88 | 0-255 | Head 6 White Fine |
| 89 | 0-255 | Cool White LED Group 16-17-18-43-44-45 |
| 90 | 0-255 | Warm White LED Group 16-17-18-43-44-45 |
| 91 | 0-255 | Head 7 Tilt |
| 92 | 0-255 | Head 7 Tilt Fine |
| 93 | 0-255 | Head 7 Zoom |
| 94 | 0-255 | Head 7 Red coarse |
| 95 | 0-255 | Head 7 Red Fine |
| 96 | 0-255 | Head 7 Green coarse |
| 97 | 0-255 | Head 7 Green Fine |
| 98 | 0-255 | Head 7 Blue coarse |
| 99 | 0-255 | Head 7 Blue Fine |
| 100 | 0-255 | Head 7 White coarse |
| 101 | 0-255 | Head 7 White Fine |
| 102 | 0-255 | Cool white 19-20-21-40-41-42 |
| 103 | 0-255 | Warm White LED Group 19-20-21-40-41-42 |
| 104 | 0-255 | Head 8 Tilt |
| 105 | 0-255 | Head 8 Tilt Fine |
| 106 | 0-255 | Head 8 Zoom |
| 107 | 0-255 | Head 8 Red coarse |
| 108 | 0-255 | Head 8 Red Fine |
| 109 | 0-255 | Head 8 Green coarse |
| 110 | 0-255 | Head 8 Green Fine |
| 111 | 0-255 | Head 8 Blue coarse |
| 112 | 0-255 | Head 8 Blue Fine |
| 113 | 0-255 | Head 8 White coarse |
| 114 | 0-255 | Head 8 White Fine |
| 115 | 0-255 | Cool White LED Group 22-23-24-37-38-39 |
| 116 | 0-255 | Warm White LED Group 22-23-24-37-38-39 |
| 117 | 0-255 | Head 9 Tilt |
| 118 | 0-255 | Head 9 Tilt Fine |
| 119 | 0-255 | Head 9 Zoom |
| 120 | 0-255 | Head 9 Red coarse |
| 121 | 0-255 | Head 9 Red Fine |
| 122 | 0-255 | Head 9 Green coarse |
| 123 | 0-255 | Head 9 Green Fine |
| 124 | 0-255 | Head 9 Blue coarse |
| 125 | 0-255 | Head 9 Blue Fine |
| 126 | 0-255 | Head 9 White coarse |
| 127 | 0-255 | Head 9 White Fine |
| 128 | 0-255 | Cool white 25-26-27-34-35-36 |
| 129 | 0-255 | Warm White LED Group 25-26-27-34-35-36 |
| 130 | 0-255 | Head 10 Tilt |
| 131 | 0-255 | Head 10 Tilt Fine |
| 132 | 0-255 | Head 10 Zoom |
| 133 | 0-255 | Head 10 Red coarse |
| 134 | 0-255 | Head 10 Red Fine |
| 135 | 0-255 | Head 10 Green coarse |
| 136 | 0-255 | Head 10 Green Fine |
| 137 | 0-255 | Head 10 Blue coarse |
| 138 | 0-255 | Head 10 Blue Fine |
| 139 | 0-255 | Head 10 White coarse |
| 140 | 0-255 | Head 10 White Fine |
| 141 | 0-255 | Cool white 28-29-30-31-32-33 |
| 142 | 0-255 | Warm White LED Group 28-29-30-31-32-33 |
| 143 | 0-255 | Cool white master dimmer |
| 144 | 0-255 | Warm White LED Group master dimmer |

5.4 216-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Head 1 Tilt (tilt direction) |
| 2 | 0-255 | Head 1 Tilt Fine |
| 3 | 0-255 | Head 2 Tilt (tilt effect amplitude) |
| 4 | 0-255 | Head 2 Tilt Fine |
| 5 | 0-255 | Head 3 Tilt |
| 6 | 0-255 | Head 3 Tilt Fine |
| 7 | 0-255 | Head 4 Tilt |
| 8 | 0-255 | Head 4 Tilt Fine |
| 9 | 0-255 | Head 5 Tilt |
| 10 | 0-255 | Head 5 Tilt Fine |
| 11 | 0-255 | Head 6 Tilt |
| 12 | 0-255 | Head 6 Tilt Fine |
| 13 | 0-255 | Head 7 Tilt |
| 14 | 0-255 | Head 7 Tilt Fine |
| 15 | 0-255 | Head 8 Tilt |
| 16 | 0-255 | Head 8 Tilt Fine |
| 17 | 0-255 | Head 9 Tilt |
| 18 | 0-255 | Head 9 Tilt Fine |
| 19 | 0-255 | Head 10 Tilt |
| 20 | 0-255 | Head 10 Tilt Fine |
| 21 | 0-255 | Tilt Speed |
| 22 | 0-255 | Tilt Effect (4 values and 1 effect for motors). 0-3 no function 4-255 (63 effects total) |
| 23 | 0-255 | CTC |
| 24 | 0-255 | White light CTC |
| 25 | 0-255 | Color Macro |
| 26 | 0-255 | Pattern |
| 27 | 0-255 | White light pattern |
| 28 | 0-255 | LED built-in effects (2 values for the main light, 1 effect). 0-1 no function 2-255 (127 effects total) |
| 29 | 0-255 | LED built-in effect speed |
| 30 | 0-255 | LED built-in effect trailing time |
| 31 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light). 0-7 no function 8-255 (31 effects total) |
| 32 | 0-255 | White light built-in effect speed |
| 33 | 0-255 | White light built-in effect trailing time |
| 34 | 0-255 | LED background color |
| 35 | 0-255 | LED background color light coarse adjustment |
| 36 | 0-255 | Master Dimmer |
| 37 | 0-255 | White Master Dimmer |
| 38 | 0-255 | Strobe |
| 39 | 0-255 | White Strobe |
| 40 | 0-255 | Head 1 Zoom |
| 41 | 0-255 | Head 2 Zoom |
| 42 | 0-255 | Head 3 Zoom |
| 43 | 0-255 | Head 4 Zoom |
| 44 | 0-255 | Head 5 Zoom |
| 45 | 0-255 | Head 6 Zoom |
| 46 | 0-255 | Head 7 Zoom |
| 47 | 0-255 | Head 8 Zoom |
| 48 | 0-255 | Head 9 Zoom |
| 49 | 0-255 | Head 10 Zoom |
| 50 | 0-255 | Function |
| 51 | 0-255 | red |
| 52 | 0-255 | green |
| 53 | 0-255 | blue |
| 54 | 0-255 | white |
| 55 | 0-255 | Cool white master dimmer |
| 56 | 0-255 | Warm White LED Group master dimmer |
| 57 | 0-255 | Head 1 Red (Effect Selection) |
| 58 | 0-255 | Head 1 Green (Effect color selection) |
| 59 | 0-255 | Head 1 Blue (effect color selection) |
| 60 | 0-255 | Head 1 White (effect color selection) |
| … | … … | … … |
| 93 | 0-255 | Head 10 Red |
| 94 | 0-255 | Head 10 Green |
| 95 | 0-255 | Head 10 Blue |
| 96 | 0-255 | Head 10 White |
| 97 | 0-255 | Cool White LED Group 1 |
| 98 | 0-255 | Warm White LED Group 1 |
| … | … … | … … |
| 215 | 0-255 | Cool white 60 |
| 216 | 0-255 | Warm White LED Group 60 |

5.5 262-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Head 1 Tilt (tilt direction) |
| 2 | 0-255 | Head 1 Tilt Fine |
| 3 | 0-255 | Head 2 Tilt (tilt effect amplitude) |
| 4 | 0-255 | Head 2 Tilt Fine |
| 5 | 0-255 | Head 3 Tilt |
| 6 | 0-255 | Head 3 Tilt Fine |
| 7 | 0-255 | Head 4 Tilt |
| 8 | 0-255 | Head 4 Tilt Fine |
| 9 | 0-255 | Head 5 Tilt |
| 10 | 0-255 | Head 5 Tilt Fine |
| 11 | 0-255 | Head 6 Tilt |
| 12 | 0-255 | Head 6 Tilt Fine |
| 13 | 0-255 | Head 7 Tilt |
| 14 | 0-255 | Head 7 Tilt Fine |
| 15 | 0-255 | Head 8 Tilt |
| 16 | 0-255 | Head 8 Tilt Fine |
| 17 | 0-255 | Head 9 Tilt |
| 18 | 0-255 | Head 9 Tilt Fine |
| 19 | 0-255 | Head 10 Tilt |
| 20 | 0-255 | Head 10 Tilt Fine |
| 21 | 0-255 | Tilt Speed |
| 22 | 0-255 | Tilt Effect (4 values and 1 effect for motors) 0-3 no function 4-255 (63 effects total) |
| 23 | 0-255 | CTC |
| 24 | 0-255 | White light CTC |
| 25 | 0-255 | Color Macro |
| 26 | 0-255 | Pattern |
| 27 | 0-255 | White light pattern |
| 28 | 0-255 | LED built-in effect (main light, 2 values, 1 effect) 0-1 no function 2-255 (127 effects total) |
| 29 | 0-255 | LED built-in effect speed |
| 30 | 0-255 | LED built-in effect trailing time |
| 31 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light) 0-7 no function 8-255 (31 effects total) |
| 32 | 0-255 | White light built-in effect speed |
| 33 | 0-255 | White light built-in effect trailing time |
| 34 | 0-255 | LED background color |
| 35 | 0-255 | LED background color light coarse adjustment |
| 36 | 0-255 | LED background tint light adjustment |
| 37 | 0-255 | Master Dimmer |
| 38 | 0-255 | Master Dimmer Fine |
| 39 | 0-255 | White Master Dimmer |
| 40 | 0-255 | Strobe |
| 41 | 0-255 | White Strobe |
| 42 | 0-255 | Head 1 Zoom |
| 43 | 0-255 | Head 2 Zoom |
| 44 | 0-255 | Head 3 Zoom |
| 45 | 0-255 | Head 4 Zoom |
| 46 | 0-255 | Head 5 Zoom |
| 47 | 0-255 | Head 6 Zoom |
| 48 | 0-255 | Head 7 Zoom |
| 49 | 0-255 | Head 8 Zoom |
| 50 | 0-255 | Head 9 Zoom |
| 51 | 0-255 | Head 10 Zoom |
| 52 | 0-255 | Function |
| 53 | 0-255 | red |
| 54 | 0-255 | Red Fine |
| 55 | 0-255 | green |
| 56 | 0-255 | Green Fine |
| 57 | 0-255 | blue |
| 58 | 0-255 | Blue Fine |
| 59 | 0-255 | white |
| 60 | 0-255 | White Fine |
| 61 | 0-255 | Cool white master dimmer |
| 62 | 0-255 | Warm White LED Group master dimmer |
| 63 | 0-255 | Head 1 Red coarse (effect color selection) |
| 64 | 0-255 | Head 1 Red Fine |
| 65 | 0-255 | Head 1 Green coarse (effect color selection) |
| 66 | 0-255 | Head 1 Green Fine |
| 67 | 0-255 | Head 1 Blue coarse adjustment (effect color selection) |
| 68 | 0-255 | Head 1 Blue Fine |
| 69 | 0-255 | Head 1 White coarse (color selection for effect) |
| 70 | 0-255 | Head 1 White Fine |
| … | … … | … … |
| 135 | 0-255 | Head 10 Red coarse |
| 136 | 0-255 | Head 10 Red Fine |
| 137 | 0-255 | Head 10 Green coarse |
| 138 | 0-255 | Head 10 Green Fine |
| 139 | 0-255 | Head 10 Blue coarse |
| 140 | 0-255 | Head 10 Blue Fine |
| 141 | 0-255 | Head 10 White coarse |
| 142 | 0-255 | Head 10 White Fine |
| 143 | 0-255 | Cool White LED Group 1 |
| 144 | 0-255 | Warm White LED Group 1 |
| … | … … | … … |
| 261 | 0-255 | Cool white 60 |
| 262 | 0-255 | Warm White LED Group 60 |

5.6 282-Channel Personality

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0-255 | Head 1 Tilt (tilt direction) |
| 2 | 0-255 | Head 1 Tilt Fine |
| 3 | 0-255 | Head 2 Tilt (tilt effect amplitude) |
| 4 | 0-255 | Head 2 Tilt Fine |
| 5 | 0-255 | Head 3 Tilt |
| 6 | 0-255 | Head 3 Tilt Fine |
| 7 | 0-255 | Head 4 Tilt |
| 8 | 0-255 | Head 4 Tilt Fine |
| 9 | 0-255 | Head 5 Tilt |
| 10 | 0-255 | Head 5 Tilt Fine |
| 11 | 0-255 | Head 6 Tilt |
| 12 | 0-255 | Head 6 Tilt Fine |
| 13 | 0-255 | Head 7 Tilt |
| 14 | 0-255 | Head 7 Tilt Fine |
| 15 | 0-255 | Head 8 Tilt |
| 16 | 0-255 | Head 8 Tilt Fine |
| 17 | 0-255 | Head 9 Tilt |
| 18 | 0-255 | Head 9 Tilt Fine |
| 19 | 0-255 | Head 10 Tilt |
| 20 | 0-255 | Head 10 Tilt Fine |
| 21 | 0-255 | Tilt Speed |
| 22 | 0-255 | Tilt Effect (4 values and 1 effect for motors). 0-3 no function 4-255 (63 effects total) |
| 23 | 0-255 | CTC |
| 24 | 0-255 | White light CTC |
| 25 | 0-255 | Color Macro |
| 26 | 0-255 | Pattern |
| 27 | 0-255 | White light pattern |
| 28 | 0-255 | LED built-in effects (2 values for the main light, 1 effect). 0-1 no function 2-255 (127 effects total) |
| 29 | 0-255 | LED built-in effect speed |
| 30 | 0-255 | LED built-in effect trailing time |
| 31 | 0-255 | Built-in white light effect (8 values and 1 effect for fill light). 0-7 no function 8-255 (31 effects total) |
| 32 | 0-255 | White light built-in effect speed |
| 33 | 0-255 | White light built-in effect trailing time |
| 34 | 0-255 | LED background color |
| 35 | 0-255 | LED background color light coarse adjustment |
| 36 | 0-255 | LED background tint light adjustment |
| 37 | 0-255 | Master Dimmer |
| 38 | 0-255 | Master Dimmer Fine |
| 39 | 0-255 | White Master Dimmer |
| 40 | 0-255 | Strobe |
| 41 | 0-255 | White Strobe |
| 42 | 0-255 | Head 1 Zoom |
| 43 | 0-255 | Head 2 Zoom |
| 44 | 0-255 | Head 3 Zoom |
| 45 | 0-255 | Head 4 Zoom |
| 46 | 0-255 | Head 5 Zoom |
| 47 | 0-255 | Head 6 Zoom |
| 48 | 0-255 | Head 7 Zoom |
| 49 | 0-255 | Head 8 Zoom |
| 50 | 0-255 | Head 9 Zoom |
| 51 | 0-255 | Head 10 Zoom |
| 52 | 0-255 | Function |
| 53 | 0-255 | red |
| 54 | 0-255 | Red Fine |
| 55 | 0-255 | green |
| 56 | 0-255 | Green Fine |
| 57 | 0-255 | blue |
| 58 | 0-255 | Blue Fine |
| 59 | 0-255 | white |
| 60 | 0-255 | White Fine |
| 61 | 0-255 | Cool white master dimmer |
| 62 | 0-255 | Warm White LED Group master dimmer |
| 63 | 0-255 | Dimming 1 coarse adjustment |
| 64 | 0-255 | Dimming 1 Fine |
| 65 | 0-255 | Head 1 Red coarse (effect color selection) |
| 66 | 0-255 | Head 1 Red Fine |
| 67 | 0-255 | Head 1 Green coarse (effect color selection) |
| 68 | 0-255 | Head 1 Green Fine |
| 69 | 0-255 | Head 1 Blue coarse adjustment (effect color selection) |
| 70 | 0-255 | Head 1 Blue Fine |
| 71 | 0-255 | Head 1 White coarse (color selection for effect) |
| 72 | 0-255 | Head 1 White Fine |
| … | … … | … … |
| 153 | 0-255 | Dimming 10 coarse adjustments |
| 154 | 0-255 | Dimming 10 Fine |
| 155 | 0-255 | Head 10 Red coarse |
| 156 | 0-255 | Head 10 Red Fine |
| 157 | 0-255 | Head 10 Green coarse |
| 158 | 0-255 | Head 10 Green Fine |
| 159 | 0-255 | Head 10 Blue coarse |
| 160 | 0-255 | Head 10 Blue Fine |
| 161 | 0-255 | Head 10 White coarse |
| 162 | 0-255 | Head 10 White Fine |
| 163 | 0-255 | Cool White LED Group 1 |
| 164 | 0-255 | Warm White LED Group 1 |
| … | … … | … … |
| 281 | 0-255 | Cool white 60 |
| 282 | 0-255 | Warm White LED Group 60 |

6  Function-Channel Reference

The function channel changes global operating behavior, reset functions, color-mixing mode, dimmer response, PWM frequency, movement limits and other fixture options. Hold reset-related ranges only as long as required by the installed firmware.

| No. | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-009 | No function |
| 2 | 010-014 | Blackout on tilt on |
| 3 | 015-019 | Blackout on tilt off |
| 4 | 020-024 | RGBW (additive) color-mixing mode |
| 5 | 025-029 | CMY (subtractive) color-mixing mode |
| 6 | 030-034 | Combine heads |
| 7 | 035-039 | Disable combine heads |
| 8 | 040-044 | Reserved for future use |
| 9 | 045-049 | Reserved for future use |
| 10 | 050-054 | Reserved for future use |
| 11 | 055-059 | Tilt reset |
| 12 | 060-064 | Zoom reset |
| 13 | 065-069 | Quiet zoom reset |
| 14 | 070-074 | All reset |
| 15 | 075-079 | Cell order 1-10 |
| 16 | 080-084 | Cell order 10-1 |
| 17 | 085-089 | Reservedđer future use |
| 18 | 090-094 | Tilt reverse |
| 19 | 095-099 | Reserved for future use |
| 20 | 100-104 | Disable tilt reverse |
| 21 | 105-109 | Reserved for future use |
| 22 | 110-114 | Reserved for future use |
| 23 | 115-119 | Reserved for future use |
| 24 | 120-134 | Reserved for future use |
| 25 | 135-139 | Dimmer fast |
| 26 | 140-144 | Dimmer smooth |
| 27 | 145-149 | Linear |
| 28 | 150-154 | Square |
| 29 | 155-159 | I Squa |
| 30 | 160-164 | S-Curve |
| 31 | 165-169 | Reserved for future use |
| 32 | 170-174 | Reserved for future use |
| 33 | 175-179 | PWM 600HZ |
| 34 | 180-184 | PWM 1200HZ |
| 35 | 185-189 | PWM 2000 HZ |
| 36 | 190-194 | PWM 4000 HZ |
| 37 | 195-199 | PWM 6000 HZ |
| 38 | 200-204 | PWM 15000 HZ |
| 39 | 205-255 | Reserved for future use |

TILT-EFFECT AMPLITUDE  If the menu Tilt Effect Amplitude is 0, use the whole-tilt fine control or Head 2 Tilt control to set the amplitude. If the menu value is 1-255, function-channel values 205-255 control amplitude; at 0-204, the saved menu amplitude is used.

6.1 Strobe Ranges

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 000-019 | Blackout |
| 2 | 020-024 | Open |
| 3 | 025-064 | Strobe, fast to slow |
| 4 | 065-069 | Open |
| 5 | 070-084 | 100-0% strobe, from fast to slow |
| 6 | 085-089 | Open |
| 7 | 090-104 | 0-100% strobe, from fast to slow |
| 8 | 105-109 | Open |
| 9 | 110-124 | Random strobe, from fast to slow |
| 10 | 125-129 | Open |
| 11 | 130-144 | Random 100-0% strobe, from fast to slow |
| 12 | 145-149 | Open |
| 13 | 150-164 | Random 0-100% strobe, from fast to slow |
| 14 | 165-169 | Open |
| 15 | 170-184 | Pulse strobe 1, from fast to slow |
| 16 | 185-189 | Open |
| 17 | 190-204 | Random pulse strobe, from fast to slow |
| 18 | 205-209 | Open |
| 19 | 210-224 | 0-100-0% strobe, from fast to slow |
| 20 | 225-229 | Open |
| 21 | 230-244 | Pulse Strobe 2, from fast to slow |
| 22 | 245-255 | Open |

6.2 Color-Temperature and Color-Macro Ranges

| Channel | DMX Value | Function |
| --- | --- | --- |
| 1 | 0 | No function |
| 2 | 001-002 | 2700 K |
| 3 | 003-004 | 3200 K |
| 4 | 005-006 | 4200 K |
| 5 | 007-008 | 5600 K |
| 6 | 009-010 | 8000 K |
| 7 | 011 | blue |
| 8 | 012-048 | Blue to cyan |
| 9 | 049 | cyan |
| 10 | 050-086 | Cyan to green |
| 11 | 087 | green |
| 12 | 088-124 | Green to yellow |
| 13 | 125 | Yellow |
| 14 | 126-162 | Yellow to red |
| 15 | 163 | red |
| 16 | 164-200 | Red to magenta |
| 17 | 201 | magenta |
| 18 | 202-238 | Magenta to blue |
| 19 | 239 | blue |
| 20 | 240-247 | Gradient effect, from fast to slow |
| 21 | 248-255 | Pulse effect, from fast to slow |

7  Routine Maintenance

ATTENTION  Disconnect power, allow the fixture to cool and ensure all connector interfaces are dry before inspection or cleaning.

Inspect the housing, lenses, seals, mounting points, safety cable, power cable and waterproof connectors before every use.

Replace damaged lenses, seals, cables and structural parts before returning the fixture to service.

Clean external optical surfaces with a soft, lint-free cloth and a suitable glass-cleaning fluid; dry every part carefully.

Keep ventilation paths clear. The housing may become hot during operation, so maintain free airflow around the fixture.

Switch the fixture off completely at the circuit breaker or plug when it is not in use to reduce wear and extend service life.

Do not open the sealed housing unless qualified and equipped to restore its IP67 protection correctly.

8  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| No light output | Verify the product-label voltage, power connection, circuit protection, control switch, available supply capacity and controller output. |
| No DMX response after reset | Confirm DMX mode, start address, selected 29/56/144/216/262/282CH personality, controller patch, cable polarity and connectors. |
| Intermittent or unstable DMX | Shorten the data run, separate signal and high-voltage cables, use shielded twisted-pair cable, add an amplifier if needed and terminate the final fixture with 120 ohms. |
| Fixture does not start | Check the supply rating and inspect for transport damage, moisture, loose internal connectors or damaged electronics. Refer internal inspection to qualified service. |
| Unexpected head movement | Remove obstructions, verify tilt direction and effect-amplitude settings, run the appropriate reset and confirm the selected personality. |
| Reduced or unstable output | Check temperature, cooling, optical cleanliness, supply quality and error messages. |
| Water or condensation suspected | Disconnect power, move the fixture to a dry service area and have seals and connectors inspected before reuse. |

SERVICE  There are no user-serviceable high-voltage or sealed parts inside. Refer internal repair, light-source replacement, seal replacement and calibration to the manufacturer, its service agent or similarly qualified personnel.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Include the product-label model, serial number, firmware version, active personality and a clear fault description when requesting support.

End of manual