ALP1260WV

12 x 60 W LED Moving Head
Waver IP65

12 INDEPENDENT TILT + ZOOM HEADS - RGBW + AUXILIARY LIGHT - USER MANUAL

Professional IP65 beam, wash and synchronized wave-effect fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Function-Channel Reference |
| 7 | Routine Maintenance |
| 8 | Troubleshooting, RDM and Service |

MODEL IDENTIFICATION  This manual is for ALP1260WV. Confirm the model, voltage rating and firmware shown on the actual fixture before installation or programming.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool before opening, cleaning, inspection or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Use only AC 100-240 V, 50/60 Hz as specified by the technical data and product label. Connect to a protected, properly grounded Class I supply.

Do not operate the fixture with a damaged housing, lens, waterproof connector, mounting point, cable or seal.

After a large temperature change, leave the fixture disconnected until it reaches ambient temperature and all condensation has evaporated.

Do not look directly into the LEDs at close range. Flashing light may trigger seizures in susceptible people; follow local restrictions.

For overhead installation, use rated clamps and an independently rated safety cable attached to the designated point.

Carry the fixture only by its handles or fixed structure. Do not lift it by the moving heads.

Keep ventilation openings clear. Never cover the fixture or operate it near flammable or explosive materials.

IP65 CONDITIONS  IP65 protection applies only when all connector caps, cable glands, covers and seals are correctly fitted and undamaged. IP65 is not an immersion rating. Do not aim high-pressure water jets at the fixture.

2  Product Overview

The ALP1260WV combines twelve independently moving 60 W RGBW heads with motorized zoom and a 432-pixel white/golden-amber auxiliary-light system. It is designed for synchronized wave, beam and wash effects in touring, stage, theatre, studio, club and outdoor-event applications where IP65 protection is required.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Rated power | 850 W (source specification) |
| Main light source | 12 x 60 W RGBW 4-in-1 LEDs |
| Auxiliary light source | 432 x 1 W bi-color LEDs: white + golden amber |
| Control | DMX512 / Auto 1 / Auto 2 / Master-Slave / Sound-active / RDM |
| DMX personalities | 26CH / 59CH / 128CH / 152CH / 182CH / 314CH / 329CH |
| Dimming | 32-bit, 0-100% linear dimming; selectable response and curves |
| Movement / optics | 12 independent tilt and motorized zoom heads |
| Primary effects | Independent wave movement, beam, wash, patterns, color macros and auxiliary-light effects |
| Operating temperature | -30 °C to 50 °C (source specification) |
| Strobe | 1-30 Hz |
| Firmware update | Via DMX connection |
| Connections | DMX512 input/output and waterproof power input/output |
| Ingress protection | IP65 |
| Construction / finish | Metal, black |

2.2 Main Functions

Twelve independently controllable tilt and zoom heads with RGBW color mixing.

White and golden-amber auxiliary LEDs with independent dimming, CTO, strobe and built-in effects.

Built-in tilt macros, color macros, patterns, LED effects and background colors.

Compact shared personalities and expanded per-head personalities up to 329 channels.

RDM support, DMX firmware updating, sound-active operation and synchronized parameter update.

Built-in alignment pins for rapid mechanical joining of compatible fixtures.

3  Setup and Installation

3.1 AC Power

Before connecting power, verify that the product-label voltage and frequency match the local supply. Complete all mounting and data connections before applying power. Keep waterproof connector caps fitted whenever a connector is unused.

3.2 Rigging

STRUCTURAL RATING  The mounting point and every attachment must be rated for at least ten times the fixture load or the higher factor required by local regulations. Keep people clear of the area below during installation and service.

Inspect the fixture, clamps, fasteners, mounting points, safety cable, seals and support structure.

Attach correctly rated clamps to the designated mounting points and tighten all fasteners.

Attach an independently rated safety cable to the designated safety point; never attach it to a moving head or handle.

Confirm clearance for the complete movement and zoom range of all twelve heads.

Recheck every attachment and waterproof connection before applying power.

3.3 DMX512 / RDM Connection

Use shielded, twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Assign a unique start address to every independently controlled fixture. RDM requires a compatible controller and bidirectional data equipment.

TERMINATION  At the last fixture in a long or electrically noisy DMX chain, install a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use MENU to return, UP/DOWN to select or change a value, and ENTER to open an item or confirm a setting. The display also shows the DMX indicator, error indicator, fixture temperature, address and active personality. Menu labels may vary slightly with firmware revision.

| Menu | Item | Option / Range | Operation |
| --- | --- | --- | --- |
| Address | DMX Start Address | 001-512 | Set the fixture start address. |
| System Settings | Operating Mode | DMX / Auto 1 / Auto 2 / Sound | Select controller, automatic or sound-active operation. |
| System Settings | Channel Mode | 26 / 59 / 128 / 152 / 182 / 314 / 329CH | Select the DMX personality. |
| System Settings | Pixel Order Reverse | Off / On | Reverse the head and pixel sequence. |
| System Settings | Tilt Reverse | Off / On | Reverse the tilt direction. |
| System Settings | DMX Hold | Off / On | Return to reset state or hold the last DMX frame after signal loss. |
| System Settings | Screen Saver | Off / On | Keep the display on or turn it off after inactivity. |
| System Settings | Display Orientation | Normal / Inverted / Auto | Set or automatically orient the display. |
| System Settings | Synchronized Update | Off / On | Copy settings/calibration to linked fixtures; disconnect the controller first. |
| System Settings | Language | Chinese / English | Select the menu language. |
| System Settings | Dimmer Curve | Linear / Square / Inverse Square / S-Curve | Select the dimming curve. |
| System Settings | Dimmer Response | Fast / Smooth | Select fast or smoothed dimming. |
| System Settings | Head ID | Off / 1-12 | Service head identification and assignment. |
| System Settings | Factory Reset | Cancel / Confirm | Restore factory settings. |
| Manual | Current Personality Channels | 000-255 | Manually test each channel in the selected personality. |
| Calibration | Password | 6688 | Open protected calibration settings. |
| Calibration | Home Position | Tilt 1-12 / Zoom 1-12, ±127 | Correct mechanical home offsets; record original values first. |
| Calibration | White Balance | Head 1-12 RGBW, 000-255 | Service-only per-head white-balance calibration; 255 = unchanged. |
| Calibration | Maximum Power | 000-255 | Set the maximum fixture power; 255 = maximum. |
| Reset | Effect / Movement / All | Confirm | Reset effect motors, movement motors or the complete fixture. |
| Information | Reset Errors | Read-only | View board communication, tilt sensor and zoom reset faults. |
| Information | DMX Monitor | Read-only | View received channel values. |
| Information | Sensors | Read-only | View zoom Hall, tilt Hall and temperature sensor status. |
| Information | Hardware / Software Version | Read-only | View installed hardware and firmware versions. |

PROTECTED SETTINGS  Home-position, white-balance, power and head-ID functions are service settings. Record original values before editing; incorrect values can cause positioning faults or abnormal output.

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. Repeated per-head blocks below are written as formulas so every channel remains unambiguous without repeating hundreds of identical rows.

| Personality | Coverage |
| --- | --- |
| 26CH | Compact shared tilt, zoom, RGBW, effects and auxiliary-light control |
| 59CH | Independent tilt/fine and zoom for 12 heads with shared color and auxiliary light |
| 128CH | Independent tilt/zoom plus per-head RGBW and two auxiliary colors |
| 152CH | Independent tilt/zoom plus per-head RGBW and four auxiliary zones |
| 182CH | 16-bit RGBW per head plus two auxiliary colors |
| 314CH | Maximum per-head tilt, zoom, dimmer, 16-bit RGBW and six auxiliary zones |
| 329CH | 16-bit shared/per-head color, dimmer, zoom and six auxiliary zones with global auxiliary effects |

5.1  26-Channel Personality

Compact shared control of all twelve heads and the auxiliary-light system.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Heads 1-12 Tilt | 000-255 | 0-100% |
| 2 | Heads 1-12 Tilt Fine | 000-255 | Fine adjustment |
| 3 | Tilt Speed | 000-255 | Fast to slow |
| 4 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 5 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 6 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 7 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 8 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 9 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 10 | LED Effect Delay | 000-255 | Fast to slow |
| 11 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 12 | Background Dimmer | 000-255 | 0-100% |
| 13 | Master Dimmer | 000-255 | 0-100% |
| 14 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 15 | Heads 1-12 Zoom | 000-255 | 0-100% |
| 16 | Control | 000-009 / 010-255 | No function / See Control reference |
| 17 | Red | 000-255 | 0-100% |
| 18 | Green | 000-255 | 0-100% |
| 19 | Blue | 000-255 | 0-100% |
| 20 | White | 000-255 | 0-100% |
| 21 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 22 | Auxiliary White | 000-255 | 0-100% |
| 23 | Auxiliary Golden Amber | 000-255 | 0-100% |
| 24 | Auxiliary CTO | 000-067 / 068-247 / 248-255 | None / CTO / None |
| 25 | Auxiliary Effect | 000-007 / 008-191 / 192-255 | None / Effect 1-23 / None; 8 values per effect |
| 26 | Auxiliary Effect Speed | 000-127 / 128-255 | Snap-change slow to fast / Fade slow to fast |

5.2  59-Channel Personality

Independent 16-bit tilt and zoom for all heads, with shared main and auxiliary color control.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1-2 | Head 1 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 3-4 | Head 2 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 5-6 | Head 3 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 7-8 | Head 4 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 9-10 | Head 5 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 11-12 | Head 6 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 13-14 | Head 7 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 15-16 | Head 8 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 17-18 | Head 9 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 19-20 | Head 10 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 21-22 | Head 11 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 23-24 | Head 12 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 25 | Tilt Speed | 000-255 | Fast to slow |
| 26 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 27 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 28 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 29 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 30 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 31 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 32 | LED Effect Delay | 000-255 | Fast to slow |
| 33 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 34 | Background Dimmer | 000-255 | 0-100% |
| 35 | Master Dimmer | 000-255 | 0-100% |
| 36 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 37-48 | Head 1-12 Zoom | 000-255 | Channel = 36 + head number; 0-100% |
| 49 | Control | 000-009 / 010-255 | No function / See Control reference |
| 50 | Red / Cyan | 000-255 | RGBW 0-100% / CMY 100-0% |
| 51 | Green / Magenta | 000-255 | RGBW 0-100% / CMY 100-0% |
| 52 | Blue / Yellow | 000-255 | RGBW 0-100% / CMY 100-0% |
| 53 | White | 000-255 | 0-100% |
| 54 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 55 | Auxiliary White | 000-255 | 0-100% |
| 56 | Auxiliary Golden Amber | 000-255 | 0-100% |
| 57 | Auxiliary CTO | 000-067 / 068-247 / 248-255 | None / CTO / None |
| 58 | Auxiliary Effect | 000-007 / 008-191 / 192-255 | None / Effect 1-23 / None |
| 59 | Auxiliary Effect Speed | 000-127 / 128-255 | Snap-change slow to fast / Fade slow to fast |

5.3  128-Channel Personality

Per-head RGBW and two auxiliary colors, with independent tilt and zoom.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1-2 | Head 1 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 3-4 | Head 2 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 5-6 | Head 3 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 7-8 | Head 4 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 9-10 | Head 5 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 11-12 | Head 6 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 13-14 | Head 7 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 15-16 | Head 8 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 17-18 | Head 9 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 19-20 | Head 10 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 21-22 | Head 11 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 23-24 | Head 12 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 25 | Tilt Speed | 000-255 | Fast to slow |
| 26 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 27 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 28 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 29 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 30 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 31 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 32 | LED Effect Delay | 000-255 | Fast to slow |
| 33 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 34 | Background Dimmer | 000-255 | 0-100% |
| 35 | Master Dimmer | 000-255 | 0-100% |
| 36 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 37 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 38-49 | Head 1-12 Zoom | 000-255 | Channel = 37 + head number; 0-100% |
| 50 | Control | 000-009 / 010-255 | No function / See Control reference |
| 51-56 | Shared RGBW + Auxiliary White/Amber | 000-255 | Red, Green, Blue, White, Auxiliary White, Auxiliary Golden Amber |
| 57-128 | Per-head 6-channel color block | 000-255 | Base = 57 + (head-1) x 6; offsets: R, G, B, W, Aux White, Aux Golden Amber |
| 57-62 | Head 1 color block | 000-255 | R, G, B, W, Aux White, Aux Golden Amber |
| 123-128 | Head 12 color block | 000-255 | R, G, B, W, Aux White, Aux Golden Amber |

5.4  152-Channel Personality

Per-head RGBW and four auxiliary-light zones, with independent tilt and zoom.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1-2 | Head 1 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 3-4 | Head 2 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 5-6 | Head 3 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 7-8 | Head 4 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 9-10 | Head 5 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 11-12 | Head 6 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 13-14 | Head 7 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 15-16 | Head 8 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 17-18 | Head 9 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 19-20 | Head 10 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 21-22 | Head 11 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 23-24 | Head 12 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 25 | Tilt Speed | 000-255 | Fast to slow |
| 26 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 27 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 28 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 29 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 30 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 31 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 32 | LED Effect Delay | 000-255 | Fast to slow |
| 33 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 34 | Background Dimmer | 000-255 | 0-100% |
| 35 | Master Dimmer | 000-255 | 0-100% |
| 36 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 37 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 38-49 | Head 1-12 Zoom | 000-255 | Channel = 37 + head number; 0-100% |
| 50 | Control | 000-009 / 010-255 | No function / See Control reference |
| 51-56 | Shared RGBW + Auxiliary White/Amber | 000-255 | Shared color control |
| 57-152 | Per-head 8-channel color block | 000-255 | Base = 57 + (head-1) x 8; R, G, B, W, Aux White Up, Aux Amber Up, Aux White Down, Aux Amber Down |
| 57-64 | Head 1 color block | 000-255 | R, G, B, W, four auxiliary zones |
| 145-152 | Head 12 color block | 000-255 | R, G, B, W, four auxiliary zones |

5.5  182-Channel Personality

16-bit per-head RGBW, two auxiliary colors, independent 16-bit tilt and independent zoom.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1-2 | Head 1 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 3-4 | Head 2 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 5-6 | Head 3 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 7-8 | Head 4 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 9-10 | Head 5 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 11-12 | Head 6 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 13-14 | Head 7 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 15-16 | Head 8 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 17-18 | Head 9 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 19-20 | Head 10 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 21-22 | Head 11 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 23-24 | Head 12 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 25 | Tilt Speed | 000-255 | Fast to slow |
| 26 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 27 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 28 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 29 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 30 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 31 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 32 | LED Effect Delay | 000-255 | Fast to slow |
| 33 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 34 | Background Dimmer | 000-255 | 0-100% |
| 35 | Background Dimmer Fine | 000-255 | Fine adjustment |
| 36 | Master Dimmer | 000-255 | 0-100% |
| 37 | Master Dimmer Fine | 000-255 | Fine adjustment |
| 38 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 39 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 40-51 | Head 1-12 Zoom | 000-255 | Channel = 39 + head number; 0-100% |
| 52 | Control | 000-009 / 010-255 | No function / See Control reference |
| 53-60 | Shared RGBW / Fine | 000-255 | R, R Fine, G, G Fine, B, B Fine, W, W Fine |
| 61-62 | Shared Auxiliary White / Amber | 000-255 | 0-100% |
| 63-182 | Per-head 10-channel color block | 000-255 | Base = 63 + (head-1) x 10; R/R Fine, G/G Fine, B/B Fine, W/W Fine, Aux White, Aux Amber |
| 63-72 | Head 1 color block | 000-255 | 16-bit RGBW plus two auxiliary colors |
| 173-182 | Head 12 color block | 000-255 | 16-bit RGBW plus two auxiliary colors |

5.6  314-Channel Personality

Maximum independent control: a 25-channel block for each of the twelve heads.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Control | 000-009 / 010-255 | No function / See Control reference |
| 2 | Tilt Speed | 000-255 | Fast to slow |
| 3 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 4 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 5 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 6 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 7 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 8 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 9 | LED Effect Delay | 000-255 | Fast to slow |
| 10 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 11 | Background Dimmer | 000-255 | 0-100% |
| 12 | Background Dimmer Fine | 000-255 | Fine adjustment |
| 13 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 14 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 15-314 | Per-head 25-channel block | 000-255 | Base = 15 + (head-1) x 25; Tilt/Fine, Zoom, Dimmer/Fine, RGBW/Fine, six Aux White/Amber zone pairs |
| 15-39 | Head 1 block | 000-255 | Tilt/Fine, Zoom, Dimmer/Fine, 16-bit RGBW, auxiliary zones 1-6 |
| 290-314 | Head 12 block | 000-255 | Tilt/Fine, Zoom, Dimmer/Fine, 16-bit RGBW, auxiliary zones 1-6 |

5.7  329-Channel Personality

Expanded shared and per-head 16-bit control with global auxiliary-light effects.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1-2 | Head 1 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 3-4 | Head 2 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 5-6 | Head 3 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 7-8 | Head 4 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 9-10 | Head 5 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 11-12 | Head 6 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 13-14 | Head 7 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 15-16 | Head 8 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 17-18 | Head 9 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 19-20 | Head 10 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 21-22 | Head 11 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 23-24 | Head 12 Tilt / Fine | 000-255 | Coarse 0-100% / fine adjustment |
| 25 | Tilt Speed | 000-255 | Fast to slow |
| 26 | Tilt Macro | 000-004 / 005-255 | No function / See Tilt Macro reference |
| 27 | Color Temperature | 000 / 001-255 | No function / 19,000 K to 2,700 K |
| 28 | Color Macro | 000 / 001-255 | No function / See Color Macro reference |
| 29 | Pattern | 000 / 001-255 | No function / Pattern 1-255 |
| 30 | Built-in LED Effect | 000-015 / 016-255 | No function / One effect per 8 values |
| 31 | LED Effect Speed | 000-127 / 128 / 129-255 | Fast to slow / Stop / Slow to fast |
| 32 | LED Effect Delay | 000-255 | Fast to slow |
| 33 | Background Color | 000 / 001-255 | No function / See Color Macro reference |
| 34 | Background Dimmer | 000-255 | 0-100% |
| 35 | Background Dimmer Fine | 000-255 | Fine adjustment |
| 36 | Master Dimmer | 000-255 | 0-100% |
| 37 | Master Dimmer Fine | 000-255 | Fine adjustment |
| 38 | Main Strobe | 000-019 / 020-255 | Off / See Main Strobe reference |
| 39 | Auxiliary Strobe | 000-255 | See Auxiliary Strobe reference |
| 40-51 | Head 1-12 Zoom | 000-255 | Channel = 39 + head number; 0-100% |
| 52 | Control | 000-009 / 010-255 | No function / See Control reference |
| 53-60 | Shared RGBW / Fine | 000-255 | R/R Fine, G/G Fine, B/B Fine, W/W Fine |
| 61-324 | Per-head 22-channel block | 000-255 | Base = 61 + (head-1) x 22; Dimmer/Fine, RGBW/Fine, six Aux White/Amber zone pairs |
| 61-82 | Head 1 block | 000-255 | Dimmer/Fine, 16-bit RGBW and auxiliary zones 1-6 |
| 303-324 | Head 12 block | 000-255 | Dimmer/Fine, 16-bit RGBW and auxiliary zones 1-6 |
| 325 | Global Auxiliary White | 000-255 | 0-100% |
| 326 | Global Auxiliary Golden Amber | 000-255 | 0-100% |
| 327 | Auxiliary CTO | 000-067 / 068-247 / 248-255 | None / CTO / None |
| 328 | Auxiliary Effect | 000-007 / 008-191 / 192-255 | None / Effect 1-23 / None |
| 329 | Auxiliary Effect Speed | 000-127 / 128-255 | Snap-change slow to fast / Fade slow to fast |

6  Function-Channel Reference

Use these tables whenever a personality row points to a shared macro, strobe or control reference.

6.1  Tilt Macro Ranges

| DMX Value | Function |
| --- | --- |
| 000-004 | No function |
| 005-009 | Tilt Macro 1 |
| 010-014 | Tilt Macro 2 |
| 015-249 | Tilt Macros 3-49 in consecutive 5-value steps |
| 250-254 | Tilt Macro 50 |
| 255 | Tilt Macro 51 |

MACRO OPERATION  When a tilt macro is active, Head 1 Tilt sets the macro start position, Head 1 Tilt Fine sets the swing amplitude, and Tilt Speed sets macro speed. Valid amplitude values are 001-255.

6.2  Color Macro Ranges

| DMX Value | Function |
| --- | --- |
| 000 | No function |
| 001-002 | White 2700 K |
| 003-004 | White 3200 K |
| 005-006 | White 4200 K |
| 007-008 | White 5600 K |
| 009-010 | White 8000 K |
| 011 | Blue |
| 012-048 | Add Green |
| 049 | Cyan |
| 050-086 | Reduce Blue |
| 087 | Green |
| 088-124 | Add Red |
| 125 | Yellow |
| 126-162 | Reduce Green |
| 163 | Red |
| 164-200 | Add Blue |
| 201 | Magenta |
| 202-238 | Reduce Red |
| 239 | Blue |
| 240-247 | Color fade, fast to slow |
| 248-255 | Color snap, fast to slow |

6.3  Main Strobe Ranges

| DMX Value | Function |
| --- | --- |
| 000-019 | Off |
| 020-024 | On |
| 025-064 | Strobe, fast to slow |
| 065-069 | On |
| 070-084 | 100%-to-0% strobe, fast to slow |
| 085-089 | On |
| 090-104 | 0%-to-100% strobe, fast to slow |
| 105-109 | On |
| 110-124 | Random strobe, fast to slow |
| 125-129 | On |
| 130-144 | Random 100%-to-0% strobe, fast to slow |
| 145-149 | On |
| 150-164 | Random 0%-to-100% strobe, fast to slow |
| 165-169 | On |
| 170-184 | Pulse strobe, fast to slow |
| 185-189 | On |
| 190-204 | Random pulse strobe, fast to slow |
| 205-209 | On |
| 210-224 | 0%-100%-0% strobe, fast to slow |
| 225-229 | On |
| 230-244 | Random pulse strobe, fast to slow |
| 245-255 | On |

6.4  Auxiliary Strobe Ranges

| DMX Value | Function |
| --- | --- |
| 000-019 | Blackout |
| 020-024 | Open |
| 025-064 | Even strobe, fast to slow |
| 065-070 | Open |
| 071-084 | Fast-on / slow-off strobe, slow to fast |
| 085-089 | Open |
| 090-104 | Slow-on / fast-off strobe, slow to fast |
| 105-109 | Open |
| 110-124 | Random strobe |
| 125-129 | Open |
| 130-144 | Random fast-on / slow-off |
| 145-150 | Open |
| 151-164 | Random slow-on / fast-off |
| 165-169 | Open |
| 170-184 | Strobe program 1 |
| 185-189 | Open |
| 190-204 | Strobe program 2 |
| 205-209 | Open |
| 210-224 | Strobe program 3 |
| 225-229 | Open |
| 230-244 | Strobe program 4 |
| 245-255 | Open |

6.5  Control Ranges

| DMX Value | Function |
| --- | --- |
| 000-055 | No function |
| 056-060 | Tilt motor reset; hold for 5 seconds |
| 061-065 | Zoom motor reset; hold for 5 seconds |
| 066-075 | Full fixture reset; hold for 5 seconds |
| 076-255 | Reserved |

7  Routine Maintenance

ATTENTION  Disconnect AC power and allow the fixture to cool completely before inspection or cleaning.

Inspect the housing, lenses, seals, mounting points, clamps, safety cable, power cable and data connectors before every use.

Replace damaged lenses, cables, covers, connector caps and structural parts before returning the fixture to service.

Clean optical surfaces with a soft, lint-free cloth and suitable optical cleaner; dry every part carefully.

Keep fans and ventilation openings free of dust, fog-fluid residue and obstructions. The source recommends monthly cleaning in dusty environments.

Inspect every tilt and zoom mechanism for unusual noise, resistance, imbalance or positioning errors.

After cleaning or service, confirm that every waterproof seal and connector is correctly refitted before outdoor use.

8  Troubleshooting, RDM and Service

| Symptom | Check |
| --- | --- |
| No light output | Verify supply voltage, power connection, circuit protection, dimmer/strobe values and controller output. |
| No DMX response after reset | Confirm DMX mode, start address, selected personality, controller patch, cable polarity and connectors. |
| Intermittent or unstable DMX | Use shielded twisted-pair cable, inspect bidirectional amplifiers, separate signal and high-voltage cables, and terminate the final fixture with 120 ohms. |
| Fixture does not start | Check supply rating, circuit protection, moisture, transport damage and loose connections. Refer internal inspection to qualified service. |
| Abnormal tilt or zoom | Remove obstructions, run the appropriate reset, inspect reversal/ID settings, and refer Hall sensors or mechanisms to qualified service. |
| Reduced or unstable output | Check temperature, ventilation, fan operation, optical cleanliness, supply quality, power calibration and error records. |
| DMX works but RDM does not | Confirm an RDM-compatible controller, bidirectional amplifier and continuity on data pins 2 and 3; all fixtures must remain in DMX mode. |

SERVICE  There are no user-serviceable high-voltage parts inside. Refer internal repair, LED-board replacement, motor service, seal replacement and calibration to the manufacturer, its service agent or similarly qualified personnel.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair, incorrect installation, compromised waterproof seals or operation outside the specified conditions may be excluded from warranty coverage. Include the model, serial number, firmware version, active personality and a clear fault description when requesting support.

End of manual