ALP3740WR

37 x 40 W LED Bee-Eye
Moving Head IP65

RGBW 4-IN-1 - 8°-48° ZOOM - CONTINUOUS FRONT-LENS ROTATION

IP65 beam, wash, pixel and kaleidoscopic-effect moving-head fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation, rigging and service must be performed by qualified personnel.

English Edition  |  Professionally translated and technically standardized

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview and Confirmed Specifications |
| 3 | Setup, Rigging and Data Connections |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Inspection, Maintenance and Cleaning |
| 7 | Troubleshooting and Service |

MODEL IDENTIFICATION  This edition is organized for model ALP3740WR. Before installation, confirm the model, mains rating, connector configuration and firmware shown on the actual fixture.

TECHNICAL VERIFICATION  The neutral source contains a contradictory 111/148-channel pixel table and physically inconsistent photometric values. This edition clearly identifies the inferred pixel mapping and omits the unreliable illuminance table. Obtain firmware and photometric confirmation before production programming or publication.

Quick Reference

| Item | Confirmed value |
| --- | --- |
| Input | AC 100-240 V, 50/60 Hz |
| Rated power | 1,450 W |
| Light source | 37 x 40 W RGBW 4-in-1 LEDs |
| Zoom | Motorized 8°-48° |
| DMX personalities | 21 / 22 / 35 / 36 / 111 / 148 channels |
| Movement | Pan 540° / Tilt 250° |
| Protection | IP65 when correctly sealed |

1  Safety Information

DANGER  Disconnect the fixture from AC power and allow it to cool before cleaning, inspection, connection changes or service.

This is a professional entertainment-lighting fixture. Installation, operation and service must be performed by trained, qualified personnel.

Connect only to AC 100-240 V, 50/60 Hz through a protected, correctly grounded Class I supply. Never defeat the protective-earth connection.

After transport or a major temperature change, leave the fixture disconnected until it reaches room temperature and all condensation has evaporated.

Do not operate the fixture with a damaged housing, lens, cable, connector, mounting point, bracket, seal or secondary attachment.

Do not direct the light output onto combustible materials.

Maintain at least 0.5 m clearance from walls and ventilation paths. Do not cover ventilation openings.

Do not look directly into the output at close range. Intense light and strobe effects can cause eye discomfort and may affect photosensitive people.

Carry the fixture by its base or approved handles, never by the moving head. Protect it from impact, severe vibration and uncontrolled movement.

Keep power and data cables away from sharp edges, heat, moving parts and public traffic. Disconnect by holding the connector, not by pulling the cable.

Disconnect power immediately if smoke, a burning odor, abnormal noise, overheating or unexpected movement occurs. Have the fixture inspected before reuse.

Do not modify the fixture or bypass protective devices. Internal repair, calibration and seal replacement require qualified service personnel.

1.1 IP65 Protection

IP65 USE  IP65 protection depends on intact seals, closed covers and caps, and correctly mated waterproof connectors. Do not immerse the fixture, expose it to high-pressure water, open it while wet, or leave a connector interface uncapped.

Before outdoor use, inspect every gasket, cover, cable gland and mating connector. Keep unused ports capped, arrange cables with drip loops where appropriate and dry the exterior before opening any cover or disconnecting a sealed interface.

1.2 Suspended Installation

The support structure and every load-bearing component must support at least ten times the fixture load, or the higher factor required by local regulations.

Use both Omega brackets with correctly rated clamps and suitable M10 clamp fasteners. Fully lock all quarter-turn fasteners.

Attach an independently rated safety cable to the designated safety point, never to a carrying handle.

Keep people clear of the area below during installation, focusing, inspection and removal. Complete all rigging before connecting power.

Allow unobstructed clearance for the full pan, tilt, zoom and front-lens rotation range.

2  Product Overview and Confirmed Specifications

The ALP3740WR combines thirty-seven individually controllable 40 W RGBW emitters with motorized zoom and continuous bidirectional front-lens rotation. Its honeycomb optical face supports narrow beams, broad washes, color effects, pixel graphics and kaleidoscopic looks in an IP65 moving-head platform.

2.1 Technical Specifications

| Parameter | Confirmed specification |
| --- | --- |
| Input voltage | AC 100-240 V, 50/60 Hz |
| Rated power | 1,450 W |
| Light source | 37 x 40 W high-power RGBW 4-in-1 LEDs; individual LED control |
| Zoom | Motorized 8°-48° |
| Front lens | Continuous bidirectional rotation; beam, wash and kaleidoscopic effects |
| Dimming | Electronic 0-100%; four curves stated, curve names not supplied |
| Strobe | Electronic 0-25 Hz; regular, pulse and random effects |
| Control | DMX512 / Sound-activated / Master-Slave |
| DMX personalities | 21 CH / 22 CH / 35 CH / 36 CH / 111 CH / 148 CH |
| Pan / tilt | 540° / 250°, both with 16-bit fine control |
| Display | LCD with four touch-sensitive keys |
| Cooling | Temperature-controlled fan and overtemperature protection |
| Ingress protection | IP65 when covers, seals and connectors are correctly secured |
| Fixture dimensions | 530 x 279 x 663 mm (W x D x H) |
| Net / gross weight | 32.2 kg / 37.8 kg |
| Carton | 79.5 x 65 x 42 cm; one fixture |
| Two-unit flight case | 104 x 58 x 88 cm; N.W. 41 kg / G.W. 110 kg (source labels) |

SPECIFICATION LIMITS  The source does not state dimming-curve names, ambient-temperature range, fuse/current rating, maximum linked load, detailed connector configuration or LED lifetime. These values are not estimated in this edition.

2.2 Package Contents

| Item | Quantity |
| --- | --- |
| ALP3740WR fixture | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |
| Omega brackets | 2 |
| Safety cable | 1 |
| User manual | 1 |

2.3 Main Functions

Thirty-seven RGBW pixels with 16-bit global RGBW and master-dimmer control in the basic engine.

Motorized 8°-48° zoom for beam-to-wash coverage.

Continuous bidirectional front-lens rotation for dynamic vortex and kaleidoscopic looks.

Built-in effect engine with foreground/background color, dimmer, strobe, transition and offset controls.

Selectable PWM frequency for camera and broadcast environments.

Temperature-controlled cooling and overtemperature protection.

PHOTOMETRIC DATA  The source illuminance table is internally inconsistent: the listed 48° values are substantially higher than the 8° values and distance scaling is irregular. Use verified photometric measurements for planning, quotations and publication.

3  Setup, Rigging and Data Connections

3.1 Unpacking and Inspection

- Unpack carefully and inspect the fixture, optical face, mounting points, seals, cables and accessories for transport damage.

- Confirm that the product-label voltage and frequency match the local mains supply.

- Record the model, serial number, firmware revision and connector configuration for controller profiles and service records.

- Do not power a fixture that appears damaged, wet inside or incomplete; retain the packaging and contact the supplier.

3.2 Rigging Procedure

- Verify the load rating and condition of the structure, clamps, M10 fasteners, Omega brackets and safety cable.

- Attach rated clamps to both Omega brackets and tighten the clamp fasteners completely.

- Insert the quarter-turn bracket fasteners into the base and rotate every fastener fully to lock.

- Attach the independently rated safety cable to the designated point on the fixture and to the support structure.

- Check all clearances and attachments before applying power; inspect the installation regularly.

3.3 DMX512 Connection

Use shielded twisted-pair cable designed for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Avoid star wiring unless an approved DMX splitter is used.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the final fixture in a long or electrically noisy DMX line, connect a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

3.4 Addressing and Personality Selection

Set the same personality in the fixture and console profile. The next non-overlapping start address is the current start address plus the selected footprint. Confirm the pixel-engine switching behavior on the installed firmware before patching 111CH or 148CH.

| Engine | Personality | Footprint |
| --- | --- | --- |
| Basic | 21CH | Channels 1-21 |
| Basic | 22CH | 21CH core + PWM-frequency channel |
| Basic | 35CH | 21CH core + effect engine |
| Basic | 36CH | 35CH map + PWM-frequency channel |
| Pixel | 111CH | Inferred standalone 37-pixel RGB map; verify firmware |
| Pixel | 148CH | Inferred standalone 37-pixel RGBW map; verify firmware |

4  Control Panel and Menu

The front panel has four keys marked M, E, ▲ and ▼. Their conventional roles are Menu/Back, Enter, Up and Down; the source does not explicitly define them, and labels or menu order may vary with firmware.

| Menu group | Item | Option / range | Purpose / normalized wording |
| --- | --- | --- | --- |
| DMX Address | Start Address | 000-512 (source) | Set the DMX start address. Verify whether firmware accepts 000; otherwise use 001-512. |
| Fixture | Basic Engine | 21 / 22 / 35 / 36 CH | Select the basic DMX personality. |
| Fixture | Pixel Engine | Off / 111 / 148 CH | Select the source-listed pixel engine; verify switching behavior. |
| Fixture | Run Mode | DMX / Master / Sound | Select controller, master/slave or sound-activated operation. |
| Fixture | Pan Invert | Off / On | Reverse pan direction. |
| Fixture | Tilt Invert | Off / On | Reverse tilt direction. |
| Fixture | Zoom Invert | Off / On | Reverse zoom direction. |
| Fixture | No Signal | Blackout / Hold | Black out or hold the last look; source uses Clear/Hold, so verify firmware. |
| Fixture | Fan Control | Auto / Silent / High | Select cooling-fan behavior. |
| Fixture | Auto Test | - | Access the automatic fixture test. |
| Fixture | PWM Frequency | 1 / 10 / 15 / 20 / 25 kHz | Select the LED PWM frequency. |
| Fixture | Sound Sensitivity | 00-99 | Set sound-trigger sensitivity. |
| Manual | Manual Test | - | Access local manual function testing. |
| Information | Operating Time | Current / Total | View current-session and total operating time. |
| Information | Power-On Count | Read only | View the source-listed power-cycle count. |
| Information | Sensors | Pan / Tilt / Zoom | View position-sensor status. |
| Information | Environment | Head Temp / Bottom Humi (source) | View source-listed readings; the meaning of Bottom Humi is not defined. |
| Information | Software Version | Panel / Motor / Dimmer1 | View module firmware versions. |
| Factory | Password | 000-255 | Qualified service access; password is not supplied. |
| Factory | Calibration | Pan / Tilt / Zoom / Lens Rotation / R / G / B / W | Service-only calibration values, 000-255. |
| Fixture Reset | Motor Reset | Off / Run | Reinitialize motors. |
| Fixture Reset | Factory Reset | Off / Run | Restore factory defaults. |
| Display | Language | Chinese / English | Select menu language. |
| Display | Display Flip | Off / On | Reverse display orientation. |
| Display | Display Mode | 60 s / Show (source) | Select the source-listed display mode; confirm firmware behavior. |

SERVICE MENUS  Calibration and factory-reset functions can change alignment, color balance and stored settings. Record current values and restrict access to qualified personnel.

5  DMX512 Channel Reference

Select the matching personality on both fixture and controller. Basic-engine channels are documented directly from the source. Undefined ranges remain explicitly unidentified instead of being assigned guessed functions.

| Mode | Documented channel map |
| --- | --- |
| 21CH | Common RGBW, master, movement, zoom and front-lens rotation core |
| 22CH | 21CH core + CH22 PWM frequency |
| 35CH | 21CH core + CH22-35 effect engine |
| 36CH | 35CH map + CH36 PWM frequency |
| 111CH | Likely standalone 37-pixel RGB engine; inferred from footprint and endpoints |
| 148CH | Likely standalone 37-pixel RGBW engine; inferred from footprint and endpoints |

5.1  Common Basic-Engine Channels 1-21

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 000-255 | Red coarse intensity |
| 2 | Red Fine | 000-255 | 16-bit red fine adjustment |
| 3 | Green | 000-255 | Green coarse intensity |
| 4 | Green Fine | 000-255 | 16-bit green fine adjustment |
| 5 | Blue | 000-255 | Blue coarse intensity |
| 6 | Blue Fine | 000-255 | 16-bit blue fine adjustment |
| 7 | White | 000-255 | White coarse intensity |
| 8 | White Fine | 000-255 | 16-bit white fine adjustment |
| 9 | CTO | 000-255 | Color-temperature control; exact mapping not supplied |
| 10 | Color Macro | 000-255 | Built-in color macro; value map not supplied |
| 11 | Strobe | Segmented | Closed, regular, pulse and random strobe; see 5.2 |
| 12 | Master Dimmer | 000-255 | Overall output, 0-100% |
| 13 | Dimmer Fine | 000-255 | 16-bit master-dimmer fine adjustment |
| 14 | Pan | 000-255 | Pan position |
| 15 | Pan Fine | 000-255 | 16-bit pan fine adjustment |
| 16 | Tilt | 000-255 | Tilt position |
| 17 | Tilt Fine | 000-255 | 16-bit tilt fine adjustment |
| 18 | Control Functions | Segmented | Pixel enable and color correction; see 5.2 |
| 19 | Fixture Reset | Segmented | Five-second reset command; see 5.2 |
| 20 | Zoom | 000-255 | Motorized zoom; narrow/wide direction not supplied |
| 21 | Front-Lens Rotation | 000-255 | Rotation direction/speed/stop bands not supplied |

5.2  Basic-Engine Value Bands

Channel 11 - Strobe

| DMX value | Function |
| --- | --- |
| 000-003 | Closed |
| 004-103 | Regular strobe, slow to fast |
| 104-107 | Open |
| 108-207 | Pulse strobe, slow to fast |
| 208-212 | Open |
| 213-225 | Random strobe, slow |
| 226-238 | Random strobe, medium |
| 239-251 | Random strobe, fast |
| 252-255 | Open |

Channel 18 - Control Functions

| DMX value | Function |
| --- | --- |
| 000-102 | No function |
| 103-105 | Enable the selected pixel mode |
| 106-109 | Undefined in source |
| 110-114 | Color correction off |
| 115-119 | Color correction on |
| 120-255 | Undefined in source |

Channel 19 - Fixture Reset

| DMX value | Function |
| --- | --- |
| 000-199 | No function |
| 200-205 | Fixture reset after maintaining the value for 5 seconds |
| 206-255 | No function |

PWM Frequency - CH22 in 22CH / CH36 in 36CH

| DMX value | PWM frequency |
| --- | --- |
| 000-009 | Default |
| 010-049 | 1 kHz |
| 050-099 | 10 kHz |
| 100-149 | 15 kHz |
| 150-199 | 20 kHz |
| 200-255 | 25 kHz |

5.3  35CH / 36CH Effect-Engine Extension

Channels 1-21 use the common basic engine. The 35CH personality adds Channels 22-35; the 36CH personality adds the same effect engine plus CH36 PWM Frequency.

| CH | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22 | Effect Pattern | 000-255 | Select a built-in pattern; complete map not supplied |
| 23 | Effect Speed | 000-255 | Effect animation speed |
| 24 | Effect Fade | 000-255 | Effect fade |
| 25 | Effect Red | 000-255 | Foreground red |
| 26 | Effect Green | 000-255 | Foreground green |
| 27 | Effect Blue | 000-255 | Foreground blue |
| 28 | Effect White | 000-255 | Foreground white |
| 29 | Effect Dimmer | 000-255 | Foreground intensity |
| 30 | Background Dimmer | 000-255 | Background intensity |
| 31 | Effect Transition | 000-255 | Effect transition control |
| 32 | Effect Offset | 000-255 | Effect offset; detailed behavior not supplied |
| 33 | Foreground Strobe | Segmented | Foreground strobe bands below |
| 34 | Background Strobe | Segmented | Background strobe bands below |
| 35 | Background Selection | 000-255 | Background preset; complete map not supplied |

CHANNEL-LABEL CONFLICT  The source primary Function column identifies CH23 as Effect Speed and CH24 as Effect Fade, while its secondary description reverses them. This edition follows the primary labels; verify both controls on the installed firmware before show programming.

Channels 33 and 34 - Effect Strobe

| DMX value | Function |
| --- | --- |
| 000-004 | Closed |
| 005-103 | Regular strobe, slow to fast |
| 104-107 | Open |
| 108-207 | Pulse strobe, slow to fast |
| 208-255 | Open |

5.4  111-Channel Pixel Engine - Inferred RGB Map

FIRMWARE VERIFICATION REQUIRED  The source incorrectly shows global Channels 1-21 before thirty-seven RGB pixels, which would total 132 channels rather than 111. The footprint and final endpoint exactly match a standalone 37 x RGB engine. The table below is therefore an evidence-based inference, not a confirmed firmware map.

For Pixel n = 1-37: Red = 3n - 2, Green = 3n - 1, Blue = 3n. Pixel 1 uses CH1-3 and Pixel 37 uses CH109-111. Physical lens positions are not supplied; verify numbering with a low-output test scene.

| CH Range | Pixel | Component Channels | DMX Value |
| --- | --- | --- | --- |
| 1-3 | Pixel 1 | R CH1 / G CH2 / B CH3 | 000-255 each |
| 4-6 | Pixel 2 | R CH4 / G CH5 / B CH6 | 000-255 each |
| 7-9 | Pixel 3 | R CH7 / G CH8 / B CH9 | 000-255 each |
| 10-12 | Pixel 4 | R CH10 / G CH11 / B CH12 | 000-255 each |
| 13-15 | Pixel 5 | R CH13 / G CH14 / B CH15 | 000-255 each |
| 16-18 | Pixel 6 | R CH16 / G CH17 / B CH18 | 000-255 each |
| 19-21 | Pixel 7 | R CH19 / G CH20 / B CH21 | 000-255 each |
| 22-24 | Pixel 8 | R CH22 / G CH23 / B CH24 | 000-255 each |
| 25-27 | Pixel 9 | R CH25 / G CH26 / B CH27 | 000-255 each |
| 28-30 | Pixel 10 | R CH28 / G CH29 / B CH30 | 000-255 each |
| 31-33 | Pixel 11 | R CH31 / G CH32 / B CH33 | 000-255 each |
| 34-36 | Pixel 12 | R CH34 / G CH35 / B CH36 | 000-255 each |
| 37-39 | Pixel 13 | R CH37 / G CH38 / B CH39 | 000-255 each |
| 40-42 | Pixel 14 | R CH40 / G CH41 / B CH42 | 000-255 each |
| 43-45 | Pixel 15 | R CH43 / G CH44 / B CH45 | 000-255 each |
| 46-48 | Pixel 16 | R CH46 / G CH47 / B CH48 | 000-255 each |
| 49-51 | Pixel 17 | R CH49 / G CH50 / B CH51 | 000-255 each |
| 52-54 | Pixel 18 | R CH52 / G CH53 / B CH54 | 000-255 each |
| 55-57 | Pixel 19 | R CH55 / G CH56 / B CH57 | 000-255 each |
| 58-60 | Pixel 20 | R CH58 / G CH59 / B CH60 | 000-255 each |
| 61-63 | Pixel 21 | R CH61 / G CH62 / B CH63 | 000-255 each |
| 64-66 | Pixel 22 | R CH64 / G CH65 / B CH66 | 000-255 each |
| 67-69 | Pixel 23 | R CH67 / G CH68 / B CH69 | 000-255 each |
| 70-72 | Pixel 24 | R CH70 / G CH71 / B CH72 | 000-255 each |
| 73-75 | Pixel 25 | R CH73 / G CH74 / B CH75 | 000-255 each |
| 76-78 | Pixel 26 | R CH76 / G CH77 / B CH78 | 000-255 each |
| 79-81 | Pixel 27 | R CH79 / G CH80 / B CH81 | 000-255 each |
| 82-84 | Pixel 28 | R CH82 / G CH83 / B CH84 | 000-255 each |
| 85-87 | Pixel 29 | R CH85 / G CH86 / B CH87 | 000-255 each |
| 88-90 | Pixel 30 | R CH88 / G CH89 / B CH90 | 000-255 each |
| 91-93 | Pixel 31 | R CH91 / G CH92 / B CH93 | 000-255 each |
| 94-96 | Pixel 32 | R CH94 / G CH95 / B CH96 | 000-255 each |
| 97-99 | Pixel 33 | R CH97 / G CH98 / B CH99 | 000-255 each |
| 100-102 | Pixel 34 | R CH100 / G CH101 / B CH102 | 000-255 each |
| 103-105 | Pixel 35 | R CH103 / G CH104 / B CH105 | 000-255 each |
| 106-108 | Pixel 36 | R CH106 / G CH107 / B CH108 | 000-255 each |
| 109-111 | Pixel 37 | R CH109 / G CH110 / B CH111 | 000-255 each |

5.5  148-Channel Pixel Engine - Inferred RGBW Map

FIRMWARE VERIFICATION REQUIRED  The source incorrectly shows global Channels 1-21 before thirty-seven RGBW pixels, which would total 169 channels rather than 148. The footprint and final endpoint exactly match a standalone 37 x RGBW engine. Confirm the installed firmware and console profile before production use.

For Pixel n = 1-37: Red = 4n - 3, Green = 4n - 2, Blue = 4n - 1, White = 4n. Pixel 1 uses CH1-4 and Pixel 37 uses CH145-148. Physical lens positions are not supplied.

| CH Range | Pixel | Component Channels | DMX Value |
| --- | --- | --- | --- |
| 1-4 | Pixel 1 | R CH1 / G CH2 / B CH3 / W CH4 | 000-255 each |
| 5-8 | Pixel 2 | R CH5 / G CH6 / B CH7 / W CH8 | 000-255 each |
| 9-12 | Pixel 3 | R CH9 / G CH10 / B CH11 / W CH12 | 000-255 each |
| 13-16 | Pixel 4 | R CH13 / G CH14 / B CH15 / W CH16 | 000-255 each |
| 17-20 | Pixel 5 | R CH17 / G CH18 / B CH19 / W CH20 | 000-255 each |
| 21-24 | Pixel 6 | R CH21 / G CH22 / B CH23 / W CH24 | 000-255 each |
| 25-28 | Pixel 7 | R CH25 / G CH26 / B CH27 / W CH28 | 000-255 each |
| 29-32 | Pixel 8 | R CH29 / G CH30 / B CH31 / W CH32 | 000-255 each |
| 33-36 | Pixel 9 | R CH33 / G CH34 / B CH35 / W CH36 | 000-255 each |
| 37-40 | Pixel 10 | R CH37 / G CH38 / B CH39 / W CH40 | 000-255 each |
| 41-44 | Pixel 11 | R CH41 / G CH42 / B CH43 / W CH44 | 000-255 each |
| 45-48 | Pixel 12 | R CH45 / G CH46 / B CH47 / W CH48 | 000-255 each |
| 49-52 | Pixel 13 | R CH49 / G CH50 / B CH51 / W CH52 | 000-255 each |
| 53-56 | Pixel 14 | R CH53 / G CH54 / B CH55 / W CH56 | 000-255 each |
| 57-60 | Pixel 15 | R CH57 / G CH58 / B CH59 / W CH60 | 000-255 each |
| 61-64 | Pixel 16 | R CH61 / G CH62 / B CH63 / W CH64 | 000-255 each |
| 65-68 | Pixel 17 | R CH65 / G CH66 / B CH67 / W CH68 | 000-255 each |
| 69-72 | Pixel 18 | R CH69 / G CH70 / B CH71 / W CH72 | 000-255 each |
| 73-76 | Pixel 19 | R CH73 / G CH74 / B CH75 / W CH76 | 000-255 each |
| 77-80 | Pixel 20 | R CH77 / G CH78 / B CH79 / W CH80 | 000-255 each |
| 81-84 | Pixel 21 | R CH81 / G CH82 / B CH83 / W CH84 | 000-255 each |
| 85-88 | Pixel 22 | R CH85 / G CH86 / B CH87 / W CH88 | 000-255 each |
| 89-92 | Pixel 23 | R CH89 / G CH90 / B CH91 / W CH92 | 000-255 each |
| 93-96 | Pixel 24 | R CH93 / G CH94 / B CH95 / W CH96 | 000-255 each |
| 97-100 | Pixel 25 | R CH97 / G CH98 / B CH99 / W CH100 | 000-255 each |
| 101-104 | Pixel 26 | R CH101 / G CH102 / B CH103 / W CH104 | 000-255 each |
| 105-108 | Pixel 27 | R CH105 / G CH106 / B CH107 / W CH108 | 000-255 each |
| 109-112 | Pixel 28 | R CH109 / G CH110 / B CH111 / W CH112 | 000-255 each |
| 113-116 | Pixel 29 | R CH113 / G CH114 / B CH115 / W CH116 | 000-255 each |
| 117-120 | Pixel 30 | R CH117 / G CH118 / B CH119 / W CH120 | 000-255 each |
| 121-124 | Pixel 31 | R CH121 / G CH122 / B CH123 / W CH124 | 000-255 each |
| 125-128 | Pixel 32 | R CH125 / G CH126 / B CH127 / W CH128 | 000-255 each |
| 129-132 | Pixel 33 | R CH129 / G CH130 / B CH131 / W CH132 | 000-255 each |
| 133-136 | Pixel 34 | R CH133 / G CH134 / B CH135 / W CH136 | 000-255 each |
| 137-140 | Pixel 35 | R CH137 / G CH138 / B CH139 / W CH140 | 000-255 each |
| 141-144 | Pixel 36 | R CH141 / G CH142 / B CH143 / W CH144 | 000-255 each |
| 145-148 | Pixel 37 | R CH145 / G CH146 / B CH147 / W CH148 | 000-255 each |

PIXEL-ENGINE ACTIVATION  The source first instructs selecting 21CH under Basic Engine, then selecting 111CH or 148CH under Pixel Engine and using Basic CH18 values 103-105 to enable it. It does not define the exact state transition or console-patch workflow. Test the complete changeover on the production fixture before programming.

6  Inspection, Maintenance and Cleaning

ATTENTION  Disconnect power, allow the fixture to cool and make sure connector interfaces are dry before inspection or cleaning.

Inspect clamps, M10 fasteners, Omega brackets, quarter-turn locks, safety attachments and mounting points for looseness, corrosion, cracks or deformation.

Inspect the housing, optical cover, seals, caps, connectors and cables for damage, contamination, overheating or water paths.

Check moving parts for abnormal noise, obstruction, wear, imbalance or loss of position.

Clean the exterior lens and housing with a soft, damp, lint-free cloth. Do not use alcohol, aggressive solvents, abrasive materials or high-pressure water.

Remove dust and residue from accessible exterior ventilation surfaces without opening the sealed enclosure.

Adjust the inspection interval to operating hours, rigging duty, dust, fog-fluid residue, salt exposure and weather conditions. Correct every safety issue before reuse.

SEALED ENCLOSURE  There are no user-serviceable high-voltage or sealed parts inside. Internal cleaning, fan service, calibration and seal replacement must be performed by qualified personnel using genuine parts.

7  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| Fixture does not power on | Verify supply voltage/frequency, protective earth, circuit protection, cable, connector and product-label rating. |
| No DMX response | Confirm DMX mode, start address, personality, controller patch, data polarity, cable condition and termination. |
| Unexpected channel behavior | Confirm the selected basic/pixel engine and firmware. Recheck CH23/24 and the inferred 111/148 maps. |
| Movement, zoom or lens-rotation error | Remove obstructions, run the appropriate motor reset and inspect the related mechanism and sensor status. |
| Reduced or unstable output | Check temperature, fan setting, exterior ventilation, optical cleanliness, master dimmer and supply quality. |
| Overtemperature warning | Stop operation, allow cooling and inspect airflow and ambient conditions. Refer persistent faults to qualified service. |
| Water or condensation suspected | Disconnect power, move the fixture to a dry service area and have seals/connectors inspected before reuse. |

7.1 Service Information

When requesting support, provide the model, serial number, firmware versions, selected personality, start address, operating conditions and a clear description of the fault. Include photographs or a short video when they help identify the issue.

SERVICE  Do not return a wet, damaged or unstable fixture to operation. Refer internal repair, electrical testing, seal replacement and calibration to qualified personnel.

End of manual