C08

19 x 15 W LED Wash
Moving Head Eco

RGBW 4-IN-1 - MOTORIZED ZOOM - USER MANUAL

Compact beam and wash moving head with three-zone color control

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition - Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX512 Channel Reference |
| 6 | Photometric Data |
| 7 | Routine Maintenance |
| 8 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is for model C08. Confirm the model, voltage rating and firmware shown on the actual fixture before installation or programming.

1  Safety Information

DANGER  Disconnect AC power and allow the fixture to cool before opening, cleaning, inspection or service.

This professional fixture must be installed, operated and serviced by qualified personnel.

Use only AC 90-240 V, 50/60 Hz as specified by the source and product label. Connect to a protected, properly grounded Class I supply.

Do not operate the fixture with a damaged housing, lens, connector, mounting point or cable.

After a large temperature change, leave the fixture disconnected until it reaches ambient temperature and all condensation has evaporated.

Do not look directly into the LEDs at close range. Flashing light may trigger seizures in susceptible people; follow local restrictions.

Keep the fixture at least 0.5 m from combustible materials and maintain unobstructed ventilation around all fans and air openings.

For overhead installation, use rated clamps and an independently rated safety cable attached to the designated point.

Carry the fixture by its handles or fixed base only. Do not lift it by the moving head.

INDOOR USE  The source identifies this as an indoor fixture. Protect it from rain, splashing water, condensation and excessive dust. Do not use outdoors without a correctly rated enclosure.

2  Product Overview

The C08 is a compact RGBW moving-head fixture with nineteen 15 W four-in-one LEDs, motorized zoom and two DMX personalities. It provides beam, wash, color-mixing, macro and three-zone pixel effects for stages, theatres, studios, clubs, mobile productions and rental applications.

2.1 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input voltage | AC 90-240 V, 50/60 Hz |
| Maximum power | 250 W |
| Light source | 19 x 15 W RGBW 4-in-1 LEDs |
| Expected LED life | 50,000 hours (source specification) |
| Control | DMX512 / Master-Slave / Auto / Sound-active |
| DMX personalities | 16CH / 24CH |
| Pan | 540°, 16-bit |
| Tilt | 270°, 16-bit |
| Zoom / beam angle | 8°-50° motorized zoom |
| Dimming | 0-100% linear dimming |
| Strobe | 1-25 Hz electronic strobe |
| Color / effects | RGBW color mixing, blackout, rainbow and built-in macro effects |
| Display | Blue LCD; Chinese / English menus; 180° inversion |
| Data connection | 3-pin XLR DMX input/output |
| Ingress protection | IP20 / indoor use |
| Net / gross weight | 6.3 kg / 8 kg |
| Packing dimensions | 360 x 260 x 460 mm |

2.2 Main Functions

High-output RGBW beam and wash with continuously variable 8°-50° zoom.

Three independently controlled RGBW zones in the 24-channel personality.

Smooth 16-bit pan and tilt with position memory and automatic correction.

Linear dimming, electronic strobe, color-temperature control and built-in macro effects.

Automatic, sound-active, master-slave and DMX512 operation.

3  Setup and Installation

3.1 AC Power

Before connecting power, verify that the product-label voltage and frequency match the local supply. Complete all mounting and data connections before applying power. Disconnect by holding the plug, not by pulling the cable.

3.2 Rigging

STRUCTURAL RATING  The mounting point and every attachment must be rated for at least ten times the fixture load or the higher factor required by local regulations. Keep people clear of the area below during installation and service.

Inspect the fixture, omega brackets, clamps, fasteners, mounting points, safety cable and support structure.

Attach correctly rated clamps to the supplied omega brackets using suitable M10 hardware.

Lock the brackets into the base and attach an independently rated safety cable to the designated safety point.

Confirm clearance for the complete 540° pan, 270° tilt and zoom range.

Recheck every attachment before applying power.

3.3 DMX512 Connection

Use shielded, twisted-pair cable intended for RS-485/DMX512. Connect the controller to DMX IN, then link DMX OUT to the next fixture. Assign a unique start address to every independently controlled fixture.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / signal common |
| Pin 2 | Data - |
| Pin 3 | Data + |

TERMINATION  At the last fixture in a long or electrically noisy DMX chain, install a 120 ohm resistor between pins 2 and 3 of a male XLR plug and insert it into DMX OUT.

4  Control Panel and Menu

Use the confirmation key to open or save a setting, UP/DOWN to select or change a value, and the return key to exit. Menu labels may vary slightly with firmware revision.

| Main Menu | Item | Option / Range | Operation |
| --- | --- | --- | --- |
| System Settings | DMX Address | 001-512 | Set the fixture start address. |
| System Settings | Encoder | Off / On | Enable or disable encoder feedback. |
| System Settings | Pan Reverse | Off / On | Reverse the pan direction. |
| System Settings | Tilt Reverse | Off / On | Reverse the tilt direction. |
| Run Mode | Operating Mode | DMX / Auto / Sound | Select controller, automatic or sound-active operation. |
| Run Mode | Channel Mode | 16CH / 24CH | Select the DMX personality. |
| Display | Language | Chinese / English | Select the menu language. |
| Display | Display Reverse | Off / On | Rotate the display by 180°. |
| Factory | Password | 138 | Open protected calibration settings. |
| Factory | Pan Calibration | Default 128 | Adjust the pan home offset. |
| Factory | Tilt Calibration | Default 128 | Adjust the tilt home offset. |
| Factory | Zoom Calibration | Default 128 | Adjust the zoom home offset. |
| Factory | RGBW Calibration | Default 220 per color | Adjust maximum red, green, blue and white output. |
| Factory | Logo | Default 000 | Select the startup logo where supported. |

PROTECTED SETTINGS  Factory calibration is service-only. Record every original value before editing; incorrect calibration can cause positioning or output errors.

5  DMX512 Channel Reference

Select the same personality on the fixture and controller. Values increase from 000 to 255 unless a narrower range is shown.

5.1  16-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-540° |
| 2 | Pan Fine | 000-255 | 16-bit fine adjustment |
| 3 | Tilt | 000-255 | 0-270° |
| 4 | Tilt Fine | 000-255 | 16-bit fine adjustment |
| 5 | Pan / Tilt Speed | 000-255 | Fast to slow |
| 6 | Master Dimmer | 000-255 | 0-100% |
| 7 | Strobe | 000-255 | Electronic strobe |
| 8 | Red | 000-255 | 0-100% |
| 9 | Green | 000-255 | 0-100% |
| 10 | Blue | 000-255 | 0-100% |
| 11 | White | 000-255 | 0-100% |
| 12 | Zoom | 000-255 | 8°-50° |
| 13 | Macro Effect | 000-255 | Built-in macro selection |
| 14 | Macro Speed | 000-255 | Fast to slow |
| 15 | Color Temperature | 000-255 | Linear color-temperature adjustment |
| 16 | Reset | 000-249 / 250-255 | No function / Reset; hold for 5 seconds |

5.2  24-Channel Personality

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | 0-540° |
| 2 | Pan Fine | 000-255 | 16-bit fine adjustment |
| 3 | Tilt | 000-255 | 0-270° |
| 4 | Tilt Fine | 000-255 | 16-bit fine adjustment |
| 5 | Pan / Tilt Speed | 000-255 | Fast to slow |
| 6 | Zoom | 000-255 | 8°-50° |
| 7 | Master Dimmer | 000-255 | 0-100% |
| 8 | Strobe | 000-255 | Electronic strobe |
| 9 | Zone 1 Red | 000-255 | 0-100% |
| 10 | Zone 1 Green | 000-255 | 0-100% |
| 11 | Zone 1 Blue | 000-255 | 0-100% |
| 12 | Zone 1 White | 000-255 | 0-100% |
| 13 | Zone 2 Red | 000-255 | 0-100% |
| 14 | Zone 2 Green | 000-255 | 0-100% |
| 15 | Zone 2 Blue | 000-255 | 0-100% |
| 16 | Zone 2 White | 000-255 | 0-100% |
| 17 | Zone 3 Red | 000-255 | 0-100% |
| 18 | Zone 3 Green | 000-255 | 0-100% |
| 19 | Zone 3 Blue | 000-255 | 0-100% |
| 20 | Zone 3 White | 000-255 | 0-100% |
| 21 | Macro Effect | 000-255 | Built-in macro selection |
| 22 | Macro Speed | 000-255 | Fast to slow |
| 23 | Color Temperature | 000-255 | Linear color-temperature adjustment |
| 24 | Reset | 000-249 / 250-255 | No function / Reset; hold for 5 seconds |

6  Photometric Data

Illuminance values below are copied from the source test data. Actual output varies with production tolerance, optical condition, mains voltage and measurement method.

6.1  8° Beam

| Color | 2 m | 4 m | 6 m | 8 m |
| --- | --- | --- | --- | --- |
| Red | 7,200 lux | 1,900 lux | 1,000 lux | 620 lux |
| Green | 14,800 lux | 3,900 lux | 1,900 lux | 1,200 lux |
| Blue | 14,800 lux | 3,950 lux | 1,950 lux | 1,200 lux |
| White | 15,200 lux | 3,850 lux | 1,950 lux | 1,200 lux |
| RGBW | 41,100 lux | 10,000 lux | 4,600 lux | 2,550 lux |

6.2  50° Wash

| Color | 2 m | 4 m | 6 m | 8 m |
| --- | --- | --- | --- | --- |
| Red | 1,150 lux | 310 lux | 200 lux | 110 lux |
| Green | 2,450 lux | 100 lux | 350 lux | 180 lux |
| Blue | 2,450 lux | 710 lux | 350 lux | 200 lux |
| White | 2,350 lux | 700 lux | 320 lux | 200 lux |
| RGBW | 6,600 lux | 2,000 lux | 1,200 lux | 700 lux |

SOURCE VALUE  The green 50° reading at 4 m is listed as 100 lux in both supplied source editions and is reproduced without alteration.

7  Routine Maintenance

ATTENTION  Disconnect AC power and allow the fixture to cool completely before inspection or cleaning.

Inspect all screws, brackets and rigging components for tightness, corrosion and deformation.

Inspect the housing, lenses, mounting points and support structure for damage or deformation.

Inspect pan, tilt and zoom movement for wear, imbalance, resistance or abnormal noise.

Inspect the power cable and connectors for cuts, fatigue, exposed conductors or heat damage.

Clean internal and external optical surfaces weekly in dusty service conditions using a soft, lint-free cloth.

Clean fans and ventilation openings weekly and have the electrical system inspected by a qualified technician every three months.

Do not use alcohol, solvents or abrasive materials on optical or plastic parts.

8  Troubleshooting and Service

| Symptom | Check |
| --- | --- |
| No light output | Verify supply voltage, power connection, circuit protection, dimmer/strobe values and controller output. |
| No DMX response after reset | Confirm DMX mode, start address, selected 16/24CH personality, controller patch, cable polarity and connectors. |
| Intermittent or unstable DMX | Use shielded twisted-pair cable, inspect connectors, separate signal and high-voltage cables, and terminate the final fixture with 120 ohms. |
| Fixture does not start | Check supply rating, circuit protection, transport damage and loose connections. Refer internal inspection to qualified service. |
| Abnormal pan, tilt or zoom | Remove obstructions, check reverse/calibration settings, run reset and refer encoders or mechanisms to qualified service. |
| Reduced or unstable output | Check ventilation, fan operation, optical cleanliness, supply quality, temperature and color calibration. |

SERVICE  There are no user-serviceable high-voltage parts inside. Refer internal repair, LED-module replacement, motor service and calibration to the manufacturer, its service agent or similarly qualified personnel.

8.1 Warranty and Support

Damage caused by misuse, ignored warnings, unauthorized repair, incorrect installation or operation outside the specified conditions may be excluded from warranty coverage. Include the model, serial number, active personality and a clear fault description when requesting support.

End of manual