W09

19 × 40 W LED Bee Eye
Moving Head Eco

USER MANUAL

Professional LED moving-head wash, beam and effect fixture

READ BEFORE USE  Keep this manual with the fixture. Installation, operation and service must be performed by qualified personnel.

English Edition · Specifications subject to change without notice

Contents

| Section | Topic |
| --- | --- |
| 1 | Safety Information |
| 2 | Product Overview |
| 3 | Setup and Installation |
| 4 | Control Panel and Menu |
| 5 | DMX Channel Reference |
| 6 | Maintenance |
| 7 | Troubleshooting and Service |

MODEL IDENTIFICATION  This manual is organized for model W09. Before connecting power or selecting a DMX personality, verify the model, electrical rating and firmware shown on the actual fixture.

Thank you for choosing the W09. Read this manual completely before installation or operation. Follow all safety instructions and retain the manual for future reference.

1  Safety Information

DANGER  Disconnect the fixture from AC power before cleaning, inspection, installation or service. The fixture contains no user-serviceable internal parts.

•  Use the fixture indoors in a dry location unless the product label explicitly states a different environmental rating.

•  Connect only to a grounded Class I supply. The protective earth conductor must never be interrupted.

•  Use only the voltage and frequency marked on the fixture. The technical rating in this manual is AC 100–240 V, 50/60 Hz.

•  Allow the fixture to reach room temperature after transport or exposure to a large temperature change. Condensation can damage internal components.

•  Inspect the power cable regularly. Do not use a cable that is pinched, cut, abraded or otherwise damaged.

•  Do not modify the fixture. Installation, maintenance and repair must be performed by qualified personnel.

•  Do not stare into the light output. Do not operate the fixture if a lens, guard or enclosure is missing or damaged.

•  Keep children and unauthorized persons away from the fixture. Do not apply excessive force during installation or operation.

Figure 1. Protect the front lens from concentrated external light.

OPTICAL HAZARD  Direct sunlight or another powerful beam entering the front lens can cause severe internal damage. Never expose the lens to direct sunlight during unpacking, installation, operation, transport or storage. Never aim one fixture directly into another.

2  Product Overview

The W09 combines a 19-pixel RGBW LED array with motorized pan, tilt, zoom and a continuously rotating front lens. It supports DMX512, sound-active and automatic operation for concerts, theaters, studios, clubs, bars and similar entertainment venues.

2.1 Package Contents

| Item | Quantity |
| --- | --- |
| W09 fixture | 1 |
| User manual | 1 |
| Power cable | 1 |
| DMX signal cable | 1 |

Unpack carefully and inspect the fixture and accessories for shipping damage. If anything is missing or damaged, stop and contact the supplier before use.

2.2 Technical Specifications

| Parameter | Specification |
| --- | --- |
| Input power | AC 100–240 V, 50/60 Hz |
| Maximum power | 800 W |
| Light source | 19 × 40 W RGBW 4-in-1 LEDs |
| Color temperature | 2,800–8,500 K |
| Rated LED life | 50,000 hours (nominal) |
| Beam angle | 3.5°–50° |
| Control | DMX512, sound-active, Auto 1, Auto 2 |
| DMX personalities | 21 / 30 / 35 / 78 / 92 / 97 / 99 channels |
| Effects | CTO, wash, beam, vortex, kaleidoscope, pixel control |
| Front lens | Bidirectional continuous rotation |
| Dimensions | 390 × 223 × 531 mm (W × D × H) |

Figure 2. Dimensions and beam-angle reference. Photometric values are nominal.

3  Setup and Installation

3.1 DMX512 Data Connection

Connect fixtures in a daisy chain using shielded twisted-pair DMX cable. Connect the controller to DMX IN on the first fixture, then connect each DMX OUT to the next DMX IN. Do not use a star connection without a suitable DMX splitter.

| 3-pin XLR | Signal |
| --- | --- |
| Pin 1 | Shield / Ground |
| Pin 2 | Data – |
| Pin 3 | Data + |

Figure 3. DMX512 daisy-chain wiring and termination.

TERMINATION  Install a 120 Ω, 1 W resistor between pins 2 and 3 of a 3-pin XLR plug and connect it to DMX OUT on the final fixture.

3.2 DMX Start Address

Assign each fixture a unique start address. The next non-overlapping address equals the current start address plus the channel count of the selected personality.

| Example: 21-channel mode | Start address |
| --- | --- |
| Fixture 1 | A001 |
| Fixture 2 | A022 |
| Fixture 3 | A043 |

CONTROLLER CAPACITY  The controller must provide enough channels for every addressed fixture. Addressing rules can vary with the console and fixture profile.

3.3 Rigging

The fixture may be placed upright, mounted at an angle or suspended upside down. The supporting structure, clamps, fasteners and safety cable must be rated for the installation and installed according to local regulations.

Figure 4. Typical mounting orientations.

•  Verify that the mounting point and support structure are stable before installation.

•  For suspended installation, attach an independently rated safety cable through a designated safety point.

•  Keep the area below the fixture clear during installation and adjustment.

•  Inspect the safety cable, clamps and fasteners regularly for wear, deformation and loosening.

4  Control Panel and Menu

Figure 5. Display and connection panel layout. Follow the markings on the actual fixture.

4.1 Display Status and Keys

| Indicator / Key | Operation |
| --- | --- |
| DMX indicator | Blue when a valid controller signal is present; off in normal stand-alone operation. |
| Error indicator | Red when the fixture reports a fault; off in normal operation. |
| Temperature | Shows LED-board temperature. “--- °C” may indicate an open sensor circuit; “*** °C” or an implausible value may indicate a short circuit. |
| Address | Displays the DMX start address from 001 to 512. |
| UP / DOWN | Move through menu items or change the selected value. |
| ENTER | Open a menu, enter edit mode or confirm and save a value. |
| MENU | Return to the previous screen or leave edit mode without saving. |

DISPLAY CARE  Do not press the display with a sharp or pointed object.

4.2 Main Menu

| Main Menu | Item | Options / Notes |
| --- | --- | --- |
| Address | DMX address | 001–512 |
| Settings | Run mode | DMX / Sound / Auto 1 / Auto 2 |
| Settings | Channel mode | 21CH / 35CH / 78CH / 92CH / 97CH / 30CH / 99CH |
| Settings | Pan invert | On / Off |
| Settings | Tilt invert | On / Off |
| Settings | Hall correction | On / Off |
| Settings | Optical correction | On / Off |
| Settings | Signal hold | On / Off |
| Settings | Screen saver | On / Off |
| Settings | Screen orientation | On / Off / Auto |
| Settings | Synchronized update | On / Off |
| Settings | Language | Chinese / English |
| Settings | Factory reset | Restore factory defaults |
| Manual | Channel control | Set the current channel value from 0–255 |
| Calibration | Password | Enter the protected fixture-calibration menu |
| Reset | Effect motors | Reset effect motors except pan/tilt |
| Reset | Movement motors | Reset pan and tilt |
| Reset | All motors | Reset the complete fixture |
| Information | Diagnostics | Reset errors, DMX monitor, Hall sensor, X/Y optical sensor |
| Information | Versions | Hardware and software versions |

4.3 Manual Control

Select a channel and press ENTER to edit. Use UP or DOWN to change the value. Press ENTER again to save, or press MENU to leave edit mode without saving.

4.4 Calibration

| Calibration Group | Parameter | Range / Reference |
| --- | --- | --- |
| Home position | Pan / Tilt / Zoom | 0–255; 127 = no offset |
| Travel | Pan / Tilt / Zoom | 0–255; 127 = no offset |
| Drive strength | Pan / Tilt and other motors | 0–255; 127 = nominal |
| Other | Microphone sensitivity | Adjust to suit the environment |
| Security | Password | Restricted to qualified service personnel |

CALIBRATION  Incorrect calibration can cause abnormal movement or mechanical stress. Record original values before adjustment and restrict this menu to qualified personnel.

4.5 Reset and Information

Use RESET to initialize the effect motors, pan/tilt motors or the entire fixture. The INFORMATION menu provides reset-error records, live DMX data, sensor status and hardware/software version data. A red ERR indication means that the corresponding subsystem requires inspection.

5  DMX Channel Reference

The W09 provides seven DMX personalities. To reduce repetition, the charts below show two common channel cores followed by the extensions used by each personality. Pixel channels are expanded in full for direct fixture-profile programming.

| Mode | Common Core | Extension |
| --- | --- | --- |
| 21CH | Channels 1–21 · Core A | None |
| 35CH | Channels 1–21 · Core A | Built-in effects, Channels 22–35 |
| 78CH | Channels 1–21 · Core A | 19-pixel RGB, Channels 22–78 |
| 92CH | Channels 1–21 · Core A | Effects 22–35 + 19-pixel RGB 36–92 |
| 97CH | Channels 1–21 · Core A | 19-pixel RGBW, Channels 22–97 |
| 30CH | Channels 1–23 · Core B | Auxiliary strip, Channels 24–30 |
| 99CH | Channels 1–23 · Core B | 19-pixel RGBW, Channels 24–99 |

FIRMWARE  Channel functions can change with firmware revision. Compare this chart with the fixture display and controller personality before programming.

5.1 Core A · Channels 1–21

Used by 21CH, 35CH, 78CH, 92CH and 97CH personalities.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Red | 0–255 | Red linear dimming from dark to full |
| 2 | Red Fine | 0–255 | Red fine dimming |
| 3 | Green | 0–255 | Green linear dimming from dark to full |
| 4 | Green Fine | 0–255 | Green fine dimming |
| 5 | Blue | 0–255 | Blue linear dimming from dark to full |
| 6 | Blue Fine | 0–255 | Blue fine dimming |
| 7 | White | 0–255 | White linear dimming from dark to full |
| 8 | White Fine | 0–255 | White fine dimming |
| 9 | Color Temperature | 0–255 | Linear color-temperature adjustment |
| 10 | Color Macro | 0–255 | Built-in color macros |
| 11 | Strobe | 0–3 | Blackout |
| 11 | Strobe | 4–103 | Synchronized strobe, slow to fast (1–25 Hz) |
| 11 | Strobe | 104–107 | Open |
| 11 | Strobe | 108–207 | Pulse strobe, slow to fast (1–25 Hz) |
| 11 | Strobe | 208–212 | Open |
| 11 | Strobe | 213–225 | Slow random strobe |
| 11 | Strobe | 226–238 | Medium random strobe |
| 11 | Strobe | 239–251 | Fast random strobe |
| 11 | Strobe | 252–255 | Open |
| 12 | Master Dimmer | 0–255 | Linear dimming from dark to full |
| 13 | Master Dimmer Fine | 0–255 | Fine master-dimmer control |
| 14 | Pan | 0–255 | 0–540° |
| 15 | Pan Fine | 0–255 | 16-bit fine adjustment |
| 16 | Tilt | 0–255 | 0–205° |
| 17 | Tilt Fine | 0–255 | 16-bit fine adjustment |
| 18 | Macro Function | 0–255 | Reserved |
| 19 | Reset | 0–25 | No function |
| 19 | Reset | 26–76 | Head reset; hold value for 5 s |
| 19 | Reset | 77–127 | Pan/tilt reset; hold value for 5 s |
| 19 | Reset | 128–255 | Full reset; hold value for 5 s |
| 20 | Zoom | 0–255 | Linear zoom from narrow to wide |
| 21 | Rotating Lens | 0–127 | 0–60° |
| 21 | Rotating Lens | 128–188 | Counterclockwise rotation, fast to slow |
| 21 | Rotating Lens | 189–193 | Stop |
| 21 | Rotating Lens | 194–255 | Clockwise rotation, slow to fast |

5.2 Effect Extension · Channels 22–35

Used by 35CH and 92CH personalities. In 35CH mode this extension completes the personality. In 92CH mode, pixel RGB channels follow at Channel 36.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 22 | Built-in Effect (with Effect Color) | 0–7 | None |
| 22 | Built-in Effect (with Effect Color) | 8 | Static Effect 1 |
| 22 | Built-in Effect (with Effect Color) | 9 | Static Effect 2 |
| 22 | Built-in Effect (with Effect Color) | 10 | Static Effect 3 |
| 22 | Built-in Effect (with Effect Color) | 11 | None |
| 22 | Built-in Effect (with Effect Color) | 12 | Static Effect 4 |
| 22 | Built-in Effect (with Effect Color) | 13 | Static Effect 5 |
| 22 | Built-in Effect (with Effect Color) | 14 | None |
| 22 | Built-in Effect (with Effect Color) | 15 | Dynamic Effect 1 |
| 22 | Built-in Effect (with Effect Color) | 16 | Dynamic Effect 2 |
| 22 | Built-in Effect (with Effect Color) | 17 | Dynamic Effect 3 |
| 22 | Built-in Effect (with Effect Color) | 18 | Dynamic Effect 4 |
| 22 | Built-in Effect (with Effect Color) | 19 | Single-color random effect |
| 22 | Built-in Effect (with Effect Color) | 20 | Full-color random effect |
| 22 | Built-in Effect (with Effect Color) | 21 | Rainbow Effect 1 |
| 22 | Built-in Effect (with Effect Color) | 22 | Rainbow Effect 2 |
| 22 | Built-in Effect (with Effect Color) | 23 | Dynamic Effect 5 |
| 22 | Built-in Effect (with Effect Color) | 24 | Dynamic Effect 6 |
| 22 | Built-in Effect (with Effect Color) | 25 | Dynamic Effect 7 |
| 22 | Built-in Effect (with Effect Color) | 26 | Dynamic Effect 8 |
| 22 | Built-in Effect (with Effect Color) | 27 | Dynamic Effect 9 |
| 22 | Built-in Effect (with Effect Color) | 28 | Dynamic Effect 10 |
| 22 | Built-in Effect (with Effect Color) | 29 | Dynamic Effect 11 |
| 22 | Built-in Effect (with Effect Color) | 30 | Dynamic Effect 12 |
| 22 | Built-in Effect (with Effect Color) | 31 | Dynamic Effect 13 |
| 22 | Built-in Effect (with Effect Color) | 32 | Dynamic Effect 14 |
| 22 | Built-in Effect (with Effect Color) | 33 | None |
| 22 | Built-in Effect (with Effect Color) | 34 | None |
| 22 | Built-in Effect (with Effect Color) | 35 | Dynamic Effect 15 |
| 22 | Built-in Effect (with Effect Color) | 36 | Dynamic Effect 16 |
| 22 | Built-in Effect (with Effect Color) | 37 | Dynamic Effect 17 |
| 22 | Built-in Effect (with Effect Color) | 38 | Dynamic Effect 18 |
| 22 | Built-in Effect (with Effect Color) | 39 | Dynamic Effect 19 |
| 22 | Built-in Effect (with Effect Color) | 40 | Dynamic Effect 20 |
| 22 | Built-in Effect (with Effect Color) | 41 | None |
| 22 | Built-in Effect (with Effect Color) | 42 | Dynamic Effect 21 |
| 22 | Built-in Effect (with Effect Color) | 43 | None |
| 22 | Built-in Effect (with Effect Color) | 44 | Dynamic Effect 22 |
| 22 | Built-in Effect (with Effect Color) | 45 | Dynamic Effect 23 |
| 22 | Built-in Effect (with Effect Color) | 46 | Dynamic Effect 24 |
| 22 | Built-in Effect (with Effect Color) | 47 | Dynamic Effect 25 |
| 22 | Built-in Effect (with Effect Color) | 48 | Dynamic Effect 26 |
| 22 | Built-in Effect (with Effect Color) | 49 | Dynamic Effect 27 |
| 22 | Built-in Effect (with Effect Color) | 50 | Dynamic Effect 28 |
| 22 | Built-in Effect (with Effect Color) | 51 | Dynamic Effect 29 |
| 22 | Built-in Effect (with Effect Color) | 52 | Dynamic Effect 30 |
| 22 | Built-in Effect (with Effect Color) | 53 | Dynamic Effect 31 |
| 22 | Built-in Effect (with Effect Color) | 54 | Dynamic Effect 32 |
| 22 | Built-in Effect (with Effect Color) | 55 | Dynamic Effect 33 |
| 22 | Built-in Effect (with Effect Color) | 56 | Dynamic Effect 34 |
| 22 | Built-in Effect (with Effect Color) | 57 | Dynamic Effect 35 |
| 22 | Built-in Effect (with Effect Color) | 58 | Dynamic Effect 36 |
| 22 | Built-in Effect (with Effect Color) | 59 | Dynamic Effect 37 |
| 22 | Built-in Effect (with Effect Color) | 60 | Dynamic Effect 38 |
| 22 | Built-in Effect (with Effect Color) | 61 | Random Position 1 |
| 22 | Built-in Effect (with Effect Color) | 62 | Random Position 2 |
| 22 | Built-in Effect (with Effect Color) | 63 | Random Position 3 |
| 22 | Built-in Effect (with Effect Color) | 64 | None |
| 22 | Built-in Effect (with Effect Color) | 65 | Random Position 4 |
| 22 | Built-in Effect (with Effect Color) | 66 | Random Position 5 |
| 22 | Built-in Effect (with Effect Color) | 67 | Dynamic Effect 39 |
| 22 | Built-in Effect (with Effect Color) | 68 | Dynamic Effect 40 |
| 22 | Built-in Effect (with Effect Color) | 69 | None |
| 22 | Built-in Effect (with Effect Color) | 70 | Dynamic Effect 41 |
| 22 | Built-in Effect (with Effect Color) | 71–255 | None |
| 23 | Effect Speed | 0–63 | Static effect |
| 23 | Effect Speed | 64–158 | Forward, fast to slow |
| 23 | Effect Speed | 159–160 | Stop |
| 23 | Effect Speed | 161–255 | Reverse, slow to fast |
| 24 | Effect Fade | 0–255 | Dynamic-effect fade |
| 25 | Effect Red | 0–255 | Dynamic-effect red |
| 26 | Effect Green | 0–255 | Dynamic-effect green |
| 27 | Effect Blue | 0–255 | Dynamic-effect blue |
| 28 | Effect White | 0–255 | Dynamic-effect white |
| 29 | Effect Dimmer | 0–255 | Pattern dimmer, 0–100% |
| 30 | Background Dimmer | 0–255 | Background dimmer, 0–100% |
| 31 | Effect Transition | 0–255 | Pattern transition control |
| 32 | Effect Translation | 0–255 | Pattern translation |
| 33 | Foreground Strobe | 0–255 | Foreground strobe; behavior follows Channel 11 |
| 34 | Background Strobe | 0–255 | Background strobe; behavior follows Channel 11 |
| 35 | Background Selection | 0–255 | Background selection |

5.3 78CH Pixel Extension · Channels 22–78

| Channel | Pixel | Color | DMX Value / Description |
| --- | --- | --- | --- |
| 22 | 1 | Red | 0–255 · Linear dimming from dark to full |
| 23 | 1 | Green | 0–255 · Linear dimming from dark to full |
| 24 | 1 | Blue | 0–255 · Linear dimming from dark to full |
| 25 | 2 | Red | 0–255 · Linear dimming from dark to full |
| 26 | 2 | Green | 0–255 · Linear dimming from dark to full |
| 27 | 2 | Blue | 0–255 · Linear dimming from dark to full |
| 28 | 3 | Red | 0–255 · Linear dimming from dark to full |
| 29 | 3 | Green | 0–255 · Linear dimming from dark to full |
| 30 | 3 | Blue | 0–255 · Linear dimming from dark to full |
| 31 | 4 | Red | 0–255 · Linear dimming from dark to full |
| 32 | 4 | Green | 0–255 · Linear dimming from dark to full |
| 33 | 4 | Blue | 0–255 · Linear dimming from dark to full |
| 34 | 5 | Red | 0–255 · Linear dimming from dark to full |
| 35 | 5 | Green | 0–255 · Linear dimming from dark to full |
| 36 | 5 | Blue | 0–255 · Linear dimming from dark to full |
| 37 | 6 | Red | 0–255 · Linear dimming from dark to full |
| 38 | 6 | Green | 0–255 · Linear dimming from dark to full |
| 39 | 6 | Blue | 0–255 · Linear dimming from dark to full |
| 40 | 7 | Red | 0–255 · Linear dimming from dark to full |
| 41 | 7 | Green | 0–255 · Linear dimming from dark to full |
| 42 | 7 | Blue | 0–255 · Linear dimming from dark to full |
| 43 | 8 | Red | 0–255 · Linear dimming from dark to full |
| 44 | 8 | Green | 0–255 · Linear dimming from dark to full |
| 45 | 8 | Blue | 0–255 · Linear dimming from dark to full |
| 46 | 9 | Red | 0–255 · Linear dimming from dark to full |
| 47 | 9 | Green | 0–255 · Linear dimming from dark to full |
| 48 | 9 | Blue | 0–255 · Linear dimming from dark to full |
| 49 | 10 | Red | 0–255 · Linear dimming from dark to full |
| 50 | 10 | Green | 0–255 · Linear dimming from dark to full |
| 51 | 10 | Blue | 0–255 · Linear dimming from dark to full |
| 52 | 11 | Red | 0–255 · Linear dimming from dark to full |
| 53 | 11 | Green | 0–255 · Linear dimming from dark to full |
| 54 | 11 | Blue | 0–255 · Linear dimming from dark to full |
| 55 | 12 | Red | 0–255 · Linear dimming from dark to full |
| 56 | 12 | Green | 0–255 · Linear dimming from dark to full |
| 57 | 12 | Blue | 0–255 · Linear dimming from dark to full |
| 58 | 13 | Red | 0–255 · Linear dimming from dark to full |
| 59 | 13 | Green | 0–255 · Linear dimming from dark to full |
| 60 | 13 | Blue | 0–255 · Linear dimming from dark to full |
| 61 | 14 | Red | 0–255 · Linear dimming from dark to full |
| 62 | 14 | Green | 0–255 · Linear dimming from dark to full |
| 63 | 14 | Blue | 0–255 · Linear dimming from dark to full |
| 64 | 15 | Red | 0–255 · Linear dimming from dark to full |
| 65 | 15 | Green | 0–255 · Linear dimming from dark to full |
| 66 | 15 | Blue | 0–255 · Linear dimming from dark to full |
| 67 | 16 | Red | 0–255 · Linear dimming from dark to full |
| 68 | 16 | Green | 0–255 · Linear dimming from dark to full |
| 69 | 16 | Blue | 0–255 · Linear dimming from dark to full |
| 70 | 17 | Red | 0–255 · Linear dimming from dark to full |
| 71 | 17 | Green | 0–255 · Linear dimming from dark to full |
| 72 | 17 | Blue | 0–255 · Linear dimming from dark to full |
| 73 | 18 | Red | 0–255 · Linear dimming from dark to full |
| 74 | 18 | Green | 0–255 · Linear dimming from dark to full |
| 75 | 18 | Blue | 0–255 · Linear dimming from dark to full |
| 76 | 19 | Red | 0–255 · Linear dimming from dark to full |
| 77 | 19 | Green | 0–255 · Linear dimming from dark to full |
| 78 | 19 | Blue | 0–255 · Linear dimming from dark to full |

5.4 92CH Pixel Extension · Channels 36–92

| Channel | Pixel | Color | DMX Value / Description |
| --- | --- | --- | --- |
| 36 | 1 | Red | 0–255 · Linear dimming from dark to full |
| 37 | 1 | Green | 0–255 · Linear dimming from dark to full |
| 38 | 1 | Blue | 0–255 · Linear dimming from dark to full |
| 39 | 2 | Red | 0–255 · Linear dimming from dark to full |
| 40 | 2 | Green | 0–255 · Linear dimming from dark to full |
| 41 | 2 | Blue | 0–255 · Linear dimming from dark to full |
| 42 | 3 | Red | 0–255 · Linear dimming from dark to full |
| 43 | 3 | Green | 0–255 · Linear dimming from dark to full |
| 44 | 3 | Blue | 0–255 · Linear dimming from dark to full |
| 45 | 4 | Red | 0–255 · Linear dimming from dark to full |
| 46 | 4 | Green | 0–255 · Linear dimming from dark to full |
| 47 | 4 | Blue | 0–255 · Linear dimming from dark to full |
| 48 | 5 | Red | 0–255 · Linear dimming from dark to full |
| 49 | 5 | Green | 0–255 · Linear dimming from dark to full |
| 50 | 5 | Blue | 0–255 · Linear dimming from dark to full |
| 51 | 6 | Red | 0–255 · Linear dimming from dark to full |
| 52 | 6 | Green | 0–255 · Linear dimming from dark to full |
| 53 | 6 | Blue | 0–255 · Linear dimming from dark to full |
| 54 | 7 | Red | 0–255 · Linear dimming from dark to full |
| 55 | 7 | Green | 0–255 · Linear dimming from dark to full |
| 56 | 7 | Blue | 0–255 · Linear dimming from dark to full |
| 57 | 8 | Red | 0–255 · Linear dimming from dark to full |
| 58 | 8 | Green | 0–255 · Linear dimming from dark to full |
| 59 | 8 | Blue | 0–255 · Linear dimming from dark to full |
| 60 | 9 | Red | 0–255 · Linear dimming from dark to full |
| 61 | 9 | Green | 0–255 · Linear dimming from dark to full |
| 62 | 9 | Blue | 0–255 · Linear dimming from dark to full |
| 63 | 10 | Red | 0–255 · Linear dimming from dark to full |
| 64 | 10 | Green | 0–255 · Linear dimming from dark to full |
| 65 | 10 | Blue | 0–255 · Linear dimming from dark to full |
| 66 | 11 | Red | 0–255 · Linear dimming from dark to full |
| 67 | 11 | Green | 0–255 · Linear dimming from dark to full |
| 68 | 11 | Blue | 0–255 · Linear dimming from dark to full |
| 69 | 12 | Red | 0–255 · Linear dimming from dark to full |
| 70 | 12 | Green | 0–255 · Linear dimming from dark to full |
| 71 | 12 | Blue | 0–255 · Linear dimming from dark to full |
| 72 | 13 | Red | 0–255 · Linear dimming from dark to full |
| 73 | 13 | Green | 0–255 · Linear dimming from dark to full |
| 74 | 13 | Blue | 0–255 · Linear dimming from dark to full |
| 75 | 14 | Red | 0–255 · Linear dimming from dark to full |
| 76 | 14 | Green | 0–255 · Linear dimming from dark to full |
| 77 | 14 | Blue | 0–255 · Linear dimming from dark to full |
| 78 | 15 | Red | 0–255 · Linear dimming from dark to full |
| 79 | 15 | Green | 0–255 · Linear dimming from dark to full |
| 80 | 15 | Blue | 0–255 · Linear dimming from dark to full |
| 81 | 16 | Red | 0–255 · Linear dimming from dark to full |
| 82 | 16 | Green | 0–255 · Linear dimming from dark to full |
| 83 | 16 | Blue | 0–255 · Linear dimming from dark to full |
| 84 | 17 | Red | 0–255 · Linear dimming from dark to full |
| 85 | 17 | Green | 0–255 · Linear dimming from dark to full |
| 86 | 17 | Blue | 0–255 · Linear dimming from dark to full |
| 87 | 18 | Red | 0–255 · Linear dimming from dark to full |
| 88 | 18 | Green | 0–255 · Linear dimming from dark to full |
| 89 | 18 | Blue | 0–255 · Linear dimming from dark to full |
| 90 | 19 | Red | 0–255 · Linear dimming from dark to full |
| 91 | 19 | Green | 0–255 · Linear dimming from dark to full |
| 92 | 19 | Blue | 0–255 · Linear dimming from dark to full |

5.5 97CH Pixel Extension · Channels 22–97

| Channel | Pixel | Color | DMX Value / Description |
| --- | --- | --- | --- |
| 22 | 1 | Red | 0–255 · Linear dimming from dark to full |
| 23 | 1 | Green | 0–255 · Linear dimming from dark to full |
| 24 | 1 | Blue | 0–255 · Linear dimming from dark to full |
| 25 | 1 | White | 0–255 · Linear dimming from dark to full |
| 26 | 2 | Red | 0–255 · Linear dimming from dark to full |
| 27 | 2 | Green | 0–255 · Linear dimming from dark to full |
| 28 | 2 | Blue | 0–255 · Linear dimming from dark to full |
| 29 | 2 | White | 0–255 · Linear dimming from dark to full |
| 30 | 3 | Red | 0–255 · Linear dimming from dark to full |
| 31 | 3 | Green | 0–255 · Linear dimming from dark to full |
| 32 | 3 | Blue | 0–255 · Linear dimming from dark to full |
| 33 | 3 | White | 0–255 · Linear dimming from dark to full |
| 34 | 4 | Red | 0–255 · Linear dimming from dark to full |
| 35 | 4 | Green | 0–255 · Linear dimming from dark to full |
| 36 | 4 | Blue | 0–255 · Linear dimming from dark to full |
| 37 | 4 | White | 0–255 · Linear dimming from dark to full |
| 38 | 5 | Red | 0–255 · Linear dimming from dark to full |
| 39 | 5 | Green | 0–255 · Linear dimming from dark to full |
| 40 | 5 | Blue | 0–255 · Linear dimming from dark to full |
| 41 | 5 | White | 0–255 · Linear dimming from dark to full |
| 42 | 6 | Red | 0–255 · Linear dimming from dark to full |
| 43 | 6 | Green | 0–255 · Linear dimming from dark to full |
| 44 | 6 | Blue | 0–255 · Linear dimming from dark to full |
| 45 | 6 | White | 0–255 · Linear dimming from dark to full |
| 46 | 7 | Red | 0–255 · Linear dimming from dark to full |
| 47 | 7 | Green | 0–255 · Linear dimming from dark to full |
| 48 | 7 | Blue | 0–255 · Linear dimming from dark to full |
| 49 | 7 | White | 0–255 · Linear dimming from dark to full |
| 50 | 8 | Red | 0–255 · Linear dimming from dark to full |
| 51 | 8 | Green | 0–255 · Linear dimming from dark to full |
| 52 | 8 | Blue | 0–255 · Linear dimming from dark to full |
| 53 | 8 | White | 0–255 · Linear dimming from dark to full |
| 54 | 9 | Red | 0–255 · Linear dimming from dark to full |
| 55 | 9 | Green | 0–255 · Linear dimming from dark to full |
| 56 | 9 | Blue | 0–255 · Linear dimming from dark to full |
| 57 | 9 | White | 0–255 · Linear dimming from dark to full |
| 58 | 10 | Red | 0–255 · Linear dimming from dark to full |
| 59 | 10 | Green | 0–255 · Linear dimming from dark to full |
| 60 | 10 | Blue | 0–255 · Linear dimming from dark to full |
| 61 | 10 | White | 0–255 · Linear dimming from dark to full |
| 62 | 11 | Red | 0–255 · Linear dimming from dark to full |
| 63 | 11 | Green | 0–255 · Linear dimming from dark to full |
| 64 | 11 | Blue | 0–255 · Linear dimming from dark to full |
| 65 | 11 | White | 0–255 · Linear dimming from dark to full |
| 66 | 12 | Red | 0–255 · Linear dimming from dark to full |
| 67 | 12 | Green | 0–255 · Linear dimming from dark to full |
| 68 | 12 | Blue | 0–255 · Linear dimming from dark to full |
| 69 | 12 | White | 0–255 · Linear dimming from dark to full |
| 70 | 13 | Red | 0–255 · Linear dimming from dark to full |
| 71 | 13 | Green | 0–255 · Linear dimming from dark to full |
| 72 | 13 | Blue | 0–255 · Linear dimming from dark to full |
| 73 | 13 | White | 0–255 · Linear dimming from dark to full |
| 74 | 14 | Red | 0–255 · Linear dimming from dark to full |
| 75 | 14 | Green | 0–255 · Linear dimming from dark to full |
| 76 | 14 | Blue | 0–255 · Linear dimming from dark to full |
| 77 | 14 | White | 0–255 · Linear dimming from dark to full |
| 78 | 15 | Red | 0–255 · Linear dimming from dark to full |
| 79 | 15 | Green | 0–255 · Linear dimming from dark to full |
| 80 | 15 | Blue | 0–255 · Linear dimming from dark to full |
| 81 | 15 | White | 0–255 · Linear dimming from dark to full |
| 82 | 16 | Red | 0–255 · Linear dimming from dark to full |
| 83 | 16 | Green | 0–255 · Linear dimming from dark to full |
| 84 | 16 | Blue | 0–255 · Linear dimming from dark to full |
| 85 | 16 | White | 0–255 · Linear dimming from dark to full |
| 86 | 17 | Red | 0–255 · Linear dimming from dark to full |
| 87 | 17 | Green | 0–255 · Linear dimming from dark to full |
| 88 | 17 | Blue | 0–255 · Linear dimming from dark to full |
| 89 | 17 | White | 0–255 · Linear dimming from dark to full |
| 90 | 18 | Red | 0–255 · Linear dimming from dark to full |
| 91 | 18 | Green | 0–255 · Linear dimming from dark to full |
| 92 | 18 | Blue | 0–255 · Linear dimming from dark to full |
| 93 | 18 | White | 0–255 · Linear dimming from dark to full |
| 94 | 19 | Red | 0–255 · Linear dimming from dark to full |
| 95 | 19 | Green | 0–255 · Linear dimming from dark to full |
| 96 | 19 | Blue | 0–255 · Linear dimming from dark to full |
| 97 | 19 | White | 0–255 · Linear dimming from dark to full |

5.6 Core B · Channels 1–23

Used by 30CH and 99CH personalities.

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0–255 | 0–540° |
| 2 | Pan Fine | 0–255 | 16-bit fine adjustment |
| 3 | Tilt | 0–255 | 0–205° |
| 4 | Tilt Fine | 0–255 | 16-bit fine adjustment |
| 5 | Pan/Tilt Speed | 0–255 | Pan/tilt speed, fast to slow |
| 6 | Zoom | 0–255 | Linear zoom from narrow to wide |
| 7 | Rotating Lens | 0–127 | 0–60° |
| 7 | Rotating Lens | 128–188 | Counterclockwise rotation, fast to slow |
| 7 | Rotating Lens | 189–193 | Stop |
| 7 | Rotating Lens | 194–255 | Clockwise rotation, slow to fast |
| 8 | Dimmer | 0–255 | Linear dimming from dark to full |
| 9 | Strobe | 0–3 | Open |
| 9 | Strobe | 4–200 | Synchronized strobe, slow to fast (1–25 Hz) |
| 9 | Strobe | 201–215 | Slow random strobe |
| 9 | Strobe | 216–234 | Medium random strobe |
| 9 | Strobe | 235–255 | Fast random strobe |
| 10 | Red | 0–255 | Red linear dimming from dark to full |
| 11 | Green | 0–255 | Green linear dimming from dark to full |
| 12 | Blue | 0–255 | Blue linear dimming from dark to full |
| 13 | White | 0–255 | White linear dimming from dark to full |
| 14 | Color Temperature | 0–255 | Linear color-temperature adjustment |
| 15 | Color Macro | 0–255 | Built-in color macros |
| 16 | Static Effect | 0–9 | None |
| 16 | Static Effect | 10–14 | Static Effect 1 |
| 16 | Static Effect | 15–19 | Static Effect 2 |
| 16 | Static Effect | 20–24 | Static Effect 3 |
| 16 | Static Effect | 25–29 | Static Effect 4 |
| 16 | Static Effect | 30–34 | Static Effect 5 |
| 16 | Static Effect | 35–39 | Static Effect 6 |
| 16 | Static Effect | 40–44 | Static Effect 7 |
| 16 | Static Effect | 45–49 | Static Effect 8 |
| 16 | Static Effect | 50–54 | Static Effect 9 |
| 16 | Static Effect | 55–59 | Static Effect 10 |
| 16 | Static Effect | 60–64 | Static Effect 11 |
| 16 | Static Effect | 65–69 | Static Effect 12 |
| 16 | Static Effect | 70–74 | Static Effect 13 |
| 16 | Static Effect | 75–79 | Static Effect 14 |
| 16 | Static Effect | 80–84 | Static Effect 15 |
| 16 | Static Effect | 85–89 | Static Effect 16 |
| 16 | Static Effect | 90–94 | Static Effect 17 |
| 16 | Static Effect | 95–99 | Static Effect 18 |
| 16 | Static Effect | 100–104 | Static Effect 19 |
| 16 | Static Effect | 105–109 | Static Effect 20 |
| 16 | Static Effect | 110–114 | Static Effect 21 |
| 16 | Static Effect | 115–119 | Static Effect 22 |
| 16 | Static Effect | 120–124 | Static Effect 23 |
| 16 | Static Effect | 125–129 | Static Effect 24 |
| 16 | Static Effect | 130–134 | Static Effect 25 |
| 16 | Static Effect | 135–139 | Static Effect 26 |
| 16 | Static Effect | 140–144 | Static Effect 27 |
| 16 | Static Effect | 145–149 | Static Effect 28 |
| 16 | Static Effect | 150–154 | Static Effect 29 |
| 16 | Static Effect | 155–159 | Static Effect 30 |
| 16 | Static Effect | 160–164 | Static Effect 31 |
| 16 | Static Effect | 165–169 | Static Effect 32 |
| 16 | Static Effect | 170–174 | Static Effect 33 |
| 16 | Static Effect | 175–179 | Static Effect 34 |
| 16 | Static Effect | 180–184 | Static Effect 35 |
| 16 | Static Effect | 185–255 | Static Effect 36 |
| 17 | Dynamic Effect | 0–9 | None |
| 17 | Dynamic Effect | 10–14 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 15–19 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 20–24 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 25–29 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 30–34 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 35–39 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 40–44 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 45–49 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 50–54 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 55–59 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 60–64 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 65–69 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 70–74 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 75–79 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 80–84 | Single-pixel, single-color random effect |
| 17 | Dynamic Effect | 85–89 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 90–94 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 95–99 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 100–104 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 105–109 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 110–114 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 115–119 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 120–124 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 125–129 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 130–134 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 135–139 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 140–144 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 145–149 | Foreground and background colors adjustable |
| 17 | Dynamic Effect | 150–154 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 155–159 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 160–164 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 165–169 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 170–174 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 175–179 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 180–184 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 185–189 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 190–194 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 195–199 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 200–204 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 205–209 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 210–214 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 215–219 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 220–224 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 225–229 | Multicolor dynamic effect |
| 17 | Dynamic Effect | 230–255 | Multicolor dynamic effect |
| 18 | Effect Speed | 0–126 | Forward, fast to slow |
| 18 | Effect Speed | 127–128 | Stop |
| 18 | Effect Speed | 129–255 | Reverse, slow to fast |
| 19 | Background Red | 0–255 | Effect background red |
| 20 | Background Green | 0–255 | Effect background green |
| 21 | Background Blue | 0–255 | Effect background blue |
| 22 | Background White | 0–255 | Effect background white |
| 23 | Reset | 0–26 | None |
| 23 | Reset | 27–76 | Head reset; hold value for 5 s |
| 23 | Reset | 77–127 | Pan/tilt reset; hold value for 5 s |
| 23 | Reset | 128–255 | Full reset; hold value for 5 s |

5.7 30CH Strip Extension · Channels 24–30

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 24 | Strip Dimmer | 0–255 | Strip dimmer, 0–100% |
| 25 | Strip Strobe | 0–4 | None |
| 25 | Strip Strobe | 5–250 | Synchronized strobe |
| 25 | Strip Strobe | 251–255 | None |
| 26 | Strip Red | 0–255 | Strip red dimmer, 0–100% |
| 27 | Strip Green | 0–255 | Strip green dimmer, 0–100% |
| 28 | Strip Blue | 0–255 | Strip blue dimmer, 0–100% |
| 29 | Strip Macro | 0–13 | None |
| 29 | Strip Macro | 14–51 | Static colors |
| 29 | Strip Macro | 52–255 | Dynamic effects |
| 30 | Effect Speed | 0–255 | Strip effect speed, slow to fast |

5.8 99CH Pixel Extension · Channels 24–99

| Channel | Pixel | Color | DMX Value / Description |
| --- | --- | --- | --- |
| 24 | 1 | Red | 0–255 · Linear dimming from dark to full |
| 25 | 1 | Green | 0–255 · Linear dimming from dark to full |
| 26 | 1 | Blue | 0–255 · Linear dimming from dark to full |
| 27 | 1 | White | 0–255 · Linear dimming from dark to full |
| 28 | 2 | Red | 0–255 · Linear dimming from dark to full |
| 29 | 2 | Green | 0–255 · Linear dimming from dark to full |
| 30 | 2 | Blue | 0–255 · Linear dimming from dark to full |
| 31 | 2 | White | 0–255 · Linear dimming from dark to full |
| 32 | 3 | Red | 0–255 · Linear dimming from dark to full |
| 33 | 3 | Green | 0–255 · Linear dimming from dark to full |
| 34 | 3 | Blue | 0–255 · Linear dimming from dark to full |
| 35 | 3 | White | 0–255 · Linear dimming from dark to full |
| 36 | 4 | Red | 0–255 · Linear dimming from dark to full |
| 37 | 4 | Green | 0–255 · Linear dimming from dark to full |
| 38 | 4 | Blue | 0–255 · Linear dimming from dark to full |
| 39 | 4 | White | 0–255 · Linear dimming from dark to full |
| 40 | 5 | Red | 0–255 · Linear dimming from dark to full |
| 41 | 5 | Green | 0–255 · Linear dimming from dark to full |
| 42 | 5 | Blue | 0–255 · Linear dimming from dark to full |
| 43 | 5 | White | 0–255 · Linear dimming from dark to full |
| 44 | 6 | Red | 0–255 · Linear dimming from dark to full |
| 45 | 6 | Green | 0–255 · Linear dimming from dark to full |
| 46 | 6 | Blue | 0–255 · Linear dimming from dark to full |
| 47 | 6 | White | 0–255 · Linear dimming from dark to full |
| 48 | 7 | Red | 0–255 · Linear dimming from dark to full |
| 49 | 7 | Green | 0–255 · Linear dimming from dark to full |
| 50 | 7 | Blue | 0–255 · Linear dimming from dark to full |
| 51 | 7 | White | 0–255 · Linear dimming from dark to full |
| 52 | 8 | Red | 0–255 · Linear dimming from dark to full |
| 53 | 8 | Green | 0–255 · Linear dimming from dark to full |
| 54 | 8 | Blue | 0–255 · Linear dimming from dark to full |
| 55 | 8 | White | 0–255 · Linear dimming from dark to full |
| 56 | 9 | Red | 0–255 · Linear dimming from dark to full |
| 57 | 9 | Green | 0–255 · Linear dimming from dark to full |
| 58 | 9 | Blue | 0–255 · Linear dimming from dark to full |
| 59 | 9 | White | 0–255 · Linear dimming from dark to full |
| 60 | 10 | Red | 0–255 · Linear dimming from dark to full |
| 61 | 10 | Green | 0–255 · Linear dimming from dark to full |
| 62 | 10 | Blue | 0–255 · Linear dimming from dark to full |
| 63 | 10 | White | 0–255 · Linear dimming from dark to full |
| 64 | 11 | Red | 0–255 · Linear dimming from dark to full |
| 65 | 11 | Green | 0–255 · Linear dimming from dark to full |
| 66 | 11 | Blue | 0–255 · Linear dimming from dark to full |
| 67 | 11 | White | 0–255 · Linear dimming from dark to full |
| 68 | 12 | Red | 0–255 · Linear dimming from dark to full |
| 69 | 12 | Green | 0–255 · Linear dimming from dark to full |
| 70 | 12 | Blue | 0–255 · Linear dimming from dark to full |
| 71 | 12 | White | 0–255 · Linear dimming from dark to full |
| 72 | 13 | Red | 0–255 · Linear dimming from dark to full |
| 73 | 13 | Green | 0–255 · Linear dimming from dark to full |
| 74 | 13 | Blue | 0–255 · Linear dimming from dark to full |
| 75 | 13 | White | 0–255 · Linear dimming from dark to full |
| 76 | 14 | Red | 0–255 · Linear dimming from dark to full |
| 77 | 14 | Green | 0–255 · Linear dimming from dark to full |
| 78 | 14 | Blue | 0–255 · Linear dimming from dark to full |
| 79 | 14 | White | 0–255 · Linear dimming from dark to full |
| 80 | 15 | Red | 0–255 · Linear dimming from dark to full |
| 81 | 15 | Green | 0–255 · Linear dimming from dark to full |
| 82 | 15 | Blue | 0–255 · Linear dimming from dark to full |
| 83 | 15 | White | 0–255 · Linear dimming from dark to full |
| 84 | 16 | Red | 0–255 · Linear dimming from dark to full |
| 85 | 16 | Green | 0–255 · Linear dimming from dark to full |
| 86 | 16 | Blue | 0–255 · Linear dimming from dark to full |
| 87 | 16 | White | 0–255 · Linear dimming from dark to full |
| 88 | 17 | Red | 0–255 · Linear dimming from dark to full |
| 89 | 17 | Green | 0–255 · Linear dimming from dark to full |
| 90 | 17 | Blue | 0–255 · Linear dimming from dark to full |
| 91 | 17 | White | 0–255 · Linear dimming from dark to full |
| 92 | 18 | Red | 0–255 · Linear dimming from dark to full |
| 93 | 18 | Green | 0–255 · Linear dimming from dark to full |
| 94 | 18 | Blue | 0–255 · Linear dimming from dark to full |
| 95 | 18 | White | 0–255 · Linear dimming from dark to full |
| 96 | 19 | Red | 0–255 · Linear dimming from dark to full |
| 97 | 19 | Green | 0–255 · Linear dimming from dark to full |
| 98 | 19 | Blue | 0–255 · Linear dimming from dark to full |
| 99 | 19 | White | 0–255 · Linear dimming from dark to full |

6  Maintenance

Regular inspection and cleaning preserve optical performance and reduce overheating risk. The required interval depends on dust, haze fluid, ventilation and operating conditions.

•  Disconnect AC power and allow the fixture to cool before cleaning or inspection.

•  Clean the front lens with a soft, lint-free cloth and a suitable optical cleaner. Do not use abrasive materials.

•  Remove dust from ventilation openings without forcing debris into the fixture.

•  Inspect the power cable, data connectors, mounting hardware, clamps and safety cable.

•  Do not open the fixture unless you are qualified and authorized to service it.

SERVICE  If the display reports a temperature-sensor fault, persistent reset error or abnormal motion, stop using the fixture until it has been inspected by qualified personnel.

7  Troubleshooting and Service

| Symptom | Possible Cause | Corrective Action |
| --- | --- | --- |
| Fixture does not respond | No AC power; damaged plug or cable; defective power switch | Verify the supply, plug and cable. Replace a defective switch through qualified service. |
| Fixture switches off suddenly | Protection device or circuit breaker tripped | Remove the cause of the overload, then reset or replace the protection device as specified. |
| Light output is interrupted | Fixture overheating | Read the error information, allow the fixture to cool, clean ventilation paths and reduce ambient temperature. |
| Fixture stops responding | DMX signal disconnected | Inspect the DMX cable and connections. |
| Abnormal DMX operation | Incorrect address or personality | Set the correct start address and DMX mode. |
| Abnormal DMX operation | Missing termination | Install a 120 Ω terminator at the final DMX OUT. |
| Abnormal DMX operation | Weak or damaged data signal | Inspect and replace damaged data cable; use a proper splitter or amplifier for long runs. |
| Abnormal DMX operation | A faulty fixture disrupts the daisy chain | Bypass fixtures one at a time to locate the fault; have the faulty unit repaired. |
| Pan/tilt movement is abnormal | Obstruction in the movement path | Disconnect power and remove the obstruction. |
| Pan/tilt movement is abnormal | Deformed mechanical part or displaced sensor magnet | Stop operation and have the affected part repaired or replaced by qualified personnel. |

7.1 After-Sales Service

Damage caused by incorrect installation, misuse, unauthorized modification or failure to follow this manual is not covered by warranty. Technical consultation is available for the service life of the product. For warranty repair, provide the warranty card and an accurate description of the fault.

END OF MANUAL  Retain this manual with the fixture. Before each installation, verify the product label, local electrical code, rigging requirements and venue safety procedures.